3GPP TSG-RAN WG2 Meeting #122         	R2-2304829

Incheon, Korea, 22nd May – 26th May 2023

Source:	vivo

Title:	Discussion on the future extensibility of sl-FreqInfoList in R16/17 NR SL Spec

Agenda Item:	5.2.1

Document for:	Discussion and Decision

- Introduction

In Rel-16 NR SL, only single carrier operation is supported, but the SL carrier configuration in SIB12 was introduced as a list, i.e. sl-FreqInfoList-r16, in Rel-16, with the motivation of future-proof to support SL CA to be introduced in later releases.

With SL CA agreed to be specified in RAN plenary [1], it is however found that such future-proof motivation cannot be realized via the existing field sl-FreqInfoList-r16 unfortunately. In this contribution, we illustrate why/how this happens, and propose way forward on how to resolve/avoid this issue.

- Discussion

The field sl-FreqInfoList-r16 is now specified in Rel-16/17 TS 38.331 [2][3] as follows:

SL-ConfigCommonNR-r16 ::= SEQUENCE { sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 OPTIONAL, -- Need R sl-UE-SelectedConfig-r16 SL-UE-SelectedConfig-r16 OPTIONAL, -- Need R sl-NR-AnchorCarrierFreqList-r16 SL-NR-AnchorCarrierFreqList-r16 OPTIONAL, -- Need R sl-EUTRA-AnchorCarrierFreqList-r16 SL-EUTRA-AnchorCarrierFreqList-r16 OPTIONAL, -- Need R sl-RadioBearerConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 OPTIONAL, -- Need R sl-RLC-BearerConfigList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 OPTIONAL, -- Need R sl-MeasConfigCommon-r16 SL-MeasConfigCommon-r16 OPTIONAL, -- Need R sl-CSI-Acquisition-r16 ENUMERATED {enabled} OPTIONAL, -- Need R sl-OffsetDFN-r16 INTEGER (1..1000) OPTIONAL, -- Need R t400-r16 ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need R sl-MaxNumConsecutiveDTX-r16 ENUMERATED {n1, n2, n3, n4, n6, n8, n16, n32} OPTIONAL, -- Need R sl-SSB-PriorityNR-r16 INTEGER (1..8) OPTIONAL -- Need R }

sl-FreqInfoList This field indicates the NR sidelink communication/discovery configuration on some carrier frequency (ies). In this release, only one entry can be configured in the list.

This “list” signalling structure was introduced with the intention to enable a later-release gNB to directly include more than one SL carriers to support SL CA.

Observation 1: In Rel-16/17 Spec, a “list” structure is used for the signalling sl-FreqInfoList-r16 with the motivation that enables a later-release gNB to directly reuse this field to include multiple SL carriers, when SL CA is supported.

However, we found this motivation cannot be achieved based on the current signalling structure along with the above field description (highlighted in yellow). The reason is simple: if a later-release gNB (say, a Rel-18 gNB) were to really include more than one entries in this field, the Rel-16/17 UEs cannot know which specific SL carrier indicated by this list should be used for its TX/RX, because the restriction of “only one entry in this release” in above field description makes the Rel-16/17 UE behaviour unspecified and thus unable to decide which carrier to select for TX/RX. This situation especially cannot be resolved by UE implementation, because UE implementation is likely to make Re-16/17 UEs not all select exactly the same carrier from the sl-FreqInfoList-r16, so that they cannot communication with each other as a result. That is, including more than one entries in sl-FreqInfoList-r16 (by a later-release in gNB) is likely to introduce inter-operability issues among Rel-16/17 UEs.

Observation 2: Based on current Spec, the future extensibility of sl-FreqInfoList-r16 to include more than one SL carriers by a later-release gNB fails to be achieved, with the following reasons:

- Rel-16/17 UE behaviour on which SL carrier to select is unspecified, when more than one entries are included in this field, so a Rel-16/17 UE cannot decide which carrier to use for subsequent SL TX/RX.

- This situation cannot be addressed by UE implementation, since UE implementation will result in Rel-16/17 UEs highly likely to select different SL carriers, leading to inter-operability issue among them.

Now that the SL CA has finally decided to be introduced since Rel-18, it is high time to decide whether/how to fix this issue, considering the multiple SL carriers anyway likely to be configured by a gNB from Rel-18 on. Basically, there can be two way-forward to address this problem:

- WF1: Revise the Rel-16/17 Spec, and add a sentence “The UE uses only SL carrier frequency indicated by the first entry in this sl-FreqInfoList-r16” in the field description.

Note that this WF is learnt from the experience of Rel-12 D2D specification, e.g. for commTxPoolNormalCommon/Dedicated or preconfigComm, where a TX pool list was introduced and it was specified that only the pool in the first entry is used (See Appendix 1). This enables such pool list directly used to include multiple TX pools in Rel-13 D2D, when PPPP associated multiple TX pools were introduced, thus realizing its future extension. However, the con of this WF is obvious: this will be a functional NBC change of course, and it is rather late stage for Rel-16/17.

- WF2: No more relying on this sl-FreqInfoList-r16 to configure multiple SL carriers for SL CA, and introduce a new field with “-v18xy”, as NCE, to do so since Rel-18.

This is a usual way that we do extension for a later-release extension to an earlier release basic feature. However, if this WF is adopted, there can forever be only one entry included in this “sl-FreqInfoList-r16” field, even if it is specified as a “List”. So, the con is that the existing “List” signalling format of sl-FreqInfoList-r16 will be totally of no sense, making this signalling look like specified in an incorrect way from very beginning and thus making people get confused on why the list was used instead of a single-entry IE.

We request RAN2 to discuss which of the above WFs should be adopted by considering their respective con.

Proposal 1: RAN2 to decide which of the above two WFs to follow to address the future extensibility problem of sl-FreqInfoList-r16.

Proposal 1a: If WF1 is agreed, add the description in the field description of sl-FreqInfoList-r16 in Rel-16/17 RRC Specs: “The UE uses only SL carrier frequency indicated by the first entry in this field”.

Proposal 1b: If WF1 is NOT agreed, RAN2 takes a note in the meeting minutes that the sl-FreqInfoList-r16 cannot be directly reused to include more than one entries to support SL CA in later releases.

- Conclusion

We discussed the future extensibility problem of sl-FreqInfoList-r16 in this contribution. The technical problem is summarized by the below Observations:

Observation 1: In Rel-16/17 Spec, a “list” structure is used for the signalling sl-FreqInfoList-r16 with the motivation that enables a later-release gNB to directly reuse this field to include multiple SL carriers, when SL CA is supported.

Observation 2: Based on current Spec, the future extensibility of sl-FreqInfoList-r16 to include more than one SL carriers by a later-release gNB fails to be achieved, with the following reasons:

- Rel-16/17 UE behaviour on which SL carrier to select is unspecified, when more than one entries are included in this field, so a Rel-16/17 UE cannot decide which carrier to use for subsequent SL TX/RX.

- This situation cannot be addressed by UE implementation, since UE implementation will result in Rel-16/17 UEs highly likely to select different SL carriers, leading to inter-operability issue among them.

Proposal 1: RAN2 to decide which of the above two WFs to follow to address the future extensibility problem of sl-FreqInfoList-r16.

Proposal 1a: If WF1 is agreed, add the description in the field description of sl-FreqInfoList-r16 in Rel-16/17 RRC Specs: “The UE uses only SL carrier frequency indicated by the first entry in this field”.

Proposal 1b: If WF1 is NOT agreed, RAN2 takes a note in the meeting minutes that the sl-FreqInfoList-r16 cannot be directly reused to include more than one entries to support SL CA in later releases.

- Reference

- RP-230077 WID revision: NR sidelink evolution

- TS 38.331, V16.12.0

- TS 38.331, V17.4.0

Appendix 1: D2D communication procedure in TS 36.331-ck0

5.10.4     Sidelink communication transmission

A UE capable of sidelink communication that is configured by upper layers to transmit sidelink communication and has related data to be transmitted shall:

1> if the conditions for sidelink operation as defined in 5.10.1a are met:

2> if in coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4, 11.4]:

3> if the UE is in RRC_CONNECTED and uses the PCell for sidelink communication:

4> if the UE is configured, by the current PCell/ the PCell in which physical layer problems or radio link failure was detected, with commTxResources set to scheduled:

5> if T310 or T311 is running; and if the PCell at which the UE detected physical layer problems or radio link failure broadcasts SystemInformationBlockType18 including commTxPoolExceptional; or

5> if T301 is running and the cell on which the UE initiated connection re-establishment broadcasts SystemInformationBlockType18 including commTxPoolExceptional:

6> configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources indicated by the first entry in commTxPoolExceptional;

5>  else:

6> configure lower layers to request E-UTRAN to assign transmission resources for sidelink communication;

4> else if the UE is configured with commTxPoolNormalDedicated:

5>  configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources indicated by the first entry in commTxPoolNormalDedicated;

3> else (i.e. sidelink communication in RRC_IDLE or on cell other than PCell in RRC_CONNECTED):

4> if the cell chosen for sidelink communication transmission broadcasts SystemInformationBlockType18:

5>  if SystemInformationBlockType18 includes commTxPoolNormalCommon:

6> configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources indicated by the first entry in commTxPoolNormalCommon;

5>  else:

6> if the last connection establishment was initiated to request sidelink communication transmission resources and resulted in T300 expiry; and

6> if the cell on which the UE initiated connection establishment broadcasts SystemInformationBlockType18 including commTxPoolExceptional:

7> from the moment T300 expired, as specified in 5.3.3.6, until receiving an RRCConnectionReconfiguration including sl-CommConfig or until receiving an RRCConnectionRelease or an RRCConnectionReject;

8> configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources indicated by the first entry in commTxPoolExceptional;

2> else (i.e. out of coverage on sidelink carrier):

3> configure lower layers to transmit the sidelink control information and the corresponding data using the pool of resources that were preconfigured i.e. indicated by the first entry in preconfigComm in SL-Preconfiguration defined in 9.3 and in accordance with the timing of the selected SyncRef UE, or if the UE does not have a selected SyncRef UE, based on the UEs own timing;