3GPP TSG-RAN WG2 Meeting #122	R2-2304863

22 – 26 May 2023, Incheon , KR

Agenda item:	7.16.2.3

Source:	Dell Technologies

Title:	AI/ML model delivery/transfer on CP RRC

Document for:	Discussion & Decision

# Introduction

The study item on ‘’AI/ML for NR air radio interface’’ has been agreed during RAN#94-e for Release-18 [1]. Three basic use cases are currently under RAN1 study for AI/ML support as follows: (1) enhanced channel state information acquisition, (2) beam management, and (3) positioning, respectively. In the SID description, the RAN2 study scope as follows:

- Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case

Furthermore, RAN2 has recently reached the following agreements on the set of potential solutions for delivering AI/ML models over the radio interfaces:

Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following: Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data. Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP).

Therefore, in this contribution, we present our views on solution 1a, for AI/ML model delivery/transfer over CP RRC signalling.

# Introduction

AI/ML model transfer implies the handling of AI/Ml model delivery over the radio interface. Based on the outcomes of RAN1#110-bis, two categories of AI/ML model transfer are defined as follows:

- Scheme 1: AI/ML model transfer transparent to 3GPP (i.e., level y)

- Scheme 2: AI/ML model transfer on 3GPP signaling (i.e., level z)

Scheme 2 denotes that AI/ML model payload is stored on servers, as OTTs or OEMs, which is outside 3GPP scope. Scheme 2 means that AI/ML model payload is stored at an arbitrary 3GPP entity (e.g., gNBs and/or various core network functions) and are delivered to devices based on 3GPP control and/or data signaling procedures, respectively.

Observation 1: Level Z of AI/ML model transfer based on 3GPP signalling is relevant to RAN2 scope, where AI/ML model payload is delivered based on 3GPP signalling. This calls for 3GPP CP or UP signalling and protocol enhancements to adapt to emerging AI/ML model delivery requirements.

# AI/ML model transfer over CP RRC signaling

Transferring the AI/ML model payload over the control plane (CP) allows the AI/ML model payload to be part of each UE context information, which offers multiple AI/ML model delivery benefits to each UE needs and AI/ML capabilities. Specifically, with this delivery option, the AI/ML model is transferred as part of the radio resource control (RRC) reconfiguration message.

Two key issues, which have been identified by companies for mode delivery over RRC, are (1) the RRC segmentation limitations in terms of segment size and number, and (2) RRC signalling not supporting mobility forwarding, respectively.

Observation 2: Delivering the AI/ML model over CP RRC signalling has the following issues: (1) the RRC segmentation limitations in terms of segment size and number, and (2) RRC signalling not supporting mobility forwarding, respectively.

Therefore, to avoid the mentioned RRC issues and to support efficient AI/ML model as part of the RRC signalling, we propose introducing a new RRC message signalling, which is specifically defined for carrying the AI/ML model payload. The new RRC message is treated in a different way than basic RRC signalling as:

- The new AI/ML model RRC signalling is separate from existing non-AI/ML RRC signalling such that it does not block critical RRC information.

- The new AI/ML model RRC signalling can adopt different RRC segmentation size thresholds and numbers compared to existing RRC signalling. This allows the RRC segment size to be match the AI/ML model sizes which can significantly vary depending on the AI/ML driven use case and model complexity.

- The new AI/ML model RRC signalling should support mobility forwarding, unlike existing RRC basic signalling. Thus, in case of handover, the RRC segments, carrying the AI/ML model payload, are exchanged among source and target cells, and hence, the remaining segments are only transmitted. This is beneficial specially for large AI/ML models by avoiding transmitting the whole RRC segment set carrying large AI/ML models.

Thus, we propose a two stage RRC signalling format, where a first RRC reconfiguration message carries the basic RRC information, while a second RRC reconfiguration message carries the actual AI/ML model information. The AI/ML RRC message is scheduled by new scheduling information/indication in the first RRC message. Thus, regardless of the size of the AI/ML model, carried over the second RRC message, the essential non-AI RRC information is not blocked by the AI/ML payload.

Proposal 1: A new RRC message signalling is defined, independently from existing RRCreconfig message, for carrying AI/ML model information as UE context information.

Proposal 2: The new AI/ML RRC message is semi-statically scheduled by basic RRCreconfig message. Hence, an additional scheduling indication/information must be included in basic RRC signalling.

Proposal 3: The new RRC signalling message, for carrying AI/ML model information, to support mobility forwarding, and different RRC segmentation configuration (segment size and number) compared to existing RRC messages, which matches the larger RRC model sizes.

# Conclusions

This contribution has presented Dell’s views on AI/ML model transfer over CP RRC signaling, respectively. The following observations and proposals are therefore made:

Observation 1: Level Z of AI/ML model transfer based on 3GPP signalling is relevant to RAN2 scope, where AI/ML model payload is delivered based on 3GPP signalling. This calls for 3GPP CP or UP signalling and protocol enhancements to adapt to emerging AI/ML model delivery requirements.

Observation 2: Delivering the AI/ML model over CP RRC signalling has the following issues: (1) the RRC segmentation limitations in terms of segment size and number, and (2) RRC signalling not supporting mobility forwarding, respectively.

Proposal 1: A new RRC message signalling is defined, independently from existing RRCreconfig message, for carrying AI/ML model information as UE context information.

Proposal 2: The new AI/ML RRC message is semi-statically scheduled by basic RRCreconfig message. Hence, an additional scheduling indication/information must be included in basic RRC signalling.

Proposal 3: The new RRC signalling message, for carrying AI/ML model information, to support mobility forwarding, and different RRC segmentation configuration (segment size and number) compared to existing RRC messages, which matches the larger RRC model sizes.

# References

[1] RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface