3GPP TSG-RAN WG2 Meeting #122	R2-2304871

Incheon, South Korea, 22– 26 May 2023

Agenda item:	5.1.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Partial resource setup failure at RRC setup

WID/SID:	NR_NewRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

RRC connection setup is used to establish connectivity towards the network for various reasons, typically for UP traffic but also for some control procedures such as NAS messaging or LPP usage. To ensure secure connectivity, SRB2 and all DRBs are only setup after AS security activation, which mitigates possibilities for obvious attacks against the users.

However, there are still some cases where a part of the connection setup fails for some reason, and failure handling has been defined for many of these cases. In this contribution, we consider one such case where security is established correctly but the setup of all PDU sessions fails, and discuss what are the expected UE/Network actions in that case.

# 2	Partial resource failure at connection setup

As indicated by the first sentence in TS38.331, clause 5.3.1.1, the RRC connection establishment involves the establishment of SRB1. This happens via Msg4, after which UE forwards the initial NAS message towards AMF (which contains request for at least one PDU session to be setup), and network initiates the AS security setup, and finally does the initial RRC reconfiguration that sets up DRBs and SRB2. All of this requires interplay and resource reservation from at least DU, CU-CP, CU-UP, SMF and AMF in the network side, all of which have to succeed.

Observation 1: The task of setting up UE connection requires successful resource reservation from multiple network entities.

In particular, if the AS security fails, the RRC indicates that network should release the RRC connection, as evidenced by the highlighted text below in TS38.331, clause 5.3.1.1 (version 15.21.0):

### 5.3.1 Introduction #### 5.3.1.1 RRC connection control RRC connection establishment involves the establishment of SRB1. The network completes RRC connection establishment prior to completing the establishment of the NG connection, i.e. prior to receiving the UE context information from the 5GC. Consequently, AS security is not activated during the initial phase of the RRC connection. During this initial phase of the RRC connection, the network may configure the UE to perform measurement reporting, but the UE only sends the corresponding measurement reports after successful AS security activation. However, the UE only accepts a re-configuration with sync message when AS security has been activated. Upon receiving the UE context from the 5GC, the RAN activates AS security (both ciphering and integrity protection) using the initial AS security activation procedure. The RRC messages to activate AS security (command and successful response) are integrity protected, while ciphering is started only after completion of the procedure. That is, the response to the message used to activate AS security is not ciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs) are both integrity protected and ciphered. After having initiated the initial AS security activation procedure, the network may initiate the establishment of SRB2 and DRBs, i.e. the network may do this prior to receiving the confirmation of the initial AS security activation from the UE. In any case, the network will apply both ciphering and integrity protection for the RRC reconfiguration messages used to establish SRB2 and DRBs. The network should release the RRC connection if the initial AS security activation and/ or the radio bearer establishment fails. A configuration with SRB2 without DRB or with DRB without SRB2 is not supported (i.e., SRB2 and at least one DRB must be configured in the same RRC Reconfiguration message, and it is not allowed to release all the DRBs without releasing the RRC Connection). The release of the RRC connection normally is initiated by the network. The procedure may be used to re-direct the UE to an NR frequency or an E-UTRA carrier frequency. The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE Inactive AS context and any configuration received from the network, and transits to RRC_INACTIVE state. If the UE is configured with SCG, the UE releases the SCG configuration upon initiating a RRC Connection Resume procedure. The RRC message to suspend the RRC connection is integrity protected and ciphered. The resumption of a suspended RRC connection is initiated by upper layers when the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state or by RRC layer to perform a RNA update or by RAN paging from NG-RAN. When the RRC connection is resumed, network configures the UE according to the RRC connection resume procedure based on the stored UE Inactive AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates AS security and re-establishes SRB(s) and DRB(s). In response to a request to resume the RRC connection, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

Table 1. Excerpt from TS38.331 v15.21.0, clause 5.3.1.1 (connection control)

Since 3GPP normally doesn’t mandate specific network actions, it is up to network what to do exactly. However, there is also an additional clause included: “and/ or the radio bearer establishment fails.”, which seems to indicate that even if AS security would succeed, network should release the UE connection if the radio bearers could not be setup.

Observation 2: According to RRC clause 5.3.1.1, if the radio bearer setup fails network should release the RRC connection.

This is also partly linked to the following part, which indicates that SRB2 cannot exist without DRBs or vice versa, so if the DRB setup fails, SRB2 cannot be setup, either. The origin of these sentences comes from way back in LTE CR RP-080361, which was motivated by the fact that S1AP did not support partial success.

Observation 3: The sentence that indicate RRC connection should be released if AS security and/or DRB setup fails was introduced in LTE because S1AP did not support partial failure.

However, both LTE S1AP and NR NGAP do support failure of some E-RABs/PDU sessions, and in particular NR has no “default bearer” like in LTE. For example, NGAP does support the case when not all PDU sessions are successfully established, as shown in below excerpt from TS38.413, clause 8.2.1 (version 15.13.0):

## 8.2 PDU Session Management Procedures ### 8.2.1 PDU Session Resource Setup #### 8.2.1.1 General The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU sessions and the corresponding QoS flows, and to setup corresponding DRBs for a given UE. The procedure uses UE-associated signalling. #### 8.2.1.2 Successful Operation Figure 8.2.1.2-1: PDU session resource setup: successful operation The AMF initiates the procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the NG-RAN node. The PDU SESSION RESOURCE SETUP REQUEST message shall contain the information required by the NG-RAN node to setup the PDU session related NG-RAN configuration consisting of at least one PDU session resource and include each PDU session resource to setup in the PDU Session Resource Setup Request List IE. Upon reception of the PDU SESSION RESOURCE SETUP REQUEST message, if resources are available for the requested configuration, the NG-RAN node shall execute the requested NG-RAN configuration and allocate associated resources over NG and over Uu for each PDU session listed in the PDU Session Resource Setup Request List IE. If the RAN Paging Priority IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state. For each requested PDU session, if resources are available for the requested configuration, the NG-RAN node shall establish at least one DRB and associate each accepted QoS flow of the PDU session to a DRB established. For each PDU session successfully established the NG-RAN node shall pass to the UE the PDU Session NAS-PDU IE, if included, and the value contained in the PDU Session ID IE received for the PDU session. The NG-RAN node shall not send to the UE the PDU Session NAS PDUs associated to the failed PDU sessions. If the NAS-PDU IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall pass it to the UE. For each PDU session the NG-RAN node shall store the UL NG-U UP TNL Information IE included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and use it as the uplink termination point for the user plane data for this PDU session. For each PDU session, if the Additional UL NG-U UP TNL Information IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may allocate for this split PDU session resources for an additional NG-U transport bearer for some or all of the QoS flows present in the QoS Flow Setup Request List IE and it shall indicate these QoS flows in the Additional DL QoS Flow per TNL Information IE in the PDU Session Resource Setup Response Transfer IE. In case the Additional DL QoS Flow per TNL Information IE is not included the SMF shall consider the proposed additional UL NG-U UP TNL information as available again. For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and the Common Network Instance IE is not present, the NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [9]. For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [9]. For each PDU session, if the PDU Session Type IE included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message is set to “ethernet” or “unstructured”, the NG-RAN node shall not perform header compression for the concerned PDU session. For each PDU session for which the Security Indication IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to “required”, then the NG-RAN node shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU session. If the NG-RAN node cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU session resources with an appropriate cause value. If the NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to “required”. For each PDU session for which the Security Indication IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to “preferred”, then the NG-RAN node should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup Response Transfer IE of the PDU SESSION RESOURCE SETUP RESPONSE message. For each PDU session for which the Maximum Integrity Protected Data Rate Downlink IE or the Maximum Integrity Protected Data Rate Uplink IE are included in the Security Indication IE in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store the respective information and, if integrity protection is to be performed for the PDU session, it shall enforce the traffic limits corresponding to the received values, for the concerned PDU session and concerned UE, as specified in TS 23.501 [9]. For each PDU session for which the Security Indication IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message: - if the Integrity Protection Indication IE is set to “not needed”, then the NG-RAN node shall not perform user plane integrity protection for the concerned PDU session; - if the Confidentiality Protection Indication IE is set to “not needed”, then the NG-RAN node shall not perform user plane ciphering for the concerned PDU session. For each PDU session for which the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store the received value in the UE context and use it when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9]. For each PDU session in the PDU SESSION RESOURCE SETUP REQUEST message, if the Additional QoS Flow Information IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may consider it for the DRB allocation process. It is up to NG-RAN node implementation to decide whether and how to use it. For each QoS flow requested to be setup the NG-RAN node shall take into account the received QoS Flow Level QoS Parameters IE. For each QoS flow the NG-RAN node shall establish or modify the resources according to the values of the Allocation and Retention Priority IE (priority level and pre-emption indicators) and the resource situation as follows: - The NG-RAN node shall consider the priority level of the requested QoS flow, when deciding on the resource allocation. - The priority levels and the pre-emption indicators may (individually or in combination) be used to determine whether the QoS flow setup has to be performed unconditionally and immediately. If the requested QoS flow is marked as “may trigger pre-emption” and the resource situation requires so, the NG-RAN node may trigger the pre-emption procedure which may then cause the forced release of a lower priority QoS flow which is marked as “pre-emptable”. Whilst the process and the extent of the pre-emption procedure are operator-dependent, the pre-emption indicators shall be treated as follows: 1. The values of the last received Pre-emption Vulnerability IE and Priority Level IE shall prevail. 2. If the Pre-emption Capability IE is set to “may trigger pre-emption”, then this allocation request may trigger the pre-emption procedure. 3. If the Pre-emption Capability IE is set to “shall not trigger pre-emption”, then this allocation request shall not trigger the pre-emption procedure. 4. If the Pre-emption Vulnerability IE is set to “pre-emptable”, then this QoS flow shall be included in the pre-emption process. 5. If the Pre-emption Vulnerability IE is set to “not pre-emptable”, then this QoS flow shall not be included in the pre-emption process. - The NG-RAN node pre-emption process shall keep the following rules: 1. The NG-RAN node shall only pre-empt QoS flows with lower priority, in ascending order of priority. 2. The pre-emption may be done for QoS flows belonging to the same UE or to other UEs. For each QoS flow which has been successfully established, the NG-RAN node shall store the mapped E-RAB ID if included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and use it as specified in TS 38.300 [8]. The NG-RAN node shall report to the AMF in the PDU SESSION RESOURCE SETUP RESPONSE message the result for each PDU session resource requested to be setup: - For each PDU session resource successfully setup, the PDU Session Resource Setup Response Transfer IE shall be included containing: 1. The NG-U UP transport layer information to be used for the PDU session and associated list of QoS flows which have been successfully established, in the QoS Flow per TNL Information IE. 2. The list of QoS flows which failed to be established, if any, in the QoS Flow Failed to Setup List IE. When the NG-RAN node reports unsuccessful establishment of a QoS flow, the cause value should be precise enough to enable the SMF to know the reason for the unsuccessful establishment. - For each PDU session resource which failed to be setup, the PDU Session Resource Setup Unsuccessful Transfer IE shall be included containing a cause value that should be precise enough to enable the SMF to know the reason for the unsuccessful establishment. Upon reception of the PDU SESSION RESOURCE SETUP RESPONSE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Setup Response Transfer IE or PDU Session Resource Setup Unsuccessful Transfer IE to the SMF associated with the concerned PDU session. The UE Aggregate Maximum Bit Rate IE should be sent to the NG-RAN node if the AMF has not sent it previously. If it is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for all Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9]. Interactions with Handover Preparation procedure: If a handover becomes necessary during the PDU Session Resource Setup procedure, the NG-RAN node may interrupt the ongoing PDU Session Resource Setup procedure and initiate the Handover Preparation procedure as follows: 1. The NG-RAN node shall send the PDU SESSION RESOURCE SETUP RESPONSE message in which the NG-RAN node shall indicate, if necessary, all the PDU session resources which failed to be setup with an appropriate cause value, e.g. “NG intra-system handover triggered”, “NG inter-system handover triggered” or “Xn handover triggered”. 2. The NG-RAN node shall trigger the handover procedure.

Table 1. Excerpt from TS38.413 v15.1.30, clause 8.2.1 (PDU session setup)

In particular, we would note that PDU sessions are established before DRBs, so the question is: Does the failure of “radio bearer” in RRC text refer to the 1) DRBs only, 2) PDU sessions or 3) both?

Proposal 1: RAN2 to clarify the meaning of “radio bearer establishment fails” in clause 5.3.1.1 of TS38.331: Does it refer to DRBs, PDU sessions or both?

Looking at the above 38.413, it is clear that specification supports at least the case where some PDU sessions may fail completely, and even if a PDU session is setup successfully, some QoS flows within it may still fail. But if we consider the case when all PDU sessions fail to be setup, i.e. the network cannot setup any DRBs. If the AS security succeeds, it is still possible to continue the UE connection as a signalling-only connection (i.e. only SRB1 is established), but network cannot configure SRB2 as long as there are no DRBs (which would require PDU session setup). Hence, network may not release the connection immediately.

#### 9.2.2.2 INITIAL CONTEXT SETUP RESPONSE This message is sent by the NG-RAN node to confirm the setup of a UE context. Direction: NG-RAN node ® AMF IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AMF UE NGAP ID; M; 9.3.3.1; YES; ignore. RAN UE NGAP ID; M; 9.3.3.2; YES; ignore. PDU Session Resource Setup Response List; 0..1; YES; ignore. >PDU Session Resource Setup Response Item; 1..<maxnoofPDUSessions>; -. >>PDU Session ID; M; 9.3.1.50; -. >>PDU Session Resource Setup Response Transfer; M; OCTET STRING; Containing the PDU Session Resource Setup Response Transfer IE specified in subclause 9.3.4.2.; -. PDU Session Resource Failed to Setup List; 0..1; YES; ignore. >PDU Session Resource Failed to Setup Item; 1..<maxnoofPDUSessions>; -. >>PDU Session ID; M; 9.3.1.50; -. >>PDU Session Resource Setup Unsuccessful Transfer; M; OCTET STRING; Containing the PDU Session Resource Setup Unsuccessful Transfer IE specified in subclause 9.3.4.16.; -. Criticality Diagnostics; O; 9.3.1.3; YES; ignore.

Observation 4: It is possible that setup of all PDU sessions fails at NR connection setup, but network may not (immediately) release the RRC connection.

This implies that the RRC sentence is incorrect in light of the current RAN3 specifications and should be corrected.

We would note that signalling-only connection (i.e. RRC connection without DRBs or SRB2) is also supported since Rel-15, so even if all PDU sessions fail to be setup, network could still retain the UE in RRC connection (but just cannot e.g. setup SRB2 or initiate UP connectivity). Hence, as the existing sentence states, network ”should” release the connection but is not mandated to do so, and the imperative is strong only for the case when AS security fails.

Proposal 2: If AS security setup fails, network should release RRC connection (as already implied by clause 5.3.1.1 in NR RRC).

Proposal 3: If the PDU session setup fails (partially or fully) but AS security activation succeeds, RAN2 to confirm it is up to network implementation whether/when to release the RRC connection and UE shall not initiate RLF due to that.

Proposal 4: Discuss the TP in Annex A for RRC to clarify the radio bearer setup failure case.

# 3	Conclusion

This document has made the following observations:

Observation 1: The task of setting up UE connection requires successful resource reservation from multiple network entities.

Observation 2: According to RRC clause 5.3.1.1, if the radio bearer setup fails network should release the RRC connection.

Observation 3: The sentence that indicate RRC connection should be released if AS security and/or DRB setup fails was introduced in LTE because S1AP did not support partial failure.

Observation 4: It is possible that setup of all PDU sessions fails at NR connection setup, but network may not (immediately) release the RRC connection.

And proposed the following:

Proposal 1: RAN2 to clarify the meaning of “radio bearer establishment fails” in clause 5.3.1.1 of TS38.331: Does it refer to DRBs, PDU sessions or both?

Proposal 2: If AS security setup fails, network should release RRC connection (as already implied by clause 5.3.1.1 in NR RRC).

Proposal 3: If the PDU session setup fails (partially or fully) but AS security activation succeeds, RAN2 to confirm it is up to network implementation whether/when to release the RRC connection and UE shall not initiate RLF due to that.

Proposal 4: Discuss the TP in Annex A for RRC to clarify the radio bearer setup failure case.

# Annex A: TP to 38.331, clause 5.3.1.1 (Rel-15 version)

### 5.3.1	Introduction

#### 5.3.1.1	RRC connection control

RRC connection establishment involves the establishment of SRB1. The network completes RRC connection establishment prior to completing the establishment of the NG connection, i.e. prior to receiving the UE context information from the 5GC. Consequently, AS security is not activated during the initial phase of the RRC connection. During this initial phase of the RRC connection, the network may configure the UE to perform measurement reporting, but the UE only sends the corresponding measurement reports after successful AS security activation. However, the UE only accepts a re-configuration with sync message when AS security has been activated.

Upon receiving the UE context from the 5GC, the RAN activates AS security (both ciphering and integrity protection) using the initial AS security activation procedure. The RRC messages to activate AS security (command and successful response) are integrity protected, while ciphering is started only after completion of the procedure. That is, the response to the message used to activate AS security is not ciphered, while the subsequent messages (e.g. used to establish SRB2 and DRBs) are both integrity protected and ciphered. After having initiated the initial AS security activation procedure, the network may initiate the establishment of SRB2 and DRBs, i.e. the network may do this prior to receiving the confirmation of the initial AS security activation from the UE. In any case, the network will apply both ciphering and integrity protection for the RRC reconfiguration messages used to establish SRB2 and DRBs. The network should release the RRC connection if the initial AS security activation fails. If the initial AS security activation succeeds but PDU session establishment fails (partially or fully), it is up to network implementation whether to release the RRC connection. A configuration with SRB2 without DRB or with DRB without SRB2 is not supported (i.e., SRB2 and at least one DRB must be configured in the same RRC Reconfiguration message, and it is not allowed to release all the DRBs without releasing the RRC Connection).

The release of the RRC connection normally is initiated by the network. The procedure may be used to re-direct the UE to an NR frequency or an E-UTRA carrier frequency.

The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE Inactive AS context and any configuration received from the network, and transits to RRC_INACTIVE state. If the UE is configured with SCG, the UE releases the SCG configuration upon initiating a RRC Connection Resume procedure. The RRC message to suspend the RRC connection is integrity protected and ciphered.

The resumption of a suspended RRC connection is initiated by upper layers when the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state or by RRC layer to perform a RNA update or by RAN paging from NG-RAN. When the RRC connection is resumed, network configures the UE according to the RRC connection resume procedure based on the stored UE Inactive AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates AS security and re-establishes SRB(s) and DRB(s).

In response to a request to resume the RRC connection, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).