3GPP TSG-RAN WG2 Meeting #122	R2-2304872

Incheon, South Korea, 22 – 26 May 2023	R2-2302771

Agenda item:	5.1.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	CSI resources coordination in NR-DC

WID/SID:	NR_NewRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

The current NR-DC assumes coordination of several resources between MN and SN to ensure UE capabilities are followed. However, there are some cases such as CSI resources that are not explicitly coordinated, which can have a significant impact on network since static values need to be assumed per cell group. We discuss how to resolve this issue in this contribution.

This topic was discussed during RAN2#121bis-e, but the conclusion was postponed as per the following:

DCCA misc

R2-2302771	CSI-RS resource coordination in NR-DC	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT-Core

[003] Noted, topic is postponed

R2-2304138	CSI-RS resource coordination in NR-DC	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.12.0	3990	2	F	NR_newRAT-Core, TEI16	R2-2304133

R2-2304140	CSI-RS resource coordination in NR-DC	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.4.0	3991	2	A	NR_newRAT-Core, TEI16	R2-2304135

[003] both postponed

We continue the discussion based on the feedback, with the section 2.3 being dedicated to that (and sections 2.1 and 2.2 are almost identical to the previous contribution, being just provided as background).

# 2	CSI resource coordination

## 2.1	Background: Resource coordination in inter-node messages

In NR-DC mode, MN and SN need to coordinate the usage of some resources as e.g. The following ones:

-	Number of UE measured frequencies (measuredFrequenciesMN/SN)

-	Number of intra-/inter frequency meas. Ids requested by SN (requestedMaxInter(/intra)FreqMeasIdSCG)

-	Max number of intra-/inter-frequency measurements allowed to SN (maxIntra(/Inter)FreqMeasIdentitiesSCG)

-	PSCell and SCell allocated frequencies in SN (pSCellFrequency and scellFrequenciesSN-NR)

-	PCell&SCell allocated frequencies in MN (servFrequenciesMN, needed e.g. for intra-band contiguity checks)

-	Band entries in the respective allowed band combination used/reserved by MN (SelectedBandEntriesMN)

The exchange of above information avoids the risk of inconsistent configurations (e.g. if UE CA class is exceeded) or of performance degradation (e.g. If UE is requested more frequencies which it can support). We would also note that these coordination possibilities relate to UE capabilities that are per UE and/or per carrier, because in such cases both MN and SN need to ensure UE capabilities are not exceeded. This allows network more dynamic control of UE resources and avoids having to have “fixed” resource coordination via OAM, which improves system performance.

Observation 1: The coordination of MN/SN resources via inter-node messages allow more optimal resource allocation across the cell groups.

However, there is one big design constraint in the inter-node messages: It is not (currently) possible to introduce late extensions to any inter-node messages except at the message level, which is equivalent of a critical extension at the message-level (i.e. a new message type). However, some parts of the inter-node messages (e.g. configuration restrictions) do allow extensions, and have been extended last time in Rel-16

Observation 2: NR inter-node messages do not allow for late extensions, e.g. introducing late Rel-15/16 extensions at message level is not possible once Rel-17 extensions have been introduced.

Observation 3: MN-SN configuration restriction IEs can be extended and have been last extended in Rel-16.

In particular, we note is that the L1 resource coordination is lacking in the inter-node message exchange. In particular, the used CSI resources are not currently coordinated between MN and SN. This presents a dilemma for network since MN does not know how many CSI resources SN wants to configure, nor does SN know how many CSI resources MN has already configured. We discuss this in the next section of the document

## 2.2	Background: UE capabilities for CSI resources

There are several UE capabilities governing the use of CSI-RS in NR Rel-15 specifications, but for the purpose of this discussion, we focus on the following SRS and CSI-RS capabilities since they illustrate the limitations UEs and networks have for configuring resources in per-UE/per-BC and per-CC manner.

Per-BC capabilities (i.e. applicable per-UE when the corresponding BC is used)

CA-ParametersNR-v1540 ::=           SEQUENCE {

simultaneousSRS-AssocCSI-RS-AllCC                       INTEGER (5..32)         OPTIONAL,

csi-RS-IM-ReceptionForFeedbackPerBandComb               SEQUENCE {

maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC            INTEGER (1..64)     OPTIONAL,

totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC     INTEGER (2..256)    OPTIONAL

}                                                                               OPTIONAL,

simultaneousCSI-ReportsAllCC                            INTEGER (5..32)         OPTIONAL,

dualPA-Architecture                                     ENUMERATED {supported}  OPTIONAL

}

Per-CC capabilities

CSI-RS-IM-ReceptionForFeedback ::=              SEQUENCE {

maxConfigNumberNZP-CSI-RS-PerCC                 INTEGER (1..64),

maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC      INTEGER (2..256),

maxConfigNumberCSI-IM-PerCC                     ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousNZP-CSI-RS-PerCC           INTEGER (1..64),

totalNumberPortsSimultaneousNZP-CSI-RS-PerCC    INTEGER (2..256)

}

CSI-RS-ProcFrameworkForSRS ::=                  SEQUENCE {

maxNumberPeriodicSRS-AssocCSI-RS-PerBWP         INTEGER (1..4),

maxNumberAperiodicSRS-AssocCSI-RS-PerBWP        INTEGER (1..4),

maxNumberSP-SRS-AssocCSI-RS-PerBWP              INTEGER (0..4),

simultaneousSRS-AssocCSI-RS-PerCC               INTEGER (1..8)

}

These are described as follows in 38.306:

| csi-RS-IM-ReceptionForFeedback Indicates support of CSI-RS and CSI-IM reception for CSI feedback. This capability signalling comprises the following parameters: - maxConfigNumberNZP-CSI-RS-PerCC indicates the maximum number of configured NZP-CSI-RS resources per CC; - maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC indicates the maximum number of ports across all configured NZP-CSI-RS resources per CC; - maxConfigNumberCSI-IM-PerCC indicates the maximum number of configured CSI-IM resources per CC; - maxNumberSimultaneousNZP-CSI-RS-PerCC indicates the maximum number of simultaneous CSI-RS-resources per CC; - totalNumberPortsSimultaneousNZP-CSI-RS-PerCC indicates the total number of CSI-RS ports in simultaneous CSI-RS resources per CC. The UE is mandated to report csi-RS-IM-ReceptionForFeedback. | Band | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |

| csi-RS-IM-ReceptionForFeedbackPerBandComb Indicates support of CSI-RS and CSI-IM reception for CSI feedback. This capability signalling comprises the following parameters: - maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC indicates the maximum number of simultaneous CSI-RS resources (irrespective of the associated codebook type) in active BWPs across all CCs, and across MCG and SCG in case of NR-DC. The network applies this limit in addition to the limits signalled in MIMO-ParametersPerBand-> maxNumberSimultaneousNZP-CSI-RS-PerCC and in Phy-ParametersFRX-Diff-> maxNumberSimultaneousNZP-CSI-RS-PerCC; - totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC indicates the total number of CSI-RS ports in simultaneous CSI-RS resources (irrespective of the associated codebook type) in active BWPs across all CCs, and across MCG and SCG in case of NR-DC. The network applies this limit in addition to the limits signalled in MIMO-ParametersPerBand-> totalNumberPortsSimultaneousNZP-CSI-RS-PerCC and in Phy-ParametersFRX-Diff-> totalNumberPortsSimultaneousNZP-CSI-RS-PerCC. The UE is mandated to report csi-RS-IM-ReceptionForFeedbackPerBandComb. | BC | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |

| simultaneousSRS-AssocCSI-RS-AllCC Indicates support of CSI-RS processing framework for SRS and the number of SRS resources that the UE can process simultaneously across all CCs, and across MCG and SCG in case of NR-DC, including periodic, aperiodic and semi-persistent SRS. This parameter may further limit simultaneousSRS-AssocCSI-RS-PerCC in MIMO-ParametersPerBand and Phy-ParametersFRX-Diff for each band in a given band combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

As can be seen from above, UE has different capabilities for total number of CSI-RS resources (i.e. AllCC) and per-carrier (i.e. PerCC), and network has to take both into account.

Observation 4: Many capabilities are common to all carriers (e.g. CSI-RS capabilities with “AllCC”) and network needs to ensure that UE capabilities are not exceeded in NR-DC configuration.

However, the inter-node messages CG-ConfigInfo and CG-Config do not allow any coordination of the resource linked to these capabilities – which in practice means that network has to use some form of “static” assumptions for MN and SN (e.g. SN cannot use more than N CSI-RS resources, rest are for MN), typically set via OAM. This is very restrictive (and often overly complex) and seems like something that has simply been overlooked at the time when these capabilities were set. And if the UE is provided with a configuration that exceeds any of the above limits, consequence would be either call drop (e.g. number of configured simultaneous CSI-RS resources or ports is exceeding) or to performance degradation (number of requested simultaneous CSI reports is exceeding). Therefore, in many cases network will underutilize the UE resources or use them inefficiently.

Observation 5: In NR-DC, MN and SN configure CSI-RS resources independently according to (known) UE capabilities. Lack of MN/SN-coordination restricts usage of CSI-RS capabilities in NR-DC, leading to complex network handling and underutilization of UE resources.

One can also wonder why this has not been considered in EN-DC? The reason is simple: CSI-RS capabilities for NR SgNB do not the LTE MeNB capabilities, and those are coordinated via the MR-DC capability containers, where UE can already take any shared resources into account and network need not consider any further limitations. Additionally, the CSI-RS configurations for NR are separate for MN and SN and can be independently configured for MCG and SCG serving cell. Same also applies for NE-DC.

Observation 6: The CSI-RS resource coordination is not an issue for EN-DC or NE-DC.

Based on this, we would propose to simply enable coordination of the CSI-RS resource via inter-node messages to enable better handling of NR-DC configurations (especially in inter-vendor cases). Since these changes are late extensions, it seems not feasible to do Rel-15 changes but adding the elements to configuration restriction framework seems viable and could be done from Rel-16 onwards. While this is not optimal, it is still fine given that many deployments are anyway using Rel-16 signalling and these changes require no UE impacts.

We also further discuss the points raised in RAN2#121bis-e in the next section.

## 2.3	Clarifications on UE capabilities requiring MN/SN coordination (e.g. CSI resources)

During the discussion (see R2-2304438), we categorize the comments raised acording to the following:

- Use cases were not clear (e.g. configuration/activation), nor were which capabilities would need to be coordinated (and which values to use)

- CLI resource coordination was claimed to work for (at least some cases), and

- The need for the coordination was unclear given these were introduced alread in Rel-15

- Some simplications to the changes could be considered (based on comments).

For convenience, the comments from network vendors (who are the ones this would impact) are shown below.

| Huawei, HiSilicon | Not sure | No | The use cases are not clear. Should it not be only for "AllCC" capabilities? Why are fields not matching any "AllCC" capability? Why are some CSI-RS "AllCC" capabilities ignored? |
| --- | --- | --- | --- |
| ZTE | Maybe | No | We raised similar issue in Rel-16 (CLI), see R2-1916128, but we proposed not to coordinate the CSI-RS capabilities, because most capabilities are defined to indicate the maximum number for “simultaneous” CSI-RS resources, in our view, it means the limitation for CSI-RS on the same slot/symbol. From network perspective, it is very difficult to do slot or symbol level coordination between MN and SN. In the CR, there are some capabilities that are not defined as allCC, we think there should be no problem for those capabilities. In addition, the capabilities listed are related to L1 CSI-RS measurement, not L3 CSI-RS measurement, for L3, the capability is maxNumberCSI-RS-RRM-RS-SINR, but as we discussed in R2-1916128, it is very hard to do slot level coordination between MN and SN. |
| Ericsson | Not sure | N | - It does not seem the MN needs to limit the capabilities defined per CC, since either MN or SN would configure them, so the inter-node message signaling could be simplified to coordinate only the UE parameters in csi-RS-IM-ReceptionForFeedbackPerBandComb and simultaneousSRS-AssocCSI-RS-AllCC. - The wording “per CG” seems to hint that the MN is informing the SN how many resources it intends to configure, while the SN could indicate how many it actually configure (in that way, if any resources are left, the MN can still configure additional resources). Is that the intention? If yes, we may need some rewording to clarify it. |
| Nokia, Nokia Shanghai Bell | Yes (proponent) | Yes (proponent) | Just to clarify: Several/most CSI-RS capabilities are both per-CC and per-UE. For example, UE might support up to 5 CSI-RS resources per-UE and 2 per-CC. Network has to ensure both limitations are followed, but this is currently not possible as neither MN nor SN knows what the other has configured. That’s why we proposed to coordinate the resources. As for the exact capabilities coordinated, what we show were the most immediate concerns. We are fine to coordinate also other capabilities if seen needed. In general we are also fine to continue discussing the exact wording that suits all. |
| CATT | Yes | No strong view |  |
| NEC | Y | Y, basically | It seems valid observations, although we can follow majority views. Some small editorial issues. Most parameters are explained as “resources per CG” in the CSI-RS-ResourceUsage field description, while the intention would be “for SCG”. Renaming can be considered? For maxSimultaneousSRS-AssocCSI-RS-PerCG, it is explained as “SN can use simultaneously across the CG”. We are wondering if it should be “for SCG” like other parameters? |

We address each of these issues in the following sub-sections.

### Use cases and exact capabilities

The use case is any NR-DC configuration where network wishes to use configurations that correspond to capabilities where UE does not support just per-CC configurations, but also has some per-UE limitations for configuration. The point from e.g. ZTE was that we should NOT introduce dynamic coordination between the nodes on used or activated resources, as that would be far too demanding. But we also see that some semi-static limitations could still be indicated for capabilities where desired to better allow for optimized performance, but those would not be dynamic. Optimizing the actual UE behaviour was also something we didn’t intend to cover, as there are already some rules for those cases in RAN1 specifications.

Observation 7: The NR-DC coordination via INM should be limited to configuration restrictions (e.g. number of CSI-RS resources per CC or per MN/SN) and does not concern activation of resources (i.e. dynamic coordination is precluded).

Additionally, it was raised that it was not clear if the capabilities in section 2.2 were the only ones requiring coordination, and why the exact same values for capabilities were not reused. For the first part, we have tried to list all the capabilities with MN/SN configuration in Annex A (with explanations on why it needs or does not need to be coordinated). For the capability values, we modified some lower bounds simply because e.g. the minimum per-UE limit might be “2”, so splitting that would require network to be able to indicate also “1”.

Observation 8: The UE capability values cannot always be used with the exact same value ranges (e.g on the lower bound) to allow splitting the resources between MN and SN.

Proposal 1: Define inter-node signalling for MN/SN resource coordination in NR-DC from Rel-16 onwards for the following capabilities:

-	BandNR capabilities activeConfiguredGrant-r16, csi-RS-IM-ReceptionForFeedback, maxNumberCSI-RS-BFD, maxNumberCSI-RS-SSB-CBD, maxNumberSSB-BFD, sps-r16, beamManagementSSB-CSI-RS and csi-RS-ForTracking;

-	CA-ParametersNR capabilities csi-RS-IM-ReceptionForFeedbackPerBandComb, simultaneousCSI-ReportsAllCC and simultaneousSRS-AssocCSI-RS-AllCC:

### CLI resource coordination

It was also raised that Rel-16 CLI WI already defined some coordination for SRS capabilities, as shown by below excerpt from RAN2#108 minutes:

CLI measurements in NR-DC R2-1916128 Remaining issues on CLI measurement ZTE Corporation, Sanechips discussion Rel-16 NR_CLI_RIM - Oppo wonders whether CLI measurements can be configured in (NG)EN-DC and NE-DC - We reconfirm that CLI measurements can be configured in (NG)EN-DC and NE-DC - Ericsson wonders whether to support proposal 2, proposal 4 is the only thing we need to do. - Huawei and Nokia support proposal 2. - QC wonders if this requires additional capabilities. - Nokia/LG do not think we need additional capabilities Agreements: - CLI measurements can be configured in NR-DC (and NR CA). No additional RAN3 work is expected for this. - In NR-DC, both MN and SN can configure CLI measurement at the same time. Network ensures the CLI measurements configured by MN and SN do not go beyond UE’s capability. - For CLI measurement coordination, MN informs SN in CG-ConfigInfo: • Maximum number of SRS RSRP resources that can be configured by SN; • Maximum number of CLI-RSSI resources that can be configured by SN.

This is correct and the CLI-SRS and CLI-RSSI coordination has been defined as shown below, but it is also clear that this coordination is only for the CLI purpose and would only allow SRS resource coordination anyway. Hence, we think it would be better to separate the two uses cases.

maxMeasSRS-ResourceSCG-r16       INTEGER(0..maxNrofCLI-SRS-Resources-r16)                         OPTIONAL,

maxMeasCLI-ResourceSCG-r16       INTEGER(0..maxNrofCLI-RSSI-Resources-r16)                        OPTIONAL,

maxMeasCLI-ResourceSCG Indicates the maximum number of CLI RSSI resources that the SCG is allowed to configure.

maxMeasSRS-ResourceSCG Indicates the maximum number of SRS resources that the SCG is allowed to configure for CLI measurement.

Observation 9: The CLI resource coordination is only valid for SRS and RSSI, which is not sufficient for most NR-DC resource coordination use case.

Proposal 2: Do not reuse of CLI resource coordination for NR-DC cases outside the CLI.

### Need for the coordination

Some companies were wondering why the need for the coordination arose only now, given RAN2 is working in Rel-18 already. The reason is simple: NR-DC deployments are only now starting to be more topical, with EN-DC being the dominant MR-DC deployment so far, so some issues are only being noticed now. Further, there are limits to what OAM  can efficiently configure (since cannot do “magic”), so the need for the coordination will likely only increase in the future.

Observation 10: The need for MN/SN resource coordination will only become larger once more NR-DC deployments are made.

### Simplifications and other changes to signalling

Finally, some comments were made that there could be simplifications to wording and/or signalling details. As always, we are receptive to such comments but first it would be necesary to agree to the intent, after which is easiest to discuss the exact wordings.

Observation 11: Wording simplifications and value ranges can be discussed and taken into account in the CR phase.

We have provided an example Rel-16 and Rel-17 CRs for this in R2-2304873 and R2-2304874 based on the proposals in this document, and would consider those as starting points for the discussion.

Proposal 3: Discuss CRs based on R2-2304873 and R2-2304874.

# 3	Conclusion

This document has made the following observations:

Observation 1: The coordination of MN/SN resources via inter-node messages allow more optimal resource allocation across the cell groups.

Observation 2: NR inter-node messages do not allow for late extensions, e.g. introducing late Rel-15/16 extensions at message level is not possible once Rel-17 extensions have been introduced.

Observation 3: MN-SN configuration restriction IEs can be extended and have been last extended in Rel-16.

Observation 4: Several CSI-RS capabilities are both per-UE and per-carrier, thus requiring that both MN and SN ensure UE capabilities are not exceeded in NR-DC.

Observation 5: In NR-DC, MN and SN configure CSI-RS resources independently according to (known) UE capabilities. Lack of MN/SN-coordination restricts usage of CSI-RS capabilities in NR-DC, leading to complex network handling and underutilization of UE resources.

Observation 6: The CSI-RS resource coordination is not an issue for EN-DC or NE-DC.

Observation 7: The NR-DC coordination via INM should be limited to configuration restrictions (e.g. number of CSI-RS resources per CC or per MN/SN) and does not concern activation of resources (i.e. dynamic coordination is precluded).

Observation 8: The UE capability values cannot always be used with the exact same value ranges (e.g on the lower bound) to allow splitting the resources between MN and SN.

Observation 10: The need for MN/SN resource coordination will only become larger once more NR-DC deployments are made.

Observation 11: Wording simplifications and value ranges can be discussed and taken into account in the CR phase.

And proposed the following:

Proposal 1: Define inter-node signalling for MN/SN resource coordination in NR-DC from Rel-16 onwards for the following capabilities:

-	BandNR capabilities activeConfiguredGrant-r16, csi-RS-IM-ReceptionForFeedback, maxNumberCSI-RS-BFD, maxNumberCSI-RS-SSB-CBD, maxNumberSSB-BFD, sps-r16, beamManagementSSB-CSI-RS and csi-RS-ForTracking;

-	CA-ParametersNR capabilities csi-RS-IM-ReceptionForFeedbackPerBandComb, simultaneousCSI-ReportsAllCC and simultaneousSRS-AssocCSI-RS-AllCC:

Proposal 2: Do not reuse of CLI resource coordination for NR-DC cases outside the CLI.

Proposal 3: Discuss CRs based on R2-2304873 and R2-2304874.

# Annex A:	UE capabilities having MCG/SCG components

#### 4.2.7.1	BandCombinationList parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ca-ParametersNRDC Indicates whether the UE supports NR-DC for the band combination. It contains the NR band combination parameters applicable across MCG and SCG. A UE indicating support for NR-DC shall support synchronous NR-DC configuration where all serving cells of the MCG are in FR1 and all serving cells of the SCG are in FR2. | BC | No | N/A | N/A |

The band combinations used for NR-DC are coordinated already

#### 4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| activeConfiguredGrant-r16 Indicates whether the UE supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. This field includes the following parameters: - maxNumberConfigsPerBWP-r16 indicates the maximum number of configured/active configured grant configurations in a BWP of a serving cell. - maxNumberConfigsAllCC-r16 indicates the maximum number of configured/active configured grant configurations across all serving cells in a MAC entity, and across MCG and SCG in case of NR-DC. The UE can include this feature only if the UE indicates support of either configuredUL-GrantType1 or configuredUL-GrantType1-v1650 and/or configuredUL-GrantType2 or configuredUL-GrantType2-v1650. NOTE: - For all the reported bands in FR1, a same X1 value is reported for maxNumberConfigsAllCC-r16. For all the reported bands in FR2, a same X2 value is reported for maxNumberConfigsAllCC-r16. - The total number of configured/active configured grant configurations across all serving cells in FR1 is no greater than X1. - The total number of configured/active configured grant configurations across all serving cells in FR2 is no greater than X2. - If the CA have some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of configured/active configured grant configurations across all serving cells is no greater than max(X1, X2). | Band | No | N/A | N/A |

| maxNumberCSI-RS-BFD Indicates maximal number of CSI-RS resources across all CCs, and across MCG and SCG in case of NR-DC, for UE to monitor PDCCH quality. In this release, the maximum value that can be signalled is 16. If the UE includes the field in an FR1 band, it shall set the same value in all FR1 bands. If the UE includes the field in an FR2 band, it shall set the same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. It is mandatory with capability signalling for FR2 and optional for FR1. | Band | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| maxNumberCSI-RS-SSB-CBD Defines maximal number of different CSI-RS [and/or SSB] resources across all CCs, and across MCG and SCG in case of NR-DC, for new beam identifications. In this release, the maximum value that can be signalled is 128. If the UE includes the field in an FR1 band, it shall set the same value in all FR1 bands. If the UE includes the field in an FR2 band, it shall set the same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. It is mandatory with capability signalling for FR2 and optional for FR1. The UE is mandated to report at least 32 for FR2. | Band | CY | N/A | N/A |
| maxNumberSSB-BFD Defines maximal number of different SSBs across all CCs, and across MCG and SCG in case of NR-DC, for UE to monitor PDCCH quality. In this release, the maximum value that can be signalled is 16. If the UE includes the field in an FR1 band, it shall set the same value in all FR1 bands. If the UE includes the field in an FR2 band, it shall set the same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. It is mandatory with capability signalling for FR2 and optional for FR1. | Band | CY | N/A | N/A |
| sps-r16 Indicates whether the UE support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group. This field includes the following parameters: - maxNumberConfigsPerBWP-r16 indicates the maximum number of active SPS configurations in a BWP of a serving cell. - maxNumberConfigsAllCC-r16 indicates the maximum number of active SPS configurations across all serving cells in a MAC entity, and across MCG and SCG in case of NR-DC. The UE can include this feature only if the UE indicates support of downlinkSPS. NOTE: - For all the reported bands in FR1, a same X1 value is reported for maxNumberConfigsAllCC-r16. For all the reported bands in FR2, a same X2 value is reported for maxNumberConfigsAllCC-r16. - The total number of active SPS configurations across all serving cells in FR1 is no greater than X1. - The total number of active SPS configurations across all serving cells in FR2 is no greater than X2. - If the CA have some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of active SPS configurations across all serving cells is no greater than max(X1, X2). | Band | No | N/A | N/A |

| csi-RS-IM-ReceptionForFeedback Indicates support of CSI-RS and CSI-IM reception for CSI feedback. This capability signalling comprises the following parameters: - maxConfigNumberNZP-CSI-RS-PerCC indicates the maximum number of configured NZP-CSI-RS resources per CC; - maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC indicates the maximum number of ports across all configured NZP-CSI-RS resources per CC; - maxConfigNumberCSI-IM-PerCC indicates the maximum number of configured CSI-IM resources per CC; - maxNumberSimultaneousNZP-CSI-RS-PerCC indicates the maximum number of simultaneous CSI-RS-resources per CC; - totalNumberPortsSimultaneousNZP-CSI-RS-PerCC indicates the total number of CSI-RS ports in simultaneous CSI-RS resources per CC. The UE is mandated to report csi-RS-IM-ReceptionForFeedback. | Band | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |

The BandNR capabilities activeConfiguredGrant-r16, csi-RS-IM-ReceptionForFeedback, maxNumberCSI-RS-BFD, maxNumberCSI-RS-SSB-CBD, maxNumberSSB-BFD and sps-r16 are (partially) across MCG/SCG but are not coordinated currently for NR-DC

#### 4.2.7.4	CA-ParametersNR

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| csi-RS-IM-ReceptionForFeedbackPerBandComb Indicates support of CSI-RS and CSI-IM reception for CSI feedback. This capability signalling comprises the following parameters: - maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC indicates the maximum number of simultaneous CSI-RS resources (irrespective of the associated codebook type) in active BWPs across all CCs, and across MCG and SCG in case of NR-DC. The network applies this limit in addition to the limits signalled in MIMO-ParametersPerBand-> maxNumberSimultaneousNZP-CSI-RS-PerCC and in Phy-ParametersFRX-Diff-> maxNumberSimultaneousNZP-CSI-RS-PerCC; - totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC indicates the total number of CSI-RS ports in simultaneous CSI-RS resources (irrespective of the associated codebook type) in active BWPs across all CCs, and across MCG and SCG in case of NR-DC. The network applies this limit in addition to the limits signalled in MIMO-ParametersPerBand-> totalNumberPortsSimultaneousNZP-CSI-RS-PerCC and in Phy-ParametersFRX-Diff-> totalNumberPortsSimultaneousNZP-CSI-RS-PerCC. The UE is mandated to report csi-RS-IM-ReceptionForFeedbackPerBandComb. | BC | Yes | N/A | N/A |

| simultaneousCSI-ReportsAllCC Indicates whether the UE supports CSI report framework and the number of CSI report(s) which the UE can simultaneously process across all CCs, and across MCG and SCG in case of NR-DC. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsAllCC includes the beam report and CSI report. This parameter may further limit simultaneousCSI-ReportsPerCC in MIMO-ParametersPerBand and Phy-ParametersFRX-Diff for each band in a given band combination. | BC | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |

| simultaneousSRS-AssocCSI-RS-AllCC Indicates support of CSI-RS processing framework for SRS and the number of SRS resources that the UE can process simultaneously across all CCs, and across MCG and SCG in case of NR-DC, including periodic, aperiodic and semi-persistent SRS. This parameter may further limit simultaneousSRS-AssocCSI-RS-PerCC in MIMO-ParametersPerBand and Phy-ParametersFRX-Diff for each band in a given band combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

The CA-ParametersNR capabilities csi-RS-IM-ReceptionForFeedbackPerBandComb, simultaneousCSI-ReportsAllCC and simultaneousSRS-AssocCSI-RS-AllCC are across MCG/SCG but are not coordinated currently for NR-DC

| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| simultaneousRxTxInterBandCAPerBandPair Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA for each band pair in the band combination. Encoded as a bitmap with size L * (L – 1) / 2, and bit N (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission and reception for band pair (x, y), where L is the number of band entries in the band combination, x and y are the indices of the band entry in the band combination (the first band entry is indexed as 0), x < y, and N = x*(2*L – x – 1)/2 + y – x – 1. If this field is included in ca-ParametersNR-ForDC, each bit of this field indicates whether the UE supports simultaneous transmission and reception between each band pair, within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. The UE does not include this field if the UE supports simultaneous transmission and reception for all applicable band pairs in the band combination (in which case simultaneousRxTxInterBandCA is included) or does not support for any band pair in the band combination. It is mandatory for certain band pairs as specified in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall consistently set the bits which correspond to the same band pair. | BC | CY | N/A | N/A |

The CA-ParametersNR capabilities simultaneousRxTxInterBandCA and simultaneousRxTxInterBandCAPerBandPair are across MCG/SCG, but need not be coordinated since they cannot be “split” so MN/SN can just follow the UE capabilities in NR-DC

### ===== Across FR, no MCG/SCG… ======

#### 4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| beamManagementSSB-CSI-RS Defines support of SS/PBCH and CSI-RS based RSRP measurements. The capability comprises signalling of - maxNumberSSB-CSI-RS-ResourceOneTx indicates maximum total number of configured one port NZP CSI-RS resources and SS/PBCH blocks that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). On FR2, it is mandatory to report >=8; On FR1, it is mandatory with capability signalling to report >=8. - maxNumberCSI-RS-Resource indicates maximum total number of configured NZP-CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] across all serving cells (see NOTE). It is mandated to report at least n8 for FR1. - maxNumberCSI-RS-ResourceTwoTx indicates maximum total number of two ports NZP CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). - supportedCSI-RS-Density indicates density of one RE per PRB for one port NZP CSI-RS resource for RSRP reporting, if supported. On FR2, it is mandatory to report either "three" or "oneAndThree"; On FR1, it is mandatory with capability signalling to report either "three" or "oneAndThree". - maxNumberAperiodicCSI-RS-Resource indicates maximum number of configured aperiodic CSI-RS resources across all serving cells (see NOTE). For FR1 and FR2, the UE is mandated to report at least n4. NOTE: If the UE sets a value other than n0 in an FR1 band, it shall set that same value in all FR1 bands. If the UE sets a value other than n0 in an FR2 band, it shall set that same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. | Band | Yes | N/A | FD |

| csi-RS-ForTracking Indicates support of CSI-RS for tracking (i.e. TRS). This capability signalling comprises the following parameters: - maxBurstLength indicates the TRS burst length. Value 1 indicates 1 slot and value 2 indicates both of 1 slot and 2 slots. In this release UE is mandated to report value 2; - maxSimultaneousResourceSetsPerCC indicates the maximum number of TRS resource sets per CC which the UE can track simultaneously; - maxConfiguredResourceSetsPerCC indicates the maximum number of TRS resource sets configured to UE per CC. It is mandated to report at least 8 for FR1 and 16 for FR2; - maxConfiguredResourceSetsAllCC indicates the maximum number of TRS resource sets configured to UE across CCs. If the UE includes the field in an FR1 band, it shall set the same value in all FR1 bands. If the UE includes the field in an FR2 band, it shall set the same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. The UE is mandated to report at least 16 for FR1 and 32 for FR2. The UE is mandated to report csi-RS-ForTracking. | Band | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |

The BandNR capabilities beamManagementSSB-CSI-RS and csi-RS-ForTracking are not per MN/SN but per FR, and are not coordinated currently for NR-DC

### ===== Across NR-DC BC =======

#### 4.2.7.10	Phy-Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pdcch-BlindDetectionMCG-UE Indicates PDCCH blind decoding capabilities supported for MCG when in NR DC. The field value is from 1 to 15. The UE sets the value in accordance with the constraints specified in TS 38.213 [11]. Additionally, if the UE does not report pdcch-BlindDetectionCA, and if X is the maximum number of CCs supported by the UE across all NR-DC band combinations then there is at least one parameter pair (X1, X2) such that X1 + X2 = X and the UE supports at least one NR-DC band combination with X1 CCs in MCG and X2 CCs in SCG and for which X1 <= pdcch-BlindDetectionMCG-UE and X2 <= pdcch-BlindDetectionSCG-UE. | UE | No | No | Yes |
| pdcch-BlindDetectionSCG-UE Indicates PDCCH blind decoding capabilities supported for SCG when in NR DC. The field value is from 1 to 15. The UE sets the value in accordance with the constraints specified in TS 38.213 [11]. Additionally, if the UE does not report pdcch-BlindDetectionCA, and if X is the maximum number of CCs supported by the UE across all NR-DC band combinations then there is at least one parameter pair (X1, X2) such that X1 + X2 = X and the UE supports at least one NR-DC band combination with X1 CCs in MCG and X2 CCs in SCG and for which X1 <= pdcch-BlindDetectionMCG-UE and X2 <= pdcch-BlindDetectionSCG-UE. | UE | No | No | Yes |

The Phy-Parameters capabilities pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE-CSI-RS are across MCG/SCG, and and are already coordinated for NR-DC

ConfigRestrictModReqSCG ::=         SEQUENCE {

requestedPDCCH-BlindDetectionSCG    INTEGER (1..15)                                   OPTIONAL,

requestedPDCCH-BlindDetectionSCG Requested value of the reference number of cells for PDCCH blind detection allowed to be configured for the SCG.

ConfigRestrictInfoSCG ::=       SEQUENCE {

pdcch-BlindDetectionSCG          INTEGER (1..15)                                      OPTIONAL,

pdcch-BlindDetectionSCG Indicates the maximum value of the reference number of cells for PDCCH blind detection allowed to be configured for the SCG.