3GPP TSG-RAN WG2 Meeting #122	R2-2304879

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.25.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Lower MSD capability

WID/SID:	NR_ENDC_RF_FR1_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN4 has been working on a WI aimed to study/specify for feasibility of lower MSD capability for inter-band CA/EN-DC/DC combinations, and concluded on the following in RAN4#106:

- The feasibility for MSD improvement for all kinds of MSD has been confirmed based on the evaluation from companies on the selected example band combinations.

- Several promising options for allowing a UE to signal improved lower MSD performance have been discussed, with which companies think it is feasible to introduce the lower MSD capability.

- Details of the lower MSD capability will be further discussed in the UE RF FR1 WI.

Based on this, the WI objective was revised and RAN2 was added as one of the responsible WGs. Due to that, RAN4 has now sent an LS in R2-2304644, requesting RAN2 to start the work.

# 2	Enabling Lower MSD capability

## 2.1	Background and definition of MSD

Since RAN2 is usually not involved with RF aspects, the RAN4 LS nicely provides a simple explanation of what “MSD” is:

MSD means maximum sensitivity degradation or reference sensitivity exceptions specified in TS 38.101-1/3 clause 7 for CA/DC respectively. The sensitivity degradation for the victim band can be caused by harmonic, harmonic mixing, intermodulation (IMD) or cross band isolation interference. For some band combinations, the MSD value could be as large as 30dB. The intention of lower MSD capability is to facilitate the network scheduling if UE can have better MSD compared to those defined in the specifications. The MSD is specified for specific conditions on the UL/DL bandwidth and the uplink power levels.

That is, MSD causes reference sensitivity degradation (i.e. increases the noise floor in DL), which can happen due to various reasons (e.g. harmonics, IMD, cross-band isolation), and RAN4 is investigating means by which UE could perform better, i.e. to have lower MSD value.

Observation 1: The RAN2 work for lower MSD seems to be mainly about capability signalling and (possibly) some configuration parameters.

RAN4 has also indicated the rough guidelines for the RAN2 work ahead as shown below:

To facilitate the initial discussion of the signalling design for lower MSD capability, the following agreements reached so far are provided, which are essential information considered by RAN4 for the capability: - Victim band - MSD type (harmonic; harmonic mixing; cross band isolation; IMD) with orders - MSD indication of corresponding threshold (or capability class) - Power class - [Aggressor UL and victim DL bandwidth] Upon the essential information, some other agreements reached in RAN4 (up to RAN4#106-bis-e) could also be referred for the signalling design: - A UE should be allowed to report the low MSD capability for any MSD mechanism and order that have been defined in the 3GPP specifications for a given band combination. - No dynamic signaling scheme will be introduced for lower MSD report in Rel-18 - Define one set of absolute multiple thresholds for lower MSD - Respective MSD indication of corresponding threshold(s) are reported with one of the thresholds listed in the set - Lower MSD capability for higher order combination is inherited from lower order fallback combinations - For 2-bands combination, the MSD indication of corresponding threshold (or capability class) are supposed to be reported separately as per victim band per MSD type per band combination - For 3-bands combination, the MSD indication of corresponding threshold (or capability class) are only reported for IMD of dual UL falling into the third band DL whose UL band is not configured with, other kinds of Lower MSD capability (harmonic/ harmonic mixing/cross band isolation/IMD due to dual UL falling into own DL) could inherit from 2-band combinations with the same power class. - For combination with more than 3 bands, no need to report the Lower MSD capability any more, the capability could inherit from the fallback combinations with the same power class. - UE is allowed to report lower MSD capability for different power classes Since a band combination may suffer reference sensitivity degradation from different MSD types and orders (e.g. CA_n3-n78 could have MSDs caused by harmonic, harmonic mixing, 2nd order IMD and 4th order IMD with different levels for certain UL/DL configurations in the operating bands), signalling overhead reduction is also an important issue discussed in RAN4.

Looking at this, it appears to be somewhat close to the IDC functionality, but with some clear distinctions: The information is per power class and MSD type, and could depend on both the victim band, victim DL bandwidth and aggressor UL bandwidth.

Observation 2: The MSD capability is not the same as IDC, even though there are some similarities.

Additionally, RAN4 has indicated that “no dynamic signalling scheme” will be introduced, but then also that there would be “one set of thresholds for lower MSD” that corresponds to the MSD indication. Also, it seems RAN4 is concerned about the signalling overhead since a single band combination could indicate MSDs for multiple MSD types, which could then create additional overhead due to large volume of band combination signalling, especially since the capability signalling could also depend on the used power class.

Observation 3: The signalling overhead of per-BC MSD capability signalling needs to be considered (as per RAN4 feedback).

Finally, RAN4 is also noting that the signalling would only be applicable for (inter-band) band combinations with 2 or and 3 band( entrie)s. RAN4 is also indicating that the higher-order band combinations will “inherit” the lower MSD capabilities from the lower-order band combinations, which is opposite of what RAN2 normally does with band combinations, wherein lower-order fallback combinations inherit the capabilities or higher-order band combinations!

Observation 4: The lower MSD capabilities of lower-order (i.e. 2- or 3-band) band combinations are inherited by the higher-order band combinations (i.e. 3 or more bands).

## 2.2	Discussion on the UE capability signalling

As the above shows and RAN4 LS also notes, the work is still ongoing and what exactly to do in RAN2 is not so clear.RAN4 is also asking for RAN2 feedback, but we think RAN2 could first consider the expected work based on current information, and only request feedback on matters that are unclear or require additional RAN4 information:

- How do the “set of absolute multiple thresholds for lower MSD” impact the capability signalling? How does that relate to the UE capability signalling? → This could require RAN4 feedback, the current sentence is not very clear. However, we do assume that RAN4 will further clarify this point anyway.

- Is the list of “harmonic; harmonic mixing; cross band isolation; IMD” all MSD types that are needed for the capability signalling? → In our understanding the RAN4 list is exhaustive, so no need to ask for further feedback.

- RAN2 normally only signals the highest-order band combinations, with UE supporting all of the defined fallback band combinations that “inherit” the UE capabilities. Since the “inheriting” support of lower MSD capabilities is done in reverse order, should the MSD capabilities be indicated separately from the existing band combination signalling? → This is a RAN2 matter since it’s about the signalling structure. RAN2 can decide on this and inform RAN4.

- Since no “dynamic signalling” is required, how does network know when the MSD capabilities apply? → If UE is capable of lower MSD and network configuration meets the necessary conditions, we think UE will always have lower MSD and no explicit configuration for “use lower MSD” is needed.

### Current band combination capability signalling

Current band combination signaling structures assume that each fallback band combination supports the same capabilities as those for the parent band combination unless explicitly indicated. For example, if CA_n1-n3-n78-nX has some capabilities, the fallback band combinations like CA_n1-n3-n78, CA_n1-n3-nX, CA_n3-n78-nX and CA_n1-n3, etc., have the same capabilities. The parent-fallback BCs are illustrated in Figure 1 below.

Figure 1: Parent and fallback BC structure

In terms of RAN4 specifications, there are no MSD requirements for band combinations whose number of UL bands is >2 and that of DL bands is > 3. More specifically, the MSD requirements are classified as follows.

| # of UL bands | # of DL bands | Root cause of MSD |
| --- | --- | --- |
| 1 | 2 | Harmonics |
| 1 | 2 | Harmonic mixing |
| 1 | 2 | Cross band isolation |
| 2 | 2 | IMD |
| 11 or 2 | 3 | IMD(for the 3rd DL bands) |
| Note 1: IMD may occur for intra-band non-contiguous CA |  |  |

Following the existing RAN2 capability signalling for band combinations consisting of four bands like CA_n1-n3-n78-nX and CA_n1-n3-n78-nY, the signaling could be defined (in simplified terms) as shown in Figure 2 below.

Figure 2: MSD reporting using current capability signalling structure

This shows that if a UE supports the above two parent band combinations, exactly the same information needs to be reported to a network for both of them.

Observation 5: Using existing capability signalling structure would duplicate a lot of UE capability signalling.

In addition, some fallback band combinations have multiple MSD sources and values like CA-n1-n3, and in fact, CA_n1-n3-n78 contains the following MSD values as summarized in Table 1.

Table 1: MSDs contained in CA_n1-n3-78

| NR-CA | UL | UL | DL | MSD dB | Source of MSD |
| --- | --- | --- | --- | --- | --- |
| CA_n1-n3 | n1 | n3 | n1 | 23 | IMD2 |
| CA_n1-n3 | n1 | N/A | n3 | 3@5MHz, … | Cross band isolation |
| CA_n1-n78 | n1 | N/A | n78 | 8@5MHz | IMD4 |
| CA_ n3-n78 | n3 | N/A | n78 | 23.9@10 MHz,… | Harmonics |
| CA_ n3-n78 | n3 | n78 | n3 | 26@ 5 MHz | IMD2 |
| CA_ n3-n78 | n3 | n78 | n3 | 8@ 5 MHz | IMD4 |
| CA_ n3-n78 | n78 | N/A | n3 | 8.1@ 5 MHz | Harmonic mixing |
| CA_n1-n3-n78 | n1 | n3 | n78 | 28.4@ 10 MHz | IMD2 |
| CA_n1-n3-n78 | n1 | n3 | n78 | 11.2@ 10 MHz | IMD4 |
| CA_n1-n3-n78 | n1 | n78 | n3 | 27.9@ 5 MHz | IMD2 |

Hence, reported MSD values for fallback combinations can be multiple under each parent band combination.

Observation 6: If a band combination consists of multiple MSD “sources”, the lower MSD capability signalling will require different entries for each.

### RAN4-requested lower MSD capability signalling

We try to illustrate the RAN4 request with a few diagrams and a concrete example band case. Assume that UE supports

-	Bands n1, n3, n5, n7, n28 and n78

-	CA (parent) band combinations CA_n1-n3-n5-n78, CA_n1-n3-n7-n78, CA_n1-n3-n28-n78 and CA_n1-n3-n8-n78

-	Lower MSD for CA_n1-n3 (but not for e.g. CA_n1-n78).

Then, what UE signals in band combination is:

-	Supported BCs: CA_n1-n3-n5-n78, CA_n1-n3-n7-n78, CA_n1-n3-n28-n78 and CA_n1-n3-n8-n78

For the lower MSD capabilities, UE should signal only CA_n1-n3 lower MSD capabilities, which are then “inherited” by the parent band combinations, e.g. as illustrated in Figure 1 below, where the green color illustrates the part where the lower MSD capabilities apply (note that the figure doesn’t consider all possible parent/fallback combinations for simplicity).

Figure 3: “Inherited” lower MSD support

Thus, it seems more efficient to only signal the lower MSD capabilities once for those band combinations where UE actually supports the lower MSD, which are then applicable for all band combinations where the lower MSD “components” are included.

Observation 7: It is more efficient to define the lower MSD capabilities outside the usual band combination capabilities, since that avoids duplication of the capabilities in multiple band combinations.

Proposal 1: RAN2 define the lower MSD capabilities outside the band combination UE capabilities to avoid repetition of the same signaling in multiple band combinations.

If the signalling is defined outside the band combinations, it seems also reasonable to consider an additional capability filter that network indicates to UE when it wishes to obtain the lower MSD information. This enables RAN2 to define only a single capability bit for the support of lower MSD capabilities,

Proposal 2: Define a new capability filter that allows network to only request the lower MSD capabilities when necessary. UE support lower MSD only indicates the detailed MSD information if requested.

Proposal 3: Define a per-UE capability bit that indicates the UE supports lower MSD capabilities for some band combinations. It is always reported regardless of the UE capability filter.

Proposal 4: Companies to provide CRs to RAN2#123 to illustrate the capability structure for the lower MSD capabilities.

# 3	Conclusion

This document has made the following observations:

Observation 1: The RAN2 work for lower MSD seems to be mainly about capability signalling and (possibly) some configuration parameters.

Observation 2: The MSD capability is not the same as IDC, even though there are some similarities.

Observation 3: The signalling overhead of per-BC MSD capability signalling needs to be considered (as per RAN4 feedback).

Observation 4: The lower MSD capabilities of lower-order (i.e. 2- or 3-band) band combinations are inherited buy the higher-order band combinations (i.e. 3 or more bands).

Observation 5: Using existing capability signalling structure would duplicate a lot of UE capability signalling.

Observation 6: If a band combination consists of multiple MSD “sources”, the lower MSD capability signalling will require different entries for each.

Observation 7: It is more efficient to define the lower MSD capabilities outside the usual band combination capabilities, since that avoids duplication of the capabilities in multiple band combinations.

And proposed the following:

Proposal 1: RAN2 define the lower MSD capabilities outside the band combination UE capabilities to avoid repetition of the same signaling in multiple band combinations.

Proposal 2: Define a new capability filter that allows network to only request the lower MSD capabilities when necessary. UE support lower MSD only indicates the detailed MSD information if requested.

Proposal 3: Define a per-UE capability bit that indicates the UE supports lower MSD capabilities for some band combinations. It is always reported regardless of the UE capability filter.

Proposal 4: Companies to provide CRs to RAN2#123 to illustrate the capability structure for the lower MSD capabilities.