3GPP TSG-RAN WG2 Meeting #122		         R2-2304889

Incheon, Republic of Korea, 22 – 26 May 2023

Agenda Item:	7.4.2.1

Source: 	MediaTek Inc.

Title:  	Open Issues for LTM Procedure

Document for:	Discussion and decision

# Introduction

In last RAN2 meeting [1], we made agreements on L1/L2-triggered mobility (LTM) procedure. In this contribution, we discuss remaining open issues for LTM procedure.

# Discussion

## Arrival indication and completion

In last meeting, we made the following assumptions and agreements for LTM UE arrival indication and LTM completion:

- R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. - In RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA and on the reception of Msg3/MsgA in CBRA, like the legacy HO. - In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE. - In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) - For RACH-based LTM, the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. - For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data.

Does UE always send RRCReconfigurationComplete message?

The assumption that UE always sent RRCReconfigurationComplete (for both RACH-based and RACH-less LTM) came from the majority view in the corresponding email discussion [2]. However, there were concerns about the feasibility of RRCReconfigurationComplete in case of RACH-less LTM (e.g., coupling L1/L2 and L3 procedures). We suggest that UE always sends RRCReconfigurationComplete for LTM, since UE does perform RRC reconfiguration in LTM execution.

Proposal 1: RAN2 confirms that UE always sends RRCReconfigurationComplete message towards target cell at each LTM execution.

Grant for the first UL message

In RACH-based LTM, the first UL message (RRCReconfigurationComplete) is sent after receiving RAR, which also provides the UL grant. In RACH-less LTM, the target cell needs a way to provide UL grant for the first UL message (assuming RRCReconfigurationComplete). Since UE needs some time for reconfiguration (even with RACH-less operation), the UL grant can be preconfigured periodically within a short duration after the cell switch command. We see the following options for UL grant provisioning:

- Configured grant in candidate configurations: The cell switch command serves as an ‘activation’ signal, and UE can access the preconfigured UL resources once it finishes reconfiguration. The UL grant is deactivated once UE receives PDCCH addressed to its C-RNTI. We may reuse existing ConfiguredGrantConfig (maybe with a new type) or define a new IE for this purpose.

- UL grant is provided in the LTM command MAC CE.

We need further discussions in RAN2 for the details (signalling, periodicity, etc.) of such UL grant.

Proposal 2: In RACH-less LTM, network should provide UL grant for the first UL message in target cell. RAN2 to discuss the method, e.g.,

- Configured grant in candidate RRC configuration, or

- UL grant field in LTM command MAC CE.

## Recovery with LTM candidates

RAN2 made the following agreement on failure handling for LTM.

- At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution.

This resembles the recovery mechanism in Rel-16 conditional handover (CHO), where UE may execute CHO towards another CHO candidate upon handover failure or CHO failure. We now discuss the details of recovery with LTM candidates:

Arrival indication

If UE does not have valid TA of the selected candidate for recovery, UE should perform RACH towards selected candidate, and the Msg1/MsgA serves as the arrival indication. Even if UE has valid TA (via early TA acquisition) of the selected candidate, it is unlikely that the candidate can provide UL grant for UE’s first UL message as arrival indication. Therefore, we suggest that UE always perform RACH when executing LTM for recovery.

CBRA or CFRA?

RACH-based LTM can be either CBRA or CFRA. UE may use CFRA resource provided in candidate configuration when the candidate is selected, as long as the CFRA resource is available. However, availability of CFRA resource in an earlier configuration may require further discussion. We suggest that CBRA be considered as baseline, and CFRA can be supported if availability of CFRA resources is guaranteed.

Proposal 3: UE always perform RACH when executing LTM for recovery. In such RACH procedure, CBRA is considered as baseline, and CFRA can be supported if availability of CFRA resources is guaranteed.

## UE-based TA measurement for candidate cells

Early TA acquisition allows UE to skip RACH upon LTM execution and thus achieves fast cell switch. In addition to sending preamble triggered by PDCCH order, UE may derive candidate cell TA based on serving cell TA and timing difference between serving and candidate cells. RAN1 made the following working in RAN1#112 [3]:

Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

Feasibility

Although physical layer details need further discussion in RAN1, from RAN2 perspective, we believe that as least for synchronous network, UE-based TA measurement for candidate cells can be supported. As illustrated in the figure below, in synchronous network, TA of neighbor cell (TA2) can be obtained by simply adding twice of Rx timing difference (Δ) to serving cell TA (TA1).

Figure 1. UE-based TA measurement in synchronous network

For asynchronous network, however, the calculated TA value needs to be adjusted with Tx timing difference between serving and candidate cells. Such Tx timing difference needs to be estimated by network and provided to UE, but the estimation is not always possible since there may not be common clock between the cells. We suggest that RAN2 confirm the feasibility of UE-based TA measurement for candidate cells for synchronous network, and leave it FFS for asynchronous network.

Proposal 4: From RAN2 perspective, UE-based TA measurement for candidate cells can be supported for synchronous network without additional information (e.g., Tx timing difference between serving and candidate cell) from network. FFS support for asynchronous network.

TA value update

Since candidate TA is derived based on serving cell TA, it can be updated when serving cell TA is updated. Moreover, UE may not need the TA for all configured candidate cells. We can introduce a MAC CE to activate/deactivate the TA derivation for a candidate cell (e.g., based on L1 measurements).

Proposal 5: As a baseline, with UE-based TA measurement, candidate cell TA values are updated when serving cell TA is updated. FFS other update mechanism.

Proposal 6: Introduce a MAC CE to activate/deactivate candidate cell TA update in UE-based TA measurement.

# Conclusion

It is proposed to discuss and decide on the following proposals:

Arrival indication and completion

Proposal 1: RAN2 confirms that UE always sends RRCReconfigurationComplete message towards target cell at each LTM execution.

Proposal 2: In RACH-less LTM, network should provide UL grant for the first UL message in target cell. RAN2 to discuss the method, e.g.,

- Configured grant in candidate RRC configuration, or

- UL grant field in LTM command MAC CE.

Recovery with LTM candidates

Proposal 3: UE always perform RACH when executing LTM for recovery. In such RACH procedure, CBRA is considered as baseline, and CFRA can be supported if availability of CFRA resource is guaranteed.

UE-based TA measurement for candidate cells

Proposal 4: From RAN2 perspective, UE-based TA measurement for candidate cells can be supported for synchronous network without additional information (e.g., Tx timing difference between serving and candidate cell) from network. FFS support for asynchronous network.

Proposal 5: As a baseline, with UE-based TA measurement, candidate cell TA values are updated when serving cell TA is updated. FFS other update mechanism.

Proposal 6: Introduce a MAC CE to activate/deactivate candidate cell TA update in UE-based TA measurement.

# Reference

- RAN2#121bis Chair’s Note

- R2-2304214 Summary of [AT121bis-e][018][eMob] Procedure Consolidation (Huawei)	Huawei, HiSilicon

- RAN1#112 Chair’s Note