3GPP TSG-RAN WG2 Meeting #122		         R2-2304890

Incheon, Republic of Korea, 22 – 26 May 2023

Agenda Item:	7.4.2.2

Source: 	MediaTek Inc.

Title:  	Open Issues for LTM RRC

Document for:	Discussion and decision

# Introduction

In last RAN2 meeting [1], we made agreements on RRC for L1/L2-triggered mobility (LTM). In this contribution, we discuss remaining open issues for LTM RRC.

# Discussion

## L1 measurements

In LTM procedure, although UE applies candidate configurations upon cell switch, some knowledge of candidate cells may be needed even when UE is still served by source cell. In last meeting, we identify the following required information and made initial agreements on how they should be configured.

Initial agreements, from RAN2 point of view (may be dep on RAN1 progress). - The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS. - RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s). Chair: the agreements above may need to be further evaluated, e.g. wrt subsequent LTM switches.

While the initial agreements were obtained from majority view in a related offline discussion [2], there were concerns about detailed operations, e.g., subsequent LTM. We now discuss the details for each bullet.

L1 measurement RS

If a list of candidate cell SSBs is configured external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells, it should contain the strongest SSBs from serving and nearby candidate cells, and these SSBs correspond to the beams that UE is currently using or likely to use in near future. A list external to candidate cell configurations help UE avoid accessing stored configurations. However, this also means that the associated candidate configuration needs to be specified. We had a similar issue in Rel-17 inter-cell beam management, where we introduced inside CSI-SSB-ResourceSet a servingAdditionalPCIList, indicating the PCI of the SSBs in the csi-SSB-ResourceList.  For Rel-18 LTM, we may consider such indexing method with the following modifications:

- Unlike Rel-17 ICBM, where serving cell does not change (and SSBs corresponding to serving cell is assigned ServingAdditionalPCIIndex = 0), serving cell does change when UE switches to a beam of another cell in Rel-18 LTM. To avoid reconfiguration of the SSB list every time after cell switch, the SSBs from serving cell needs not to have index = 0. The index can simply be candidate configuration ID (serving cell is also a candidate).

- To avoid confusion with Rel-17 ICBM, the index may be configured for each CSI-SSB-ResourceSet (instead of each SSB)

Proposal 1: SSBs in the list (external to serving and candidate cell configurations) are associated with indices identifying the corresponding candidate cells.

Proposal 2: Current serving cell is also a candidate, and the cell index for SSBs from current serving cell needs not to be zero. RAN2 may discuss if the index should start from 0 or 1.

Proposal 3: The candidate cell indices are configured for each CSI-SSB-ResourceSet.

Moreover, as the list is also outside serving cell, there may also be a list in serving cell configuration (e.g., csi-SSB-ResourceSetToAddModList in serving cell’s CSI-MeasConfig). We need to discuss how UE handles the L1 measurements on current serving cell when there are two lists containing SSBs (i.e., one inside and another outside serving cell configuration). We see three options:

- Option 1: UE follows the list inside serving cell configuration (and ignores serving cell SSBs in the list outside)

- Option 2: UE follows the list outside serving cell configuration (and ignores the list inside)

- Option 3: UE takes the union of elements in the two lists

Our preference is Option 1, since it allows network to configure SSBs for intra-cell mobility in the ‘inside’ list, and SSBs for inter-cell mobility in the ‘outside’ list.

Proposal 4: When serving cell configuration contains a list of SSBs for L1 measurement, UE ignores the serving cell SSBs in the list outside.

L1 reporting

In last meeting, RAN1 also made agreements on L1 measurement report.

Agreement For the beam selection for SSB based L1-RSRP measurement report, - Beam selection is performed across the L cells from configured (or activated, if introduced) cells, i.e. M beams for each of the L cells - FFS: How to select the L cells and M beams per cells is up to UE - M x L beams are reported in a single report instance - Max values of M and L are based on UE capability, and at least M x L=4 is supported as a UE capability, other UE capabilities are FFS - FFS if UE is allowed to report less than M x L beams - The values of M and L are configured to the UE in the reporting configuration - FFS: The following configurability is introduced in the report configuration - 1) Whether serving cell is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 - 2) at least one of the inter-frequency cells is always selected in the L cell selection performed by the UE, and applicable when a UE is configured with L>=2 and at least one cell in inter-frequency

If the cells to be reported can be selected by the UE itself, the reporting configuration can be included in each candidate, and applied when a candidate become serving cell. Since this is still FFS in RAN1, our suggestion is that RAN2 keeps our assumption at this moment and re-evaluate it once RAN1 has their conclusion.

Proposal 5: RAN2 keeps the assumption that for L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s) and re-evaluate it once RAN1 has their conclusion.

Based on the above proposals, Figure 1 shows an example structure of related configurations. In this example, CSI-ResourceConfig is kept inside the CSI-MeasConfig of serving and candidate configurations, together with CSI-ReportConfig, since they are highly related. The list of CSI-SSB-ResourceSet is external to serving and candidate configurations, following RAN2 agreement.

Figure 1. L1 measurement / reporting configurations inside and outside serving / candidate configurations

Control with L1/L2 signalling

Another issue is the need of L1/L2 signalling to control or change L1 measurement/reporting for LTM

- Whether to assume L1/L2 signaling to control or change L1 measurement/reporting for LTM needs further discussion (parts may be discussed in RAN1). RAN2 assumes that such control would be limited to certain aspect that need frequent update and restricted by RRC configuration.

As mentioned earlier, the SSBs in the ‘common’ list correspond to the beams seen by a UE. The list needs not to be too big, and it can be gradually updated based on L3 measurements. Therefore, we do not see the needed to have additional L1/L2 signalling to activate/deactivate elements in the list.

Proposal 6: Do not introduce L1/L2 signalling to control or change L1 measurement/reporting for LTM.

L1 measurements on deactivated SCell

RAN2 left it FFS whether L1 measurement for LTM can be performed

- FFS if the LTM specific L1 measurements of an LTM candidate SCell is independent of its activation status.

Since it is possible to have an SCell as LTM candidate PCell (i.e., a ‘PCell-SCell swapping’ scenario), we believe that L1 measurement should be applicable to a currently deactivated SCell. In other words, LTM-specific L1 measurements of an LTM candidate SCell is independent of its activation status

Proposal 7: LTM-specific L1 measurements of an LTM candidate SCell is independent of its activation status.

## TCI states

In April meeting, both RAN2 and RAN1 made some progress related to TCI states. RAN2 made the following assumption:

- RAN2 assumes that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration).

Meanwhile, RAN1 made the following agreements:

Agreement - Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported) , - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE - Alt. 2: By indicating Rel-17 TCI state index Agreement For the Rel-17 unified TCI based beam indication in Rel-18 LTM, at least Alt 1 is supported: - Alt 1: TCI state activation of a candidate cell is received before the reception of beam indication of the candidate cell, - Alt 2: TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command - Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability FFS: signalling details for TCI state activation FFS: For Alt 1, whether/how TCI state activation for candidate cell(s) is allowed Note: If scenarios 1 and 3 are to be supported other beam indication/TCI activation timing relationships are not precluded.

Per RAN1 agreements, it would be straightforward to configure TCI states in a ‘common’ list external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration). In this way, a TCI state can be activated (before beam indication/ cell switch) and indicated (upon cell switch) via its index. Therefore, we suggest RAN2 confirm the corresponding assumption made in last meeting.

Proposal 8: RAN2 confirms that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells.

## ASN.1 decoding and validity check

Regarding ASN.1 decoding and validity check for received candidate configurations, the following text is captured in Chair’s note:

- The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

We discussed similar issues for Rel-16 conditional handover (CHO), and we decided to leave it for UE implementation when to perform ASN.1 decoding and validity check for a CHO candidate.

For LTM, our view is as follows

- ASN.1 decoding upon reception of candidate configuration is not a must since candidate information needed before cell switch (L1 measurement RS, TCI states, and RACH configurations for early TA acquisition) is external to the candidate configuration.

- Validity check can then be postponed to LTM execution if UE has concern about unnecessary RRC re-establishment due to an invalid candidate configuration.

The only drawback of late ASN.1 decoding and validity check is a slightly longer interruption after cell switch command. However, we do not pursue 0ms interruption in LTM so this should be acceptable. We suggest that the timing for ASN.1 decoding and validity check be left for UE implementation.

Proposal 9: When to perform ASN.1 decoding and validity check is left for UE implementation.

Then regarding the FFS issues captured in Chair’s note, our views are as follows.

- Whether early validity check triggers early re-establishment: It should be very rare case that a candidate configuration is invalid. Therefore, we suggest that invalid candidate configuration trigger RRC reestablishment, i.e., we do not introduce new signalling for UE to inform network when early validity check fails.

- Possible timing:  It is left for UE implementation

- Subset of cells: UE may perform early decoding and early validity check for all candidate cells (not limited to a subset of cells)

- If need to specify anything or just up to UE impl: Just up to UE implementation

- If other signalling to notify network is needed: No need to notify network whether UE performs early ASN.1 decoding and validity check

We have the following proposals:

Proposal 10: An invalid candidate configuration triggers RRC reestablishment. Do not introduce any new signalling for UE to inform network when early validity check fails.

Proposal 11: UE may perform early decoding and early validity check for all candidate cells (not limited to a subset of cells)

Proposal 12: UE does not notify network whether it performs early ASN.1 decoding and validity check.

## L2 reset indication

For L2 reset indication, we made the following agreement in RAN2#121.

- To determine if to reset L2 or not is based on RRC configuration (e.g., set of cells). FFS if separate for RLC, MAC, PDCP).

The agreement means that the L2 reset indication is not indicated in the cell switch command MAC CE (i.e., not like reestablishRLC or recoverPDCP in RRC-based handover command). Depending on which cell is the serving cell, a pre-configured candidate cell can be intra- or inter-DU. Therefore, there must be a mechanism for UE to decide if L2 reset is needed. We suggest that a DU-like identifier is included in each candidate configuration. Then UE knows that a LTM executions inter-DU if the source and target have different identifiers, and intra-DU otherwise. The maximum number of such identifier needs to be discussed in RAN2.

Proposal 13: A DU-like identifier is included in each candidate configuration. Then UE knows that a LTM executions inter-DU if the source and target have different identifiers, and intra-DU otherwise. FFS maximum number of identifiers.

For simplicity, we may have one identifier applied for all layers. That is, for inter-DU LTM, UE always performs MAC reset, RLC reestablishment and PDCP recovery; for intra-DU LTM, UE always performs MAC partial reset.

Proposal 14: There is a single DU-like identifier, and it applies for MAC, RLC, and PDCP.

# Conclusion

It is proposed to discuss and decide on the following proposals:

L1 measurements

Proposal 1: SSBs in the list (external to serving and candidate cell configurations) are associated with indices identifying the corresponding candidate cells.

Proposal 2: Current serving cell is also a candidate, and the cell index for SSBs from current serving cell needs not to be zero. RAN2 may discuss if the index should start from 0 or 1.

Proposal 3: The candidate cell indices are configured for each CSI-SSB-ResourceSet.

Proposal 4: When serving cell configuration contains a list of SSBs for L1 measurement, UE ignores the serving cell SSBs in the list outside.

Proposal 5: RAN2 keeps the assumption that for L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s) and re-evaluate it once RAN1 has their conclusion.

Proposal 6: Do not introduce L1/L2 signalling to control or change L1 measurement/reporting for LTM.

Proposal 7: LTM-specific L1 measurements of an LTM candidate SCell is independent of its activation status.

TCI states

Proposal 8: RAN2 confirms that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells.

ASN.1 decoding and validity check

Proposal 9: When to perform ASN.1 decoding and validity check is left for UE implementation.

Proposal 10: An invalid candidate configuration triggers RRC reestablishment. Do not introduce any new signalling for UE to inform network when early validity check fails.

Proposal 11: UE may perform early decoding and early validity check for all candidate cells (not limited to a subset of cells)

Proposal 12: UE does not notify network whether it performs early ASN.1 decoding and validity check.

L2 reset indication

Proposal 13: A DU-like identifier is included in each candidate configuration. Then UE knows that a LTM executions inter-DU if the source and target have different identifiers, and intra-DU otherwise. FFS maximum number of identifiers.

Proposal 14: There is a single DU-like identifier, and it applies for MAC, RLC, and PDCP.

# Reference

- RAN2#121bis Chair’s Note

- R2-2304548, [AT121bis-e][019][eMob] L1 Measurements (Qualcomm)	Qualcomm

- R2-2302039, 38.300 running CR for introduction of NR further mobility enhancements, MediaTek, May 2023