3GPP TSG-RAN WG2 Meeting #122                                                       R2-2304894

Incheon, KR, 22nd – 26th May, 2023

Source:	CATT

Title:	Discussion on GNSS operation in connected mode

Agenda Item:	7.6.2.2

Document for:	Discussion and Decision

Introduction

For the GNSS operation in connected mode, some agreements have been achieved during the last RAN1 and RAN2 meeting, and some open issues were left after discussion in RAN2. In this contribution, we give some discussion on the GNSS operation for IoT NTN UE in long connection mode, based on RAN1 and RAN2 agreements, and open issues.

Discussion

Negotiation for long connection mode

In our contribution [1] of last meeting, we have analyzed that, the network need to know whether UE supports the improved GNSS operation, to avoid invalid configuration related with improved GNSS operation, for example, to trigger connected UE perform GNSS operation. And the UE need also to know whether the network supports the improved GNSS operation, to avoid unnecessary report of GNSS position fix time duration for measurement.

During the offline discussion of [AT121][101][IoT NTN enh] GNSS operation (CATT) [2], companies thought that, UE capability may be needed, but can be discussed later. And network indication may be needed, but no network capability should be defined.

So maybe RAN2 can achieve a common understanding that, indications to indicate whether the UE/network supports the improved GNSS operation is needed, leaving the capability definition or implicit/explicit indication open.

Proposal 1: Indications are needed to indicate whether the network/UE supporting the improved GNSS operations.

- FFS whether UE capability is needed

- FFS implicit/explicit indication

Based on the indication above, network supporting the improved GNSS operations can configure an UE supporting the improved GNSS operations to re-acquire GNSS position fix, aperiodically or autonomously (when configured by the network), if the network wants the UE continue to keep RRC_CONNECTED after the current GNSS position becoming out-of-date, according to the following RAN1 agreements:

Agreement For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap - FFS details of gap configuration The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement - FFS based on configured timing

If the network does not configure the UE to re-acquire GNSS position fix, regardless the network does not support the improved GNSS operations, or the network does want the UE to keep RRC_CONNECTEDD after the GNSS position becoming out-of-date, the UE should leave RRC_CONNECTED state after the GNSS position becoming out-of-date, as what the Rel-17 IoT NTN UE does.

Proposal 2: By default, Rel-18 IoT NTN UE (supporting the improved GNSS operations) will leave RRC_CONNECTED state when current GNSS position becoming out-of-date, unless it has been indicated by network to perform GNSS measurement.

We can discuss the case of UE requiring performing GNSS measurement actively, if RAN1 has any agreements on this case.

When to start new GNSS measurement

The following agreement was achieved:

Agreements via email – from offline 104 – second round:

- UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. FFS whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry

As discussed above, network supporting the improved GNSS operations can configure an UE supporting the improved GNSS operations to re-acquire GNSS position fix, aperiodically or autonomously (when configured by the network), if the network wants the UE continue to keep RRC_CONNECTED after the current GNSS position becoming out-of-date.

And for the case of eNB aperiodically triggering connected UE to make GNSS measurement, RAN1 has the following agreements in last meeting:

Agreement For the GNSS measurement gap aperiodically triggered with MAC CE, the duration for the GNSS measurement gap can be configured by eNB. - The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB.

And

Agreement On when the aperiodic GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot - FFS: details of X, e.g. predefined value or configured value, considering HARQ feedback for the MAC CE, etc

So for the case of eNB aperiodically triggering connected UE to make GNSS measurement, the UE will actually start to make GNSS measurement at the start of the determined GNSS measurement gap, which is configured by network or equal to the latest reported GNSS position fix time duration for measurement. And the start of the GNSS measurement gap is determined by the number n of MAC CE receiving subframe/slot and a predefined or configured value X, regardless the expiry of the current GNSS validity duration.

For the case of UE re-acquiring GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement, it is not reasonable for the UE to perform GNSS measurement when the current GNSS is still valid, because the UE can still perform data transmission. So for the case of UE re-acquiring GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement, the UE should start the new GNSS measurement upon the current GNSS validity duration expiry.

Proposal 3: The new GNSS measurement shall be started:

- for the case of eNB aperiodically triggering connected UE to make GNSS measurement, at the start of the GNSS measurement gap

- for the case of UE re-acquiring GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement, upon the current GNSS validity duration expiry

Security issue of MAC CE

RAN1 has achieved agreements that, UE reports the GNSS validity duration via MAC CE, and eNB aperiodically triggers UE to make GNSS measurement via MAC CE. And RAN2 achieved very little agreement after a long and hard discussion:

Agreements online:

- For the NB-IoT CP solution, UE will report the GNSS validity duration by using a MAC CE

We think that, the agreements are from RAN1, and the security issue is determined by SA3, RAN2 cannot change the RAN1 agreements directly, or determine the security issue directly. We prefer to follow the RAN1 agreements as at least working assumption in RAN2, and send LS to RAN1 to check the motivation, and send LS to SA3 to check if they have any security concern on this issue.

Proposal 4: RAN2 has the following working assumption:

- Defined new MAC CE for eNB aperiodically triggering UE to make GNSS measurement

- Defined new MAC CE for GNSS validation duration report

Send LS to RAN1 and SA3 to check the motivation of RAN1 agreements and security concern on the working assumption above.

GNSS validity duration report

For GNSS validity report, RAN2 achieved the following working assumption:

Working Assumption:

- GNSS validity duration UE reported after GNSS measurement is the remaining validity duration

Some companies have concern whether the UE will report the remaining validity duration every time after GNSS measurement. We think that, if the GNSS validation duration is remaining validation duration, it may be changed every time the UE report it after the GNSS measurement, considering the possible reporting delay of the available UL resources. So for simplification, we think the UE should report the validity duration every time the UE completing the GNSS measurement. But as compromise, we also think this can be left to UE implementation to report the GNSS validity duration or not, if the remaining validity duration is not changed compared with the latest reported remaining validity duration.

Proposal 5: RAN2 confirms the following working assumption:

GNSS validity duration UE reported after GNSS measurement is the remaining validity duration

And

Proposal 6: It is UE implementation to report the GNSS validity duration or not, if the remaining validity duration is not changed compared with the latest reported remaining validity duration. No spec impact is expected.

Other issues for GNSS validation duration MAC CE can be further studied. For example, whether LCID or eLCID can be used, the MAC CE format, whether this MAC CE will trigger SR or RACH if there is no uplink grant to carry this MAC CE, etc.

GNSS position fix time duration report in RRC connected state

In last meeting, the following proposal was discussed:

(13/17) Proposal 1: Working Assumption: The UE does not need to report GNSS position fix time duration in RRC connected state with the assumption that this value doesn’t change. This can be revised based on RAN1 further input if necessary.

Whether the GNSS position fix time duration will be changed, e.g., when the surroundings of the UE is changed because of the UE moving, should be evaluated by RAN1. But RAN1 has very little discussion on this issue up to now. We think RAN2 can have a working assumption on this issue, and ask RAN1 if they have concern on the working assumption. Because we are talking about IoT NTN UEs, which may be fixed or have lower mobility.

Proposal 7: RAN2 to have the following working assumption:

The UE will not report GNSS position fix time duration in RRC connected state with the assumption that this value doesn’t change. FFS for the LS to check the RAN1 view on this working assumption.

GNSS measurement during inactive state of C-DRX

RAN2 has agreed to discuss the case of UE autonomously reacquire GNSS during inactive state of C-DRX in last meeting:

Agreements online:

- RAN2 can discuss UE autonomously reacquire GNSS during inactive state of C-DRX based on RAN1’s input in the next RAN2 meeting

The first issue is why the UE wants/needs to autonomously reacquire GNSS during inactive state of C-DRX. As we discussed in section 2.1, the default behavior is leave RRC_CONNECTED state when current GNSS position becoming out-of-date, unless configured by network, or maybe required by UE itself. So some configuration from network or some indication form UE itself may be needed to triggered UE autonomously to reacquire GNSS during inactive state of C-DRX. But this can also be left to UE implementation entirely, because anyway the UE will not perform data transmission during the inactive state of C-DRX, so the UE can trigger GNSS measurement by itself, if the inactive duration of the C-DRX is long than the GNSS position fix time duration.

Another issue is the RRC need to send an indication to GNSS module, to perform GNSS measurement, as GNSS module is independent with 3GPP module. But the indication is not specific for the case of UE autonomously reacquiring GNSS during inactive state of C-DRX, for the case of the network aperiodically triggering connected UE to make GNSS measurement, or for the case of UE re-acquiring GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement, the indication from RRC to GNSS module is also needed.

So we think for UE autonomously reacquiring GNSS during inactive state of C-DRX, no specific enhancement is needed.

Proposal 8: RAN2 will not introduce specific enhancement for UE autonomously reacquiring GNSS during inactive state of C-DRX.

Closed loop time correction

For the closed loop time correction, RAN1 has the following agreement:

Agreement At least for the case when frequency error is within frequency error requirements, study the mechanisms and conditions to allow UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration. - FFS: with legacy closed loop time correction or enhanced closed loop time correction - This mechanism is enabled/configured by eNB - FFS: whether such mechanism will be specified depends on the outcome of this study

If the frequency error is within frequency error requirements, supporting closed loop time correction can reduce GNSS measurement time, which is beneficial for valid data transmission and power saving.  RAN2 should also start the study on this mechanism and give RAN2 outcome to help to determine whether such mechanism can or need to be specified.

Obviously, according to the RAN1 agreement above, an indication or configuration is needed to enable or configure this mechanism by eNB, i.e., to let UE to know it can continue UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration.

Another issue, how to determine the duration during which the UE can continue UL transmission after original GNSS validity duration expires without GNSS re-acquisition. One option is the eNB configure a duration when enable/configure the mechanism, another option may be the UE can continue UL transmission after original GNSS validity duration expires without GNSS re-acquisition, until the eNB gives another indication, e.g, the eNB triggers the UE perform GNSS measurement, or the eNB release RRC connection.

Proposal 9: RAN2 to discuss the mechanism of closed loop time correction at least on the following issues:

- How to enable/configure the mechanism

- How to determine the duration during which the UE can continue UL transmission without GNSS re-acquisition after original GNSS validity duration expires

Other issues:

Some other issues have been under discussion of RAN1, for example:

- Whether the UE will re-acquire UL synchronization after GNSS measurement, considering the GNSS measurement duration can be last 1s-10s depending on the GNSS module state (hot re-start or warm re-start) and the GNSS channel state

- The detail mechanism (e.g. based on configured timing) of UE re-acquiring GNSS autonomously (when configured by network)

- Whether allow the UE to recover RRC connection before the end of the gap when the UE successfully complete the GNSS measurement, and how the UE can confirm the RRC connection has been recovered

RAN2 can just wait the further input from RAN1.

Conclusion

In this contribution, we give some discussion on the GNSS operation for IoT NTN UE in long connection mode, based on RAN1 and RAN2 agreements, and open issues, and the following observations and proposals are given:

Proposal 1: Indications are needed to indicate whether the network/UE supporting the improved GNSS operations.

- FFS whether UE capability is needed

- FFS implicit/explicit indication

Proposal 2: By default, Rel-18 IoT NTN UE (supporting the improved GNSS operations) will leave RRC_CONNECTED state when current GNSS position becoming out-of-date, unless it has been indicated by network to perform GNSS measurement.

Proposal 3: The new GNSS measurement shall be started:

- for the case of eNB aperiodically triggering connected UE to make GNSS measurement, at the start of the GNSS measurement gap

- for the case of UE re-acquiring GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement, upon the current GNSS validity duration expiry

Proposal 4: RAN2 has the following working assumption:

- Defined new MAC CE for eNB aperiodically triggering UE to make GNSS measurement

- Defined new MAC CE for GNSS validation duration report

Send LS to RAN1 and SA3 to check the motivation of RAN1 agreements and security concern on the working assumption above.

Proposal 5: RAN2 confirms the following working assumption:

GNSS validity duration UE reported after GNSS measurement is the remaining validity duration

Proposal 6: It is UE implementation to report the GNSS validity duration or not, if the remaining validity duration is not changed compared with the latest reported remaining validity duration. No spec impact is expected.

Proposal 7: RAN2 to have the following working assumption:

The UE will not report GNSS position fix time duration in RRC connected state with the assumption that this value doesn’t change. FFS for the LS to check the RAN1 view on this working assumption.

Proposal 8: RAN2 will not introduce specific enhancement for UE autonomously reacquiring GNSS during inactive state of C-DRX.

Proposal 9: RAN2 to discuss the mechanism of closed loop time correction at least on the following issues:

- How to enable/configure the mechanism

- How to determine the duration during which the UE can continue UL transmission without GNSS re-acquisition after original GNSS validity duration expires

Reference

- R2-2302558 Discussion on GNSS operation in connected mode, RAN2# 121bis meeting

- R2-2301951 Report of AT121101IoT NTN enh GNSS operation (CATT), RAN2# 121 meeting