3GPP TSG-RAN WG2 Meeting #122		R2-2304898

Incheon, KR, May 22 – 26, 2023

Source:	CATT, IPLOOK

Title:	Discussion on the cell reselection enhancement for earth-moving cell

Agenda Item:	7.7.4.1.2

Document for:	Discussion and Decision

Introduction

In RAN2#121bis-e meeting, the NTN-NTN enhancements were discussed with some leftover issues. In this contribution, the following issues will be further discussed, including:

- Format of reference location

- Time/location-based measurement initiation

- Whether to support location-based cell reselection criteria

Discussion

  - Format of reference location

In RAN2#121bis-e meeting, it was agreed that, in earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. It can be seen that, new IE will be introduced for the reference location of serving cell, and the distanceThresh in SIB19 will be reused. How to define the new IE for reference location of serving cell needs to be further studied.

- RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (FFS whether a new epochTime IE is needed). RAN2 understanding is that both PVT and orbital parameters can be used for this. FFS if additional information is needed to allow more accurate measurements.

- For earth-moving cell, new IE is introduced to indicate the reference location of serving cell.

- For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19.

Figure 1: One method for calculating the timely reference location of the serving cell

One option (option1) of defining the reference location of serving cell is to give the position of the absolute reference location of serving cell, i.e. use the definition of the R17 ReferenceLocation-r17. According to [1], the method for UE to calculate the timely reference location of the serving cell is shown in Figure 1. However, the steps of this method are redundant. The objective for the network to provide the serving cell’s reference location at epochTime is used for UE to calculate the relative position between the serving cell’s reference location and the sub-satellite point. Hence, the first step and the second step can be simplified as the network provides the relative position between the serving cell’s reference location and the sub-satellite point directly.

Hence another option (option 2) to define the reference location of the serving cell is raised. That is, the new IE introduced for the reference location of the serving cell is defined as a relative position between the serving cell’s reference location and the sub-satellite point.

Comparing with the option 1, the step 1 and step 2 is omitted. UE could get the timely reference location of the serving cell directly by the timely position of the sub-satellite point and the relative position between the serving cell’s reference location and the sub-satellite point. Furthermore, considering the overhead, for option 1, the reference location is defined as R17 ReferenceLocation-r17, the overhead is 48 bits, for option 2, the value range of the relative position is less than the absolute position defined as R17 ReferenceLocation-r17, the signaling overhead of the option 2 is less than option 1.

Observation 1: Defining the serving cell’s reference location as the relative position between the serving cell’s reference location and the sub-satellite point has lower signaling overhead and this method has simpler UE behavior.

Proposal 1: New IE introduced for the reference location of serving cell is defined as a relative position between the serving cell’s reference location and the sub-satellite point.

There is another FFS whether a new epochTime IE is needed. Considering the NW can update the reference location of serving cell together with the update of ephemeris and TA common related information, if needed. The existing epochTime can be reused.

Proposal 2: A new epochTime IE for the earth-moving serving cell’s reference location is not needed.

  - Location/time-based measurement initiation

In RAN2#121-bis-e meeting, the enhancements on measurement initiation for earth moving cell were discussed. It was agreed that, location-based measurement initiation is introduced for earth moving cell, which is mainly used for serving link switch case. Time-based measurement initiation is introduced for feeder link switch case.

- For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19.

- For cell (re)selection in earth-moving system, time-based measurement initiation is used to address feeder-link switch case.

- Time-based cell reselection criteria is not pursued in R18.

In Rel-17, the time-based initiation solution and location-based initiation solution were introduced for quasi-earth-fixed cell.

Based on TS 38.304 [2], the location-based solution introduced for quasi-earth fixed cell is defined as UE will keep measuring higher priority frequency and may choose not perform neighbor cell measurements of equal or low priority frequency if the conditions of distance threshold and RSRP/RSRQ threshold are fulfilled.

For earth moving cell, with knowing the real-time reference location of serving cell, the location-based mechanism made for quasi-earth fixed cell is also applicable.

Proposal 3: For earth-moving cell, reuse the Rel-17 location-based measurement initiation solution for RRC_IDLE and RRC_INACTIVE UE:

- UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell real-time reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

- UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the distance between UE and serving cell real-time reference location.

Based on the existing protocol TS 38.304 [2], the time-based solution introduced for quasi-earth fixed cell is defined as UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service and UE shall always perform higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133.

For earth moving cell, the existing t-Service parameter can be reused for feeder link switch time. With knowing the time point, the time-based mechanism made for quasi-earth fixed cell is also applicable for earth moving cell.

Proposal 4: For earth-moving cell, the parameter t-Service is reused to address feeder-link switch case.

Proposal 5: For earth-moving cell, reuse the Rel-17 time-based measurement initiation solution for RRC_IDLE and RRC_INACTIVE UE:

- UE should start measurements on neighbour cells before the serving cell stops covering the current area, regardless of (the distance between UE and serving cell real-time reference location) or (if legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

- UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the remaining serving time.

  - Whether to support location-based cell reselection criteria

In R17, the location-based cell reselection criteria are discussed and the following agreement was achieved.

RAN2#118e Agreements:

- Location based cell reselection method is not introduced in NR NTN Rel-17

This possible enhancement was proposed by company again in Rel-18. However, it is not clear whether to take the location information into considering will bring obvious benefits. Reselect to a cell with longer serving time doesn’t mean the whole reselection times during a long time can be reduced. Under a certain constellation, the cells which UE will pass by is predictable, camp on a cell with a longer time means camp on another cell with shorter time. The total times for UE to perform cell reselection is not reduced. Additionally, it is very likely that UE can only detect the next upcoming cell.

What’s more, to support this enhancement, some assistance information of neighbour cells need to be provided to UE, which will bring additional signalling overhead. And the calculation consumption of UE is also raised. Meanwhile, if the cell reselection takes both the location information and the radio link quality into consideration, the method for UE to determine the suitable cell will be complex.

Based on these aspects, we prefer not to support location-based cell reselection criteria for NTN cell.

Proposal 6: Location-based cell reselection criterion is not supported.

- Conclusion

In this document, we discuss the enhancement on cell reselection. According to the analysis in section 2, the following observation and proposals are provided:

Format of reference location

Observation 1: Defining the serving cell’s reference location as the relative position between the serving cell’s reference location and the sub-satellite point has lower signaling overhead and this method has simpler UE behavior.

Proposal 1: New IE introduced for the reference location of serving cell is defined as a relative position between the serving cell’s reference location and the sub-satellite point.

Proposal 2: A new epochTime IE for the earth-moving serving cell’s reference location is not needed.

Location/time-based measurement initiation

Proposal 3: For earth-moving cell, reuse the Rel-17 location-based measurement initiation solution for RRC_IDLE and RRC_INACTIVE UE:

- UE may choose not to perform neighbour cell measurements of “NR intra-freq or inter-freq with equal or lower priority, or inter-RAT freq with lower priority”, if (the distance between UE and serving cell real-time reference location is shorter than a threshold) and (legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

- UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the distance between UE and serving cell real-time reference location.

Proposal 4: For earth-moving cell, the parameter t-Service is reused to address feeder-link switch case.

Proposal 5: For earth-moving cell, reuse the Rel-17 time-based measurement initiation solution for RRC_IDLE and RRC_INACTIVE UE:

- UE should start measurements on neighbour cells before the serving cell stops covering the current area, regardless of (the distance between UE and serving cell real-time reference location) or (if legacy Srxlev/Squal condition is met, i.e., serving cell’s Srxlev/Squal is better than a threshold).

- UE shall perform neighbour cell measurements of “higher priority NR inter-frequency or inter-RAT frequencies” regardless of the remaining serving time.

Whether to support location-based cell reselection criteria

Proposal 6: Location-based cell reselection criterion is not supported.

- Reference

- R2-2303140 Report of [Post121][106][NR NTN enh] NTN-NTN cell reselection (ZTE).

- 3GPP TS 38.304 User Equipment (UE) procedures in Idle mode and RRC Inactive state.