3GPP TSG-RAN WG2 Meeting #122		R2-2304904

Incheon, KR, May 22nd – May 26th 2023

Source:	CATT

Title:	Discussion on UP issues for MSG1 repetition

Agenda Item:	7.21.3

Document for:	Discussion and Decision

Introduction

In RAN2#121bis-e meeting, preliminary agreements on MSG1 repetition were agreed [1]. In this document, we continue to discuss the UP issues for MSG1 repetition.

Discussion

2.1	CBRA/CFRA

MSG3-based on-demand SI

In RAN2#112bis-e meeting, it was agreed that:

- RAN2 assumes that MSG1 repetition can be applicable to all 4-step CBRA procedures (FFS for SI request)

So, it can be seen that it is still FFS for whether MSG1 repetition can be applicable to SI request in 4-step CBRA.

Regarding procedure for Msg3 based SI request, the UE uses the common PRACH resources to initiate RACH procedure. After receiving UL grant in RAR, the UE sends RRCSystemInfoRequest to the network requiring the intended SI. From the UE’s perspective, the PRACH repetition increases the successful rate of MSG1 reception, which helps the UE to acquire the intended SIB. Hence, it is proposed that:

Proposal 1: MSG1 repetition can be applicable to Msg3 based on-demand SI.

CFRA

The typical scenarios for CFRA including, for example, reconfiguration with sync, downlink data arrival without UL synchronization, SCell addition, BFR and etc. While, downlink data arrival without UL synchronization and SCell addition are triggered by PDCCH order which involves DCI design. Hence, we can leave this to RAN1.

Proposal 2: MSG1 repetition in CFRA which is triggered by PDCCH order is pending on RAN1 discussion.

But for reconfiguration with sync, BFR as well as MSG1 based on-demand SI, the PRACH resource is sent to the UE by RRC signalling, we can firstly discuss these 3 cases.

- Reconfiguration with sync

For reconfiguration with sync, the PRACH resource is included in HO command. Since the target cell is aware of the channel condition, it can assign the proper MSG1 repetition number for the UE. If MSG1 repetition is supported in HO scenario, the PRACH repetition number can also be included together with PRACH resource.

- BFR

And for BFR, the PRACH resource which may be configured in BeamFailureRecoveryConfig is also delivered in RRC signaling, before the occurrence of BFR. And the PRACH resources are sent to the UE individually. If MSG1 repetition is supported in BFR, this greatly aggravates the issue of PRACH resource fraction. Hence, it can be studied with low priority.

- Msg1 based on-demand SI

For Msg1 based on-demand SI, the network configures the PRACH resource in si-RequestConfig which is in SIB1. Hence, different from Reconfiguration with sync in which the PRACH repetition number can be configured based on the channel condition of the UE, for Msg1 based on-demand SI, the UE needs to evaluate the channel like CBRA. Hence, if Msg1 based on-demand SI is supported, separate PRACH resources need to be reserved apart from legacy PRACH resources for Msg1 based on-demand SI.

One of the benefits supporting MSG1 repetition for these scenarios is that MSG1 repetition can improve the coverage of MSG1. But on the other hand, it also aggravates issue of the PRACH resources fraction. For the latter issue, from the network point of view, it may configure only a couple of features for MSG1 repetition. So, the PRACH resources fraction issue can be alleviated based on network implementation. Further investigation is needed.

Hence, we propose

Proposal 3: Support MSG1 repetition for Reconfiguration with sync and FFS for BFR and MSG1 based on-demand SI.

2.2 SUL/NUL

In RAN2#121bis-e meeting, it was agreed that:

Agreements - RAN2 assumes that MSG1 repetition can be applicable to NUL - RAN2 assumes that MSG1 repetition can be applicable to SUL

Based on the current specification, if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL, the UE selects the SUL carrier for performing Random Access procedure. Otherwise, the UE selects the NUL carrier. If MSG1 repetition is supported to both NUL and SUL, whether to define one separate RSRP threshold for MSG1 repetition needs to be studied.

If we define one new rsrp-ThresholdSSB-SUL for MSG1 repetition (the rsrp-ThresholdSSB-SUL-Msg1Repetition in the following figure), the value is probably lower than the legacy rsrp-ThresholdSSB-SUL. On the one hand, the UEs served by SUL (the UEs in the grey circle) are offloaded to NUL, which definitely aggravates the issue of resource congestion of NUL. On the other hand, since MSG1 repetition is performed on NUL, it indicates that the channel condition is poor. Hence, it is high probability that repetition for PUSCH/PUCCH is needed to avoid incorrect decoding. So this also brings repetition for PUSCH/PUCCH. Otherwise, imbalance for repetition number may happen for MSG1 and PUSCH/PUCCH. This should be avoided for MSG1 repetition.

Figure 1: SUL/NUL selection for MSG1 repetition

Furthermore, the RACH procedure defined for RACH partitioning in Rel-17 is that the UE firstly selects carrier before feature selection. Hence, if one separate RSRP threshold is defined for MSG1 repetition will change the common procedure for RACH partitioning. In order to simplify the specification work, the procedure should not be modified.

Based on the reason analysed above, it is proposed that:

Proposal 4: No separate RSRP threshold for NUL/SUL selection is defined for MSG1 repetition.

2.3 RSRP threshold for MSG1 repetition

For MSG1 repetition, the UE needs to decide when to initiate MSG1 repetition. In RAN1#111 meeting, it has been agreed that [2]:

Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

It can be seen that at least SSB-RSRP threshold(s) used to determine the number of PRACH transmission with the same Tx beam is introduced for PRACH CE.

And it was also agreed in RAN1#112 meeting that [3]:

Agreement For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. - If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported.

Agreement Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams.

Based on the RAN1 agreement, it can be concluded that the number of multiple PRACH transmissions with same TX beams can be 2, 4 and 8. So, the SSB-RSRP threshold(s) can be different values for the UE to distinguish which repetition number is used for multiple PRACH transmissions.

Some companies may argue that this may impact the fall-back procedure, e.g. fall-back from MSG1 repetition with lower number to MSG1 repetition with higher number. However, the RSRP can be used to evaluate MSG1 repetition number at initiation procedure for CFRA and 4-step CBRA. Once the UE has selects RACH procedure with MSG1 repetition, the UE can follow the fall-back procedure from MSG1 repetition with lower number to MSG1 repetition with higher number. Hence, we see no issues here. Therefore, we propose that:

Proposal 5: Define different RSRP thresholds for different MSG1 repetition number.

2.4 RA-RNTI

In RAN2#121bis-e meeting, it was agreed that:

Agreements - RAN2 waits for further inputs from RAN1 for how to associate RA-RNTI to the PRACH occasion for multiple PRACH transmissions and also for ra-ResponseWindow start point

And in RAN1#112bis-e meeting [4], it was agreed that:

Agreement The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213. Note: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not

Based on the agreements made in RAN1, RAR window is started after the last symbol of the last valid RO in the RO group corresponding to the MSG1 repetition.

For RA-RNTI, it is associated with the PRACH occasion in which the Random Access Preamble is transmitted. Per RAN1 agreements, we think the RA-RNTI can be computed using the last valid RO in the RO group corresponding to the MSG1 repetition which is similar to the mechanism in LTE.

Hence, it is proposed that:

Proposal 6: RA-RNTI is associated with the last valid RO in the RO group corresponding to the MSG1 repetition.

2.5 Fallback procedures

It was agreed in RAN2#121bis-e meeting that:

- RAN2 will not support the fallback from legacy RA to Msg1 repetition and vice versa; Other fall back scenarios are FFS

In the following, we analyse the other fall back scenarios.

Fall-back from MSG1 repetition with lower number to MSG1 repetition with higher number

In LTE [5]-[6], maxNumPreambleAttemptCE is defined as the maximum number of preamble transmission attempts per CE level. When the preamble transmission counter per CE, i.e. PREAMBLE_TRANSMISSION_COUNTER_CE equals to maxNumPreambleAttemptCE + 1, the UE will initiate RACH procedure in the next enhanced coverage level if supported. In NR, we think we can follow the same mechanism in LTE.

- if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage: - increment PREAMBLE_TRANSMISSION_COUNTER_CE by 1; - if PREAMBLE_TRANSMISSION_COUNTER_CE = maxNumPreambleAttemptCE for the corresponding enhanced coverage level + 1: - reset PREAMBLE_TRANSMISSION_COUNTER_CE; - consider to be in the next enhanced coverage level, if it is supported by the Serving Cell and the UE, otherwise stay in the current enhanced coverage level;

Proposal 7: RAN2 to support the fall-back from MSG1 repetition with lower number to MSG1 repetition with higher number.

If proposal 7 is supported, one parameter indicating the maximum attempt number of the preamble transmission per MSG1 repetition number needs to be defined.

Proposal 8: Define one parameter to indicate the maximum attempt number of the preamble transmission per MSG1 repetition number.

Fall-back from 2-step RA to 4-step RA with Msg1 repetition

Normally, when the UE selects 2-step RA, it implies the the RSRP of the downlink pathloss reference is above msgA-RSRP-Threshold when the BWP selected for Random Access procedure is configured with both 2-step and 4-step RA type. Hence, when the UE fails for 2-step RA, it may due to the terrible channel condition, uplink synchronization lost or preamble collision.

Reevaluating the channel condition to figure out which MSG1 repetition number is selected is helpful. But this does not align with current MAC procedure. For example, the UE has to check the availability of the set of RA resources, when the UE fallback to 4-step RACH which is not supported by current spec.

Hence, it is proposed the check the feasibility and specification impacts for fallback from 2-step RA to 4-step RA with Msg1 repetition.

Proposal 9: RAN2 to check the specification impacts for fall-back from 2-step RA to 4-step RA with Msg1 repetition.

Fall-back from CFRA to CBRA with Msg1 repetition

In current RACH procedure, when the UE fails in one CRRA attempt, the UE will revaluate the condition for RACH. But the UE does not check the feature during this procedure. If fall-back from CFRA to CBRA with Msg1 repetition is supported, this will bring procedure impacts. Hence, it is proposed that:

Proposal 10: RAN2 to check the specification impacts for fall-back from CFRA to CBRA with Msg1 repetition.

2.6 Association with Msg3 repetition

In Rel-17, Msg3 repetition was agreed. And one new RSRP threshold, i.e. rsrp-ThresholdMsg3 was introduced to determine whether Msg3 repetition can be initiated by the UE. Besides, in order to notify the network that the UE supports Msg3 repetition, PRACH resources for Msg3 repetition can be reserved in SIB. When the RSRP is below rsrp-ThresholdMsg3 and PRACH resources are configured for Msg3 repetition, the UE may select PRACH resources corresponding to Msg3 repetition and initiate corresponding RACH procedure.

Normally, the UE notifies the network that it supports Msg3 repetition when the UE selects the PRACH resource configured for Msg3 repetition. But there is ambiguity whether the UE can indicates the network that it supports both MSG1 repetition as well as MSG3 repetition when it selects PRACH resources for MSG1 repetition.

One of the drawbacks regarding the above solution is that the network can’t figure out the separate capability/request for MSG1 repetition and MSG3 repetition. And it makes mandatory for the UE to support MSG3 repetition when it supports MSG1 repetition.

On other hand, if separate PRACH resources are configured for each combination, i.e. Msg3 repetition, multiple PRACH transmissions, and Msg3 together multiple PRACH transmissions, this brings PRACH resource fractions. Besides, considering the MSG3 which transmitted on PUSCH which has higher requirements on channel condition, it is likely the condition of MSG3 repetition has been satisfied when MSG1 repetition is triggered. This means the Msg3 repetition should be applied when MSG1 repetition condition has been satisfied.

Hence, we slightly prefer that UE implicitly indicates it supports/requests Msg3 repetition when it selects PRACH resources for multiple PRACH transmissions.

Proposal 11: The UE implicitly indicates it supports/requests Msg3 repetition when it selects PRACH resources for multiple PRACH transmissions.

Another issue is whether repetition number for Msg3 is associated with the repetition number for PRACH. For Msg3 repetition, the repetition is signalled in RAR UL grant, which has been specified in Rel-17. And multiple PRACH transmissions number can be determined by UE based on RSRP. It is not flexible to associate repetition number for Msg3 to the repetition number for PRACH.

Considering analysis above, it is proposed that:

Proposal 12: Msg3 repetition number is not associated with the repetition number of MSG1, i.e., Msg3 repetition number is indicated in RAR like Rel-17 when both MSG1 repetition and Msg3 repetition are triggered.

Conclusion

In this document, we analyse issues for PRACH coverage enhancement, and we find the observations and proposls as following:

Proposal 1: MSG1 repetition can be applicable to Msg3 based on-demand SI.

Proposal 2: MSG1 repetition in CFRA which is triggered by PDCCH order is pending on RAN1 discussion.

Proposal 3: Support MSG1 repetition for Reconfiguration with sync and FFS for BFR and MSG1 based on-demand SI.

Proposal 4: No separate RSRP threshold for NUL/SUL selection is defined for MSG1 repetition.

Proposal 5: Define different RSRP thresholds for different MSG1 repetition number.

Proposal 6: RA-RNTI is associated with the last valid RO in the RO group corresponding to the MSG1 repetition.

Proposal 7: RAN2 to support the fallback from MSG1 repetition with lower number to MSG1 repetition with higher number.

Proposal 8: Define one parameter to indicate the maximum attempt number of the preamble transmission per MSG1 repetition number.

Proposal 9: RAN2 to check the specification impacts for fall-back from 2-step RA to 4-step RA with Msg1 repetition.

Proposal 10: RAN2 to check the specification impacts for fall-back from CFRA to CBRA with Msg1 repetition.

Proposal 11: The UE implicitly indicates it supports/requests Msg3 repetition when it selects PRACH resources for multiple PRACH transmissions.

Proposal 12: Msg3 repetition number is not associated with the repetition number of MSG1, i.e., Msg3 repetition number is indicated in RAR like Rel-17 when both MSG1 repetition and Msg3 repetition are triggered.

Reference

- RAN2#121bis-e meeting report;

- R1-2300001, Final Report of 3GPP TSG RAN WG1 #111 v1.0.0;

- Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0

- RAN1#112bis-e meeting report;

- TS 36.321 V17.4.0, Medium Access Control (MAC) protocol specification;

- TS 36.331, V17.4.0, Radio Resource Control (RRC) protocol specification;