3GPP TSG-RAN WG2 Meeting #122		R2-2304906

Incheon, Korea, 22nd May – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1616 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SDT with sparate initial BWP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Huawei, HiSilicon, Guangdong Genius |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core, NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For CG-SDT performed in RedCap, PDCCH monitoring and UL transmission on RedCap specific initial BWP is not clear according to the current specification. It should be clarified that CG-SDT and corresponding PDCCH monitoring should be performed on RedCap specific initial BWP, if configured. - For RA-SDT performed in RedCap, in case there is SDT configuration on legacy initial BWP, while there is no SDT configuration on the RedCap specific initial BWP, RA-SDT would be initiated according to the current specification in section 5.27.1. But RA-SDT cann’t be performed as there is no RA-SDT configuration on separate initial BWP. Thus, SDT type selection should be performed based on the RACH resource on RedCap specific initial BWP, if configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that CG-SDT for RedCap UE should be performed on RedCap specific initial UL BWP, if configured. - Clairify that RA-SDT type selection should be performed based on the RA resource configured on RedCap specific initial BWP, if configured. - Clarify that PDCCH monitoring for CG-SDT for RedCap UE should be performed on RedCap specific initial DL BWP, if configured. Impact analysis Impacted 5G architecture options: SA Impacted functionality SDT for RedCap Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap UE may initiate SDT procedure on legacy initial BWP when RedCap specific initial BWP is configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.27	Small Data Transmission

5.27.1	General

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer. The SDT procedure can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold; or

1>	if sdt-RSRP-Threshold is not configured:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if, for each RB having data available for transmission, configuredGrantType1Allowed, if configured, is configured with value true for the corresponding logical channel; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for a RedCap UE; otherwise, on the BWP configured by initialUplinkBWP, according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for a RedCap UE; otherwise, on the BWP configured by initialUplinkBWP:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If RA-SDT is selected above and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the RA-SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI, on the BWP configured by initialDownlinkBWP-RedCap, if configured for a RedCap UE; otherwise, on the BWP configured by initialDownlinkBWP, as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the CG-SDT procedure is terminated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

End of change