3GPP TSG-RAN WG2 Meeting #122		R2-2304911

Incheon, Korea, 22nd May – 26th May 2023

Source:	vivo

Title:	RRC configuration for LTM

Agenda Item:	7.4.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#121bis-e meeting [1], the candidate cell configuration for LTM and the reference configuration were discussed. And the following agreements were made.

Discuss terminology for the TS in the RRC stage-3 discussions when/if needed (not at current meeting). Whether the Reference configuration is a complete configuration or not is up to the network implementation. Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration. The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current). Confirm that only the replacement procedure (the “full config without L2 reset”) is supported for Execution of LTM cell switch. The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed. Initial agreements, from RAN2 point of view (may be dep on RAN1 progress). The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS. RAN2 assumes that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration). RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s). RAN2 assumes that whether filtering, hysteresis, and time-to-trigger are needed for LTM specific L1 measurements is up to RAN1. FFS if the LTM specific L1 measurements of an LTM candidate SCell is independent of its activation status. Whether to assume L1/L2 signaling to control or change L1 measurement/reporting for LTM needs further discussion (parts may be discussed in RAN1). RAN2 assumes that such control would be limited to certain aspect that need frequent update and restricted by RRC configuration.

In this paper, we provide our views on the configuration for LTM including the L1 measurement and report configuration, reference configuration and other issues related to RRC configuration.

- Discussion

  - L1 measurement and report

In the last meeting, the following agreements were made for L1 measurement and report.

The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS.

RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s).

Since the L1 measurement configuration and the report configuration for candidate cells are not in the same place, the report configuration under the ServingCellConfig of the serving cell(s) should refer to the L1 measurement configuration of the candidate cells in the separate list. In this way, the L1 measurement results of the candidate cells can be reported in the serving cell.

Proposal 1: To support L1 measurement report of candidate cells, the report configuration of current serving cell(s) should refer to the L1 measurement configuration of the candidate cells in the separate list.

As discussed in the last meeting, if the report configuration is only included in the current serving cell(s), then subsequent LTM cannot be supported without RRC reconfiguration because the network will need to reconfigure the L1 measurement report when UE handover to a new serving cell. To support subsequent LTM, we think the report configuration for LTM candidate cells should be included in the ServingCellConfig  of every candidate cell, not only the current serving cell(s). In this way, the UE can report the L1 measurement results of other candidate cells in the target cell after LTM without RRC reconfiguration. The report configuration in the candidate cells should also refer to the L1 measurement configuration of other candidate cells in the separate list, just the same as in the current serving cell(s).

We show the RRC configuration structure for L1 measurement and report in Figure 1 to clearly demonstrate the configuration method. In Figure 1, we assume that two LTM candidate cells are configured to the UE, one is the candidate cell 1 and the other is candidate cell 2(current serving cell).

Figure 1. Structure of L1 measurement and report for supporting subsequent LTM

Proposal 2: To support L1 measurement report for subsequent LTM, the report configuration for LTM candidate cells should be included in the ServingCellConfig of each candidate cell.

Proposal 3: The report configuration in the ServingCellConfig of candidate cells should refer to the L1 measurement configuration of other candidate cells in the separate list.

To support the L1 measurement and report for LTM discussed above, it is necessary to perform L1 measurement configuration exchanging between different DU(s) in the preparation phase for inter-DU LTM. For example, the S-DU should know the L1 measurement configuration of candidate cells in the candidate DU(s) so that the serving cell can generate the corresponding report configuration for the L1 measurement of candidate cells. Similarly, each candidate cell should also know the L1 measurement configuration of other candidate cells to generate the corresponding report configuration for LTM.

The procedure for source and candidate cell(s) to generate the L1 measurement configuration/ServingCellConfig is shown in Figure 2 below.

Figure 2. Structure of L1 measurement and report for supporting subsequent LTM

Proposal 4: For one UE configured with multiple LTM candidate cells, each candidate cell needs to be informed about L1 measurement configuration of serving cell and/or other candidate cells, to generate the corresponding report configuration for LTM.

Proposal 5: If P4 is agreed, send an LS to inform RAN3 about the agreement.

  - Reference candidate configuration

Reference cell configuration

In the previous meeting, reference configuration is agreed to be introduced for LTM to reduce the signaling overhead. In our view, there are mainly three options for providing the reference CG configuration.

- Option 1: One reference CG configuration that contains a PCell and multiple SCell

- Option 2: One reference CG configuration containing one reference PCell configuration and one reference SCell configuration

- Option 3: One reference CG configuration containing only one reference PCell configuration

With Option 1, the reference configuration is provided as the whole CG. For example, if the candidate LTM cell group has more SCells than the reference CG configuration, the extra SCells in the candidate LTM cell group must be configured with complete cell configuration explicitly. Figure3 illustrates the scenario how Option 1 works.

Figure 3. Reference configuration for option 1

With Option 2, one reference PCell configuration and one reference SCell configuration are provided in the reference CG configuration. In this way, delta configuration of candidate PCell will be provided based on the configuration of the reference PCell, and delta configuration of each candidate SCell will be provided based on the configuration of the reference SCell. Figure 4 illustrates how Option 2 reduces the signaling overhead for candidate CG configuration.

Figure 4. Reference configuration for option 2

With Option 3, only one reference PCell configuration is provided in the reference CG configuration. In this way, both PCell and SCell(s) in the candidate CG will reference to the reference PCell configuration. Compared with Option 2, the size of reference CG configuration in Option 3 is smaller . However, PCell configuration and SCell configuration may have significant differences, e.g. PUCCH configuration may not exist in the SCell configuration. Therefore, Option 3 may result in more signaling overhead than Option 2. Figure 5 illustrates how option 3 works.

Figure 5. Reference configuration for option 3

From the analysis of the 3 Options, we observe that Option 2 is the most flexible method for providing reference configuration and brings the least signaling overhead for LTM candidate configurations. We therefore propose to include one reference PCell configuration and one reference SCell configuration in the reference CG configuration.

Proposal 6: The reference CG configuration for LTM contains one reference PCell configuration and one reference SCell configuration. Delta configuration of candidate PCell is provided based on the configuration of the reference PCell, and delta configuration of each candidate SCell associated with the candidate PCell is provided based on the configuration of the reference SCell.

Complete configuration work procedure

When reference configuration is provided and the candidate CG configuration is delta configuration, UE will apply the candidate delta configuration on top of the reference configuration to form a complete candidate configuration before the LTM cell switch. However, the network may provide the UE with a complete configuration instead of a delta configuration when there is a large difference between the candidate CG configuration and the reference configuration. We prefer to provide enough flexibility for the network to provide the candidate CG configuration. Therefore, a CompleteConfig flag should be introduced. For a candidate CG configuration includes the CompleteConfig IE, UE should consider the candidate CG configuration is a complete configuration which can be applied without combining with the reference configuration.

Proposal 7: Introduce CompleteConfig flag in LTM candidate CG configuration. If the flag is not present,  UE considers the candidate CG configuration as delta configuration on top of the reference configuration; otherwise, UE considers the candidate CG configuration as a complete configuration which can be applied without combining with the reference configuration.

  - RRC configured L2 partial reset

In RAN2#121, it was agreed that whether to reset L2 or not is based on RRC configuration:

- To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

For subsequent LTM, a candidate cell may be in the same DU with the serving cell at one time, and the candidate cell maybe in a different DU with the serving cell at another time. Therefore, a single candidate configuration should support both intra-DU LTM and inter-DU LTM scenarios.

To support subsequent LTM without reconfiguration, several cell sets, which are configured external to the configurations of serving and candidate cells, can be provided to the UE. Each cell set includes identities of cells that belongs to the same DU and UE determines whether to perform L2 partial reset based on whether the source and target cell belongs to the same cell set.

Proposal 8: Several cell sets are pre-configured to UE and the cell sets are configured external to the configurations of serving and candidate cells.

Proposal 9: Each cell set includes a list of cell identities and UE determines whether to perform L2 reset based on whether the source and target cell belongs to the same cell set.

The L2 operation for source cell and target cell  in the same cell set or different cell set during LTM may work as below:

- For MAC: If the source PCell and target PCell is in the same cell set, UE performs partial MAC reset. Otherwise, UE performs MAC reset.

- For RLC: If the source PCell and target PCell is in the same cell set, UE performs RLC reestablishment for SRB and not performs RLC reestablishment for DRB. Otherwise, UE performs RLC reestablishment for SRB and DRB.

- For PDCP

  - For SRB: UE always performs PDCP discard.

For DRB: If the source PCell and target PCell is in the same cell set, UE does not perform PDCP data recovery. Otherwise, PDCP data recovery is performed according to the existing IE recoverPDCP for DRB.

Proposal 10：When performing LTM, if the source cell and target cell is in the same cell set:

- For MAC: UE performs partial MAC reset.

- For RLC: UE performs RLC reestablishment for SRB and not performs RLC reestablishment for DRB.

- For PDCP

  - For SRB: UE always performs PDCP discard.

  - For DRB: UE does not perform PDCP data recovery.

Proposal 11：When performing LTM, if the source cell and target cell is in the different cell set:

- For MAC: UE performs MAC reset.

- For RLC: UE performs RLC reestablishment

- For PDCP

  - For SRB: UE performs PDCP discard.

  - For DRB: UE determines whether to perform PDCP data recovery according to the existing IE recoverPDCP for the DRB

  - BWP determination and SCell activation/deactivation

BWP determination

In RAN2#119bis-e meeting, we have a remaining issue on BWP determination after the LTM as follows:

- FFS how the UE determines the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility.

In legacy, there are two parameters firstActivateDownlinkBWP-Id and firstActivateUplinkBWP-Id configured within ServingCellConfig to indicate the activated BWPs upon the SpCell change or SCell activation, and this field is mandatory present for a SpCell upon reconfiguration with reconfigurationWithSync.

Since the firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id are already included in ServingCellConfig, and the LTM candidate configuration could include the ServingCellConfig, that is to say, the two IEs, i.e. firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id is also mandatorily included in the pre-configured candidate configuration.

RAN1 has discussed to include the active DL and UL BWPs for the target cell in the LTM MAC CE. However, if the first active BWP is selected by source cell, the CFRA resources in the target cell used for LTM needs to be reserved for multiple potential first active BWPs, which brings much waste of resources. Considering the legacy configuration could be re-used directly without extra effort, further optimization is not needed in our understanding.

Proposal 12: UE determines the BWPs (for DL and UL) to be activated upon the execution of LTM based on the firstActivateDownlinkBWP-Id and firstActivateUplinkBWP-Id within the configuration of target cell(s).

SCell activation/deactivation

There is also an existing IE, i.e. the sCellState IE in the current ScellConfig of CellGroupConfig to indicate whether the SCell shall be considered to be in activated state upon SCell configuration. Since the CellGroupConfig is mandatorily configurated in candidate configuration, the sCellState IE could also be included in candidate configuration.

Similarly, UE could perform SCell activation/deactivation based on this IE when UE switches to this cell. Hence, we propose:

Proposal 13: Upon the reception of LTM cell switch command, UE performs target SCell activation/deactivation based on the indication (i.e. sCellState field) within the pre-configured RRC configuration of target SCells.

  - Maintenance of candidate configurations

In RAN2#119bis-e meeting, we have agreed that the candidate cell configuration can only be modified/released by network. However, it is FFS whether other optimization should be applied for release.

- RAN2 assumes that candidate cell configuration can only be modified/released by Network (FFS later whether some optimization should be applied e.g. for release).

In our understanding, the potential optimization here may include UE releasing the candidate cell configuration when a timer expired or when UE moves out of a valid area pre-configured.

It seems reasonable that UE could release the invalid candidate configuration, however, additional enhancements have to be introduced in support of the UE triggered releasing considering UE and network must be consistent on the releasing of candidate cell configuration. For example, the network needs to configure the timer or valid area for UE, however, it is different for the network to determine an exact timer value since it is related to the motion track and velocity of the UE. Besides, configurations for different candidate cells may have different valid areas, hence introducing optimization, e.g. timer or valid area, will bring large configuration overhead and complexity. Hence, we prefer to introduce no optimization for LTM release.

Proposal 14: UE automatically modifies/releases the stored candidate cell configuration for LTM is not allowed.

  - Early decoding and Validity Check on Pre-configured RRC configurations of Candidate Cells

In the last meeting, it was agreed that UE may perform early decoding and early validity check for candidate cell configurations. If UE performs early validity check before LTM cell switch, potential RLF may happen and RRC reestablishment will be performed if validity check failed on a candidate cell. To resolve this issue, some companies proposed to introduce a new signaling to inform the network that configuration mistakes happened and avoid performing RRC reestablishment.

However, there is no need to do any enhancement for the early decoding and early validity check for LTM in our view. Because the LTM candidate cells is configured by the same CU as serving cell, the probability of configuration errors is very small and there is no need to do any optimization.

Observation 1: UE’s serving cell and LTM candidate cells shares the same CU, so the probability of configuration errors is very small.

Besides, if a configuration error is detected on a candidate cell before the cell switch, UE can choose to ignore it and perform RRC reestablishment only after receiving an LTM cell switch command with the target cell same as the validity check failed candidate cell. In this way, the RLF caused by configuration mistakes before the LTM cell switch can be avoided.

Proposal 15: If UE performs early validity check for the LTM candidate cell(s) and failed, based on UE implementation, RRC re-establishment can be triggered when validity check is performed or after receiving the cell switch command and the target cell is the candidate cell which fails to pass the validity check. No other optimization (e.g. report validity check failure information to network) is introduced since validity check failure is a rare case.

- Conclusion

In this contribution, we provide our view on some issues related to RRC. We have the following observations and proposals:

L1 measurement and report

Proposal 1: To support L1 measurement report of candidate cells, the report configuration of current serving cell(s) should refer to the L1 measurement configuration of the candidate cells in the separate list.

Proposal 2: To support L1 measurement report for subsequent LTM, the report configuration for LTM candidate cells should be included in the ServingCellConfig of each candidate cell.

Proposal 3: The report configuration in the ServingCellConfig of candidate cells should refer to the L1 measurement configuration of other candidate cells in the separate list.

Proposal 4: For one UE configured with multiple LTM candidate cells, each candidate cell needs to be informed about L1 measurement configuration of serving cell and/or other candidate cells, to generate the corresponding report configuration for LTM.

Proposal 5: If P4 is agreed, send an LS to inform RAN3 about the agreement.

Reference candidate configuration

Proposal 6: The reference CG configuration for LTM contains one reference PCell configuration and one reference SCell configuration. Delta configuration of candidate PCell is provided based on the configuration of the reference PCell, and delta configuration of each candidate SCell associated with the candidate PCell is provided based on the configuration of the reference SCell.

Proposal 7: Introduce CompleteConfig flag in LTM candidate CG configuration. If the flag is not present,  UE considers the candidate CG configuration as delta configuration on top of the reference configuration; otherwise, UE considers the candidate CG configuration as a complete configuration which can be applied without combining with the reference configuration.

RRC configured L2 partial reset

Proposal 8: Several cell sets are pre-configured to UE and the cell sets are configured external to the configurations of serving and candidate cells.

Proposal 9: Each cell set includes a list of cell identities and UE determines whether to perform L2 reset based on whether the source and target cell belongs to the same cell set.

Proposal 10：When performing LTM, if the source cell and target cell is in the same cell set:

- For MAC: UE performs partial MAC reset.

- For RLC: UE performs RLC reestablishment for SRB and not performs RLC reestablishment for DRB.

- For PDCP

  - For SRB: UE always performs PDCP discard.

  - For DRB: UE does not perform PDCP data recovery.

Proposal 11：When performing LTM, if the source cell and target cell is in the different cell set:

- For MAC: UE performs MAC reset.

- For RLC: UE performs RLC reestablishment

- For PDCP

  - For SRB: UE performs PDCP discard.

  - For DRB: UE determines whether to perform PDCP data recovery according to the existing IE recoverPDCP for the DRB

Other issues

Observation 1: UE’s serving cell and LTM candidate cells shares the same CU, so the probability of configuration errors is very small.

Proposal 12: UE determines the BWPs (for DL and UL) to be activated upon the execution of LTM based on the firstActivateDownlinkBWP-Id and firstActivateUplinkBWP-Id within the configuration of target cell(s).

Proposal 13: Upon the reception of LTM cell switch command, UE performs target SCell activation/deactivation based on the indication (i.e. sCellState field) within the pre-configured RRC configuration of target SCells.

Proposal 14: UE automatically modifies/releases the stored candidate cell configuration for LTM is not allowed.

Proposal 15: If UE performs early validity check for the LTM candidate cell(s) and failed, based on UE implementation, RRC re-establishment can be triggered when validity check is performed or after receiving the cell switch command and the target cell is the candidate cell which fails to pass the validity check. No other optimization (e.g. report validity check failure information to network) is introduced since validity check failure is a rare case.

# References

- R2_121bis-e Chair Notes (MediaTek)