3GPP TSG-RAN WG2 Meeting #122                 	 R2-2304912

Incheon, Korea, 22nd May – 26th May 2023

Source:	vivo, MediaTek Inc., Samsung

Title:	Remaining issues on partial MAC reset

Agenda Item:	7.4.2.3

Document for:	Discussion and Decision

- Introduction

In previous meetings, RAN2 discussed the partial MAC reset issue. And the following agreements were made in RAN2#120 and RAN2#121 respectively [1][2].

- RAN2 to have the mindset to have a common design for partial MAC reset for different cell change cases in intra-DU scenario (as far as reasonable) - The summary in [R2-2213336] could be considered as the starting point for partial reset in intra-DU.

- No consensus to support HARQ continuation (and in order to resume discussion some new input may be needed, e.g. quantitative evidence of a serious problem). - To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

In this paper, we will discuss the remaining partial MAC reset issues in detail.

- Discussion

  - Remaining partial MAC reset issue

During the discussion of partial MAC reset in RAN2#120 meeting [3], companies expected a common partial MAC reset design should be applied for different cell change cases. The related agreement is as follows:

- RAN2 to have the mindset to have a common design for partial MAC reset for different cell change cases in intra-DU scenario (as far as reasonable)

Regarding the detailed partial MAC reset, companies agreed that the summary in [4] could be considered as the starting point for partial reset in intra-DU. Hence, we will further discuss partial reset based on [4] and the summarized conclusion as follows in [3]:

Proposal 3: Some “high” MAC parts, which is not cell specific, could be considered for partial reset in intra-DU, FFS which parts below should be included, e.g. - Keep the triggered BSR - Maintain logical channel Bj value - Maintain DL HARQ soft buffer - Maintain NDI for UL HARQ processes - Not cancel Triggered Recommended bit rate query procedure

In the following sections, we discuss the remaining MAC partial reset issues.

BSR

According to the current specification, when the BSR is triggered, MAC entity will instruct the Multiplexing and Assembly procedure to generate the BSR MAC CE and start or restart the related timer.  It could be observed that the BSR procedure is not related to the serving cell. Besides, cell switch doesn’t impact the data volume calculation for UL data available for transmission, hence a triggered BSR should not be reset and needs to be reported to network to assist scheduling after LTM. Otherwise, UE’s MAC entity has to generate same BSR MAC CE for UL transmission and the UL data transmission will be delayed. Therefore, the triggered BSR procedure could be kept in intra-DU LTM.

Proposal 1: Triggered BSR is kept when UE performs partial MAC reset, to report buffer status in target cell timely.

logical channel Bj value

Every logical channel maintains a Bj, MAC entity shall initialize Bj of the logical channel to 0 when the logical channel is established, and increment the value of Bj by the product PBR * T, in which the PRB is the Prioritized Bit Rate configured for the logical channel and T is the time elapsed since Bj was last incremented. Besides, MAC entity shall decrement the value of Bj by the total size of MAC SDUs served to this logical channel when there are resources allocated to this logical channel.

Based on the above, the value of Bj is maintained to ensure the QoS of the associated logical channel, it makes sense to not affect the Bj when cell switch occurs. Hence, we propose:

Proposal 2: Maintain logical channel Bj value when UE performs partial MAC reset, to ensure the QoS of traffic of the associated logical channel.

UL time alignment timer with source cell/candidate cell

In our view, the TAT for candidate cells is essential as we have explained in our companion paper [5] and the upper value of the new introduced TAT should at least be 10.24s which is current TAT value of the serving cell. UE may perform cell switch several times within the 10.24s considering the handover frequency for LTM. If the UE doesn’t reset the TAT of candidate cells or serving cells at cell switch, the UE could skip RACH for a target cell at the next cell switch assuming the TAT of the target cell is still valid at the time when UE performs the second LTM without the new serving cell re-triggers early TA for the target cell. Besides, since the partial MAC reset only intends to intra-DU scenario, there is no additional effort for UE and DU to maintain the TAT value. Hence, we propose:

Proposal 3: Continue the UL time alignment timer with source cell/candidate cell when UE performs partial MAC reset, to ensure the UE could skip RACH during LTM as much as possible.

Cell specific MAC procedures

In current MAC reset, some cell-specific procedures will be canceled, the same principle should be applied for partial MAC reset since the serving cell is changed. These procedures are the following:

- Ongoing RACH procedure: RA configuration and CFRA resources are cell-specific. So the ongoing RACH procedure should be stopped and MsgA/Msg3 buffer should be flushed.

- Triggered SR: Triggered SR is related to the PUCCH resource of the source cell. Hence the triggered SR should be canceled after LTM.

- Triggered PHR: PHR is used to provide the difference between the nominal UE maximum transmit power and the estimated power in the current serving cell, which is useless after the serving cell changes.

- Triggered consistent LBT failure: The parameters for consistent LBT failure detection is configured per serving cell. According to the current mechanism, the consistent LBT failure is canceled after consistent LBT failure detection parameters are reconfigured. Hence, triggered consistent LBT failure should be canceled after cell switch.

- Triggered BFR: BFR is a cell-specific procedure, when LTM is performed, the serving cell is changed, and the trigger BFR for source cell means nothing to target cell.

- Triggered configured uplink grant confirmation: It is useless to send configured uplink grant confirmation in target cell as all the CG resources in source cell are released during the LTM.

Proposal 4: When UE performs partial MAC reset, cell-specific procedures including ongoing RACH procedure, triggered SR, triggered PHR, triggered consistent LBT failure, triggered BFR, and triggered configured uplink grant confirmation are cancelled.

  - HARQ continuation

Necessity of HARQ continuation

During the discussion in RAN2#121, the topic of HARQ continuation was discussed, but no consensus was reached as following agreement. In this section, we will further discuss the HARQ continuation during intra-DU LTM.

- No consensus to support HARQ continuation (and in order to resume discussion some new input may be needed, e.g. quantitative evidence of a serious problem.

In the legacy mechanism, the UE MAC entity is always reset during the handover procedure. When performing MAC reset, the UE sets the NDIs for all uplink HARQ processes to the value 0 and flushes the soft buffers for all DL HARQ processes, making HARQ continuation after handover not possible.

To reduce the latency of legacy handover, LTM is introduced. In LTM, cell switch is triggered by L1 measurement results. However, since the quality of L1 signal changes rapidly, LTM is characterized by a higher cell switch frequency with a shorter ToS compared to legacy handover. If HARQ continuation is not supported during LTM, the UE will experience more data loss than legacy handover.

In TS 23.501, the mapping between Standardized 5QI to QoS characteristics is defined. The QoS characteristics for 5QI = 2 is as follows:

Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms (NOTE 11, NOTE 13) | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 | (NOTE 1) | 40 | 150 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |

Usually, conversational video is transferred via RLC UM mode due to its stringent latency requirement. It means that loss of application packet occurs every time a MAC TB not acknowledged is flushed by the sender. Let’s assume that 1000 MAC TBs of conversational video are sent to the UE per second, and the UE performs a cell switch once per second. This implies that the QoS of conversational video cannot be met if more than one TB, which has not been successfully received, is flushed.

Therefore, it is desired that the HARQ processes in both DL and UL can continue after cell switch to ensure that the QoS of services are not impacted by LTM. Additionally, HARQ continuation can avoid duplicated HARQ retransmissions, which can help improve radio resource efficiency and reduce packet delivery latency.

Observation 1: QoS of RLC UM mode service may not be met if HARQ doesn’t continue during LTM.

Concerns analysis of HARQ continuation

In the last meeting, some concerns were raised regarding HARQ continuation [6][7][8][9][10], mainly including the following issues:

Overdue signaling in MAC TB [6][7]

Issue description: MAC PDU stored in the HARQ buffer may contain one or more UL/DL MAC CE before cell switching, and these overdated UL/DL MAC CE may cause errors at the UE side or mislead the NW.

Analysis: Since the NW knows the initial transmission of one UL TB is scheduled in which cell (i.e. source or target), it can decide whether a UL MAC CE received is outdated or not. In DL, the NW can schedule a new transmission with a HARQ process to flush the DL soft buffer associated with the HARQ process if the TB in the DL soft buffer carries overdue DL MAC CE. Moreover, not all MAC CEs are outdated after cell switching. For example, the BSR MAC CE that was initially transmitted in the source cell can still be valid if it is finally received in the target cell with cross-cell HARQ retransmission.

Observation 2: It could be up to NW implementation to determine whether re-transmit the MAC CE on a HARQ process whose initial transmission is scheduled in source cell after LTM is completed, UE retains the soft buffer during LTM and UE could perform soft buffer flush after LTM if NW schedules a new transmission on this HARQ process.

TB size may be changed after cell switch [6][ 8]

Issue description: The SINR in the target cell should be significantly better than in the source cell, which is why LTM is performed. However, maintaining the DL HARQ buffer in the target cell may result in the retransmission of the original TB from the source cell, even though it is possible to transmit a much larger TB to the UE in the same time and frequency resource due to the higher SINR in the target cell. This leads to wastage of radio resources in the target cell.

Analysis: This is an issue that already exists. In Rel-17 ICBM, a neighboring cell is modeled as a TRP of the serving cell. Hence, when the UE communicates with the neighboring cell through ICBM, its SINR improves and the HARQ buffers are retained. It should be noted that even if the UE's HARQ buffers are retained, the NW still has complete flexibility to flush the UE's UL HARQ and DL soft buffer by scheduling a new transmission through PDCCH. For example, when the NW is confident that the QoS of the ongoing service can be met, it can opt to give up HARQ continuation to avoid wasting radio resources.

Observation 3: It could be up to NW implementation to determine the TB size of a HARQ process after LTM is completed, UE retains the soft buffer during LTM and UE could perform soft buffer flush after LTM if NW schedules a new transmission on this HARQ process.

Number of HARQ processes is different in source and target cell [8][9][10]

Issue description: The number of HARQ processes could differ between the source PCell and target PCell. Therefore, it is necessary to map the HARQ process ID between the source and target cells so that both the UE and NW understand the content in each HARQ process buffer.

Analysis: HARQ continuation is only possible in an intra-DU scenario where the MAC peers in the UE and NW are unchanged. Thus, we believe it is uncommon for the number of HARQ processes to differ between the source PCell and the target PCell. Even if it does occur, and the target PCell is configured with fewer HARQ processes than the source PCell, the UE can easily identify the exceeded HARQ processes and flush the related HARQ buffers. If the target PCell is configured with more HARQ processes than the source PCell, we see no issue that needs to be fixed.

Observation 4: It is uncommon for the number of HARQ processes to differ between the source PCell and the target Pcell. If it happens and the target PCell is configured with fewer HARQ processes than the source PCell, the UE can flush the buffers of the exceeded HARQ processes.

Mapping of HARQ entity between serving and target cell group [8]

Issue description: In the current model, each serving cell has its own HARQ entity, and each HARQ entity has its own HARQ buffers. To maintain HARQ buffers during LTM in CA cases, it is necessary to consider which source HARQ entity should be maintained and mapped to which target HARQ entity. However, the number of serving cells in the source cell group may differ from the target cell group, and even if they are the same, the SCell serving cell ID may be different. Thus, an association or a mapping between source cell/HARQ entity and target cell/HARQ entity needs to be defined so that both the UE and NW understand the content in each HARQ entity buffer.

Analysis: We acknowledge that this issue needs resolving. To simplify matters, we suggest defining a simple mapping rule in the specification. For SpCell, the HARQ entity of the source SpCell is inherited by the target SpCell. For SCell, we propose to map based on SCellIndex. That is, the HARQ entity of the source SCell is inherited by the target SCell configured with the same SCellIndex as the source SCell.

Observation 5: Simple mapping rule could resolve the issue that the number of serving cells changes after LTM.

Taking the above analysis into account, we think HARQ continuation is indeed crucial to maintain QoS during LTM as it occurs more frequently than legacy handover. To support HARQ continuation, it is necessary to define a rule for mapping HARQ entities between cells in the serving and target cell groups. This will ensure that the content in each HARQ buffer is understood by both the UE and NW, enabling seamless transmission of data during LTM.

Proposal 5: Maintain DL HARQ soft buffer and NDI for UL HARQ processes when UE performs partial MAC reset.

- Conclusion

In this contribution, we discuss the remaining partial MAC reset issues and HARQ continuation. Based on the discussion, we have the following observations and proposals.

Observations are as follows:

Observation 1: QoS of RLC UM mode service may not be met if HARQ doesn’t continue during LTM.

Observation 2: It could be up to NW implementation to determine whether re-transmit the MAC CE on a HARQ process whose initial transmission is scheduled in source cell after LTM is completed, UE retains the soft buffer during LTM and UE could perform soft buffer flush after LTM if NW schedules a new transmission on this HARQ process.

Observation 3: It could be up to NW implementation to determine the TB size of a HARQ process after LTM is completed, UE retains the soft buffer during LTM and UE could perform soft buffer flush after LTM if NW schedules a new transmission on this HARQ process.

Observation 4: It is uncommon for the number of HARQ processes to differ between the source PCell and the target Pcell. If it happens and the target PCell is configured with fewer HARQ processes than the source PCell, the UE can flush the buffers of the exceeded HARQ processes.

Observation 5: Simple mapping rule could resolve the issue that the number of serving cells changes after LTM.

Proposals are as follows:

Proposal 1: Triggered BSR is kept when UE performs partial MAC reset, to report buffer status in target cell timely.

Proposal 2: Maintain logical channel Bj value when UE performs partial MAC reset, to ensure the QoS of traffic of the associated logical channel.

Proposal 3: Continue the UL time alignment timer with source cell/candidate cell when UE performs partial MAC reset, to ensure the UE could skip RACH during LTM as much as possible.

Proposal 4: When UE performs partial MAC reset, cell-specific procedures including ongoing RACH procedure, triggered SR, triggered PHR, triggered consistent LBT failure, triggered BFR, and triggered configured uplink grant confirmation are cancelled.

Proposal 5: Maintain DL HARQ soft buffer and NDI for UL HARQ processes when UE performs partial MAC reset.

- References

- R2#120 Chair Notes (MediaTek)

- R2#121 Chair Notes (MediaTek)

- R2-2213335, Report of #033 on Partial MAC reset for intra-DU LTM vivo

- R2-2213336, Potential Partial MAC Reset for intra-DU LTM vivo, MediaTek, Xiaomi

- R2-2304910, Remaining issues on early TA acquisition, vivo

- R2-2300403, Discussion on LTM cell switch, Intel Corporation

- R2-2301790, Further Considerations on Intra-DU LTM and Partial MAC Reset, ZTE Corporation, Sanechips

- R2-2301151, L2 behaviours and cell switch solutions in LTM, Huawei, HiSilicon

- R2-2300381, Discussion on partial MAC reset for LTM, OPPO

- R2-2300093, Discussions on Cell Switch, CATT