3GPP TSG-RAN WG2 Meeting #122		   R2-2304913

Incheon, KR, 22nd – 26th May, 2023

Agenda item:	7.4.3

Source:	vivo

Title:	Remaining issues for NR-DC with selective activation cell of groups

Document for:		Discussion & Decision

# 1. Introduction

In this contribution, we will further discuss some remaining issues which are related to #121 meeting and #121bis- meeting agreements:

- How to update the execution conditions in SN-initiated inter-SN SCG selective activation;

- How to avoid evaluating the current serving cell when it is configured as one candidate;

- Whether/how to differentiate the CPA and CPC configuration in SCG selective activation;

- What are the possible conditions of Rel-18 CPAC configuration Release.

- Our opinions on other miscellaneous FFS issues in #121bis-e meeting.

# 2. Discussion

## 2.1 Execution condition update for SN-initiated SCG selective activation

As the SN-initiated SCG selective activation has been agreed to be supported, we would like to further discuss some details about how the configuration for subsequent CPC evaluation/execution in SN-initiated SCG selective activation can be updated.

Let’s take the SN-initiated conditional PSCell Change procedure as an example, in the CPC preparation phase, the step 1 (source SN sending SN Change Required to MN) is as follows[1]:

## 10.5 Secondary Node Change (MN/SN initiated) ### 10.5.2 MR-DC with 5GC SN initiated conditional SN Change <omitted…> 1. The source SN initiates the conditional SN change procedure by sending SgNB Change Required message which contains a CPC initiation indication. The message also contains candidate SN ID(s) information and may include the SCG configuration (to support delta configuration), and contains the measurement results related to the candidate SN(s). The message also includes a list of proposed PSCell candidates recommended by the source SN, including execution conditions, the upper limit for the number of PSCells that can be prepared by each candidate SN, and may also include the SCG measurement configurations for CPC (e.g. measurement ID(s) to be used for CPC).

Observation 1: For SN-initiated inter-SN CPC procedure, the source SN decides and sends candidate SN ID(s) and the associated execution conditions to MN. And SCG measurement configuration may also be included for MN to interpret the execution conditions.

Also, in TS 38.331, it can be seen that what is provided by MN as follows[2]:

CG-Config-v1700-IEs ::= SEQUENCE { candidateCellInfoListCPC-r17 CandidateCellInfoListCPC-r17 OPTIONAL, twoPHRModeSCG-r17 ENUMERATED {enabled} OPTIONAL, nonCriticalExtension CG-Config-v1730-IEs OPTIONAL } CandidateCellInfoListCPC-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF CandidateCellInfo-r17 CandidateCellInfo-r17 ::= SEQUENCE { ssbFrequency-r17 ARFCN-ValueNR, candidateList-r17 SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF CandidateCell-r17 } CandidateCell-r17 ::= SEQUENCE { physCellId-r17 PhysCellId, condExecutionCondSCG-r17 OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL }

In order to support the SN-initiated selective activation, we understand from the NW point of view, after the PSCell change, the target SN can be provided by MN with at least the following information that is also included in CandidateCellInfoListCPC-r17 from source SN to MN:

- SSB frequency and physical cell ID for candidate PSCell(s).

- Associated execution conditions.

With the above information, the target SN may decide whether to update the subsequent CPC configuration to UE e.g. reconfigure the execution conditions or release the candidate PSCell(s).

However, for the execution condition, if the SCG measConfig configured by target SN (new serving SN) is different from the source SCG measConfig, the mismatch happens between the execution conditions configured to UE (before this PSCell change) and the newly configured SCG measConfig. The following is an example:

- Source SN configure the CPAC configuration with an execution condition in which

  - MeasID=1 associated with MeasObject= frequency 1

- In Target SN the configured serving cell measurement configuration is

  - MeasID=1 associated with MeasObeject= frequency 2

  - MeasID=2 associated with MeasObeject= frequency 1

I.e., MeasID associated MeasObject in target SN measurement configuration may be not aligned with measurement configuration from source SN. To resolve this issue, either the execution conditions are configured based on e.g. a common measurement configuration, or Target SN should perform the SCG measurement reconfiguration. On one hand, if this common measurement configuration is applied, e.g. execution conditions are associated with the MeasConfig in reference configuration, all the UE can know the correct mapping between the MeasObject and the execution condition. On the other hand, if we just rely on the NW procedure to update the execution condition, it can cause a lot of signalling overhead between UE and network especially when such coordination happens a lot e.g. upon each successful PSCell change in SCG selective activation, if the target SN decides to update the configuration for execution condition, there would be RRC reconfiguration to the UE to do that. Therefore, we propose to agree that the RRCReconfiguration is not necessarily needed upon each PSCell change, and we can consider to introduce a common measurement configuration to be used for understanding the execution condition during SN-initiated SCG selective activation when the serving SN keeps changing.

Observation 2: It is not desirable to rely on network updating the execution conditions in SN-initiated SCG selective activation upon PSCell change, because it would cause a lot of signalling between the network and the UE, which violates the intention of this WI objective.

Proposal 1: RAN2 aims to support SN-initiated inter-SN SCG selective activation without RRC reconfiguration to update the execution conditions upon each PSCell change.

Proposal 2: To solve the mismatch between the configuration of execution condition from source SN and the measurement configuration from target SN, all the execution conditions can be associated with a common MeasConfig (e.g. the MeasConfig in reference configuration).

## 2.2 Evaluation when serving PSCell serves as one candidate

In RAN2 #121 meeting there is an agreement:

It is assumed that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell.

The above agreement indicates that the current PSCell can be configured as one candidate. However, when that happens, whether/how the UE evaluates this ‘candidate PSCell’ (which is actually also the serving PSCell) is not clear.

Observation 3: It is not clear whether/how the UE evaluates the candidate PSCell which is actually also the serving PSCell.

In our understanding, the most straightforward method is that the UE does not evaluate this cell. Otherwise, it would be tricky how the UE treats the A4 or A3/A5 event when the candidate PSCell is also the current serving PSCell. Therefore, we propose:

Proposal 3: RAN2 to confirm that, when the candidate PSCell is the current serving PSCell, the UE does not evaluate that cell for CPAC.

## 2.3 Differentiation for CPA and CPC Configuration

Another agreement in #121 meeting:

R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

To support on configuration for both CPA and CPC and with different triggering conditions, it seems two execution conditions would be needed as in following example:

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condExecutionCond-r18            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]]

}

When there are two execution conditions, the UE can know which is for CPA and which is for CPC, based on e.g. whether the associated event is A3/A5 or not.

However, when there is only one execution condition and the associated event is A4, it is not clear how the UE can know it is for CPA or CPC.

In our understanding, there should be any indication for this case. E.g., if the UE consider one triggering condition for A4 event in a conditional configuration means it can be used for both CPA and CPC, it is not aligned with previous agreement.

Observation 4: If there is only one triggering condition for A4 event in a conditional configuration in SCG selective activation, it is not clear how the UE knows the configuration is for CPA or CPC. UE cannot assume it can be used for both CPA and CPC because the network would provide two triggering conditions for that case.

Therefore, we propose:

Proposal 4: An indication is needed in R18 CPAC configuration for SCG selective activation, for network to explicitly indicate whether the configuration is for CPA or CPC.

## 2.4 Keeping and releasing for subsequent CPAC configuration

The agreement in RAN2 #121bis-e is as follows:

UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

During online discussions, how the UE should determine whether the R18 CPC configurations should be released, has been discussed a lot. As the default behaviour of UE is to keep R18 CPC configurations after CPC execution, the key point is that whether we should support a per-configuration indication for indicating whether an R18 CPC configuration should be released. Then, even if a CPC candidate is not released explicitly by RRC reconfiguration procedure, it can be released autonomously after CHO/CPAC execution, which can further reduce the signalling between network and the UE in selective activation of SCG.

Observation 5: If the indication to release an R18 CPC configuration is included in conditional reconfiguration, the UE does not need to only rely on explicitly RRC reconfiguration procedure to release a R18 CPC configuration.

Therefore, we propose:

Proposal 5: An indication can be included in an R18 CPC configuration to denote whether this R18 CPC configuration should be kept or released after its execution.

## 2.5 Other FFS issues

#121bis-e meeting Agreement: - For the reference configuration for SCG Selective Activation, aim at following similar design as LTM. - For inter-SN SCG Selective Activation, the RRC reconfiguration message containing the Rel-18 CPC configurations provided to the UE is in MN format. - For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC. - FFS on the following options for subsequent CPC: - Option 1: Source MN generates the execution conditions for all subsequent CPC. - Option 2: Candidate SN may generate execution conditions for subsequent CPC. - For SN initiated inter-SN SCG selective activation, source SN generates the execution conditions for the initial CPC. FFS if Candidate SN may generate/modify execution conditions for subsequent CPC - Assume for now that there is only one reference configuration. - The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations: - Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty. - FFS whether MCG configuration is included. - FFS RRC model for the reference configuration. - Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included. - 3. The execution conditions associated with each candidate target PSCell. - a. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. - b. For SN initiated procedure, execution conditions based on events A3/A5 are supported. - UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

For the above highlighted FFS, our preference is as follows. As most of them have been discussed in the post-email discussion of RAN2 #121, we didn’t give exact proposals on each of them but just give our preference for each issue.

| FFS | Our preference |
| --- | --- |
| For MN initiated inter-SN SCG selective activation: FFS on the following options for subsequent CPC: Option 1: Source MN generates the execution conditions for all subsequent CPC. Option 2: Candidate SN may generate execution conditions for subsequent CPC. | - Not support Candidate SN to generate execution conditions for subsequent CPC as it is not aligned with legacy design |
| For SN initiated inter-SN SCG selective activation: FFS if Candidate SN may generate/modify execution conditions for subsequent CPC | - Candidate SN may generate/modify execution conditions for subsequent CPC only after it becomes serving SN |
| For reference configuration: FFS whether MCG configuration is included. | - It is allowed to have the MCG configuration included in the reference configuration which can be used for delta configuration for MCG. - It is up to gNB implementation whether to include the MCG configuration or not. |
| FFS RRC model for the reference configuration. | - As in legacy CPAC, the RRC model for the reference configuration is an RRCReconfiguration with at least SCG configuration is included |
| Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included. | - It is up to gNB implementation whether to include the MCG configuration. It can be anyway reconfigured by the network explicitly after CPC execution. |
| For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. | - No need to support A3/A5 as this is not supported in legacy and they can be covered by SN-initiated procedure. |

# 3. Conclusion

In this contribution, we discussed about the remaining issues for NR-DC with selective activation cell of groups, mainly based on the previous agreement for RAN2 #121 and RAN2#121bis-e.

We have the following observations and proposals:

For SN-initiated SCG selective activation

Observation 1: For SN-initiated inter-SN CPC procedure, the source SN decides and sends candidate SN ID(s) and the associated execution conditions to MN. And SCG measurement configuration may also be included for MN to interpret the execution conditions.

Observation 2: It is not desirable to rely on network updating the execution conditions in SN-initiated SCG selective activation upon PSCell change, because it would cause a lot of signalling between the network and the UE, which violates the intention of this WI objective.

Proposal 1: RAN2 aims to support SN-initiated inter-SN SCG selective activation without RRC reconfiguration to update the execution conditions upon each PSCell change.

Proposal 2: To solve the mismatch between the configuration of execution condition from source SN and the measurement configuration from target SN, all the execution conditions can be associated with a common MeasConfig (e.g. the MeasConfig in reference configuration).

For Evaluation when serving PSCell serves as one candidate

Observation 3: It is not clear whether/how the UE evaluates the candidate PSCell which is actually also the serving PSCell.

Proposal 3: RAN2 to confirm that, when the candidate PSCell is the current serving PSCell, the UE does not evaluate that cell for CPAC.

Differentiation for CPA and CPC Configuration

Observation 4: If there is only one triggering condition for A4 event in a conditional configuration in SCG selective activation, it is not clear how the UE knows the configuration is for CPA or CPC. UE cannot assume it can be used for both CPA and CPC because the network would provide two triggering conditions for that case.

Proposal 4: An indication is needed in R18 CPAC configuration for SCG selective activation, for network to explicitly indicate whether the configuration is for CPA or CPC.

For Keeping and releasing for subsequent CPAC configuration

Observation 5: If the indication to release an R18 CPC configuration is included in conditional reconfiguration, the UE does not need to only rely on explicitly RRC reconfiguration procedure to release a R18 CPC configuration.

Proposal 5: An indication can be included in an R18 CPC configuration to denote whether this R18 CPC configuration should be kept or released after its execution.

# 4. References

3GPP TS 37.340 V17.4.0 (2023-03)

3GPP TS 38.331 V17.4.0 (2023-03)