3GPP TSG-RAN WG2 Meeting #122		R2-2304916

Incheon, Korea, 22nd May – 26th May 2023

Source:	vivo

Title:	Analysis on introducing H-SFN for DRX formulas

Agenda Item:	7.5.3

Document for:	Discussion and Decision

- Introduction

In the last meeting, the following agreement was made for SFN wrap-around issue:

To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter

In this contribution, we discuss the different solutions of introducing a counter in the DRX formula for resolving the SFN wrap-around issue. And we analyze the RAN2 specification impact of different solutions and try to find the solution with the least spec impact.

- Discussion

According to the current specification, the DRX on-duration timer starts when SFN and subframe number satisfy

the following DRX formula for long DRX cycle:

[(SFN × 10) + subframe number] mod (drx-LongCycle) = drx-StartOffset.             (Eq1)

In this formula, SFN takes the value of 0~1023, i.e., wraps around every 10240ms. Therefore, when the DRX cycle is set to a value when 10240ms is not integer times of DRX cycle (e.g. 50ms), there will be a problem that the start point of the first DRX on-duration after SFN wrap-around is shifted with a wrong offset and then propagates this offset to the following cycles. The unexpected offset causes a mismatch between DRX on-duration and XR traffic arrival time and leads to extra latency.

Observation 1: Mismatch happens when SFN wrap-around if the DRX cycle is set to a value when 10240ms is not integer times of DRX cycle.

To handle this issue, RAN2 agreed to introduce a counter in the DRX formula that increments at every SFN wrap around. As for the counter used in the DRX formula, companies have proposed mainly two approaches as follows:

- Approach 1: Introducing H-SFN in SIB1 in the C-DRX formulas

The hyper-SFN-r17 parameter in SIB1 introduced in Rel-17 to support e-DRX can be reused in the C-DRX formulas to resolve the issue of SFN wrap-around. The DRX formula for long DRX cycle is modified as below

[(SFN + 1024* H-SFN) × 10+ subframe number] mod (drx-LongCycle) = drx-StartOffset       (Eq2)

Since the existing Hyper SFN in SBI1 is reused, this approach only brings little MAC specification impact for the C-DRX formula and no new parameters need to be introduced in the current RRC specification. Besides, with this approach, the reference SFN is not needed, because the SIB1 that carries the H-SFN are sent in the SI window and there won’t be any ambiguity for the UE about when the H-SFN was transmitted by the network.

One may argue that this approach does not resolve the issue perfectly, because the range of Hyper SFN is 0~1023 and H-SFN wrap-around will happen if the DRX cycle is set to a value when 1024*10240ms is not integer times of DRX cycle. In our view, the network can reconfigure DRX configuration when H-SFN wrap-around happens to align with the XR traffic. Since H-SFN takes nearly 3 hours to wrap around, we think not much signaling overhead will be introduced for DRX reconfiguration.

Observation 2: The approach of introducing H-SFN in the C-DRX formulas does not require a reference SFN and no new parameters need to be introduced in the RRC specification.

Observation 3: H-SFN takes nearly 3 hours to wrap around, so not much signaling overhead will be introduced for DRX reconfiguration.

- Approach 2: Introduce a new counter(E-SFN) in the DRX formulas[1]

This is similar to approach 1, except that it introduces a new counter instead of using the existing H-SFN to calculate the start time of DRX on-duration. The initial value of the E-SFN is indicated to UE via dedicated RRC signaling and UE should increment the E-SFN every time SFN wrap around. The DRX formulas for long DRX cycle are modified as below.

[(SFN + 1024* E-SFN) × 10+ subframe number] mod (drx-LongCycle) = drx-StartOffset       (Eq3)

Since the initial value of the E-SFN is indicated in the dedicated RRC signaling, there may be ambiguity for UE and network in initializing the E-SFN initial value when the RRC signaling is sent just before the SFN wrap around and received by the UE after the SFN wrap around because of HARQ/ARQ retransmissions. Therefore, a reference SFN needs to be indicated to the UE so that UE can know if the E-SFN value is sent in the last SFN cycle or the current SFN cycle. Besides, the maximum value of the new counter E-SFN also needs to be defined in the RRC specification. Therefore, Approach 2 has more MAC specification and more RRC specification impact than Approach 1.

Observation 4: The approach of introducing E-SFN in the C-DRX formulas brings more MAC specification and RRC specification impact than the approach of introducing H-SFN in the C-DRX formulas.

The specification impact for the two approaches is summarized as below:

|  | MAC specification impact | RRC specification impact | Other impact |
| --- | --- | --- | --- |
| Approach 1: Existing H-SFN in SIB1 | - Introduce H-SFN in the C-DRX formulas | Not required | Need to reconfigure DRX configuration when H-SFN wrap-around |
| Approach 2: E-SFN in dedicated RRC signaling | - Introduce E-SFN in the C-DRX formulas - Introduce reference SFN in the C-DRX formulas or the E-SFN value is determined by the reference SFN | - Need to indicate the initial value of E-SFN - Need to provide the reference SFN - Need to specify or configure the maximum value of E-SFN | None |

Therefore, we propose to introduce the H-SFN already existing in SIB1 in the C-DRX formula to resolve the SFN wrap-around issue.

Proposal 1: To handle the SFN wrap-around issue, introduce hyper frame number already existing in SIB1  in the C-DRX formulas. The C-DRX formulas can be enhanced as below:

- [(SFN + 1024* H-SFN) × 10+ subframe number] modulo (drx-LongCycle) = drx-StartOffset

- [(SFN + 1024* H-SFN) × 10+ subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle)

- Conclusion

In this contribution, we discuss the solutions to address SFN wrap-around issue. Based on the discussion, we have the following observations and proposal:

Observation 1: Mismatch happens when SFN wrap-around if the DRX cycle is set to a value when 10240ms is not integer times of DRX cycle.

Observation 2: The approach of introducing H-SFN in the C-DRX formulas does not require a reference SFN and no new parameters need to be introduced in the RRC specification.

Observation 3: H-SFN takes nearly 3 hours to wrap around, so not much signaling overhead will be introduced for DRX reconfiguration.

Observation 4: The approach of introducing E-SFN in the C-DRX formulas brings more MAC specification and RRC specification impact than the approach of introducing H-SFN in the C-DRX formulas.

Proposal 1: To handle the SFN wrap-around issue, introduce hyper frame number already existing in SIB1  in the C-DRX formulas. The C-DRX formulas can be enhanced as below:

- [(SFN + 1024* H-SFN) × 10+ subframe number] modulo (drx-LongCycle) = drx-StartOffset

- [(SFN + 1024* H-SFN) × 10+ subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle)

- References

- R2-2303302, SFN wrap-around solution for XR DRX, MediaTek Inc., CATT, LGE, Ericsson, NEC, DENSO