3GPP TSG-RAN WG2 Meeting #122e	R2-2304932

# Incheon, Korea, 22 - 26 May, 2023

Agenda Item:    7.13.3

Source:         Xiaomi

Title:         	Considerations on MDT override enhancement for E-UTRAN

Document for:	Discussion and Decision

Introduction

For the MDT override enhancement in inter-RAT scenario, RAN2 achieved the following agreements in last meeting [1].

Solution 1: Override protection for logged MDT by simultaneous LTE and NR configuration for logged MDT.

Solution 2: Override protection by cross-RAT signaling but no cross-RAT reporting of LTE logged MDT report from NR to LTE.

Solution 3: Both solution 1 and 2 are supported for different UE implementation.

- Solution 2 is chosen for further specification work.

Agreements For solution 2:

1	Extend the LTE LoggedMeasurementConfiguration to include Logged MDT type indication information

2	NR signaling is needed to inform the gNB that signaling based MDT is configured by E-UTRA.

3	Try to reuse R17 NR signaling by the UE to inform gNB whether signaling based MDT is configured even when it is configured by E-UTRA.

- FFS in RAN3 the details and the Need of differentiating the RAT type. Further discuss whether priority handling for signalling logged MDT configuration between different RAT types is needed or not.

In this contribution, we provide our considerations on the details of cross-RAT signaling for the MDT override enhancement in the inter-RAT scenario and provide the stage-3 draft TP as the annex based on the previous agreements and our further proposals.

Discussion

In previous meeting, for the s-based logged MDT override protection mechanism in the inter-RAT scenario where UE configured with the s-based logged MDT in LTE reselects to a NR cell, RAN2 agreed to adopt a similar mechanism as that for R17 intra-NR case. It is agreed to reuse R17 NR signalling by the UE to inform gNB whether the s-based logged MDT is configured even when it is configured by E-UTRA. However, the details on reusing are still FFS.

In R17, s-based logged MDT override protection is supported only for the intra-NR scenario, UE indicates the s-based logged MDT protection flag to gNB according to the quoted procedure from TS 38.331. As the TS 38.331 is specific to NR, all highlight parameters (e.g. the T330 timer and UE variables for logged MDT) below are only for the s-based logged MDT in NR.

2>	if the sigLoggedMeasType in VarLogMeasReport is included:

3>	if T330 timer is running and the logged measurements configuration is for NR:

4>	set sigLogMeasConfigAvailable to true in the RRCReestablishmentComplete message;

3>	else:

4>	if the UE has logged measurements available for NR:

5>	set sigLogMeasConfigAvailable to false in the RRCReestablishmentComplete message;

As the instance of T330 timer and the UE variables are RAT specific parameters, to support to MDT override protection in inter-RAT scenario, it should be supported that UE checks that parameters for the s-based logged MDT in E-UTRA to decide whether to reuse the R17 NR signaling to inform gNB whether s-based MDT is configured by E-UTRA.

In details, UE indicates the LTE s-based logged MDT configuration availability if following conditions are fulfilled.

- the sigLoggedMeasType in VarLogMeasReport for E-UTRA is included

- T330 timer for E-UTRA is running

- the logged measurements configuration is for E-UTRA

And UE indicates the LTE s-based logged MDT report availability to gNB if

- the UE has logged measurements available for E-UTRA

Observation 1: In Rel-17, before forwarding the override protection flag to gNB, UE checks the T330 timer and s-based logged MDT parameters in UE variable, all of which are for NR.

Observation 2: The instance of T330 timer and UE variable for logged MDT is specific to RAT.

Proposal 1: For MDT override protection in the inter-RAT scenario, UE should check the T330 timer and s-based logged MDT parameters in the UE variables, all of which are for E-UTRA, before reusing NR signaling to inform gNB whether the s-based MDT is configured by E-UTRA.

Proposal 2: RAN2 considers the proposed TP in the annex as baseline.

Conclusions

During the discussion above, we have the following observations and proposals:

Observation 1: In Rel-17, before forwarding the override protection flag to gNB, UE checks the T330 timer and s-based logged MDT parameters in UE variable, all of which are for NR.

Observation 2: The instance of T330 timer and UE variable for logged MDT is specific to RAT.

Proposal 1: For MDT override protection in the inter-RAT scenario, UE should check the T330 timer and s-based logged MDT parameters in the UE variables, all of which are for E-UTRA, before reusing NR signaling to inform gNB whether the s-based MDT is configured by E-UTRA.

Proposal 2: RAN2 considers the proposed TP in the annex as baseline.

References

- R2-2209xxx Report of 3GPP TSG RAN WG2 meeting #121bis-e, Online.

Draft TP

- TS 38.331

- 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

<Omit>

2>	if the sigLoggedMeasType is included in VarLogMeasReport of TS 36.331[10] and if the UE is capable of cross-RAT signalling, or

2> if the sigLoggedMeasType in VarLogMeasReport is included:

3>	if T330 timer is running and the logged measurements configuration is for NR, or

3> if T330 timer for E-UTRA is running and the logged measurements configuration is for E-UTRA

4>	set sigLogMeasConfigAvailable to true in the RRCSetupComplete message;

3>	else:

4>	if the UE has logged measurements available for NR or E-UTRA:

5>	set sigLogMeasConfigAvailable to false in the RRCSetupComplete message;

- 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

<Omit>

3>	if the sigLoggedMeasType is included in VarLogMeasReport of TS 36.331[10] and if the UE is capable of cross-RAT signalling, or

3>	if the sigLoggedMeasType in VarLogMeasReport is included:

4>	if T330 timer is running and the logged measurements configuration is for NR, or

4> if T330 timer for E-UTRA is running and the logged measurements configuration is for E-UTRA

5>	set sigLogMeasConfigAvailable to true in the RRCReconfigurationComplete message;

4>	else:

5>	if the UE has logged measurements available for NR or E-UTRA:

6>	set sigLogMeasConfigAvailable to false in the RRCReconfigurationComplete message;

- 5.3.7.5	Reception of the RRCReestablishment by the UE

The UE shall:

<Omit>

2>	if the sigLoggedMeasType is included in VarLogMeasReport of TS 36.331[10] and if the UE is capable of cross-RAT signalling, or

2>	if the sigLoggedMeasType in VarLogMeasReport is included:

3>	if T330 timer is running and the logged measurements configuration is for NR:

3> if T330 timer for E-UTRA is running and the logged measurements configuration is for E-UTRA

4>	set sigLogMeasConfigAvailable to true in the RRCReestablishmentComplete message;

3>	else:

4>	if the UE has logged measurements available for NR or E-UTRA:

5>	set sigLogMeasConfigAvailable to false in the RRCReestablishmentComplete message;

- 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

<Omit>

2>	if the sigLoggedMeasType is included in VarLogMeasReport of TS 36.331[10] and if the UE is capable of cross-RAT signalling, or

2>	if the sigLoggedMeasType in VarLogMeasReport is included:

3>	if T330 timer is running and the logged measurements configuration is for NR, or:

3> if T330 timer for E-UTRA is running and the logged measurements configuration is for E-UTRA

4>	set sigLogMeasConfigAvailable to true in the RRCResumeComplete message;

3>	else:

4>	if the UE has logged measurements available for NR or E-UTRA:

5>	set sigLogMeasConfigAvailable to false in the RRCResumeComplete message;

- TS 36.331

- 5.6.6.3	Reception of the LoggedMeasurementConfiguration by the UE

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	store the received targetMBSFN-AreaList, if included, in VarLogMeasConfig;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-NameList, if included, in VarLogMeasConfig;

1>	store the received loggedEventTriggerConfig, if included, in VarLogMeasConfig;

1>	store the received measUncomBarPre, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

1>	store the received sigLoggedMeasType, if included, in VarLogMeasReport

#### –	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by E-UTRAN to configure the UE to perform logging of measurement results while in RRC_IDLE or to perform logging of measurement results for MBSFN while in both RRC_IDLE and RRC_CONNECTED. It is used to transfer the logged measurement configuration for network performance optimisation, see TS 37.320 [60].

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

LoggedMeasurementConfiguration message

-- ASN1START

<omit>

LoggedMeasurementConfiguration-v1700-IEs ::= SEQUENCE {

loggedEventTriggerConfig-r17	LoggedEventTriggerConfig-r17	OPTIONAL,	--Need OR

measUncomBarPre-r17				ENUMERATED {true}				OPTIONAL,	--Need OR

nonCriticalExtension			LoggedMeasurementConfiguration-v1800-IEs	OPTIONAL

}

LoggedMeasurementConfiguration-v1800-IEs ::= SEQUENCE {

sigLoggedMeasType-r18           ENUMERATED {true}   OPTIONAL,	--Need OR

nonCriticalExtension			SEQUENCE {}						OPTIONAL

}

<omit>

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ identities.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

measUncomBarPre If configured, the UE attempts to perform the uncompensated Barometeric pressure measurement in RRC_IDLE as defined in TS 37.355 [109].

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

targetMBSFN-AreaList Used to indicate logging of MBSFN measurements and further restrict the area and frequencies for which the UE performs measurement logging for MBSFN. If both MBSFN area id and carrier frequency are present, a specific MBSFN area is indicated. If only carrier frequency is present, all MBSFN areas on that carrier frequency are indicated. If there is no entry in the list, any MBSFN area is indicated.

sigLoggedMeasType If included, the field indicates a signalling based logged measurements (See TS 37.320 [60]).

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [58].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [58]

#### –	VarLogMeasReport

The UE variable VarLogMeasReport includes the logged measurements information.

VarLogMeasReport UE variable

-- ASN1START

VarLogMeasReport-r10 ::=				SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10			OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-Identity-r10					PLMN-Identity,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

VarLogMeasReport-r11 ::=			SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10		OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-IdentityList-r11				PLMN-IdentityList3-r11,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

VarLogMeasReport-r18 ::=			SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10		OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-IdentityList-r11				PLMN-IdentityList3-r11,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

sigLoggedMeasType-r18               ENUMERATED {true}

}

LogMeasInfoList2-r10 ::=				SEQUENCE (SIZE (1..maxLogMeas-r10)) OF LogMeasInfo-r10

-- ASN1STOP