3GPP TSG-RAN WG2 Meeting #122		R2-2304936

Incheon, KR, May 22nd – May 26th 2023

Source:	CATT

Title:	Discussion on LP-WUS in RRC_IDLE&INACTIVE state

Agenda Item:	7.22.2

Document for:	Discussion and Decision

Introduction

In RAN2#121bis-e meeting, preliminary agreements on LP-WUS were achieved as follows [1]:

Confirm that we follow R1 and include RRC idle/inactive/connected. Ultra-deep-sleep = R2 understands for now that this is a power saving state (introduced by R1) to denote a state when the Main Receiver (MR) may sleep/turn off. In scope: Use LPWUS with Idle / Inactive UE camping with reception of paging and other necessary transmissions (from serving cell), reusing if possible/reasonable concepts from earlier releases, where the LPWUS either wakes the UE to receive by MR, or it conveys information by itself, or both.

In this document, we continue to discuss the RAN2 impact for LP-WUS in RRC_IDLE/RRC_INACTIVE state.

Discussion

2.1	LP-WUS functionality

In RRC Idle/Inactive, it is expected that the UE monitors LP-WUS per paging cycle. Different LP-WUS design options would allow such channel/signal to carry different amounts of payload, as captured in above RAN2 agreement (Section 1).

And in RAN1#112bis-e meeting, it was agreed that

Agreement - For IDLE/INACTIVE mode study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g. UE-group, -subgroup or -ID - FFS: cell information - FFS: SI change and ETWS/CMAS information, tracking area information, and RAN area information

This leaves the door open to different options in terms of LP-WUS functionality in RRC Idle/Inactive, being discussed in RAN1. Namely:

Option 1: LP-WUS replaces the PEI (minimum payload)

Option 2: LP-WUS replaces the paging PDCCH and Paging messages (more ambitious payload)

The information carried by LP-WUS can also be studied for each corresponding mechanism.

- 2.1.1	LP-WUS replacing the PEI

In Rel-17, power saving for Idle/Inactive state UEs was studied and PEI which reduces UE power consumption due to reduced false paging alarms was adopted. The group of UEs monitoring the same PO can be divided into multiple subgroups. With subgrouping, a UE shall monitor PDCCH in its PO for paging if the subgroup to which the UE belongs is paged as indicated via associated PEI [2].

In this option, LP-WUS is used in place of the PEI to indicate whether UE should decode the Paging PDCCH/message on PDSCH in the subsequent PO. When the UE is woken-up by LP-WUS, the UE will wake up from ultra-deep sleep and start receiving the Paging afterwards. And the sub-group information can be carried in LP-WUS. The procedure is shown in Figure 1 for the case where the paging triggers RACH.

Figure 1: Using LP-WUS instead of PEI

Assuming LP-WUS replaces the PEI functionality, the impact on the Idle/Inactive procedures will be quite small.

Observation 1: Assuming LP-WUS replaces the PEI functionality, the impact on the Idle/Inactive procedures will be quite small.

For PEI like solution, UE will be divided in to subgroups, and the subgroup information will be carried in PEI. Hence, based on the agreements made in RAN1#112bis-e meeting, it can be supported that:

Proposal 1: For IDLE/INACTIVE mode, at least, UE-subgroup information can be carried by the LP-WUS when LP-WUS can be applied to replace PEI.

- 2.1.2	LP-WUS replacing the paging PDCCH and Paging messages

In RRC_IDLE/INACTIVE state, the UE may use DRX to reduce power consumption. The UE monitors one PO per DRX cycle. A PO is a set of PDCCH monitoring occasions where paging DCI can be sent.

When LP-WUS replaces Paging, The MR is in ultra-deep sleep and the UE only monitors LP-WUS instead of Paging PDCCH and Paging messages. When the UE receives the LP-WUS, it will leave ultra-deep sleep state. Then, the UE may enter into RRC_CONNECTED mode and exchange information with the network via MR. In this option, the information carried in LP-WUS should include the legacy information from the paging PDCCH (UE identity, SI change indication, ETWS/CMAS indications) as well as the legacy content of the Paging message. When these are aggregated into one single channel (LP-WUS) RAN2 should study some optimization (e.g. remove redundant information). As illustrated in Figure 2, when Paging purpose is triggering RRC Setup, the LP-WUS may result in directly triggering RRC setup/resume procedure i.e. triggering RACH.

Figure 2: Using LP-WUS instead of Paging PDCCH and Paging messages

Observation 2: From RAN2 perspective, it should be analysed the procedure impact from “merging” the multiple legacy signals (PEI, paging PDCCH, paging msg on PDSCH) into one signal.

Different from PEI like solution, when LP-WUS is used to replace paging, more information is needed to be carried in LP-WUS. This will depend on RAN1 design, and will also add complexity in RAN2 considering more advanced adaptation of the paging procedure. LP-WUS reliability and coverage aspects need also to be analysed when LP-WUS conveys the critical ETWS/CMAS indications. Clearly, even at first glance, this option where LP-WUS replaces PEI, paging PDCCH and the paging msg altogether should bring significantly higher power saving benefit compared with the simple PEI replacement option. Hence, it is proposed that:

Proposal 2: RAN2 to study LP-WUS procedure in RRC_IDLE/INACTIVE mode including the following options (to be carefully compared):

- LP-WUS replaces the PEI;

- LP-WUS replaces the paging PDCCH and Paging messages.

2.2	Conditions for LP-WUS monitoring in RRC Idle/Inactive

When the UE is in RRC_IDLE/INACTIVE state, the serving cell is not aware whether the UE is under the cell’s coverage. In order to notify that the serving cell supporting LP-WUS, the straightforward solution is to broadcast this information in system information which is similar to the mechanism of PEI. That is, if LP-WUS configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting LP-WUS can go into ultra-deep sleep and monitor LP-WUS using LP-WUS parameters from system information.

Proposal 3: Receiving LP-WUS configuration broadcasted in system information is one of the conditions for the UE to enable LP-WUS monitoring in RRC_IDLE/Inactive state.

It is suggested by some companies to define some conditions for the UE to check before entering/leaving ultra-deep sleep, for example, checking the signal quality, signal power, detection rate of LP-WUS/synch signal of LP-WUS, the UE’s speed and etc. However, whether/how to define some of these conditions rely on the progress in RAN1. Hence, we suggest that:

Proposal 4: RAN2 waits for more inputs from RAN1 for the pre-defined conditions for entering/leaving LP-WUS monitoring state in RRC_IDLE/Inactive state.

2.3	RRM

In previous RAN1 meetings [3]-[4], the following agreements were made for RRM for LP-WUS:

RAN1#111 Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

RAN1#112 Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements

Regarding RRM for serving cell, some companies propose to use LP-WUS to offload RRM measurement. One of the benefits is to reduce the power consumption for RRM. If LP-WUS is applied to RRM measurement, RAN2 needs to study, for instance,

- the entering as well as the leaving condition for using LP-WUS for RRM;

- the coexistence with legacy reference signal.

Observation 3: If LP-WUS is applied to RRM, the following issues can be further studied:

- the entering as well as the leaving condition for using LP-WUS;

- the coexistence with legacy reference signal.

For neighbor cell measurement using LP-WUS, the key issue is whether the LP-WUS has the same coverage as the legacy reference signal. If the coverage provided by LP-WUS is smaller than the legacy reference signal, the LP-WUS cannot be used for neighbor cell measurement since there will be some areas which are not covered by LP-WUS of the neighbor cells. It was still under discussion in RAN1 on the target coverage as well as evaluation on the coverage performance. It was agreed in RAN1#112bis-e meeting for coverage issue that:

Agreement - Study techniques/mechanisms to enhance coverage performance of LP-WUS - Study potential gains available as well as drawback(s) of the technique(s)/mechanisms(s), e.g. system overhead, increased complexity network energy consumption etc… - Study potential issues and corresponding solutions for the case when LP-WUS coverage is insufficient - At least study fallback mechanisms where the Main Radio switches to legacy operation in case the channel condition of LP-WUS is not sufficient, e.g. below threshold.

Therefore, we think it is still too early for RAN2 to discuss neighbor cell measurements based on LP-WUS. Hence, it is proposed that:

Proposal 5: RAN2 to study RRM for serving cell in RRC_IDLE/RRC_INACTIVE mode.

Proposal 6: RAN2 waits for more inputs from RAN1 to study RRM for neighbour cells in RRC_IDLE/RRC_INACTIVE mode.

Conclusion

In this document, we analyse issues for LP-WUS procedures in RRC_IDLE/INACTIVE state, and we find the observations and proposls as following:

Observations:

Observation 1: Assuming LP-WUS replaces the PEI functionality, the impact on the Idle/Inactive procedures will be quite small.

Observation 2: From RAN2 perspective, it should be analysed the procedure impact from “merging” the multiple legacy signals (PEI, paging PDCCH, paging msg on PDSCH) into one signal.

Observation 3: If LP-WUS is applied to RRM, the following issues can be further studied:

- the entering as well as the leaving condition for using LP-WUS;

- the coexistence with legacy reference signal.

Proposals:

Proposal 1: For IDLE/INACTIVE mode, at least, UE-subgroup information can be carried by the LP-WUS when LP-WUS can be applied to replace PEI.

Proposal 2: RAN2 to study LP-WUS procedure in RRC_IDLE/INACTIVE mode including the following options (to be carefully compared):

- LP-WUS replaces the PEI;

- LP-WUS replaces the paging PDCCH and Paging messages.

Proposal 3: Receiving LP-WUS configuration broadcasted in system information is one of the conditions for the UE to enable LP-WUS monitoring in RRC_IDLE/Inactive state.

Proposal 4: RAN2 waits for more inputs from RAN1 for the pre-defined conditions for entering/leaving LP-WUS monitoring state in RRC_IDLE/Inactive state.

Proposal 5: RAN2 to study RRM for serving cell in RRC_IDLE/RRC_INACTIVE mode.

Proposal 6: RAN2 waits for more inputs from RAN1 to study RRM for neighbour cells in RRC_IDLE/RRC_INACTIVE mode.

Reference

- RAN2#121bis-e meeting report;

- TS 38.300 V17.4.0, NR and NG-RAN Overall Description; Stage 2;

- R1-2300001, Final Report of 3GPP TSG RAN WG1 #111 v1.0.0;

- Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0