3GPP TSG-RAN WG2 Meeting #122		R2-2304937

Incheon, KR, May 22nd – May 26th 2023

Source:	CATT

Title:	Discussion on LP-WUS in RRC_CONNECTED state

Agenda Item:	7.22.3

Document for:	Discussion and Decision

Introduction

In RAN2#121bis-e meeting, preliminary agreements on LP-WUS were agreed, primarily focusing on RRC Idle/Inactive, however it was also clarified: “Confirm that we follow R1 and include RRC idle/inactive/connected”[1]. In this document, we continue to discuss the RAN2 impact for LP-WUS in RRC_CONNECTED mode.

Discussion

2.1	LP-WUS monitoring in RRC Connected

In Rel-16, DCP was specified so that a UE configured with C-DRX may be indicated by DCP whether it is required to monitor or not the PDCCH during the next occurrence of the on-duration according to the configuration. If the UE does not detect a DCP, it does not monitor the PDCCH during the next occurrence of the on-duration, unless it is explicitly configured to do so in that case [2].

In RAN1#112bis-e meeting [3], it was agreed that:

Agreement - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode - Study RRC connected mode LP-WUS functionality/purpose/procedures - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different - In RRC connected, study the relationship between LP-WUS and legacy UE power saving techniques.

It can be seen that MR will not be allowed to go to Ultra-deep sleep in RRC_CONNECTED mode. Hence, when the UE is woken-up by LP-WUS, the latency using LP-WUS may be similar to the latency as DCP. Hence, LP-WUS can be used to replace DCP in RRC connected mode. Based on the agreement above, the following issues can be further studied.

LP-WUS procedure

If LP-WUS is used to replace DCP, the UE can monitor LP-WUS before the occurrence of the on-duration. If the UE does not detect LP-WUS, the UE will not monitor the PDCCH for the next occurrence of the on-duration. The procedure is illustrated in Figure 1.

Figure 1: LP-WUS replacing DCP

Similar to the PEI case in Idle/Inactive, assuming LP-WUS exactly replaces the DCP functionality, the impact on the connected mode procedures will be quite small.

Observation 1: Assuming LP-WUS exactly replaces the DCP functionality, the impact on the connected mode procedures will be quite small.

Proposal 1: RAN2 to study LP-WUS procedure in RRC_CONNECTED where LP-WUS replaces DCP.

Information carried in LP-WUS

And it was agreed in RAN1#112bis-e meeting that:

- For CONNECTED mode, study at least following candidates for content of LP-WUS - information on which user(s) is/are targeted by the LP-WUS - e.g UE-group, -subgroup or -ID

From RAN2 perspective, when LP-WUS is applied to replace DCP, it is likely the PS-RNTI/C-RNTI like information may be carried in LP-WUS. However, whether the UEs are divided into groups is up to the network implementation. From the network’s perspective, multiple UEs may be divided into one group based on the traffic pattern and one X-RNTI may be assigned to the UEs in the group. Moreover, the current DCP explicitly tells the UE to start / not start the drx-onDurationTimer for the next long DRX cycle. However whether this explicit indication also needs to be provided in LP-WUS or would the UE derive this information by other means (e.g. presence/absence of the LP-WUS or of the UE-ID information in the payload) depends on the final LP-WUS design. Hence, at this stage, it is proposed that:

Proposal 2: For CONNECTED mode, at least some UE-group or UE-ID information can be carried by the LP-WUS when LP-WUS intends to replace DCP.

2.2	LP-WUS configuration

If DCP like solution is applied, unlike the UEs in RRC_IDLE/RRC_INACTIVE state, the network is aware of the UE capability and traffic. Hence, the LP-WUS can be configured via dedicated-signaling by the network considering

-	The traffic of the UE;

-	Whether the UE supports LP-WUS, i.e. the UE capability.

When the UE is configured with LP-WUS, the UE monitors LP-WUS. Otherwise, the UE does not monitor LP-WUS.

Proposal 3: If LP-WUS can be applied in CONNECTED mode to replace DCP, the UE monitors LP-WUS when it receives the LP-WUS configuration via UE-dedicated signaling.

2.3	RRM

In previous RAN1 meetings [3]-[4], the following agreements were made for RRM for LP-WUS:

RAN1#112 Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements

RAN1#112bis-e Agreement - For RRC connected mode, the following is assumed for LP-WUS study in RAN1 - RLM/BFD/CSI are performed by UE Main Radio (MR) - RRM measurements are performed by UE Main Radio (MR) - Ultra-deep sleep state is not allowed for MR. - Study additional support of RRM measurement by LP-WUR for RRC connected mode - Study RRC connected mode LP-WUS functionality/purpose/procedures - Study RRC connected mode LP-WUS activation/deactivation procedures. - Study RRC connected mode LP-WUS BW, whether same as IDLE/Inactive mode or different - In RRC connected, study the relationship between LP-WUS and legacy UE power saving techniques.

Based on the agreements highlighted in yellow, it can be seen that RRM measurement in RRC_CONNECTED mode is performed by UE MR and further study on additional support of RRM measurement by LP-WUR for RRC connected mode will continue. But whether additional support of RRM measurement is needed by LP-WUS highly depends on the design in RAN1 and RAN4. The issues mentioned in RAN1 agreements still require further investigation in RAN1, for instance:

  - Whether LP-WUS and signals/channels used by MR can be different carriers in the band

  - Details on the LP-WUS location within a carrier

  - Band can be different than band of signals/channels used by MR

  - LP-WUS association with BWP

  - LP-WUS can be configurable within guard-band of a band (like NB-IoT)

Hence, it is proposed that:

Proposal 4: RAN2 waits for further RAN1 progress before studying the use of LP-WUS for RRM measurements in RRC_CONNECTED mode.

Conclusion

In this document, we analyse issues for LP-WUS procedures in  RRC_CONNECTED state, and we find the observations and proposls as following:

Observations:

Observation 1: Assuming LP-WUS exactly replaces the DCP functionality, the impact on the Connected mode procedures will be quite small.

Proposals:

Proposal 1: RAN2 to study LP-WUS procedure in RRC_CONNECTED where LP-WUS replaces DCP.

Proposal 2: For CONNECTED mode, at least some UE-group or UE-ID information can be carried by the LP-WUS when LP-WUS intends to replace DCP.

Proposal 3: If LP-WUS can be applied in CONNECTED mode to replace DCP, the UE monitors LP-WUS when it receives the LP-WUS configuration via UE-dedicated signaling.

Proposal 4: RAN2 waits for further RAN1 progress before studying the use of LP-WUS for RRM measurements in RRC_CONNECTED mode.

Reference

- RAN2#121bis-e meeting report;

- TS 38.321 V17.4.0, Medium Access Control (MAC) protocol specification;

- RAN1#112bis-e meeting report;

- RAN1#112 meeting report;