3GPP TSG-RAN WG2 Meeting #122	R2-2304951

Incheon, Korea, May 22 – 26, 2023

Agenda item:	7.4.2.1

Source: 	Fujitsu

Title: 	General aspects for L1/L2 triggered mobility procedure

Document for:	Discussion and decision

1. Introduction

Regarding the procedure of L1/L2 triggered mobility, RAN2 has reached some further agreements at last e-meeting [1]:

R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. In RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA and on the reception of Msg3/MsgA in CBRA, like the legacy HO. In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) For RACH-based LTM, the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data.

This contribution will discuss the general aspects for L1/L2 triggered mobility procedure, involving:

- Modifications to the procedure of LTM in 38.300 running CR

- RAN3 LS on approaches during execution for inter-DU LTM

2. Discussion

2.1 Modifications to the procedure of LTM

At RAN2#121 meeting, Figure 1 is endorsed as baseline for the procedure of LTM:

Figure 1: Signalling procedure for LTM [2]

According to RAN2 agreements, in RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE and RRCReconfigurationComplete message can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival. So, we propose to change the step 8 in the signaling procedure for LTM in the running CR to the transmission of RRCReconfigurationComplete message.

Proposal 1: The step 8 (LTM completion) is changed to a UL RRC message, i.e. RRC reconfiguration complete.

If Proposal 1 is agreed, the signalling procedure for RACH-less LTM can be revised as:

Figure 2: Signalling procedure for RACH-less LTM (revised for P1)

In addition, there is an Editor’s note to capture that FFS whether a uplink signal or message after the UE has switched to the target cell is used to indicate successful completion of the LTM cell switch. According to these RAN2 agreements, i.e. RRCReconfigurationComplete message can be the content of the first UL MAC PDU/transmission to indicate UE arrival and no need to introduce any new signaling to indicate UE arrival, the FFS can be removed for the UL message to indicate successful completion of the LTM cell switch, but we think we have not discussed whether a uplink signal is used or not. So, we propose to keep the FFS for the uplink signal. For example, SRS transmission can be used for early indication.

Proposal 2: Modify the Editor' notes to "FFS whether the UE access to the target cell based on an uplink signal in case of RACH-less LTM procedure."

According to RAN2 agreements, in RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA or on the reception of Msg3/MsgA in CBRA while the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. So, in case of RACH-based LTM, some steps in RACH procedure, e.g. the reception of preamble in CFRA or the reception of Msg3/MsgA in CBRA, are a part of LTM completion phase rather than LTM execution phase. However, RACH procedure is included in the LTM execution phase in the signaling procedure for LTM in the running CR. So, we propose to modify this to align with RAN2 agreements.

Proposal 3: RAN2 to discuss which one of the following changes will be applied:

- Alt.1: to remove the box of LTM completion

- Alt.2: to include RACH Procedure in the box of LTM completion

Proposal 3bis: RAN2 to include two figures to capture the procedures for RACH-less LTM and RACH-based LTM respectively.

If Alt.1 in Proposal 3 is agreed, the signalling procedure for RACH-based LTM can be revised as:

Figure 2: Signalling procedure for LTM (revised for Alt. 1 of P3)

If Alt.2 in Proposal 3 is agreed, the signalling procedure for RACH-based LTM can be revised as:

Figure 2: Signalling procedure for LTM (revised for Alt. 2 of P3)

2.2 RAN3 LS on approaches during execution for inter-DU LTM

Following approaches are asked by RAN3 [3]:

RAN3 has discussed the following two approaches to support inter-DU LTM cell switch during execution. Approach 1: the serving gNB-DU triggers the execution by transmitting LTM cell switch command to the UE and then informs the gNB-CU of the serving cell switch. Approach 2: the serving gNB-DU first requests information from target DU before triggering LTM cell switch command to the UE. RAN3 would like to get feedback from RAN2 about the above-mentioned approaches, and provide suggestion if there is any other possibility identified.

In our understanding, in Approach 2, the serving gNB-DU first requests information from target DU immediately before triggering the LTM cell switch command to the UE. It means that Approach 2 introduces addition latency to the LTM execution phase to make LTM decision at the serving gNB-DU and the latency on F1 interface between the serving gNB-DU, gNB-CU and the target gNB-DU will be counted in the mobility latency though it has no impact to the interruption time. Besides, the latency could incur the failure of the LTM cell switch command transmission and thus the LTM cell switch cannot be triggered.

Regarding Approach 1, the information exchange will happen before the LTM execution phase, e.g. during the LTM preparation phase and thus no addition latency is introduced to the mobility latency. So, we think that Approach 1 is better.

Observation: Approach 1 in the RAN3 LS is RAN2 preference.

In addition, to perform inter-DU LTM cell switch, the information which can be exchanged on F1 interface between the serving gNB-DU, gNB-CU and the target gNB-DU at least include RACH resources and TA value(s) or RAR for early UL synchronization, TCI states pool for beam application before or at the LTM cell switch and the L1 measurement RS configuration.

Proposal 4: To support inter-DU LTM cell switch, target DU needs to provide the following information to serving DU, e.g. in LTM preparation phase:

- RACH resources

- TA value(s) or RAR

- TCI states pool

- L1 measurement RS

Considering RAN3 is the WG to design the signalling on F1 interface, the related conclusions should be informed to RAN3, if reached.

Proposal 5: In the LS to RAN3, RAN2 can include the above observation and Proposal 4.

3. Conclusion

In this contribution, we have some discussions on the general aspects for L1/L2 triggered mobility procedure and the following observation and proposals are made:

Proposal 1: The step 8 (LTM completion) is changed to a UL RRC message, i.e. RRC reconfiguration complete.

Proposal 2: Modify the Editor' notes to "FFS whether the UE access to the target cell based on an uplink signal in case of RACH-less LTM procedure."

Proposal 3: RAN2 to discuss which one of the following changes will be applied:

- Alt.1: to remove the box of LTM completion

- Alt.2: to include RACH Procedure in the box of LTM completion

Proposal 3bis: RAN2 to include two figures to capture the procedures for RACH-less LTM and RACH-based LTM respectively.

Observation: Approach 1 in the RAN3 LS is RAN2 preference.

Proposal 4: To support inter-DU LTM cell switch, target DU needs to provide the following information to serving DU, e.g. in LTM preparation phase:

- RACH resources

- TA value(s) or RAR

- TCI states pool

- L1 measurement RS

Proposal 5: In the LS to RAN3, RAN2 can include the above observation and Proposal 4.

References

- Chairman notes for RAN2#121bis-e

- R2-2302039 38.300 running CR for introduction of NR further mobility enhancements, MediaTek Inc., RAN2#121

- R2-2302458 LS on Approaches during execution for inter-DU LTM (R3-230889; contact: Ericsson) RAN3	LS in