3GPP TSG RAN WG2 #122                                R2-2304959

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	7.16.2.1

Source:	Fujitsu

Title:   	Discussions on Model-ID and Functionality-ID based LCM

Document for:	Discussion/Decision

1. Introduction

Mode-ID and Functionality-ID have invoked several rounds of discussions in both RAN1 and RAN2, the debates are mainly focused on the following aspects:

- Applicability and benefits for applying model ID and/or functionality ID.

- Functionality and model ID definitions.

- Mechanisms of functionality and model identification.

- Model ID format and related meta information contents.

In this contribution, we give our views on further discussion of model ID and functionality-ID based LCM, the proposals given here are based on the agreements from both RAN1 and RAN2 meetings.

# 2. Discussion

# We would like to organize our discussions on the basis of the above-mentioned four aspects.

2.1 Applicability and benefits for applying model ID and functionality ID.

From very beginning of the AI/ML discussion, model ID was proposed as one essential part of AI/ML model lifecycle management since the models must be identified during status change, especially for model switching/selection. In the original consideration, many companies assumed the model ID can be assigned by the model deployment entity in a certain format, the ID will be reported to network if the current LCM requires and will be discarded when the LCM ceases to exist. However, there are some objections from other companies, they believe that both NW-side and UE-side model should be transparent to the other side, e.g., NW may only need to know the purpose of the UE application and make corresponding configurations without knowing if AI/ML or legacy methods will be used to realize the application. Therefore, the applicability of model ID becomes if model identification is necessary, and if model identification is inevitable, model ID is naturally used for the identification procedure.

Another hot topic being discussed is model transfer/delivery, the model has to be identified before transfer, or in other word, the model details have to be informed to the other side otherwise the entire transmission may be failed with high possibility. If all model information transmitted in a random way via air interface, the efficiency and effectiveness cannot be guaranteed and the information exchange between UE and network will be chaotic. Therefore, the benefit of model ID (or model identification) is to improve the network efficiency during the LCM procedures at minimum information exchange overhead.

Similar to model ID, functionality ID was proposed on the basis of model ID discussion, the purpose of functionality ID is to merge the model identification procedure into the current specification framework without explicit model ID design. It regards the AI/ML model training, inference, monitoring or other LCM procedure as configurable application request based on entity capabilities. Therefore, the applicability of functionality ID can be regarded as implicit model identification.

The benefit for applying functionality ID is to avoid too many new designs on model-related conceptions, the AI/ML model is one sort of implementation to address certain use cases, since AI/ML is an emerging technology with extremely rapid development, the fixed design such as RRC message or DCI format may lag behind the state-of-the-art AI/ML technic so it is unwise to have some inflexible pre-assumption on the model description format or contents.

Based on the analysis above, we have the following observations:

Observation 1 Both model ID and functionality ID are applicable for model identification which triggered by AI/ML related use case applications.

Observation 2 The model identification is always necessary for AI/ML model related issues, even AI/ML model will not be used after the identification.

Observation 3 The benefit by applying model ID is to exchange model-related information in the most efficient way.

Observation 4 The benefit by applying functionality ID is to avoid brand new design for AI/ML model implementation with rapid development.

It is obvious that both functionality ID and model ID are applicable if model identification is necessary, and both solutions have their pros and cons during the model identification procedures. Therefore, we suggest not to preclude either of the two solutions, and the co-existence framework to mitigate the two IDs should be studied.

Proposal 1 Both functionality ID and model ID are supported to deal with model identification issues. The co-existence framework for the two IDs should be studied.

2.2 Functionality and model ID definitions

Model

It seems that model ID definition is much clearer due to the definition of model itself is clear. In one word, the models with different IDs should be considered as different models, so the issue becomes how to distinguish one model from another model. There has already been some discussions and the consensus have been reached without an official agreement, the models with 1) different model structure or 2) same model structure with different parameters can be regarded as two different models, so the model ID must reflect the difference and the reason behind the difference, e.g., the model parameter change can be resulted from either model re-training or fine-tuning, or two models with same structure (the structure type may be limited by use case) but initially trained for different scenarios.

Observation 5 Different AI/ML models can be distinguished by two cases: 1) with different structures; 2) with same structure but different parameters.

Observation 6 The reasons for models with different structure or parameters are important for model LCM functions.

Proposal 2 Rather than a simple index, model ID should be designed to be able to reflect the model structure or parameter selection or change during the model LCM procedure, such as:

- Why the current model structure and parameters are selected.

- Why the model structure or parameters are updated or modified.

These messages can be included in either the model ID or the related model meta information depends on use cases and wireless conditions.

Functionality

On the other hand, functionality ID is not straightforward to be designed since how to define the AI/ML functionality is difficult. One option is to use LCM such as model training, inference, monitoring, updating to define the functionality, but these are more like model actions which includes a set of signaling exchange and configurations. After several rounds of discussion in RAN1, the following agreement is given:

Agreement For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. Agreement AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Based on the above agreement, the definition of functionality becomes the definition of AI/ML-enabled Feature. It is a quite broad conception because AI/ML may involve many different aspects to be used. A rough classification is to divide the Feature into two classifications: capability-based and requirement based. The capability-based feature refers to the entity itself, e.g., if one UE can support AI/ML training, while the requirement-based feature refers to the user application requirement, e.g., positioning accuracy and latency requirements.

Proposal 3 The AI/ML-enable functionalities may have capability-based and requirement-based, the, the capability-based functionality may be reported in static mode and requirement-based may be reported in more dynamic mode, Mechanisms on the reporting of both the two sorts of functionalities should be studied in RAN2.

Proposal 4 Potential functionality identifier should be able to reflect the capability and requirement-based feature or feature groups.

2.3 Model ID and Functionality identification mechanisms

Model ID

There are multiple solutions to implement model identification by model ID or functionality identification, as we described above, model ID is the explicit way to identify AI/ML model while functionality identification is the implicit way.

For model ID, RAN2 has agreed to have one global ID and the details are still pending for FFS.

- RAN2 assumes that Model ID is unique “globally”, e.g., in order to manage test certification each retrained version need to be identified.

In the last email discussion [1] on model ID, the ID globality called for some discussion and the following agreement is given:

How to achieve globality of the Model ID is FFS. - Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point: - Direction1: Pre-defined/hard-coded global unique model ID - Direction3: Assigned global unique model ID via specific ID management node. - Note: Other global unique model ID definition is not precluded.

There are some clarifications need to be made on the “globality”, one option is to consider the model ID as “real global” which means once a model ID has been assigned then this model ID will be unique all over the word and for all the time, it is similar to the QR code which one generated pattern will be unique and will never be generated again forever. If so, then direction 1 may be applicable to let all the vendors to maintain one table as the PLMN and then the model ID will include vendor information, device information and more AI/ML specific information.

However, it is not efficient to assign the global model in this way, first of all, the model ID will be extremely overhead consuming which may be one long random sequence for the network to capture useful information carried by the sequence, secondly, actually the globality for AI/ML model may not be such “global”, e.g., if one AI/ML model used in an European UE in 2023 share the same model ID with a model used in an American UE in 2033, there will be no problem at all.

Therefore, we propose to further study the 3rd direction to make the model ID “manageable”, the model ID can be created and erased, it can also be reassigned to another model if the previous model has been deactivated or re-trained. The details of the specific ID management node should be further studied, e.g., how many UEs or models can one such node be used to manage their IDs, how to align information between the node with vendors and the networks, and how to guarantee the globality during mobility.

Observation 7 The Direction 1 is basically offline and may be out of RAN scope.

Proposal 5 Direction 3, which allows model ID to be assigned and managed by specific node should be further studied, especially on the details of the node implementation.

On the other hand, even if the “globality” can be assured by management procedure, but it still cannot meet misc. requirement for multiple LCM functions, in order to maintain the globality, the model ID has to contain multiple information at least for the vendor, cell or use case information, if the global ID has to be reported on each LCM function, the overhead will still be very high for the air interface.

Therefore, we propose to maintain the hierarchical structure for model ID, it can be realized by one global ID and one local ID (or local index), or one global ID and multiple local IDs, the details which information should be carried by different IDs can be further studied. The purpose of hierarchical model ID structure is to save air interface overhead and improve network efficiency.

Proposal 6 In addition to global model ID, one or more local IDs should be supported as supplementary of the global model ID, the details such as the information carried by local IDs can be further studied.

Functionality Identification

It has already agreed on the UE capability-based functionality identification mechanism, the basic procedure is UE report its supported or preferred functionalities via UE capability. In an unofficial email discussion arranged by RAN1, two alternatives are given for down selection.

Alt 1: They have the same meaning, as “functionality identification” already reflects NW capability/interest, so “identified functionalities” refers to what NW configures to UE. Alt 2: “Configured functionalities” is a subset of “identified functionalities”, as “identified functionalities” refers to what NW could potentially configure to UE (i.e., intersection of conditions indicated by UE capability and NW’s capability), and “configured functionalities” refers to what NW actually configures (e.g., via RRC) to UE as a subset of “identified functionalities” (i.e., intersection of conditions indicated by UE capability, NW’s capability, and NW’s interest).

Alt1 basically prefers one-side reporting as the final functionality while alt2 prefers to let the network also indicates its interests or preferences to UE, the configuration of the final functionality will be the intersection of the two preferences.

Generally speaking, we propose to prioritize alternative 2 for further study since the NW has flexibility to optimize or configure its preference so both UE and NW sides capability can be considered together.

Proposal 7 It is suggested to consider both UE and NW capabilities (interests) as the basis of functionality identification configuration, how to figure out the intersections of UE and NW capabilities should be studied.

The most straightforward way to find the intersection of UE and NW capabilities is to follow the proposal below:

Proposal 8 Study potential RAN2 framework to realize the following procedure as the baseline of functionality identification:

- UE reports its capabilities via UE capability to network.

- NW configures functionalities based on reported UE capabilities based and its own interests.

- UE may update its applicable capabilities later to NW.

Furthermore, some enhancements should also be considered, first of all, UE capability should not be considered as the only mechanism to report functionality, other mechanisms should not be precluded; secondly, NW may consider having some early configuration to UE before UE capability reporting to limit the contents reported from UE in order to save overhead, thirdly, the granularity of the functionality should be carefully considered, otherwise, big expansion will be necessary for the UE capability entities for AI/ML.

Proposal 9 Study potential RAN2 signaling and framework on the following enhancements of functionality identification.

- More frameworks for UE functionality reporting.

- NW pre-requests for the UE functionality reporting contents.

- Granularity of functionality information which need to be expanded in UE capability.

2.4 Model meta information

In the offline email discussion [1], several possibilities for the AI/ML model meta information are given for discussion:

Open issue 6: RAN2 to further study the content of meta data, the following bullets can be considered as the starting point for future study: a, model input info; b, model output info. c, model version info; d, model format info; e, required AI capability. f, vendor info. g, applicable scenario, configuration, site information. h, computational complexity: FLOPs, level of pre-/post-processing. i, model complexity: number of real-value model parameter, number of real-value operations; j, model size; k, model performance: Model accuracy, model bias, model variance; l, model functionality; m, use case. Note: The coordination with RAN1 is needed.

Which information is most important depends on specific AI/ML use cases, however, we think model input, model output, applicable scenario, configuration, site information and model size should be included in meta information because they are inevitable in most LCM and use cases, e.g., model input/output information are necessary for training/inference/monitoring/updating and even model transfer/delivery because the receiver side needs to know whether the model input can be supported. Model format information are also important, but it depends on which format (e.g., vendor-specific or globally aligned) will model ID and meta information apply.

Proposal 10 The following information should be included into model ID meta information.

- Model input/output.

- Model applicable scenario, configuration, site information.

- model size information.

Other information such as model format is considered as FFS.

# 3. Conclusion

Based on the above discussions, we give the following observations:

Observation 1 Both model ID and functionality ID are applicable for model identification which triggered by AI/ML related use case applications.

Observation 2 The model identification is always necessary for AI/ML model related issues, even AI/ML model will not be used after the identification.

Observation 3 The benefit by applying model ID is to exchange model-related information in the most efficient way.

Observation 4 The benefit by applying functionality ID is to avoid brand new design for AI/ML model implementation with rapid development.

Observation 5 Different AI/ML models can be distinguished by two cases: 1) with different structures; 2) with same structure but different parameters.

Observation 6 The reasons for models with different structure or parameters are important for model LCM functions.

Observation 7 The Direction 1 is basically offline and may be out of RAN scope.

Based on the above discussions, we give the following proposals:

Proposal 1 Both functionality ID and model ID are supported to deal with model identification issues. The co-existence framework for the two IDs should be studied.

Proposal 2 Rather than a simple index, model ID should be designed to be able to reflect the model structure or parameter selection or change during the model LCM procedure, such as:

- Why the current model structure and parameters are selected.

- Why the model structure or parameters are updated or modified.

Proposal 3 The AI/ML-enable functionalities may have capability-based and requirement-based, the, the capability-based functionality may be reported in static mode and requirement-based may be reported in more dynamic mode, Mechanisms on the reporting of both the two sorts of functionalities should be studied in RAN2.

Proposal 4 Potential functionality identifier should be able to reflect the capability and requirement-based feature or feature groups.

Proposal 5 Direction 3, which allows model ID to be assigned and managed by specific node should be further studied, especially on the details of the node implementation.

Proposal 6 In addition to global model ID, one or more local IDs should be supported as supplementary of the global model ID, the details such as the information carried by local IDs can be further studied.

Proposal 7 It is suggested to consider both UE and NW capabilities (interests) as the basis of functionality identification configuration, how to figure out the intersections of UE and NW capabilities should be studied.

Proposal 8 Study potential RAN2 framework to realize the following procedure as the baseline of functionality identification:

- UE reports its capabilities via UE capability to network.

- NW configures functionalities based on reported UE capabilities based and its own interests.

- UE may update its applicable capabilities later to NW.

Proposal 9 Study potential RAN2 signaling and framework on the following enhancements of functionality identification.

- More frameworks for UE functionality reporting.

- NW pre-requests for the UE functionality reporting contents.

- Granularity of functionality information which need to be expanded in UE capability.

Proposal 10 The following information should be included into model ID meta information.

- Model input/output.

- Model applicable scenario, configuration, site information.

- model size information.

Other information such as model format is considered as FFS.

# 4. Reference

[1] R2-2304195 Report of [AT121bis-e][014] [AIML18] Model ID (incl meta data) progress (OPPO)_v35_Summary2