3GPP TSG-RAN WG2 Meeting #122                                                     R2-2304973

Incheon, KR, May 22-26, 2023

Source:              CATT

Title:	             Discussion on RAN feedback

Agenda Item:	7.23.2

Document for:	Discussion and Decision

Introduction

RAN feedback for BAT adaptation is an objective of R18 URLLC [1]. RAN2 issue is focused on the reactive RAN feedback for UL scheduling adaptation.

3. Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2]: a. RAN enhancements in order for application to adapt scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication. Note 3: Reactive RAN feedback for upstream scheduling is pending RAN2 conclusion on burst arrival time (BAT) offset derivation.

Below agreement was reached in RAN2#121bis-e:

Agreements

=>	We will wait for XR to progress and see if we can use the existing mechanism. RAN2 will strive to re-use existing mechanism or rely on gNB to determine the information (i.e. aim to not introduce new UE specific BAT reporting)

This contribution will discuss this issue based on XR progress.

Discussion

RAN feedback for BAT offset is an effective mechanism to guarantee GFBR of URLLC services. The BAT offset is used to align the arrival of the traffic bursts with the next expected transmission opportunity over the air interface. For the periodic deterministic communication, it means adjust the periodicity of transmission occasions, or burst sending time according to the provided BAT offset.

The key point to discuss in RAN2 is how gNB acquires the BAT offset. In last meeting, some solutions were considered: BAT reporting [3], BSR report (may consider delay report) [4][5][6], and the report of BAT offset value plus periodicity [7].

However, in last meeting, RAN2 agreed UE can report UL jitter information [8], similar to CN providing such jitter information for DL flows. Hence, similar to DL, the UL jitter information can be associated with a definite periodicity which can be acquired by gNB from CN [2].

3.	UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting.

The design of the jitter information report is yet to be finalized in XR WI, however it seems it can be 100% reused for the purpose of BAT reporting. That is, such information is sufficient for the gNB to derive the offset between the burst arrival time and the transmissions occasions. Hence no other solution or information is needed for reactive RAN feedback.

Proposal 1: As a baseline, reusing the jitter information report in XR, gNB can derive the BAT offset accurately. No other UE specific BAT reporting is needed.

Conclusion

According to the analysis in section 2, we provide below proposal.

Proposal 1: As a baseline, reusing the jitter information report in XR, gNB can derive the BAT offset accurately. No other UE specific BAT reporting is needed.

References

- RP-230754, New WID on NR Timing Resiliency and URLLC enhancements

- TS 23.501, System architecture for the 5G System (5GS), V18.1.0

- R2-2302690	Discussion on UL reactive RAN feedback for burst sending time adjustment	Huawei

- R2-2303866	Reactive RAN feedback for upstream scheduling	Nokia

- R2-2302933	UL BAT Reporting and Adjustment	Qualcomm

- R2-2304153	Adaptive Upstream Scheduling Based on RAN Feedback	Samsung

- R2-2302722	Discussion on RAN Feedback for Low Latency Communication 	vivo

- R2-2304202, Report from session on LTE legacy, XR, QoE and Multi-SIM

Annex: Reactive RAN feedback for BAT adaptation in TS 23.501

#### 5.27.2.5	RAN feedback for Burst Arrival Time offset and adjusted Periodicity

##### 5.27.2.5.1	Overview

If the NG-RAN receives a TSCAI containing a BAT Window or the Capability for BAT adaptation for a QoS Flow, the NG-RAN can determine a BAT offset in order to align the arrival of the traffic bursts with the next expected transmission opportunity over the air interface in each direction (i.e. DL or UL). The BAT offset can take a positive or a negative values.

If the NG-RAN receives a TSCAI containing a Periodicity Range for a QoS Flow, the NG-RAN can determine an adjusted Periodicity along with above specified BAT offset, in order to align the periodicity of the traffic bursts with the expected time interval between subsequent transmission opportunities over the air interface in each direction (i.e. DL or UL).

NG-RAN may support the following feedback mechanisms:

-	Proactive RAN feedback for adaptation of Burst Arrival Time and Periodicity: NG-RAN may provide a Burst Arrival Time offset and an adjusted Periodicity as part of QoS flow establishment or modification as illustrated in clause 5.27.2.5.2;

-	Reactive RAN feedback for Burst Arrival Time adaptation: NG-RAN may provide a Burst Arrival Time offset after QoS flow establishment as illustrated in clause 5.27.2.5.3.

##### 5.27.2.5.3	Reactive RAN feedback for Burst Arrival Time adaptation

If the RAN receives the capability for BAT adaptation without a Burst Arrival Time in the TSCAI and notification control is enabled for this QoS Flow, the 5GS will perform the following actions:

-	If NG-RAN determines that the PDB of the QoS flow cannot be fulfilled in DL and UL direction, then if supported, NG-RAN shall determine a BAT offset value which reduces the time between the arrival of the traffic bursts and the time of the next possible transmission over the air interface for DL and UL, respectively. NG-RAN shall not provide a BAT offset with the same value until the PDB of the QoS Flow can be fulfilled again.

NOTE:	NG-RAN determines BAT offset value in reference to the current arrival time of the bursts experienced by RAN in DL and by UE in UL. Further details on BAT offset determination for DL and UL will be defined by RAN WG2.

-	The BAT offset is provided from NG-RAN to the SMF when sending the notification towards the SMF that the "GFBR can no longer be guaranteed" described in clause 5.7.2.4. The SMF provides the BAT offset to the PCF and the PCF provides the BAT offset to the AF as part of notifying the AF as described in clause 6.1.3.23a of TS 23.503 [45]