3GPP TSG-RAN WG2 Meeting #122	R2-2304974

Incheon, Korea, 22nd– 26th April 2023

Agenda Item:	7.24.2

Source: 	vivo

Title:         	      Discussion on MUSIM paging cause forwarding

Document for: 	Discussion and Decision

# Introduction:

Rel-17 MUSIM feature supports MUSIM PagingRecord-v1700 forwarding on Uu as follows:

1> if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message, or 1> if in RRC_IDLE, for each of the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE: 2> if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers: 3> if upper layers indicate the support of paging cause: 4> forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers; 3> else: 4> forward the ue-Identity and accessType (if present) to the upper layers;

On another hand in Rel-17 SL relay Relay UE also supports of paging record forwarding to remote UE as follows.

1> if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message, or 1> if in RRC_IDLE, for each of the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE: 2> if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers: 3> if upper layers indicate the support of paging cause: 4> forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers; 3> else: 4> forward the ue-Identity and accessType (if present) to the upper layers;

Paging message -- ASN1START -- TAG-PAGING-START Paging ::= SEQUENCE { pagingRecordList PagingRecordList OPTIONAL, -- Need N lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension Paging-v1700-IEs OPTIONAL } Paging-v1700-IEs ::= SEQUENCE { pagingRecordList-v1700 PagingRecordList-v1700 OPTIONAL, -- Need N pagingGroupList-r17 PagingGroupList-r17 OPTIONAL, -- Need N nonCriticalExtension SEQUENCE {} OPTIONAL } PagingRecordList ::= SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord PagingRecordList-v1700 ::= SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1700 PagingGroupList-r17 ::= SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 PagingRecord ::= SEQUENCE { ue-Identity PagingUE-Identity, accessType ENUMERATED {non3GPP} OPTIONAL, -- Need N ... } PagingRecord-v1700 ::= SEQUENCE {pagingCause-r17 ENUMERATED {voice} OPTIONAL -- Need N } PagingUE-Identity ::= CHOICE { ng-5G-S-TMSI NG-5G-S-TMSI, fullI-RNTI I-RNTI-Value, ... } -- TAG-PAGING-STOP -- ASN1STOP

But, current SL relay specification does not support MUSIM PagingRecord-v1700 forwarding from Relay UE to Remote UE.

At last RAN#121nis-e meetings, RAN2 has discussed [1] adding PagingRecord-v1700 in UuMessageTransferSidelink message to support MUSIM paging cause forwarding from Relay UE to Remote UE. RAN2 has agreed that:

Agreement:

Forwarding paging cause by L2 U2N Relay UE is not supported in Rel-17.

This contribution further discusses scenarios for SL Relay UE forwarding of MUSIM paging cause scenarios in Rel-18.

# Discussion

Figure 1: Basic assumption for SL Relay and MUSIM Remote UE

As rel-17 SL relay as baseline deployment of MUSIM Remote UE deployment can be considered as shown in Fig 1:

- NWA and NWB are 2 independent networks without any coordination

- MUSIM UE (UEsim1) can connect with NWA only through Relay r to NWA gNB.

- MUSIM UE (UEsim2) can connect with NWB gNB Uu

Based on above assumption the following cases can be considered:

- Case 1: - Remote UEsim1 reselects to Relay r in NWA and stays in RRC_IDLE/INACTIVE or RRC_CONNECTED in NWA and Usim2 in RRC_IDLE/INACTIVE in NWB.

Figure 2: MUSIM UEsim1 in RRC_IDLE/INACTIVE in NWA through Relay UE.

NWA may page UEsim1 with MUSIM paging cause:

- For r in RRC IDLE or RRC_INACTIVE or RRC_CONNECTED on an active BWP with common search space configured including pagingSearchSpace, r can monitor remote UEsim’s PO.

- For r in RRC_CONNECTED on an active BWP without pagingSearchSpace configured: in this case r cannot monitor remote UE’s PO, instead the NWA can provide the UEsim’s Paging message container within RRCReconfiguration message

- Case 2: Remote UEsim1 reselects to Relay r in NWA and stays in RRC_IDLE/INACTIVE or RRC_CONNECTED in NWA and UEsim2 in RRC connected in NWB

Figure 3: MUSIM UEsim2 in RRC_CONNECTED in NWB

NWA may page UEsim1 with MUSIM paging cause. Similar to Case 1,

- For r in RRC IDLE or RRC_INACTIVE on an active BWP with common search space configured including pagingSearchSpace, r can monitor remote UEsim1’s PO.

- For r in RRC_CONNECTED on an active BWP without pagingSearchSpace configured: in this case r cannot monitor remote UE’s PO, instead the NWA can provide the UEsim1’s Paging message container within RRCReconfiguration message

Observation 1	NW may page a MUSIM Remote UE in following cases:

    - MUSIM remote UE in RRC_IDLE/INACTIVE in NWA with sim1 (via a relay UE) and in RRC_IDLE/INACTIVE in NWB with sim2 (via Uu)

    - MUSIM remote UE in RRC_IDLE/INACTIVE in NWA with sim1 (via a relay UE) and in RRC_CNNECTED in NWB with sim2 (via Uu)

Observation 2	In both cases described in Observation 1, relay UE may forward MUSIM paging record to MUSIM remote UE.

Therefore,

- RAN2 to discuss and agree to further specify SL relay UE forwarding MUSIM paging record in the following cases

  - MUSIM remote UE in RRC_IDLE/INACTIVE in NWA with one sim card (via a relay UE) and in RRC_IDLE/INACTIVE in NWB with another sim card (via Uu)

  - MUSIM remote UE in RRC_IDLE/INACTIVE in NWA with sim card (via a relay UE) and in RRC_CNNECTED in NWB with another sim card (via Uu)

# Conclusion

In this paper, we discuss scenarios for SL Relay UE forwarding of MUSIM paging cause scenarios in and the following proposals are given:

Observation 1	NW may page a MUSIM Remote UE in following cases:

    - MUSIM remote UE in RRC_IDLE/INACTIVE in NWA with sim1 (via a relay UE) and in RRC_IDLE/INACTIVE in NWB with sim2 (via Uu)

    - MUSIM remote UE in RRC_IDLE/INACTIVE in NWA with sim1 (via a relay UE) and in RRC_CNNECTED in NWB with sim2 (via Uu)

Observation 2	In both cases described in Observation 1, relay UE may forward MUSIM paging record to MUSIM remote UE.

And

- RAN2 to discuss and agree to further specify SL relay UE forwarding MUSIM paging record in the following cases

  - MUSIM remote UE in RRC_IDLE/INACTIVE in NWA with one sim card (via a relay UE) and in RRC_IDLE/INACTIVE in NWB with another sim card (via Uu)

  - MUSIM remote UE in RRC_IDLE/INACTIVE in NWA with sim card (via a relay UE) and in RRC_CNNECTED in NWB with another sim card (via Uu)

# References

- RAN2#121bis e, Chairman’ Minutes;