3GPP TSG-RAN WG2 Meeting #122	 R2-2304988

Incheon Korea, May 22 – May 26, 2023

Agenda Item:	7.22.2

Source: 	NEC

Title:  	Discussion on the considerations for LPWUS in RRC_IDLE INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#121b meeting [2], we had the agreement as below:

Aim to do every Q: Collect RAN2 text proposals in a single document during the following meeting(s) and send the document to RAN1 as the input to the TR 38.869.

Confirm that we follow R1 and include RRC idle/inactive/connected.

Ultra-deep-sleep = R2 understands for now that this is a power saving state (introduced by R1) to denote a state when the Main Receiver (MR) may sleep/turn off.

In scope: Use LPWUS with Idle / Inactive UE camping with reception of paging and other necessary transmissions (from serving cell), reusing if possible/reasonable concepts from earlier releases, where the LPWUS either wakes the UE to receive by MR, or it conveys information by itself, or both.

It can be seen that for the current stage, RAN2 will analyse the feasibility / design for LPWUS applied in all RRC states including RRC_CONNECTED. In this paper, we discuss the issues when LPWUS applied in RRC_IDLE/INACTIVE.

- 2 Discussion

- 2.1 Common issues

2.1.1 The definition of LPWUS mode

Fig.1 LPWUS framework

As shown in Fig.1, in this mechanism, a new separated/integrated receiver (i.e., low power wake up receiver, LPWUR) is introduced to monitor low power wake up signal (LPWUS). And main radio (MR) is to describe the regular communication device (i.e., NR device today), in which operations related to RRC_IDLE/INACTIVE/CONNECTED state are performed. The fundamental of LPWUS mechanism is that UE may turn off its MR or keep it in ultra-deep-sleep when there is no any data/signal coming but only using a separated LPWUR to monitor a simple signal, i.e., LPWUS for possible data/signal so that more power saving gain is achieved.

It is noted that in legacy, UE who is accessing network anyway needs to stay in at least one of RRC states, but now since LPWUR is introduced and the regular device, i.e., MR could be turned off, so in this kind of “LPWUS” mode, what UE needs to do maybe only to monitor LPWUS (or LP-synchronization signal, reference signal if possible) without actions related to RRC_IDLE/INACTIVE, e.g., paging/PEI reception, SSB/SI reception.

Therefore it is required to firstly discuss/determine whether this new “LPWUS” mode is regarded as a sub-state of the current RRC state or is a “new state” outside/independent of the current RRC state. To our understanding, maybe the former one (i.e., sub-state of RRC) is a promising solution as there would be a big impact to introduce a new state instead of the current RRC states. And this determination could eventually be done based on the requirement of LPWUS specific design and KPI.

Proposal 1: RAN2 is suggested to determine whether this new “LPWUS mode” is regarded as a sub-state of the RRC states, or is outside/independent of the RRC states.

2.1.2 Transition from RRC_IDLE/INACTIVE to LPWUS mode

There is a scenario that, first, when there is no data/signal reception for a RRC_CONNECTED UE for a while, such a UE could be released to RRC_IDLE/INACTIVE for power saving. Second, if still no data/signal comes for a longer time (e.g., the whole day), such a RRC_IDLE/INACTIVE UE may enter LPWUS mode for more power saving.

The obvious characteristic in RRC_IDLE/INACTIVE is that there is no dedicated/individual signalling reaching to a specific UE in RRC_IDLE/INACTIVE, UE is only able to receive common signal such as SSB, system information, and even for paging is a group signal. Thus how such a UE is informed to enter LPWUS mode should be studied.

Proposal 2: RAN2 is suggested to consider whether/how network informs UE of entering LPWUS mode from RRC_IDLE/INACTIVE.

However, besides network directly informing such a UE of entering LPWUS mode, there could be cases that UE can determine entering LPWUS mode by some mechanism like pre-configured criterion, then UE can enter LPWUS mode by satisfying the criterion. But more issues need to be considered like what if state misalignment between network side and UE side happens.

2.1.3 False alarm

In legacy RRC_IDLE/INACITVE, UE needs to monitor paging for possible data/signal coming. Furthermore since there is no dedicated/individual channel in RRC_IDLE/INACTIVE, multiple UEs may use the same paging resource/occasion to monitor its own paging message. However even though one UE receive the paging indication, sometimes the paging message including specific UE-ID may not indicate that UE but for other UEs, which cause unnecessary paging reception (unnecessary power consumption), i.e., false alarm issue.

Similarly, UE dedicated/individual channel in LPWUS mode may not be assumed, so UE may have the same problem as paging reception. This is more related to how we design LPWUS and which resource we will use, for example, whether all UEs in one cell is to monitor a simple (e.g., OOK/ASK) common signal without UE specific/group ID or not and which time/frequency domain including duty-cycle or non-duty-cycle we will use for the LPWUS monitoring. Anyway we are not expecting that all UEs monitor the same LPWUS, some mechanisms can be further studied to solve the false alarm issue for better power saving.

Proposal 3: RAN2 is suggested to consider the LPWUS false alarm issue, i.e., avoid massive UEs monitoring the same LPWUS.

2.1.4 Mobility

In legacy mobility for RRC_IDLE/INACTIVE, cell reselection is performed by UE to let the UE camp on a suitable cell, and once downlink data/signal is available, network use paging mechanism to find this UE in one or multiple cells in TA list or RNA. That is to say, mobility is supported by cell reselection and paging in other cells.

First, for a mobility capable UE, synchronization signal (including Cell ID) and reference signal (for measurement) are required to support cell reselection mechanism. However in LPWUS mode, if we only design a simple LPWUR which is only used to monitor LPWUS, mobility can not be supported due to lack of synchronization/reference signal. It also means only stationary UE (i.e., no cell change) can support this feature. Otherwise a moving UE who is not performing intra-cell measurement could lost the coverage of the anchor cell and not performing inter-cell measurement could not support cell change in LPWUS mode.

Second, once LPWUS capable UE supports mobility, not only the anchor cell but also others cells should be able to send LPWUS to find the UE in multiple cells. Thus some RAN3 related work is required referring to XnAP RAN Paging message.

Proposal 4: To support mobility in LPWUS, RAN2 is suggested to consider the necessity of introducing synchronization/reference signal for cell reselection in LPWUS mode.

Furthermore, if we introduce synchronization/reference signal in LPWUS mode. There could be two types of reference signal available for measurement, i.e., this new reference signal and SSB. Thus we need to further consider the co-existence issue, for example, sometimes MR is used to measure SSB and sometimes LPWUR is used to measure new reference signal. And some problems need to be solved like whether the receiving sensitivity between two types of signal varies a lot, whether both signals can be evaluated together for the same cell reselection criterion, etc.

Proposal 5: If new reference signal is introduced in LPWUS mode, RAN2 is suggested to consider the issue on co-existence between new reference signal and SSB.

2.1.5 Content of LPWUS

The payload size of LPWUS should be determined by RAN1, and RAN2 needs to consider how to use it. In principal, the payload size of LPWUS should not be large as we try to design a simple signal for power saving reason.

Some potential contents of LPWUS are:

1. Wake up indication

2. Group indication

3. Synchronization/reference signal, e.g., Cell ID (if merged into LPWUS)

4. Some early indication/information, e.g., short message

To our understanding, there is no controversial for 1. Wake up indication, but the monitoring behaviour can be further discussed like whether it is explicit or implicit. For 2. group indication, a reasonable capacity for group indication in one LPWUS is needed to avoid false alarm issue, as this can further reduce UE power consumption. For 3. Synchronization/reference signal, which can provide the Cell ID to support cell reselection, is also needed if mobility is supported and signal is merged into LPWUS. But for 4. Some early indication/information (e.g., short message) we think at least for short message this is an over-optimized mechanism, UE anyways can monitor paging after LPWUS reception, and we are concerning that more and more contents added into LPWUS could increase the complexity of LPWUS design. Also it is noted that even for PEI (Paging early indication) mechanism, there is no short message in PEI.

Proposal 6: RAN2 is suggested to consider whether the content of LPWUS, besides wake-up indication, should include group indication and Cell ID (if synchronization/reference signal is merged into LPWUS).

Proposal 7: Not introduce short message into to LPWUS.

- 2.2 Specific consideration for LPWUS in RRC_IDLE/INACTIVE

2.2.1 PEI (Paging early indication)

In legacy, PEI is introduced to indicate UE whether there is paging available or not before monitoring paging, so that avoiding unnecessary paging reception. Furthermore, with group enhancement in PEI, false alarm issue can also be alleviated. But since now another power saving mechanism, i.e., LPWUS is introduced, we need to consider the co-existence between LPWUS and PEI.

There could be two alternatives:

1. Both signal (LPWUS and PEI) can be used simultaneously

2. Both signal (LPWUS and PEI) cannot be used simultaneously

Intuitively, there is no need to use both signal simultaneously as they has the similar function (i.e., indicate whether there is paging/data/signal available), monitoring both can increase unnecessary power consumption. However due to false alarm issue, it is also not clear that whether only to use LPWUS (i.e., exclusive of PEI) is good enough, for example, if group capacity in LPWUS is insufficient, unnecessary LPWUS reception increase the UE power consumption instead. But if the group capacity of LPWUS is same as PEI’s, there is no need to use both then. Therefore, it can be determined based on the specific LPWUS design.

Proposal 8: RAN2 is suggested to consider whether both LPWUS and PEI can be used simultaneously based on the specific LPWUS design.

2.2.2 Procedure for RRC_IDLE/INACTIVE after LPWUS reception

Since monitoring LPWUS is in a new mode, e.g., LPWUS mode, and the main radio could be turned off or in ultra-deep-sleep, for now it is not clear what UE needs to do when back to normal RRC_IDLE/INACTIVE, for example, before monitoring paging, whether the DL synchronization is good enough (e.g., to support decoding PDCCH/PDSCH), whether UE needs to perform cell selection, etc. So we need to consider some potential specific procedures after LPWUS reception.

Proposal 9: RAN2 is suggested to analyse what specific procedures (e.g., DL sync, cell selection) are required after LPWUS reception for RRC_IDLE/INACTIVE.

- 3 Conclusions

Proposal 1: RAN2 is suggested to determine whether this new “LPWUS mode” is regarded as a sub-state of the RRC states, or is outside/independent of the RRC states.

Proposal 2: RAN2 is suggested to consider whether/how network informs UE of entering LPWUS mode from RRC_IDLE/INACTIVE.

Proposal 3: RAN2 is suggested to consider the LPWUS false alarm issue, i.e., avoid massive UEs monitoring the same LPWUS.

Proposal 4: To support mobility in LPWUS, RAN2 is suggested to consider the necessity of introducing synchronization/reference signal for cell reselection in LPWUS mode.

Proposal 5: If new reference signal is introduced in LPWUS mode, RAN2 is suggested to consider the issue on co-existence between new reference signal and SSB.

Proposal 6: RAN2 is suggested to consider whether the content of LPWUS, besides wake-up indication, should include group indication and Cell ID (if synchronization/reference signal is merged into LPWUS).

Proposal 7: Not introduce short message into to LPWUS.

Proposal 8: RAN2 is suggested to consider whether both LPWUS and PEI can be used simultaneously based on the specific LPWUS design.

Proposal 9: RAN2 is suggested to analyse what specific procedures (e.g., DL sync, cell selection) are required after LPWUS reception for RRC_IDLE/INACTIVE.

- 4 References

[1] RP-213645		New SID on low-power WUS WUR for NR

[2] R2-121bis-e	Chair Notes EOM		Johan