3GPP TSG-RAN WG2 Meeting #122               	                   R2-2304990

Incheon, South Korea, May 22 - May 26, 2023	   Revision of R2-2303760

Agenda Item:	7.16.2.1

Source: 	MediaTek Inc.

Title:  	Architecture and general for AI/ML

Document for:	Discussion and decision

Introduction

In this contribution, we first discuss the globality and the usefulness of global model ID based on current agreement. Then we try to identify the meta information for AIML model identification and the relationship with model ID. Subsequently, we discuss the functional framework in accordance with the previous meeting's agreement and present our understanding on AI/ML functional architecture. Finally, we provide our views on the functions that really matter to the system architecture and discuss the function mapping to physical entities.

Discussion

Model ID

In previous RAN2 meetings, RAN2 made following agreements related to Model ID:

- RAN2#119bis-e: - R2 assumes that a model is identified by a model ID. Its usage is FFS. RAN2#120: - R2 assumes that model ID can be used to identify which AI/ML model is being used in LCM including model delivery. - R2 assumes that model ID can be used to identify a model (or models) during model selection/activation/deactivation/switching (can later align with R1 if needed). RAN2#121: - RAN2 assumes that Model ID is unique “globally”, e.g., in order to manage test certification each retrained version need to be identified. RAN2#121bis-e: - Model ID can be used to identify model or models for the following LCM purposes: - model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step). (e.g. for so called “model ID based LCM”) - If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose.

RAN1 is now discussing two kinds of LCM, i.e., functionality-based LCM and Model-ID-based LCM. In the last RAN1#112bis-e meeting, it is FFS the relationship between functionality and model, as well as the relationship between these two kinds of LCM. From our views, for model-ID-based LCM, model is identified by model ID. For functionality-based LCM, network control the AI/ML functionalities via 3GPP signaling but may not identify different models. In this case, UE may perform model-level LCM and it’s unclear whether and how much awareness/interaction network should have. However, it’s possible that AI/ML functionality is identified by certain functionality ID.

Although RAN1 discussion blurs the boundary between functionality and model, RAN2 doesn’t need to differentiate functionality and model at the time being. We can focus on to enable the development of a set of specific models for the specific scenarios/configurations/sites and discuss the usage of AI/ML model ID in a general way regardless of whether it’s functionality-based LCM or model-ID-based LCM.

Observation 1: RAN2 can discuss the usage of model ID in a general way regardless of whether it’s functionality-based or model-ID-based LCM.

The Globality of model ID

In the email discussion [AT121bis-e][014][AIML] Model ID (incl meta data) progress (OPPO), it was discussed whether to not consider the data collection as one of the use cases for model ID. Some companies shared the view that data collection, as well as model training and development, occurs before a model is registered/ identified and should not be assigned a model ID.

We have the same understanding that during data collection, model training and model development, it’s very likely that different vendors may have different sets of AI/ML models even for the same functionality. Considering the AI/ML model is kept proprietary, model IDs in each vendor’s repository for model development and management is implementation and vendor specific. For each model, it may be identified by an identifier and even a version number considering the model can be updated. We call it original model ID in Figure 1 to avoid potential confusion.

Observation 2: The original model ID in the stage of model development and management period is up to vendor’s implementation, therefore the identifier for this stage should be vendor specific.

Figure 1 Different types of model IDs for different purposes and managed by different network entities

In RAN2#121 meeting, RAN2 assumed that Model ID is unique “globally”, e.g., in order to manage test certification each retrained version need to be identified. If the network is responsible for model control, i.e., model activation/deactivation/switching, it needs to re-structure the UE vendor specific model ID lists to another form of model IDs, which are manageable by the network. We call it global model ID in Figure 1. Each global model ID of this type should be able to uniquely identify each AI model known to the network.  Certain network functions or OAM may be responsible for model ID management, where the linkage between the original vendor-specific model IDs and the global model IDs are maintained. Therefore, the model ID globally identifying each AI/ML model is a permanent ID, which is assigned and controlled by the network.

Observation 3: The original model IDs from different vendors are re-structured to global model IDs manageable at the network side.

In email discussion [AT121bis-e][014] Model ID (incl meta data) progress, companies discussed the support for UE capability reporting as one of the model ID use cases. Although we are awaiting RAN1's progress, we can still make headway based on the companies' perspectives. In the discussion, majority companies think that to enable UE capability reporting using a model ID, it should be globally unique and synchronized between the network and UE, allowing the UE to link the model ID to the relevant AI/ML capabilities.

Observation 4: In order to support UE capability reporting via model ID, the global model ID should be unique and valid across UEs and network.

Global model ID for UE capability reporting

Even though RAN1 hasn't reached a conclusion on functionality and model ID, we can still discuss on using identifier for the UE capability reporting in a general context. If each AI/ML model is identified by a permanent global model ID at the network side, it can be used in UE capability reporting, indicating which AI/ML model is supported by the UE. For collaboration level y without model transfer, indicating the model ID in UE capability also implies the availability of the model at the UE. For collaboration level z with model transfer, model may not be available at the UE and needs to be downloaded from the network, so indicating the model ID in UE capability may imply the availability of a model on UE, or the executability of a model (which has not been available yet) before the model is downloaded. But common for both collaboration level y and collaboration level z, indicating the model ID in UE capability implies the availability of the model at the UE side. It is FFS on how to indicate the support and executability of a model before model transfer. During model transfer, the global model ID is used to identify the AI/ML model which is being transferred from the network to the UE. Model registration/identification and signaling for LCM are needed for both collaboration level y and collaboration level z which should be done before model transfer/delivery.

Observation 5: For collaboration level y without model transfer, indicating the global model ID in UE capability implies the availability of the model at the UE.

Observation 6: For collaboration level z with model transfer, indicating the global model ID in UE capability may imply the availability of the model at the UE. It is FFS on how to indicate the support and executability of a model before model transfer.

Observation 7: Model registration/identification and signaling for LCM are needed for both collaboration level y and collaboration level z which should be done before model transfer/delivery.

For two-sided model, the global model ID is also used to pair the AI/ML models between the UE and the network. When a global model ID is assigned to a UE part model of two-sided model, the network pairs the UE part model with a network part model.

Base on the above discussions, we propose the following for the global model ID.

Proposal 1: The global model ID is a permanent ID, which is assigned and managed by the network.

Proposal 2: The global model ID for each AI/ML model is used for the following purposes:

- Model test certification

- UE capability reporting to indicate which AI/ML model is available at the UE

- Model paring of the AI/ML models between the UE and network for two-sided model

Proposal 3: Model Identification/Registration should be done before model transfer/delivery.

Model index for local indication

In the same email discussion, it was discussed whether to introduce local model index for less overhead for local signaling, with the majority of companies supporting the idea of a temporary/local model index for certain LCM purposes, such as model activation/deactivation/switch/selection.

From our view, the global model ID may have multiple fields and very long length, making it inefficient to use directly for model control.  During model control in model-ID-based LCM, an appropriate AI/ML model among a set of AI/ML models is chosen for usage, which can fit the scenario/configuration/site. Given that model activation/deactivation/switch/selection only occurs when UE is in CONNECTED state, a temporary model index can be assigned to each model through model configuration. Model activation/deactivation/switch/selection can rely on the model index, similar as SCell index for corresponding actions. Based on the model index, both UE and network knows which AI/ML is being in use and monitored.

Observation 8: A temporary model ID, i.e., a model index, is sufficient for model activation/deactivation/switch/selection since these operations only occur when the UE is in CONNECTED state with UE specific RRC context.

Observation 9: Global and temporary model IDs are required for different purposes and managed by different entities.

Proposal 4: A model index is assigned to each model for model activation/deactivation/switch/selection by the network through model configuration.

Model identification and meta information

In last RAN2 meetings, RAN2 made following agreements related to meta information:

- RAN2#119bis-e: R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. RAN2#121bis-e: R2 assumes that Information such as FFS:vendor info, applicable conditions, model performance indicators, etc. may be required for model management and control, and should, as a starting point, be part of meta information.

In email discussion [AT121bis-e][014] Model ID (incl meta data) progress, companies also discussed the use case for meta information, and the relationship between model ID and model meta information.

In terms of the content for meta information, companies’ views are diverse. In our perspective, the purpose of meta information is to help networks and UE better understand the model, therefore at least the applicable condition (e.g., feature and scenario) should be involved to help UE to know the applicable area and ensure the proper use of the model. Besides, the nominal input and output information should be also provided to better understand the required data type and to have better interpretation of the results. This information also helps describe the model's specific scope. The expected performance information should be also provided for UE to establish expectations when using certain model, helping UE to improve and update model with certain criteria.

Proposal 5: For each AI/ML model, at least following meta information needs to be known. FFS on other information.

- Applicable AI/ML-enabled Feature/FG

- The applicability scenario, configuration and site information

- Information on model input

- Information on model output

- Expected model performance for monitoring

Functional Framework

The framework defined in [1] for RAN intelligence includes the functions of data collection, model training, model inference, and Actor. It can be considered as a starting point for Rel-18 AI/ML over air interface.

Based on RAN1’s agreement, RAN1 agreed to study the following aspects in LCM: data collection, model training, model deployment, model inference, model selection, activation, deactivation, switching, and fallback, model monitoring, model update, model transfer and UE capability. Some of the LCM aspects can be considered as functions, while others are considered as procedures to support the interactions between those functions.

During the recent RAN2 meetings, progress was made on the AI/ML functional framework that the general AI/ML framework consists of data collection, model training, model management, model inference, and model storage. However, the detailed functions of these entities and their interactions are still to be FFS, and this paper further discusses these aspects.

The function of data collection is the most fundamental one. In RAN1#110b-e Meeting, RAN1 drew the conclusion that data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. In our understanding, data collection should provide input data to model training, model inference and model management functions.

Observation 10: The function of data collection provides input data to model training, model inference and model management functions.

The function of model training performs AI/ML model training, validation and testing. RAN1 is discussing different model transfer/delivery cases considering the possibilities that model storage and model training may or may not be co-located. If model training and model storage are not performed at the same entity, model transfer/delivery is required to deliver the well-tested model from model training to model storage. Then the model will be uploaded/deployed to model inference entity for the further predictions or classification based on new (inference) data.

Observation 11: The function of model training performs AI/ML model training, validation and testing. The well-tested model will be delivered to model storage entity for future use.

Observation 12: The stored model is uploaded/deployed to model inference entity for further predictions or classification based on inference data.

Besides the functions of data collection, model training and model inference, one additional function needs to be considered is model management, which is used to monitoring the performance of the AI/ML model and triggers the corresponding control of the AI/ML model including model activation/deactivation, fallback to non-AI operation, or even triggers model retraining and update. The model management may require signaling and procedures to collect data and provide the performance feedback to the peer entity. The outcome of the model management is used for model control. The performance feedbacks can be intermediate KPIs that are used to evaluate the AI/ML model performance, and they can also be the system-level KPIs that are used to evaluate the overall system performance.

Observation 13: The function of model management performs monitoring performance, triggering control actions, and providing performance feedback for model control and system performance evaluation.

Model Inference

Model Training

Model Management

Model transfer/ delivery

Model Performance Feedback

Outputs

Training data

Inference data

Data collection

Model Storage

Model Deployment/Model delivery

Model control

Monitoring data

Model Inference

Model Training

Model Management

Model transfer/ delivery

Model Performance Feedback

Outputs

Training data

Inference data

Data collection

Model Storage

Model Deployment/Model delivery

Model control

Monitoring data

Figure 2 Example of Functional Framework for AI/ML over air interface

Just as illustrated in Figure 2, the AI/ML functional framework for air interface includes the functions of data collection, model training, model storage, model inference and model management. Besides the functions in the framework, the interactions and processes between those functions should be considered. The major steps regarding these data flows are mentioned below:

- Training data needs to be sent from data collection to model training.

- Similarly, inference data (different from training data) needs to be sent from data collection to model inference.

- Similarly, monitoring data (different from training data, inference data) needs to be sent from data collection to model management.

- Model deployment/delivery need to be propagated from model training to model storage.

- Model transfer/delivery need to be propagated from model storage to model inference.

- Model performance feedback is sent from model management to model training for model update.

- Outputs of the model needs to be sent from model inference to model management to monitor the model performances.

- Model management sends the model control decision to model inference.

Proposal 6: The AI/ML functional framework for air interface includes the functions of data collection, model training, model storage, model inference and model management. RAN2 takes Figure 2 as starting point for functional framework discussion and inform RAN1.

Mapping of Functions to Physical Entities

Data collection

Since data collection is used for different purposes, the function of data collection may not be a single functional entity but may be multiple functional entities serving different purposes of functions in LCM. For example, the datasets for model training may be collected and stored somewhere. For model inference, the inputs are live data collected over the air interface. For model management, the ground-true data may also need to be collected depending on whether intermediate KPIs are evaluated and data distribution for input and/or output is observed. It is desired that the function entities of data collection for different purposes is located with the corresponding functions to avoid dataset transferring over the air interface. In other words, if model training and update is performed at the UE side, UE should be responsible for data collection used for model training. If model management is performed at the UE side, UE should be responsible for data collection used for model management. This principle is also applicable to network side model and two-sided model.

Observation 14: If model training, model inference and/or model management is performed at the UE side, UE should also be responsible for the data collection used for the corresponding model training, model inference and model management. The same principle also applicable to network side model and two-sided model.

For UE-sided model, if it is trained at the UE side, data collection should be located at UE side; if it is trained at the network side, data collection should be located at the network side. For network sided model, it is assumed that data collection for model training is located at the network side. According to RAN3, the model training can be located in OAM or gNB (or gNB-CU). If the model training is located in gNB, the data collection should be deployed together. If the model training is located in OAM, then the data collection can be set in local gNB, which may delivery the collected data to OAM.

For two-sided model, where data collection is located depends on which type of model training is concerned. For type 1 training, data collection is located at either UE side or network side (gNB or OAM), depending where model training is performed. For type 2 training, considering that dataset between UE side and network side is aligned, it is assumed that data collection is located both UE side and network side. For type 3 training, dataset is shared and transferred between the UE and the network.

Data collection for inference should be located with the function of inference. Therefore, for UE side model, data collection for inference is located at UE side. For network side model, data collection for inference is located at the network side. Since RAN3 made progress that model inference is located in gNB (or gNB-CU), we assume the data collection should be deployed together to provide the inference data for model inference. For two-sided model, data collection for UE part inference is located at the UE side and data collection for network part inference is located at the network side.

Model training

For UE-sided model, there are two approaches for model training. One way is that AI/ML model is training at the UE side over OTT server; the other way is that AI/ML model is trained at the network side, which transfers the AI/ML model to UE. For network-sided model, it is assumed that AI/ML model is trained at the network side.

For two-sided model, where model training is performed depends on which type of model training is concerned. For type 1 training, it is possible that the model training is performed at either UE side or network side. For type 2 and type 3 training, the training process requires the involvement of both UE side and network side.

For offline training, where model training takes place is not important, what matters is how it is ensured that a model works as intended and how to allocate the related responsibility. For conventional UE-based algorithm design, it is the chipset/UE vendor who develops the algorithm to make sure that the algorithm works properly, and requirements fulfilment and test compliance is part of this responsibility. As a baseline, the same principle should be applied to AI/ML algorithm development for UE-sided model, if it is trained at the UE side. It’s FFS on two-sided model and UE-sided model if it is trained at the network side. For two-sided model, different types of model training methods require different collaborative procedures between the UE and the network.

Proposal 7: For UE-sided model, if it is trained at the UE side, it is assumed that the chipset/UE vendor who develops the AI/ML algorithm takes the responsibility of requirements fulfilment and test compliance. FFS on two-sided model and UE-side model if it is trained at the network side.

Model Inference

For model inference, the function mapping to physical entities is straightforward and obvious based on the definition and the discussion above. For UE-sided model, model inference is performed at the UE. For network-sided model, model inference is performed as the network (gNB or gNB-CU). For two-side model, model inference is performed jointly on both the UE and the network.

Model monitoring

For collaboration level x, it is very likely that model management is performed on the entity where model training/inference is performed. The whole LCM is implementation-specific and without specification impact.

For collaboration level y and z, it is by default that model control is performed at the network, which also needs to perform model management for the overall system performance. For UE-sided model and two-sided model, UE may also perform model management and send the monitored performance reports to gNB. The gNB will take the UE report into consideration and perform model control accordingly. For network-sided model, we assume that model management and model controlled is performed at the gNB. We can FFS whether OAM can also perform model management function. The measurement quantities for model management are use case specific and should be decided by RAN1. RAN2 also needs use case specific effort to discuss the measurement/reporting configuration for model management.

Proposal 8: Model monitoring, measurement/report configurations for model management are by default located at the network. UE may perform model monitoring and send performance reports to the network for UE-sided model and two-sided model.

Table 1 summarizes the functions mapping to UE side and network side. We think this table can be used as starting point for further discussion.

Proposal 9: RAN2 takes table 1 as starting point for function mapping to physical entities.

Table 1 Functions mapping to physical entities

|  | Inference | Data collection for inference | Monitoring | Data collection for monitoring | Training | Data collection for training | Use cases |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UE-sided model | UE side | UE side | gNB (FFS OAM); (Optional) UE side | gNB; (Optional) UE side | gNB | gNB | BM, CSI prediction, POS-Case 1, POS-Case 2a |
|  |  |  |  |  | OAM | gNB/OAM |  |
|  |  |  |  |  | UE side | UE side |  |
| Network-sided model | gNB | gNB | gNB (FFS OAM) | gNB | gNB | gNB | BM, Positioning POS-Case 3a, |
|  |  |  |  |  | OAM | gNB/OAM |  |
|  | LMF | LMF | LMF (optional) UE side | LMF | LMF | LMF | POS-Case 2b, POS-Case 3b |
| Two-sided model | UE side and gNB | UE side and gNB | gNB (FFS OAM), (optional) UE side | gNB; (optional) UE side | Type 1 training: UE or network side (gNB/OAM) | Type 1 training: UE or network side (gNB/OAM) | CSI compression |
|  |  |  |  |  | Type 2 training: UE and network side (gNB/OAM) | Type 2 training: UE and network side(gNB/OAM) |  |
|  |  |  |  |  | Type 3 training: both UE and network side (gNB/OAM) | Type 3 training: UE or network side. (gNB/OAM) Dataset needs to be shared |  |

Conclusion

We have following observations and proposals:

Model ID

Observation 1: RAN2 can discuss the usage of model ID in a general way regardless of whether it’s functionality-based or model-ID-based LCM.

Observation 2: The original model ID in the stage of model development and management period is up to vendor’s implementation, therefore the identifier for this stage should be vendor specific.

Observation 3: The original model IDs from different vendors are re-structured to global model IDs manageable at the network side.

Observation 4: In order to support UE capability reporting via model ID, the global model ID should be unique and valid across UEs and network.

Observation 5: For collaboration level y without model transfer, indicating the global model ID in UE capability implies the availability of the model at the UE.

Observation 6: For collaboration level z with model transfer, indicating the global model ID in UE capability may imply the availability of the model at the UE. It is FFS on how to indicate the support and executability of a model before model transfer.

Observation 7: Model registration/identification and signaling for LCM are needed for both collaboration level y and collaboration level z which should be done before model transfer/delivery.

Observation 8: A temporary model ID, i.e., a model index, is sufficient for model activation/deactivation/switch/selection since these operations only occur when the UE is in CONNECTED state with UE specific RRC context.

Observation 9: Global and temporary model IDs are required for different purposes and managed by different entities.

Proposal 1: The global model ID is a permanent ID, which is assigned and managed by the network.

Proposal 2: The global model ID for each AI/ML model is used for the following purposes:

- Model test certification

- UE capability reporting to indicate which AI/ML model is available at the UE

- Model paring of the AI/ML models between the UE and network for two-sided model

Proposal 3: Model Identification/Registration should be done before model transfer/delivery.

Proposal 4: A model index is assigned to each model for model activation/deactivation/switch/selection by the network through model configuration.

Model identification and meta information

Proposal 5: For each AI/ML model, at least following meta information needs to be known. FFS on other information.

- Applicable AI/ML-enabled Feature/FG

- The applicability scenario, configuration and site information

- Information on model input

- Information on model output

- Expected model performance for monitoring

Functional Framework

Observation 10: The function of data collection provides input data to model training, model inference and model management functions.

Observation 11: The function of model training performs AI/ML model training, validation and testing. The well-tested model will be delivered to model storage entity for future use.

Observation 12: The stored model is uploaded/deployed to model inference entity for further predictions or classification based on inference data.

Observation 13: The function of model management performs monitoring performance, triggering control actions, and providing performance feedback for model control and system performance evaluation.

Proposal 6: The AI/ML functional framework for air interface includes the functions of data collection, model training, model storage, model inference and model management. RAN2 takes Figure 2 as starting point for functional framework discussion and inform RAN1.

Mapping of Functions to Physical Entities

Observation 14: If model training, model inference and/or model management is performed at the UE side, UE should also be responsible for the data collection used for the corresponding model training, model inference and model management. The same principle also applicable to network side model and two-sided model.

Proposal 7: For UE-sided model, if it is trained at the UE side, it is assumed that the chipset/UE vendor who develops the AI/ML algorithm takes the responsibility of requirements fulfilment and test compliance. FFS on two-sided model and UE-side model if it is trained at the network side.

Proposal 8: Model monitoring, measurement/report configurations for model management are by default located at the network. UE may perform model monitoring and send performance reports to the network for UE-sided model and two-sided model.

Proposal 9: RAN2 takes table 1 as starting point for function mapping to physical entities.

Reference

- TR 37.817 V17.0.0 Study on enhancement for Data Collection for NR and EN-DC