3GPP TSG-RAN WG2 Meeting #122											R2-2304995

Incheon, 22th May– 26th May, 2023

Agenda item:       6.7.3

Source: 	LG Electronics

Title: 	Summary on user plan corrections for NR SL enhancements

Document for:	Discussion and Decision

Introduction

This document summarizes the contributions submitted to AI 6.7.3. Rapporteur’s recommendations on how to treat them are provided.

Corrections & Proposals

The following correction CRs and proposals could be discussed together.

Table 1: Corrections & Proposals

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2304845 | Correction on 38.321 for SL enhancements | Huawei | Reason for change: In RAN2#121 meeting, it was agreed to add the transmission of UC-based DCR message case for the use of the sl-drx-StartOffset and sl-drx-SlotOffset equations. However, default CG/BC DRX configuration is also used for discovery message, and the cast type of discovey message is not indicated by upper layes, but is specified as broadcast in current TS 38.321. Thus, using the sl-drx-StartOffset and sl-drx-SlotOffset equations for discovery message should be considered also. Change: When the cast type is groupcast or broadcast as indicated by upper layer, or the cast type is unicast for the reception of Direct Link Establishment Request message [28] or ProSe Direct Link Establishment Request message [29] as indicated by upper layer, or for the reception of discovery message [26], the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations: sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-DRX-GC-BC-Cycle (ms). sl-drx-SlotOffset (ms) = (Destination Layer-2 ID modulo the number of slots in one subframe) / (the number of slots in one subframe) (ms). | Rapporteur agree proponent’s observation and correction. Correction can be discussed. |
| R2-2305224 | Discussion on the usage of default CBR values for case 2b | Xiaomi | The handling of case 2b should be aligned with case 3, if the correction of the case 2b is adopted, corresponding change for case 3 also should be adopted. Otherwise, the default R16 CBR value is only applicable for exceptional case but actually RAN2 already agreed case 3 is valid, i.e., in R17 normal pool if sensing result is not available, R16 default CBR value should be applied. Agreement: RAN2 confirm the validity of Case-3 (usage of R16 default CBR for full sensing in normal pool). But no spec change for R16 at least.. Since RAN2 has agreed to have no change for case 3, RAN2 should not adopt the change for exceptional pool. Generally, this part is kept the same as R16 specification and aligned with the guidance from RAN1. Proposal 4 [13/15] R17 spec change for Case 3 (usage of R16 sl-defaultTxConfigIndex for random selection in R16 normal pool) is not needed.. Proposal1: For case 2b, RAN2 does not agree the change as proposed in R2-2302619/R2-2302647/R2-2303215. | Rapporteur has the same understanding of the RAN2 agreements about the usage of default CBR values with the proponent. Proposal can be discussed. |
| R2-2305226 | Miscellaneous corrections on TS 38.321 for NR sidelink | Xiaomi | Reason for change: In section 5.28.2, to start the inactivity timer for groupcast, the RX UE should determine if the Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-2 ID or not. But in current specification, it is based on the intended Destination Layer-1 ID. Change In section 5.28.2, change Destination Layer-1 ID to Destination Layer-2 ID. 3> if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-12 ID and the cast type indicator in the SCI is set to groupcast: 4> start or restart sl-drx-InactivityTimer for the corresponding Destination Layer-2 ID in the first slot after SCI reception. | Proponent’s observation is correct. Correction should be adopted. |
| R2-2305278 | Correction on resource (re-)selection for NR sidelink | CATT | Reason for change: In RAN2#117 meeting, the following agreements on IUC were reached: 1: A standalone MAC CE for UE-A’s IUC information is transmitted through HARQ Feedback disabled MAC PDU. 2: When a MAC CE for IUC information is multiplexed with MAC SDU(s), the HARQ attribute of a MAC PDU is determined by following sl-HARQ-FeedbackEnabled being set to enabled or disabled for the highest priority logical channel included in the MAC PDU.. Based on the above agreements, it is obvious that when IUC information needs to be transmitted, two cases need to be considered: - Case 1: There is only standalone IUC information transmission needs to be transmitted. - Case 2: There is both IUC information and SL data/SL-CSI. For Case 2, the current MAC is clear. But for Case 1, according to the current description in spec: “the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and/or the latency requirement of the triggered SL-CSI and the latency requirement of the Sidelink Inter-UE Coordination Information transmission”. Only “and” is used between the greed part and the blue part. It is obvious that Case 1 cannot be reflected in the spec Change Add “(if any)” for SL data part and triggered SL-CSI part when UE select resources for transmission of Sidelink Inter-UE Coordination Information. 3> if sl-InterUE-CoordinationScheme1 enabling reception/transmission of preferred resource set and non-preferred resource set is configured by RRC and when the UE determines the resources for Sidelink Inter-UE Coordination Information transmission upon explicit request from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources, the remaining PDB of SL data available in the logical channel(s) allowed on the carrier (if any), and/or the latency requirement of the triggered SL-CSI (if any) and the latency requirement of the Sidelink Inter-UE Coordination Information transmission. | In the #121bis-e meeting, we discussed this correction, and the discussion result is as follows: [Summary Q1] Out of 16 companies Agree: 1 (proponent) Disagree: 15 Rapporteur summary: Besides the proponent, there is no support for this correction. The majority view is that this correction is not essential. (1, 15) Proposal 1. Change in R2-2302618 is not agreed. Most companies think that the correction is not essential. Also, even in the case of CSI reporting, there is no additional text in the MAC specification, such as this correction for standalone transmission. Correction can be discussed. |
| R2-2306311 | MAC PDU filtering | Nokia | Reason for change: The NOTE in S5.22.2.2.2 covers two cases: - If the first unicast PC5-S message is received for SL-SRB0/1, the MAC entity delivers the received TB by filtering based on the LCID 0 or 1, even if the Source Layer-2 ID is unknown. (agreed in R2-2206287) - If Discovery Response message is received (cast type set to broadcast), the MAC entity delivers the received TB by filtering based on the LCID 58, even if the DST ID is set to its own source L2 ID. (agreed in R2-2302195) This shouldn’t be a NOTE according to the TS 21.801 S6.5.1 since the current NOTE is not just additional information but rather contradicting behaviour to the normative text. In other words, this is an exceptional case to be followed when performing MAC PDU filtering. Thus, it should be in the normative text, otherwise, it is confusing which to follow. Futher, the current NOTE specifies when to deliver the MAC PDU with joint conditions having multiple ‘and’ and ‘or’ in one condition, which is very unreadable. Change Split the note into procedural text covering the three cases for for unicast and broadcast where the decoded MAC PDU should be delivered to the disassembly and demultiplexing entity: - For unicast: 1) Legacy behaviour i.e. filtering only data for which SRC field and destination ID matches, and 2) If the first unicast PC5-S message is received for SL-SRB0/1 - For broadcast: 2) Legacy behaviour i.e. filtering only data for which DST field and destiantion ID matches, and 2) If Discovery Response message is received 1> if the data for this TB was successfully decoded before: 2> if this is the first successful decoding of the data for this TB: 3> if this TB is associated to unicast and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI: 4> if the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI; or 4> if this TB is corresponding to the logical channel with LCID equal to 0 or 1 and determined to be the first TB: 5> deliver the decoded MAC PDU to the disassembly and demultiplexing entity. 3> if this TB is associated to groupcast or broadcast: 4> if the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI; or 4> if this TB is corresponding to the logical channel with LCID equal to 58, and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI: 5> deliver the decoded MAC PDU to the disassembly and demultiplexing entity. NOTE: Whether the TB is the first TB can be determined based on the Source Layer-2 ID and Destination Layer-2 ID pair. 2> consider the Sidelink process as unoccupied. | There is nothing technically wrong with UE behavior in the current NOTE. In addition, RAN2 agreed in a previous meeting to specify the RAN2 agreement in the NOTE approach rather than the normative text approach with regard to this MAC filtering UE behavior. Therefore, this correction violates the RAN2 consensus. In conclusion, rapporteur believes that this correction is not necessary. |

Proposal: Discuss the proposals and CRs listed in Table 1 offline.

- Conclusions

The contributions submitted to AI 6.7.3 are summarized, Rapporteur’s recommendations are as follows.

Proposal: Discuss the proposals and CRs listed in Table 1 offline.