3GPP TSG-RAN WG2 Meeting #122 	R2-2305024

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.4.2.2

Source:	Panasonic

Title:	Remaining issues on L1 measurement configuration for LTM

Document for:	Discussion

# Introduction

It has been agreed that L1 measurements are used to trigger cell switch for LTM. In the previous meeting (RAN2#121bis-e), the following progress was achieved:

- Initial agreements, from RAN2 point of view (may be dep on RAN1 progress). The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS. RAN2 assumes that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration). RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s). Chair: the agreements above may need to be further evaluated, e.g. wrt subsequent LTM switches. RAN2 assumes that whether filtering, hysteresis, and time-to-trigger are needed for LTM specific L1 measurements is up to RAN1. FFS if the LTM specific L1 measurements of an LTM candidate SCell is independent of its activation status. Whether to assume L1/L2 signaling to control or change L1 measurement/reporting for LTM needs further discussion (parts may be discussed in RAN1). RAN2 assumes that such control would be limited to certain aspect that need frequent update and restricted by RRC configuration.

In this paper, we provide our views on some remaining issues on the design for L1 measurement configurations.

# Discussion

## Reporting configuration

In the previous meeting, we made one agreement regarding the location of reporting configuration:

RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s).

One implication is that, in order to support subsequent LTM, the report configuration is also required to be included in each candidate cell configuration, such that it can be used when the candidate cell becomes the current serving cells. Some concerns were raised because of this implication. Specifically, according to current spec, it is the DU who generates the CSI-MeasConfig including CSI-ReportConfig. In such case, some companies mentioned that each candidate DU needs to know the L1 measurement RS configuration from other DUs, to generate the right report configuration under the candidate cell configuration, which may require complicated inter-DU coordination among serving and candidate DUs.

In our opinion, such concerns can be alleviated by letting CU to manage the RS measurement resource sets. The reporting configuration (CSI-ReportConfig) prepared by the DU can simply point to such RS measurement resource set (e.g. using a resource configuration ID). Note that we have already agreed that “the location of RS configuration for candidate cells is external to current serving cells and external to the candidate cell configuration”. The following Fig.1 illustrates the idea:

Fig.1. Report configuration points to the external measurement resource set

The following diagram shows one example of how CU can maintain the RS measurement resource set.

Fig.2. The RS resource sets for LTM are maintained by CU only

Note that in above procedure, once serving DU receives the RS resource set configuration prepared by CU, serving DU simply forwards it to UE without interpretation. In other words, DU is not aware of the content of RS resource configuration of candidate cells. When configuring CSI-ReportConfig, DU simply refers to a “unknown” measurement resource set that is supposed to be provided by CU to the UE (see steps 5 and 6 in Fig.2). Finally, in step 8, UE links the CSI-ReportConfig to the RS resource set to perform measurement and reporting.

Alternatively, the RS resource sets of LTM candidate cells can be controlled by both CU and DU. The following Fig.3 shows the high level procedures. This gives more flexibility for DU to control which RS to include in the reporting configuration. But it increases the DU processing complexity. Another drawback is the signaling overhead because once RS resource set is updated, it needs to be propagated to all candidate DU in order for DU to configure new RS configuration and CSI-ReportConfig..

Fig.3. The RS resource sets for LTM are controlled by both CU and DU

Proposal 1: RAN2 confirms that the agreements related to L1 measurement/reporting configuration made in previous meeting can work in case of subsequent LTM.

Proposal 2: RAN2 to discuss the following operations regarding the RS resource configuration for LTM candidate cells

Option 1: The RS resource set for LTM candidate cells are maintained by CU only.

Option 2: The RS resource sets for LTM candidate cells are controlled by both CU and DU.

## L1/L2 signalling to control or change L1 measurement/reporting

In the last meeting, the following issue is still pending.

Whether to assume L1/L2 signaling to control or change L1 measurement/reporting for LTM needs further discussion (parts may be discussed in RAN1). RAN2 assumes that such control would be limited to certain aspect that need frequent update and restricted by RRC configuration.

We understand the motivation of using L1/L2 signaling to control or change L1 measurement/reporting is to prevent excessive measurement and reporting by dynamically adjusting the measurement/reporting objects. As UE moves across different cells in subsequent LTM, it can be imagined that the set of relevant candidate cells are also changing. Therefore, it would be beneficial that the RRC configured measurement/reporting configurations can be modified by .e.g, MAC CE.

On the other hand, the discussion seems coupled with the operations mentioned in section 2.1. For example, if the measurement RS configuration of candidate cells is only maintained at CU and not visible to DU, it seems a natural choice to rely on RRC signaling to select a subset of the measurement objects. However, if the measurement RS configuration is also visible at DU (e.g. DU to do the mapping between reporting configuration and RS configuration), L1/L2 signalling can be considered for the DU to dynamically activate/select a subset of measurement/reporting objects. For the signaling aspect, MAC CE is preferred because of being less costly than DCI.

Therefore, we propose to study more general aspect of controlling or changing L1 measurement/reporting as follows:

Proposal 3: Further study the following signaling methods to activate/select a subset of L1 measurement/reporting objects that have been configured by RRC

- RRC signaling

- MAC CE

# Conclusion

In this paper, we discuss the L1 measurement configuration. The proposals are as follows.

Proposal 1: RAN2 confirms that the agreements related to L1 measurement/reporting configuration made in previous meeting can work in case of subsequent LTM.

Proposal 2: RAN2 to discuss the following operations regarding the RS resource configuration for LTM candidate cells

Option 1: The RS resource set for LTM candidate cells are maintained by CU only.

Option 2: The RS resource sets for LTM candidate cells are controlled by both CU and DU.

Proposal 3: Further study the following signaling methods to activate/select a subset of L1 measurement/reporting objects that have been configured by RRC

- RRC signaling

- MAC CE