3GPP TSG-RAN WG2 Meeting #122	R2-2305025

Incheon, South Korea, May 22nd – May 26th, 2023

Agenda Item:	7.9.3

Source: 	Canon Research Centre France

Title:	Discussion on lossless path switching for Sidelink Relay

Document for:	Discussion

# Introduction

At the previous RAN2#121bis-e meeting, RAN2 discussed and listed some potential solutions for lossless data delivery for path switch through email discussion R2-2304305 [1].

For uplink communication, 6 solutions have been proposed:

•	Solution- U1: Relay UE delays its RLC feedback to Remote UE

•	Solution- U2: Remote UE’s PDCP retransmission based on remaining packets in the buffer

•	Solution- U3: Remote UE’s PDCP retransmission based on DL PDCP Status Report from target gNB

•	Solution-U4: Enhancing RLC status report to Remote UE

•	Solution-U5: Source Relay UE continues to transmit UL data to source gNB and gNB forwards to the target gNB

•	Solution-U6: Source gNB sends a PDCP status report to the Remote UE before SN status transfer

For downlink communication, 5 solutions have been proposed:

•	Solution-D1: Relay UE delays its RLC feedback to source gNB

•	Solution-D2: Relay UE indicates the packet transmission status to source gNB

•	Solution-D3: A PDCP status report sent from Remote UE to the source gNB

•	Solution-D4: Enhanced Data forwarding from source gNB to target gNB per target gNB request (legacy PDCP status report based)

•	Solution-D5: Proactive Data forwarding from source gNB to target gNB

Eventually, a down selection among the solutions has been done and the following agreements have been made [2]:

For uplink lossless data delivery for path switch, continue considering solutions U3 and U5 from R2-2304305. Other solutions are not pursued. For downlink lossless data delivery for path switch, Solution-D4 is taken as the baseline solution and keep Solution-D3/D5 on the table for further decision at the next meeting.

In this paper, we discuss aspects related to the lossless delivery for sidelink relay inter-gNB path switching.

# Discussion

RAN2 agreed at RAN2 #121 meeting that lossless data delivery in the inter-gNB i2x cases needs to be addressed, considering that a data loss may result from inter-gNB indirect to direct (i2d) or indirect to indirect (i2i) path switching if the source node (gNB for DL or Remote UE for UL) acknowledges the PDCP data PDU based on the RLC acknowledgements supplied by the relay UE.

Indeed, these acknowledgements indicate the transmission status at the first hop of the indirect path, without considering the transmission status over the second hop. Therefore, the PDCP entity of the source node is not aware of the actual reception status of the data at the destination node (Remote UE for DL or gNB for UL) that may not have been successfully delivered to the destination node.

As a result, when the discard timer expires, the source node may discard data acknowledged by the relay UE but not successfully received by the destination node, while such data should have been retransmitted to/from the target gNB after the completion of the inter-gNB path switch process.

## Uplink communication case

For the uplink communication case, the solution U3 proposes that the remote UE PDCP retransmits data based on PDCP Status Report provided by the target gNB. As already stated in the TS38.300, the target gNB may send a report to the UE after a handover completion to minimize the unnecessary over-the-air retransmission, thus, a slight modification of the specification permits to support this solution. This solution allows the target gNB to request the remote UE to retransmit the exact PDCP SDU necessary to ensure a lossless path switch.

Proposal 1: RAN2 to consider the solution U3 as a potential solution for the lossless inter-gNB path switch.

However, the remote UE should still have the requested PDCP SDU in its PDCP buffer in order to be able to retransmit them. Thus, to ensure the lossless behaviour of the path switch procedure, the remote UE shall be prevented to discard data which have only been acknowledged by its lower layer upon expiry of its PDCP discard timer.

Observation 1: The efficiency of solution U3 may be impacted by some potential PDCP SDUs discarding at the remote UE due to potential discard timer expiry.

Proposal 2a: For UL transmission, during path switching, when considering solution U3, the discard timer of the remote UE should be (re)configured so as to prevent the discarding of PDCP SDU that need to be retransmitted. To do so, the two following options may be considered:

- Option 1: the source gNB configures the remote UE PDCP discard timer to a value that would ensure a lossless data delivery for path switch e.g., an infinity value;

- Option 2: the remote UE suspends its discard timer. The remote UE may suspend its discard timer upon reception of a path switch command from the source gNB or upon sending some measurement report to the source gNB.

Upon completion of the path switch process and the retransmission of the PDCP SDUs (if any) to the target gNB, the discard timer should be resumed.

Proposal 2b: For UL transmission, when considering solution U3, once the path switch has completed, the discard timer of the remote UE should be resumed to a normal usage. To do so, the two following options may be considered:

- Option 1: the target gNB (re)configures the remote UE’s PDCP discard timer once the retransmitted missing PDCP SDUs have been received from the remote UE;

- Option 2: the remote UE resumes its PDCP discard timer after retransmission of the missing PDCP SDUs to the target gNB.

While solution U3, enhanced with some appropriate discard timer management as discussed above, may ensure an inter-gNB path switch without data loss, this solution requires the remote UE to buffer unnecessary PDCP SDUs until the reception of the PDCP status report from the target gNB.

In this respect, having the source gNB sending a PDCP status report to the remote UE before initiating the SN status transfer between the source and the target gNB, as proposed in solution U6, would allow to “clean up” the PDCP buffer from the useless data before the path switch of the remote UE.

Proposal 3: RAN2 to consider the solution U6 as a potential solution for the lossless inter-gNB path switch.

One main challenge for solution U6 is for the source gNB to send the PDCP status report to the remote UE at the appropriate timing, i.e., as close as possible to the effective path switch operated by the remote UE, so as to provide the remote UE with an accurate PDCP status reporting while ensuring that such PDCP status reporting would not be missed by the remote UE due to some link quality drop that could prevent the remote UE from actually receiving the PDCP status report.

Observation 2a: Solution U6 may prevent unnecessary buffering at remote UE.

Observation 2b: When applying solution U6, there is a risk of experiencing PDCP status report reception failure at the remote UE.

Based on the above considerations, it appears that a good solution for lossless data delivery for path switch could be to mix the solution U6 (to minimize the required buffering at the remote UE) and the solution 3 (to ensure that the PDCP status report is actually received by the remote UE).

Proposal 4: RAN2 to consider a complete solution based on both U6 and U3 so as to minimize buffering constraints while mitigating the risk of PDCP status report reception failure.

Concerning solution U5, some companies have reported at the last meeting that this solution may have some limitations, for instance when considering a Uu link with poor quality. Some other companies also insisted on the complexity that would result from the need to keep Remote UE and Relay UE context even after the Remote UE’s handover. Based on these considerations, it may be reasonable to withdraw solution U5.

Proposal 5: RAN2 not to pursue the solution U5 for the lossless delivery issue.

## Downlink case

For the downlink communication case, the solution D4 proposes that the target gNB, after receiving the legacy PDCP status report from the remote UE upon PDCP re-establishment or PDCP recovery (following path switch completion), may request the source gNB to forward the missing DL PDCP SDUs. This is particularly true when the target gNB detects an inconsistency between the received PDCP status report and the DL SN status transfer previously received from the source gNB.

Proposal 6: Based on D4 solution, the target gNB requests the source gNB to forward missing DL packets when detecting a SN mismatch between the PDCP status report received from the remote UE and the last SN status transfer received from the source gNB.

This solution allows the target gNB to request the source gNB the exact PDCP SDUs required to ensure a lossless data delivery for path switch. However, in order to be able to retransmit the missing PDCP SDUs, the source gNB should still have them in its PDCP buffer. Thus, solution D4 should rely on extra buffering at the source gNB while preventing the source gNB to discard the PDCP SDUs for which the successful delivery of the corresponding PDCP Data SDU has not been confirmed by PDCP status report either in the source gNB before path switch or in the target gNB after path switch that may result from discard timer expiry.

Observation 3: Solution D4 efficiency may be impacted by some potential PDCP SDUs discarding at the source gNB due to potential discard timer expiry.

Similar to the remote UE for UL, the source gNB have to manage its discard timer during the path switch procedure to avoid unexpected data discarding.

Proposal 7a: For DL transmission, when considering solution D4, the discard timer of the source gNB should be disabled during path switching operation in order to prevent the discarding of PDCP SDUs that need to be retransmitted. To do so, the two following options may be considered:

- Option 1: The source gNB configures its discard timer to a value that would ensure a lossless data delivery for path switch e.g., infinity value;

- Option 2: The source gNB suspends its discard timer upon reception of a path switch acknowledgement from the target gNB or upon the sending of a path switch command to the remote UE.

After the completion of the path switch and the retransmission of the PDCP SDUs to the target gNB (if any), the discard timer should be resumed.

Proposal 7b: For DL transmission, after path switch, the discard timer of the source gNB should be resumed to a normal usage:

- Option 1: source gNB configures its PDCP discard timer after the forwarding of the missing PDCP SDUs to the target gNB;

- Option 2: source gNB resumes its discard timer after the forwarding of the missing PDCP SDUs to the target gNB.

While solution D4, enhanced with some appropriate discard timer management as discussed above, may ensure a lossless inter-gNB path switch without data loss, this solution requires the source gNB to buffer unnecessary PDCP SDUs until the reception of the request from the target gNB.

In this respect, having the source gNB receiving a PDCP status report from the remote UE before the SN status transfer between the source and the target gNB, as proposed in solution D3, would allow to “clean up” the PDCP buffer from the useless data. For instance, the remote UE could send its PDCP status report upon reception of the path switch command to provide an up-to-date status to the source gNB.

Nevertheless, solution D3 may suffer from link quality issues that prevent the source gNB to receive the PDCP status report. Therefore, solution D3 should be considered in combination with solution D4 so as to handle the PDCP status report failure case. The failure could be detected by the target gNB as described above based on a detection of a mismatch between the SN reported by the remote UE and the SN status transfer from the source gNB.

Proposal 8: RAN2 to consider a complete solution based on both D4 and D3 so as to minimize buffering constraints while minimizing the effects of PDCP status report reception failure at the source gNB.

Concerning solution D5, many companies suggest that solution as a valid candidate and do not see the extra forwarding over Xn as a blocking issue. But in addition, to the waste of bandwidth over Xn, this extra forwarding may also lead to duplication over the air interface, as the target gNB has to prioritize all downlink data forwarded by the source gNB. Based on these considerations, it may be reasonable to withdraw solution D5.

Proposal 9: RAN2 not to pursue the solution D5 for the lossless delivery issue.

# Conclusion

This document has made the following proposals:

Proposal 1: RAN2 to consider the solution U3 as a potential solution for the lossless inter-gNB path switch.

Observation 1: The efficiency of solution U3 may be impacted by some potential PDCP SDUs discarding at the remote UE due to potential discard timer expiry.

Proposal 2a: For UL transmission, during path switching, when considering solution U3, the discard timer of the remote UE should be (re)configured so as to prevent the discarding of PDCP SDU that need to be retransmitted. To do so, the two following options may be considered:

- Option 1: the source gNB configures the remote UE PDCP discard timer to a value that would ensure a lossless data delivery for path switch e.g., an infinity value;

- Option 2: the remote UE suspends its discard timer. The remote UE may suspend its discard timer upon reception of a path switch command from the source gNB or upon sending some measurement report to the source gNB.

Proposal 2b: For UL transmission, when considering solution U3, once the path switch has completed, the discard timer of the remote UE should be resumed to a normal usage. To do so, the two following options may be considered:

- Option 1: the target gNB (re)configures the remote UE’s PDCP discard timer once the retransmitted missing PDCP SDUs have been received from the remote UE;

- Option 2: the remote UE resumes its PDCP discard timer after retransmission of the missing PDCP SDUs to the target gNB.

Proposal 3: RAN2 to consider the solution U6 as a potential solution for the lossless inter-gNB path switch.

Observation 2a: Solution U6 may prevent unnecessary buffering at remote UE.

Observation 2b: When applying solution U6, there is a risk of experiencing PDCP status report reception failure at the remote UE.

Proposal 4: RAN2 to consider a complete solution based on both U6 and U3 so as to minimize buffering constraints while mitigating the risk of PDCP status report reception failure.

Proposal 5: RAN2 not to pursue the solution U5 for the lossless delivery issue.

Proposal 6: Based on D4 solution, the target gNB requests the source gNB to forward missing DL packets when detecting a SN mismatch between the PDCP status report received from the remote UE and the last SN status transfer received from the source gNB.

Observation 3: Solution D4 efficiency may be impacted by some potential PDCP SDUs discarding at the source gNB due to potential discard timer expiry.

Proposal 7a: For DL transmission, when considering solution D4, the discard timer of the source gNB should be disabled during path switching operation in order to prevent the discarding of PDCP SDUs that need to be retransmitted. To do so, the two following options may be considered:

- Option 1: The source gNB configures its discard timer to a value that would ensure a lossless data delivery for path switch e.g., infinity value;

- Option 2: The source gNB suspends its discard timer upon reception of a path switch acknowledgement from the target gNB or upon the sending of a path switch command to the remote UE.

Proposal 7b: For DL transmission, after path switch, the discard timer of the source gNB should be resumed to a normal usage:

- Option 1: source gNB configures its PDCP discard timer after the forwarding of the missing PDCP SDUs to the target gNB;

- Option 2: source gNB resumes its discard timer after the forwarding of the missing PDCP SDUs to the target gNB.

Proposal 8: RAN2 to consider a complete solution based on both D4 and D3 so as to minimize buffering constraints while minimizing the effects of PDCP status report reception failure at the source gNB.

Proposal 9: RAN2 not to pursue the solution D5 for the lossless delivery issue.

# References

[1] R2-2304305 - Summary of [AT121bis-e][432] Candidate solutions for lossless delivery

[2] Chairman notes of RAN2#121bis-e meeting