3GPP TSG-RAN WG2 #122	R2-2305028

Incheon, KR, May 22nd – 26th, 2023

Agenda Item:	7.15.3

Source:	Ericsson

Title:	Resource selection and reselection for SL-U

Document for:	Discussion, Decision

# 1	Introduction

Studying and specifying support of sidelink on unlicensed spectrum (i.e., SL-U) is one of the objectives of SL evolution in Rel.18 [1], where channel access mechanisms is one important aspect to be investigated for SL-U. This paper will discuss some issues including further updates/details on e.g., SL resource (re)selection with SL LBT impact, etc.

# 2	Discussion

In RAN2#112bis, RAN2 has discussion whether the UE is triggered to perform reselection of resource pool and/or RB set upon detection of consistent LBT failure in an RB set. There was no consensus due to diverse views among companies.

In our view, LBT failure detection and recovery is an important feature for SL-U, which can enable the UE to switch transmissions from a channel which is subject to LBT failure to other free channels. By doing so, the transmissions will not be subject to LBT failures. This is vital for services with critical QoS requirements (e.g., V2X) to ensure its QoS satisfaction.

UE switches transmissions from a channel which is subject to LBT failure to other free channels, which can ensure the SL transmissions to be not blocked by channel congestion, and therefore, services’ QoS are better guaranteed.

Therefore, it is necessary to reselect resource pools and/or RB sets if the currently selected resource pool and RB sets are subject to consistent LBT failure.

In addition, there are similar mechanisms in NR-U where UE switches between BWPs to get rid of detected LBT failures. The WID has already recommended to reuse NR-U mechanisms for SL-U. There is no reason to not follow NR-U rules to allow the UE to switch from a resource pool or RB set to another resource pool and/or RB sets which are free.

There are similar mechanisms in NR-U where UE switches between BWPs to get rid of detected LBT failures. The WID has already recommended to reuse NR-U mechanisms for SL-U. There is no reason to not follow NR-U rules to allow the UE to switch from a resource pool or RB set to another resource pool and/or RB sets which are free.

In addition, regarding whether the MAC layer or the PHY layer shall model the reselection procedure, we think the MAC layer shall take the responsibility to model reselection of resource pool given below reason

- The selection of resource pool is modelled in the MAC layer. It is natural to also model reselection of resource pool in the MAC layer.

In addition, selection and reselection of RB set is more suitable to be modelled at the PHY layer, since RB set was originally introduced in NR-U and only visible at the PHY layer. For SL-U, it is beneficial to limit the visibility of RB set in the MAC layer. In this case, whether and how to perform RB set selection and reselection shall be up to RAN1 decision.

From RAN2 perspective, the UE shall perform resource pool reselection upon detection consistent LBT failure in the currently selected resource pool and/or RB sets.

Whether and how to perform RB set selection and reselection upon detection of consistent LBT failure shall be up to RAN1 decision.

(Re)selection of resource pool is modelled in the MAC layer.

Consistent LBT failure detection is operated at the MAC layer. The PHY layer has no direct knowledge of consistent LBT failure. Therefore, the MAC layer needs to inform the PHY layer of the information of consistent LBT failure on RB sets, based on that the PHY may consider triggering RB set selection and reselection.

Therefore, we make the below proposal

The MAC layer informs the PHY layer of the information of consistent LBT failure on RB sets.

In case a currently selected resource pool contains/overlaps with one single RB set, detection of consistent LBT failure in the RB set would trigger the UE to reselect resource pool.

In case a currently selected resource pool contain/overlaps with multiple RB set, detection of consistent LBT failure in one RB set shall first trigger reselection to other RB set in the resource pool. Reselection of resource pool shall be triggered only when the rest RB sets cannot provide sufficient resources to the UE.

In case a selected resource pool contains/overlaps with one single RB set, detection of consistent LBT failure in the RB set of the resource pool would trigger the UE to reselect another resource pool.

In case a selected resource pool contains/overlaps with multiple RB sets, detection of consistent LBT failure in all the RB sets of the resource pool would trigger the UE to reselect another resource pool.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Detection and reporting of consistent LBT failures to gNB can be performed even if detection of LBT failure is per BWP.

Based on the discussion in the previous sections we propose the following:

Proposal 1	From RAN2 perspective, the UE shall perform resource pool reselection upon detection consistent LBT failure in the currently selected resource pool and/or RB sets.

Proposal 2	Whether and how to perform RB set selection and reselection upon detection of consistent LBT failure shall be up to RAN1 decision.

Proposal 3	(Re)selection of resource pool is modelled in the MAC layer.

Proposal 4	The MAC layer informs the PHY layer of the information of consistent LBT failure on RB sets.

Proposal 5	In case a selected resource pool contains/overlaps with one single RB set, detection of consistent LBT failure in the RB set of the resource pool would trigger the UE to reselect another resource pool.

Proposal 6	In case a selected resource pool contains/overlaps with multiple RB sets, detection of consistent LBT failure in all the RB sets of the resource pool would trigger the UE to reselect another resource pool.