3GPP TSG-RAN WG2 #122	R2-2305033

Incheon, KR, May 22nd – 26th, 2023

Agenda Item:	7.7.3

Source:	Ericsson

Title:	Discussion on network verified UE location

Document for:	Discussion, Decision

# 1	Introduction

In NR Rel-18 WI on NTN enhancement, there are FFSs pending for further discussions. We address these FFSs in this contribution.

# 2	Discussion

## 2.1 On mirror point ambiguity

RAN2 has discussed this issue and made the below agreement in RAN2#121

- RAN2 assumes that in general the mirror point issue can be resolved by properly configuring neighbor cell measurement to UE, for example, measurement of two neighbor cells in the opposite side of a satellite beam. FFS if there are any cases that require anything in the specs

From RAN2 understanding, the issue can be addressed through UE neighbour cell measurement which is configured by the network. Since the network has the knowledge on the locations where the UE may have mirror point ambiguity issue, the network can just rely on the legacy procedure and signalling to configure the UE to measure neighbour cells well before the UE has moved to those locations. Therefore, there is no additional spec change foreseen.

In order to resolve the mirror point ambiguity issue, the network relies on the legacy signaling and procedure to configure NTN UE to measure and report neighbor cells (e.g., neighbour cells in the opposite side of a satellite beam). No spec changes are needed.

## 2.2 UE capabilities

In the RAN2#120, RAN2 has agreed that

- RAN2 agrees the re-use of the LCS framework of the LMF for the network verification of UE reported location information in NTN.

- RAN2 will work on the details of radio protocol aspects of the verification procedure based on the solution investigated by RAN1

From the above agreements, it is observed that the NTN network will rely on the RAT dependent positioning methods to perform the verification for UE.

In addition, RAN2 has agreed in RAN2#121 that

Agreements:

For network verified UE location, the verification procedure can only be triggered by the CN.

It means that the feature of network verified UE location is a network initiating feature meaning that the network determines whether and when to enable the verification procedure for a UE. Therefore, it is unnecessary to define any additional UE behaviors beyond RAT dependent positioning framework. No additional UE capability is needed for UE to indicate whether the UE supports the feature of network verified UE location. In this way, design efforts for RAN2 can be reduced.

Network verified UE location is a NW-feature which does not require any additional UE behaviour (beyond RAT-dependent positioning), e.g., no additional UE capability is needed for indicating whether UE supports the feature of network verified UE location.

# 3 Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

Proposal 1	In order to resolve the mirror point ambiguity issue, the network relies on the legacy signaling and procedure to configure NTN UE to measure and report neighbor cells (e.g., neighbour cells in the opposite side of a satellite beam). No spec changes are needed.

Proposal 2	Network verified UE location is a NW-feature which does not require any additional UE behaviour (beyond RAT-dependent positioning), e.g., no additional UE capability is needed for indicating whether UE supports the feature of network verified UE location.