3GPP TSG-RAN WG2 #122	R2-2305035

Incheon, KR, May 22nd – 26th, 2023

Agenda Item:	7.10.2 FDM solution enhancements

Source:	Ericsson

Title:	IDC configuration and report in MR-DC

Document for:	Discussion, Decision

# 1	Introduction

In [1], companies have expressed their view on whether coordination between MN and SN is needed for MR-DC. This issue is relevant to both a FDM solution and a TDM solution for IDC in NR Rel-18 in this document we further elaborate details on this topic.

# 2	Discussion

Whether enhancements need to be introduced for MR-DC has been also discussed in [1]. In our view, it is beneficial to let UE to only report IDC to the affected node, which will address the issue. Therefore, beneficial to allow SN to configure IDC on frequency regions which only affects SCG. Then, UE can report IDC on frequency regions which only affects SCG to SN.

In addition, we think it is sufficient that MN only configures IDC on frequency regions which only affects MCG. In this way, we can avoid MN and SN to coordinate for configuring the UE for IDC, which would add additional design complexity.

It is beneficial to avoid MN and SN to coordinate for configuring the UE for IDC, which would add additional design complexity.

Given for EN-DC, it was already supported that SN can indicate its frequencies of candidate serving cells to MN via candidateServingFreqListNR in CG-Config. Meanwhile, upon reception of the IDC assistance information from UE, MN can forward the IDC assistance information to SN via affectedCarrierFreqCombInfoListMRDC in CG-ConfigInfo. Such existing mechanism can be directly applied for EN-DC without further enhancement.

RAN2 has agreed in RAN2#120 that

- no additional co-ordination is needed for IDC configuration, apart from the existing mechanism between MN and SN (i.e. candidateServingFreqListNR in CG-Config for EN-DC).

It is concluded that no additional co-ordination is needed for IDC configuration, apart from the existing mechanism between MN and SN (i.e. candidateServingFreqListNR in CG-Config for EN-DC).

For NR-DC, there is FFS on whether coordination is needed between MN and SN.

Companies have raised concerns on whether coordination between MN and SN is needed for IMD issue. IMD issue may be detected by UE in the below cases

Case 1: IMD issue between simultaneous TX of carriers in MCG and non-3GPP

Case 2: IMD issue between simultaneous TX of carriers in SCG and non-3GPP

Case 3: IMD issue between simultaneous TX of MCG carriers and SCG carriers, and non-3GPP

For Case 1 and Case 2, UE can just report IDC issues for combination of carriers in MCG or in SCG using existing signalling of combinations of candidates. There is no coordination needed between MN and SN.

Coordination is not needed between MN and SN for IMD issue only concerning carriers in the CG.

For case 3, there is no coordination needed either. In this case, it is sufficient to leave up to UE implementation to only report the affected MN candidate frequencies to MN or affected SN candidate carriers to the SN. For the former, MN would deactivate the affected carriers in MCG, For the latter, SN would deactivate the affected carriers in SCG. If the UE decides to send the IDC indication to the MN, the MN would de-configure/similar the MN-frequencies which are part of causing the IMD issues, and if the UE decides to send the IDC indication to the SN, the SN would de-configure/similar the SN-frequencies which are part of causing the IMD issues. An alternative to this would be for the UE to indicate a combination of MN candidate frequencies and SN candidate frequencies to the MN and/or SN and the MN and SN would coordinate to decide if the MN should address the IMD issues by de-configuring/similar the MN carriers, or whether the SN should address the IMD issues. However, we believe that it is sufficient to leave it to the UE implementation to send the IDC indication to either the MN or the SN, and hence we can avoid additional work of specifying coordination between nodes. In addition, it has been agreed to support per CG TDM pattern. Therefore, it would be beneficial to avoid further coordination between MN and SN even for IMD issue. Thus, we can have a unified framework for both FDM solutions and TDM solutions.

Therefore, we make the below proposal

Upon detecting IMD issue for carriers across MCG and SCG, it is up to UE implementation to determine to report only the combinations of affected frequencies and bandwidth in the MCG to the MN or the combinations of affected frequencies and bandwidth in SCG to the SN.

As a further step, we think it is beneficial to capture this the behaviour of Proposal 2 in the spec and it is likely sufficient with a description in the Stage 2 spec. This rule is applicable to both EN-DC and NR-DC.

Currently, IDC is captured in TS 36.300. Meanwhile NR spec TS 38.300 refers to the IDC clause in TS 36.300. Given this, it is up to the WI rapporteur to decide how to capture the behaviour of Proposal 3 in the specs (i.e., whether to capture the behaviour of Proposal 3 in both LTE spec and NR spec, or only capture it in LTE spec).

Capture in the Stage 2 spec (i.e., TS 36.300 and/or TS 38.300) that it is up to UE implementation to determine to report only the affected frequencies and bandwidth in the MCG to the MN or affected frequencies and bandwidth in the SCG to the SN, upon detecting IMD issue for carriers across MCG and SCG.

In addition,

- The current agreed framework is sufficient for UE to report IDC issues for NR-DC. Any coordination mechanism can be categorized as optimization. It can be up to NW implementation or OAM to address the need of any coordination.

- Due to limited work time in Rel-18, there is no time to work on any further enhancement, which would require additional design efforts.

Therefore, we make the below proposal

For NR-DC, RAN2 not to standardize coordination mechanism between MN and SN for either IDC configuration or IDC assistance information report.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is beneficial to avoid MN and SN to coordinate for configuring the UE for IDC, which would add additional design complexity.

Observation 2	It is concluded that no additional co-ordination is needed for IDC configuration, apart from the existing mechanism between MN and SN (i.e. candidateServingFreqListNR in CG-Config for EN-DC).

Observation 3	Coordination is not needed between MN and SN for IMD issue only concerning carriers in the CG.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Upon detecting IMD issue for carriers across MCG and SCG, it is up to UE implementation to determine to report only the combinations of affected frequencies and bandwidth in the MCG to the MN or the combinations of affected frequencies and bandwidth in SCG to the SN.

Proposal 2	Capture in the Stage 2 spec (i.e., TS 36.300 and/or TS 38.300) that it is up to UE implementation to determine to report only the affected frequencies and bandwidth in the MCG to the MN or affected frequencies and bandwidth in the SCG to the SN, upon detecting IMD issue for carriers across MCG and SCG.

Proposal 3	For NR-DC, RAN2 not to standardize coordination mechanism between MN and SN for either IDC configuration or IDC assistance information report.