3GPP TSG-RAN WG2 #122	R2-2305036

Incheon, KR, May 22nd – 26th, 2023

Agenda Item:	7.25.1

Source:	Ericsson

Title:	Discussion on MAC-CE based indication for Cross-RRH TCI state switch

Document for:	Discussion, Decision

# 1	Introduction

RAN WG4 is working on the Release 18 NR_HST_FR2_enh WI on enhanced support for HST scenario in FR2. One of the objectives of the Release 18 WI [WID, RP-220985] is to

- Specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2].

In the recent RAN4 meeting, RAN4 has made the below agreement

- Introduce MAC-CE based solution with 1bit indication to inform UE on the TCI state switch across RRHs

Based on that agreement, RAN4 has sent a LS R4-2306399 to RAN2 for asking RAN2 to do the design work for the MAC CE. Therefore, we discuss potential RAN2 impact for this LS in this paper.

# 2	Discussion

## 2.1 RAN4 LS

In the RAN4 LS R4-2306399, RAN WG4 has introduced the support for High Speed Train (HST) scenarios in NR Frequency Range (FR2) in Release 17 NR_HST_FR2 WI.

As a result Release 17 discussion, a RAN4 requirement on One shot large UL timing adjustment for FR2 Power Class 6 UE was introduced in Clause 7.1.2.3 of TS 38.133.

RAN WG4 is working on the Release 18 NR_HST_FR2_enh WI on enhanced support for HST scenario in FR2. One of the objectives of the Release 18 WI [WID, RP-220985] is to

- Specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2].

RAN4 has concluded that NW signalling assistance is needed to indicate when the UE shall apply one-shot large uplink timing adjustment at TCI state switch. The following was agreed at RAN4#106bis-e:

- Introduce MAC-CE based solution with 1bit indication to inform UE on the TCI state switch across RRHs

Action 1: RAN4 respectfully asks RAN2 to introduce MAC-CE 1bit indication to inform UE on the TCI state switch across non-collocated RRHs.

## 2.2 Discussion on the new MAC CEs

In HST scenario, TCI state of a UE needs to be frequently updated as the train moves along the track.

The update of TCI state can be categorized to 2 types:

- TCI state switch within same (remote radio head) RRH/(transmission reception point)TRP

- TCI state switch among different RRHs/TRPs

Currently, the beam/TCI state switching procedure assumes that the same Timing Advance (TA) is applicable to the UE in source and target beam in the same cell.  Therefore, with the existing procedure, a UE cannot distinguish whether the TCI state switch occurs for the same RRH or different RRHs.

In the legacy, upon reception of a TCI state switch command, the UE cannot distinguish the target TCI state and the old TCI state belong to the same or different RRHs/TRPs

However, in high speed train (HST) scenario, it is important for the UE to be aware of whether the TCI state switch occurs concerning the same or different RRHs/TRPs, based on which the UE can determine whether to perform one-shot large TA adjustment which has been already introduced in 3GPP Rel-17.

In high speed train (HST) scenario, it is important for the UE to be aware of whether the TCI state switch occurs concerning the same or different RRHs/TRPs, based on which the UE can determine whether to perform one-shot large TA adjustment which has been already introduced in 3GPP Rel-17.

Therefore, RAN4 in the LS indicates that the NW shall indicate the UE on the TCI state switch across non-collocated RRHs

RAN4 LS indicates that the NW shall be able to indicate the UE on whether the TCI state switch is across non-collocated RRHs via a MAC CE.

We understand that RAN4 discussions concerns TCI state for PDCCH monitoring. Whenever the UE has moved in coverage area of a target RRH/TRP, the UE needs to first decode PDCCH successfully with a correct RX beam in order to perform subsequent DL receptions and UL transmissions. Therefore, it is sufficient to only extend/update the MAC CEs for indicating target TCI state for PDCCH monitoring.

Whenever the UE has moved in coverage area of a target RRH/TRP, the UE needs to first decode PDCCH successfully with a correct RX beam in order to perform subsequent DL receptions and UL transmissions.

Therefore, we would like to make the below proposal.

RAN2 to confirm that the RAN4 LS only affects the MAC CEs intended for indicating target TCI state for PDCCH monitoring.

Therefore, the relevant MAC CEs are TCI State Indication for UE-specific PDCCH MAC CE and Enhanced TCI States Indication for UE-specific PDCCH MAC CE.

In the first MAC CE, there is no R field which can be used for the additional bit. In the second MAC CE, the MAC CE carries two target TCI states, which mean that we need two additional bits for the two targets TCI states wherein each bit indicates whether the corresponding target TCI state belongs to the same or different RRHs as the old TCI state. Since there is only 1 R field which can be used for the first target TCI state, we need to introduce one more R field/bit for the second target TCI state.

There is no R bit/field which can be used for the additional bit in TCI State Indication for UE-specific PDCCH MAC CE.

Enhanced TCI States Indication for UE-specific PDCCH MAC CE carries two target TCI states. therefore two additional bits for the two targets TCI states need to be introduced wherein each bit indicates whether the corresponding target TCI state belongs to the same or different RRHs as the old TCI state. Since there is only 1 R field which can be used for the first target TCI state, we need to introduce one more R field/bit for the second target TCI state.

Based on the above analyses, it seems clear that at least TCI State Indication for UE-specific PDCCH MAC CE is to be affected. RAN2 can further check whether Enhanced TCI States Indication for UE-specific PDCCH MAC CE is also affected. Alternatively, RAN2 checks with RAN4 whether Enhanced TCI States Indication for UE-specific PDCCH MAC CE is to be affected. Enhanced TCI States Indication for UE-specific PDCCH MAC CE is used by the gNB to indicate two target TCI states to the UE for PDCCH monitoring. So the UE can monitor PDCCHs simulateneously transmitted at different beams according to the two TCI states. This MAC CE was introduced for enhanced MIMO operation. Whether it is supported for high speed train needs to be checked with RAN4.

Given there is no sufficient R bit left in the existing MAC CEs, it is reasonable to define new MAC CEs to extend the existing TCI State Indication for UE-specific PDCCH MAC CE and Enhanced TCI States Indication for UE-specific PDCCH MAC CE.

Therefore, we would like to make the below proposal.

Define a new MAC CE (e.g., named as Cross-RRH TCI State Indication for UE-specific PDCCH) to enhance the existing TCI State Indication for UE-specific PDCCH MAC CE by introducing one additional bit indicating whether the target TCI state belongs to the same or different RRH than the old TCI state.

RAN2 sends a LS to RAN4 asking whether the enhanced TCI state indication (i.e., indication of two target TCI states) is supported for high-speed train scenario.

In addition, for byte alignment purpose, the new MAC CE would contain one more byte compared to the existing MAC CE.

The new MAC CE (e.g., named as Cross-RRH TCI State Indication for UE-specific PDCCH) has one more byte in the payload compared to the corresponding existing MAC CE.

Accordingly, two LCIDs are needed to be defined for the two MAC CEs respectively. It is natural to define LCIDs in the eLCID space, since there are limited values left in the LCID space.

Define new LCID in the eLCID space for the new MAC CE.

## 2.3 Impacts to the MAC procedure

HighSpeedConfigFR2-r17 ::=  SEQUENCE {

highSpeedMeasFlagFR2-r17                    ENUMERATED {set1, set2}                       OPTIONAL,   -- Need R

highSpeedDeploymentTypeFR2-r17              ENUMERATED {unidirectional, bidirectional}    OPTIONAL,   -- Need R

highSpeedLargeOneStepUL-TimingFR2-r17       ENUMERATED {true}                             OPTIONAL,   -- Need R

...

}

HighSpeedConfig field descriptions

HighSpeedDemodCA-Scell If the field is present and UE supports demodulationEnhancementCA-r17, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4 [59]. This parameter only applies to SCell.

highSpeedDemodFlag If the field is present and UE supports demodulationEnhancement-r16, the UE shall apply the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h as specified in TS 38.101-4 [59]. This parameter only applies to SpCell.

highSpeedDeploymentTypeFR2 If the field is present, and field value is unidirectional, the UE shall assume uni-directional deployment or if field value is birectional the UE shall assume bidirectional deployment for FR2 up to 350km/h as specified in TS 38.133 [14].

highSpeedLargeOneSteptUL-TimingFR2 If the field is present, large one step UE autonomous uplink transmit timing adjustment for FR2 up to 350km/h as specified in TS 38.133 [14] is enabled.

In 3GPP Rel-17, there is a RRC parameter highSpeedDeploymentTypeFR2 introduced which indicates whether the large one-shot UE autonomous TA adjustment is enabled or not by the UE.

While the new MAC CEs give a more flexible way to the network to signal the UE on whether the one-shot UE autonomous TA adjustment shall be applied or not upon reception of a MAC CE based indication.

It is reasonable to let the UE to check both the RRC parameter and the MAC CE to determine whether the UE shall apply one-shot autonomous TA adjustment. Therefore, the logic should be defined as the below

If the RRC parameter highSpeedDeploymentTypeFR2 is present (true) and the MAC CE carries the bit with value 1

The UE applies one shot autonomous TA adjustment

Else if the RRC parameter highSpeedDeploymentTypeFR2 is absent (false) and the MAC CE carries the bit with value 1

The UE applies the RA procedure to obtain the TA value.

Else

The UE follows the legacy procedure i.e., the TA command received from the gNB

End

However, it is necessary to check with RAN4 what the intended UE behavior upon reception of this new MAC CE is.

RAN2 sends a LS to RAN4 asking what the intended UE behavior upon reception of the MAC CE is.

Based on the above proposals, we have also prepared the corresponding draft CR [1].

Adopt the changes captured in the draft CR R2-2305037.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	In the legacy, upon reception of a TCI state switch command, the UE cannot distinguish the target TCI state and the old TCI state belong to the same or different RRHs/TRPs

Observation 2	In high speed train (HST) scenario, it is important for the UE to be aware of whether the TCI state switch occurs concerning the same or different RRHs/TRPs, based on which the UE can determine whether to perform one-shot large TA adjustment which has been already introduced in 3GPP Rel-17.

Observation 3	RAN4 LS indicates that the NW shall be able to indicate the UE on whether the TCI state switch is across non-collocated RRHs via a MAC CE.

Observation 4	Whenever the UE has moved in coverage area of a target RRH/TRP, the UE needs to first decode PDCCH successfully with a correct RX beam in order to perform subsequent DL receptions and UL transmissions.

Observation 5	There is no R bit/field which can be used for the additional bit in TCI State Indication for UE-specific PDCCH MAC CE.

Observation 6	Enhanced TCI States Indication for UE-specific PDCCH MAC CE carries two target TCI states. therefore two additional bits for the two targets TCI states need to be introduced wherein each bit indicates whether the corresponding target TCI state belongs to the same or different RRHs as the old TCI state. Since there is only 1 R field which can be used for the first target TCI state, we need to introduce one more R field/bit for the second target TCI state.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that the RAN4 LS only affects the MAC CEs intended for indicating target TCI state for PDCCH monitoring.

Proposal 2	Define a new MAC CE (e.g., named as Cross-RRH TCI State Indication for UE-specific PDCCH) to enhance the existing TCI State Indication for UE-specific PDCCH MAC CE by introducing one additional bit indicating whether the target TCI state belongs to the same or different RRH than the old TCI state.

Proposal 3	RAN2 sends a LS to RAN4 asking whether the enhanced TCI state indication (i.e., indication of two target TCI states) is supported for high-speed train scenario.

Proposal 4	The new MAC CE (e.g., named as Cross-RRH TCI State Indication for UE-specific PDCCH) has one more byte in the payload compared to the corresponding existing MAC CE.

Proposal 5	Define new LCID in the eLCID space for the new MAC CE.

Proposal 6	RAN2 sends a LS to RAN4 asking what the intended UE behavior upon reception of the MAC CE is.

Proposal 7	Adopt the changes captured in the draft CR R2-2305037.

# 4 References

- R2-2305037 Draft CR on Correction to MAC on MAC-CE based indication for Cross-RRH TCI state switch, Ericsson