3GPP TSG-RAN WG2 Meeting #122	 R2-2305038

Incheon, KR, May 22nd – 26th, 2023

Title:	[Draft] LS reply to RAN4 LS R4-2306339

Response to:	R4-2306399

Release:	Rel-18

Work Item:	NR_HST_FR2_Enh

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact person:	Min Wang

min.w.wang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 has concluded that NW signalling assistance is needed to indicate when the UE shall apply one-shot large uplink timing adjustment at TCI state switch. The following was agreed at RAN4#106bis-e:

- Introduce MAC-CE based solution with 1 -bit indication to inform UE on the TCI state switch across RRHs

In RAN4 LS R4-2306399, RAN4 respectfully asks RAN2 to introduce MAC-CE 1bit indication to inform UE on the TCI state switch across non-collocated RRHs.

RAN2 understands that RAN4 discussions concern TCI state switch for PDCCH monitoring. Whenever the UE has moved to the coverage area of a target RRH/TRP, the UE needs to first decode PDCCH successfully with a correct RX beam in order to perform subsequent DL receptions and UL transmissions. Therefore, the RAN4 LS only affects the MAC CEs intended for indicating target TCI state for PDCCH.

In addition, in the current MAC spec, there is also a MAC CE for enhanced TCI state indication for PDCCH. The gNB can use that MAC CE to indicate two target TCI states to the UE for PDCCH monitoring. Whether  the functionality related to this MAC CE is supported for high speed train use case needs to be checked with RAN4.

In addition, there is an existing RRC parameter highSpeedDeploymentTypeFR2 introduced in 3GPP Rel-17 which indicates whether the large one-shot UE autonomous TA adjustment is enabled or not by the UE. Both this RRC parameter and the MAC CE can indicate the UE on whether the one-shot UE autonomous TA adjustment shall be applied or not. In such case, the intended UE behavior needs to be defined in relation to the RRC parameter highSpeedDeploymentTypeFR2 and the MAC CE with 1-bit indication on the TCI state switch across RRHs.

Therefore, RAN2 would like to seek RAN4’s input for the below questions:

- Question 1: Is it correct RAN2 understanding that the RAN4 LS only affects the MAC CEs intended for indicating target TCI state for PDCCH?

- Question 2: Whether the enhanced TCI state indication (i.e., the MAC CE indicates two target TCI states) is intended to be supported for the high-speed train use case? I.e., whether the MAC CE related to enhanced TCI state indication for PDCCH is also to be affected by the RAN4 LS?

- Question 3: What is the intended UE behavior upon reception of the MAC CE with indication on the TCI state switch across RRHs?

# 2	Actions

To RAN4:

ACTION: 	RAN2 respectfully asks RAN4 to provide responses to the above questions.

# 3	Dates of next TSG-RAN WG2 meetings

TSG RAN WG2 Meeting #123	August 21 – August 25, 2023	Toulouse, FR

TSG RAN WG2 Meeting #123-bis	October 9 – October 13, 2023	Xiamen, CN