3GPP TSG-RAN WG2 Meeting #122	R2-2305042

Incheon, Korea, May 22-26, 2023

Title: 	Discussion on miscellaneous issues for mobile IAB

Source: 	ZTE, Sanechips

Agenda item:	7.12.3

Document for:	Discussion and Approval

Introduction

In this paper, we will analyze some miscellaneous issues for the support of mobile IAB, such as TAC/RNAC configuration, PCI collision and ANR.

Discussion

TAC and RNAC configuration

TA configuration

During RAN3#119 meeting, RAN3 agreed that TAC/RANAC broadcast by the mobile IAB-DU can be changed in order to reflect the mIAB-node’s physical location. Moreover, RAN3 agreed in RAN3#119bis meeting that the IAB-DU’s TAC can have the same or a different value than the TAC of the mIAB-MT’s serving cell.

In our opinion, TAC update of mobile IAB node may occur when the mobile IAB-node perform intra-gNB partial migration or inter-gNB partial migration or full migration. The new TAC value can be configured to mobile IAB-DU via F1 signaling or RRC signaling (e.g. RRCReconfiguration) and then mobile IAB-MT transport the TAC value to its collocated mobile IAB-DU. For inter-donor partial migration or full migration with mobile IAB-MT and mobile IAB-DU migrating to different donors, if gNB determines to configure a new TAC value to mobile IAB node via F1 signaling, then mobile IAB-DU’s donor need to acquire some essential TAC information from mobile IAB-MT’s donor via Xn or NG, which means more specification impact should be considered.

With regard to the RRC signalling based approach, two options are mentioned during last meeting as follows:

- Option 1: Mobile IAB-MT reads the TAC broadcast by its parent cell and delivers it to the co-located DU

- Option 2: Mobile IAB-MT receives the TAC configured by target donor CU via dedicated signalling and delivers it to the co-located DU

For Option 1, if the legacy TAC broadcast by mobile IAB-MT’s parent cell is directly used for mobile IAB-DU, it means that the TAC of mobile IAB-DU’s cell aligned with the TAC broadcast by mobile IAB-MT’s parent cell. However, it’s not optimal since there may be some problem in forbidden Tracking Area mechanism. Based on TS 23.501, for the UE that does not support CAG functionality, NG-RAN and 5GC are allowed to use not only CAG mechanism but also the other existing mechanism e.g. forbidden Tracking Area, to manage its access to MBSR. In order to differentiate the TA neighboring cells, it would be better for the mobile IAB-MT’s parent cell broadcast a new TAC specific for mobile IAB node in addition to the legacy TAC.

For Option 2, the when mobile IAB-MT performs handover, the target donor CU may deliver the new TAC to mobile IAB-MT via RRCReconfiguration message and then the mobile IAB-MT delivers this new TAC value to its collocated mobile IAB-DU. Finally, the mobile IAB-DU broadcast this new TAC to UEs served by this mobile IAB-DU.

Proposal 1: Mobile IAB-MT may receive a new TAC value for mobile IAB node either from system information or from dedicated signalling from donor CU during mobile IAB-MT HO and then mobile IAB-MT delivers the new TAC value specific for mobile IAB node to its collocated mobile IAB-DU for SIB broadcasting.

RNA configuration

For NR system, the UE is allowed to enter RRC_INACTIVE state. Before entering the RRC_INACTIVE state, the UE is configured by the last serving gNB with an RNA. The RNA can cover one or multiple cells, and should be a subset of or equal to a CN Tracking Area. To be specific, the RNA may be configured via a list of cells or a list of RAN Area Code (RANAC) or TAC for different PLMNs. The involved cells will broadcast one RANAC in the SIB1 if the RNA is defined via RANAC. UE needs to perform RNAU when the UE re-selects a cell that does not belong to its previously configured RNA. When UE performs RNAU, it may initiate the RRC connection resumption procedure toward gNB, providing the I-RNTI allocated by the last serving gNB and appropriate cause value.

With it comes to the mobile IAB scenario, the mobile IAB node moves together with the vehicles, which may be across different RNAs. As agreed in RAN3, the TAC/RANAC broadcast by the mobile IAB-DU can be changed in order to reflect the mIAB-node’s physical location. To be specific, when the mobile IAB-node performs HO from source donor to target donor, the mobile IAB-MT may detect that the RANAC/TAC of target donor’s cell is different from the RANAC/TAC broadcast by co-located mobile IAB-DU. In order to keep the UE reachability within RAN area, the mobile IAB-DU’s cells may change the RANAC values broadcast in system information to keep align with the cell of parent donor. Alternatively, the target donor CU may send the RANAC/TAC configuration to mobile IAB-MT via RRC signalling during mobile IAB-MT migration procedure and then mobile IAB-DU’s cell broadcast this RANAC/TAC value in system information.

In order to reduce the RNAU of RRC_INACTIVE UEs served by the mobile IAB-DU’s cell, it is suggested that the donor CU of mobile IAB node provide UEs served by the mobile IAB-DU’s cell with a list RANACs, which potentially will be broadcast by the mobile IAB node along the movement and where the Xn connection is available between donor CU and potential target CUs for mobile IAB node. In this way, the UEs served by the mobile IAB node do not need to perform RNAU when the mobile IAB-DU changes RANACs.

Proposal 2: To support the dynamic change of RANAC, the following two options may be considered:

- 1)The mobile IAB-DU’s cells may change the RANAC values broadcast in system information to keep align with the RANAC of parent donor.

- 2)The target donor CU may send the RANAC configuration to mobile IAB-MT via RRC signalling and then mobile IAB-DU’s cell broadcast this RANAC value in system information.

- Proposal 3: In order to reduce the RNAU of RRC_INACTIVE UEs served by the mobile IAB-DU’s cell, it is suggested that the donor CU provide UEs served by the mobile IAB-DU’s cell with a list RANACs, which potentially will be broadcast by the mobile IAB nodes along the movement.

- On the other hand, RNA may be configured via a list of NCGIs. Suppose the UE is configured with a list of cells for RNA when entering into RRC_INACTIVE state, the UEs served by the mobile IAB-DU may detect that the NCGI change of mobile IAB-DU when the mobile IAB node performs HO. It is very likely that the new NCGI of mobile IAB-DU is not included in the list of cells for UE's RNA and the RRC_INACTIVE UE served by the mobile IAB-DU will perform the RNAU procedure. In order to reduce unnecessary RNAU, it is suggested that the RRC_INACTIVE UE may be notified of the NCGI change of the cells of mobile IAB-node-DU and UE replaces the old NCGI with the new one in its cell list of RAN area.

Proposal 4: For the RNA configuration via a list of NCGIs, in order to reduce unnecessary RNAU, the  RRC_INACTIVE UE may be notified of the NCGI change of the cells of mobile IAB-node-DU and UE may then replace the old NCGI with new NCGI of mobile IAB-node-DU’s cell for the cells included in the RNA configuration.

PCI collision

The PCI of mobile IAB-DU cell may collide with other cells due to the movement of mobile IAB-MT. During RAN3#117 bis meeting , it has been agreed that PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node. For example, the donor CU should be able to know the PCI used by the mobile IAB-DU via the F1 Setup procedure. During the full migration, the target donor CU could detect PCI collision upon receiving the F1 setup request (which include the PCI used by the mobile IAB-DU) from mobile IAB-DU. Suppose the PCI collision is detected, the target donor CU may request a new PCI from OAM or select a new PCI value from the preconfigured PCI list, and then send it to the mobile IAB-DU.

When it comes to the partial migration, the mobile IAB-DU has only F1 connection with F1-terminating donor CU and the F1-terminating donor CU may be not aware of the PCIs used in the neighbouring cells of the mobile IAB cell. It is doubtful how the F1-terminating donor CU detects the PCI collision of mobile IAB-DU. In our opinion, if there is Xn connection between the F1-terminating donor CU and target donor CU, the F1-terminating donor CU can be aware of PCIs of serving cells and neighbour cells of the target donor CU via existing XnAP signaling, e.g., Xn SETUP/NG-RAN NODE CONFIGURATION UPDATE. Thus F1-terminating donor CU can detect potential PCI collision and then reconfigure the PCI for mobile IAB-DU’s cell when necessary. However, if the Xn connection is not available, it is hard for the F1-terminating donor CU to be aware of the PCI collision. On the other hand, the PCI reconfiguration after the F1 connection SETUP or NG-RAN node CONFIGURATION UPDATE may lead to longer latency and cause interference to neighbor cells.  So it needs to be discussed how to predict PCI collision so that PCI collision can be avoided.

Based on the above analysis, it is suggested that the new PCI be reconfigured as soon as possible before the potential collision happens. To be specific, the PCI used by mobile IAB DU’s cells may be sent by source donor CU to the target donor CU during HO preparation phase of co-located mobile IAB-MT. If the target donor CU receives the PCI of mobile IAB DU’s cells and predicts PCI collision, the target donor CU may re-assign the PCI for the mobile IAB-DU’s cells and send it to source donor CU. Upon receiving the Reconfigurationwithsync from source donor CU, the mobile IAB-MT delivers the new PCI configuration of mobile IAB cells to the co-located mobile IAB-DU, which will then activate another set of cells with the new PCIs.

Proposal 5: RAN2 to consider the PCI re-assignment for mobile IAB-DU’s cell during the HO preparation phase of co-located mobile IAB-MT.

ANR

As we know, the ANR function resides in the gNB, which manages the Neighbour Cell Relation Table (NCRT). Generally speaking, the NCR may contain a list of entries for each source cell which include the NR CGI, NR PCI of target cell and attributes such as Xn availability, HO, etc. Located within ANR, the Neighbour Detection Function finds new neighbors and adds them to the NCRT. The gNB may obtain the NCR information via OAM configuration, neighbor information exchange via Xn setup or gNB Configuration Update procedure. In addition, gNB may request UE to use the newly discovered PCI as parameter, to read all the broadcast NCGI(s) /ECGI(s), TAC(s), RANAC(s), PLMN ID(s) and, for neighbour NR cells, NR frequency band(s). Then UE report these information of neighboring NR cells to gNB. gNB may decide to add this neighbour relation to NCRT. In addition, gNB may use PCI and NCGI(s) to lookup a transport layer address to the new NG-RAN node and setup a new Xn interface towards this NG-RAN node.

When it comes to the mobile IAB scenario, the mobile IAB node may notify the cells information to donor CU via F1 interface once the mobile IAB node MT has connected or handover to donor CU. This donor CU may then notify the neighbor gNB with the updated served cell info via Xn interface. For the other neighbor gNB which has no Xn interface with donor CU, the UE may detect the PCI of mobile IAB cells and report to the neighbor gNB.

As a matter of fact, it is not recommended for the neighboring UE served by non-mobile neighboring cells to HO to the mobile IAB cells. Therefore it is also not necessary to add the mobile IAB cells to the neighboring gNB’s NCRT. To support this, when UE report the mobile IAB indication together with other NCGI(s) /ECGI(s), TAC(s), RANAC(s), PLMN ID(s) and, NR frequency band(s) information. Based on this information, the neighboring gNB decides not to add the entry for mobile IAB cells into the NCRT.

Proposal 6: To avoid neighbor gNB to add the mobile IAB cells into NCRT, it is suggested UE indicates to gNB that the cell is for mobile IAB when UE is requested to report more information for a PCI associated with mobile IAB cell.

Conclusion

In this contribution, we focus on the remaining issues for mobile IAB and present our point of view. The following proposals are given:

Proposal 1: Mobile IAB-MT may receive a new TAC value for mobile IAB node either from system information or from dedicated signalling from donor CU during mobile IAB-MT HO and then mobile IAB-MT delivers the new TAC value specific for mobile IAB node to its collocated mobile IAB-DU for SIB broadcasting.

Proposal 2: To support the dynamic change of RANAC, the following two options may be considered:

- 1)The mobile IAB-DU’s cells may change the RANAC values broadcast in system information to keep align with the RANAC of parent donor.

- 2)The target donor CU may send the RANAC configuration to mobile IAB-MT via RRC signalling and then mobile IAB-DU’s cell broadcast this RANAC value in system information.

- Proposal 3: In order to reduce the RNAU of RRC_INACTIVE UEs served by the mobile IAB-DU’s cell, it is suggested that the donor CU provide UEs served by the mobile IAB-DU’s cell with a list RANACs, which potentially will be broadcast by the mobile IAB nodes along the movement.

Proposal 4: For the RNA configuration via a list of NCGIs, in order to reduce unnecessary RNAU, the  RRC_INACTIVE UE may be notified of the NCGI change of the cells of mobile IAB-node-DU and UE may then replace the old NCGI with new NCGI of mobile IAB-node-DU’s cell for the cells included in the RNA configuration.

Proposal 5: RAN2 to consider the PCI re-assignment for mobile IAB-DU’s cell during the HO preparation phase of co-located mobile IAB-MT.

Proposal 6: To avoid neighbor gNB to add the mobile IAB cells into NCRT, it is suggested UE indicates to gNB that the cell is for mobile IAB when UE is requested to report more information for a PCI associated with mobile IAB cell.

Reference

- RAN2#121 Chairman notes.