3GPP TSG-RAN WG2 Meeting #122                                                                R2-2305046

# Incheon, KR, May 22th~ 26th, 2023

Title: 	Further discussion on the support of multi-path relaying

Source: 	ZTE, Sanechips

Agenda item:	7.9.4

Document for:	Discussion and Approval

Introduction

In this paper, we will focus on the remaining issues specific for scenario1 or common for scenario 1 and scenario 2. Our point of view are presented.

FFS if there are cases where the configuration of non-split SRBs over indirect path is useful. In scenario 2, if both remote and relay UE are in RRC_CONNECTED, the remote UE reports relay UE’s ID to gNB for indirect path addition. Need for reporting in the idle/inactive case can be further discussed. FFS what ID is used. A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment. FFS whether to apply the same behaviour 1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE. This agreement does not imply any conclusion on non-split SRB1 on indirect path.

Discussion

Non-split SRBs over indirect path

In last RAN2 meeting, there is one FFS issue that if there are cases where the configuration of non-split SRBs over indirect path is useful. Additionally, another similar case is that if there are cases where the primary RLC channel for split SRB1 can be configured on indirect path. From our view, following cases can be considered:

- Case1: when both direct path and indirect path are available for UE to initiate initial access, according to current specification, it’s up to UE implementation to select the path used for initial access. It is possible that UE will select indirect path to perform initial access, then the SRB1/2 bearer type is indirect bearer. After adding a direct path by gNB, gNB may do not want to switch the bearer type of SRB1 and SRB2. In this case, the non-split SRB is kept on indirect path.

- Case2: direct path experiences bad channel condition or direct path is congested. For example, the RSRP of direct path RSRP is below the threshold for relay discovery. In this case, although direct path is available, it is still reasonable for UE to be configured with indirect path SRB.

Furthermore, some companies think if SRB1 is configured as non-split indirect bearer, when RLF is detected on indirect path, direct path can not be used for indirect path RLF report. It looks strange to perform RRC re-establishment since direct path is available. From our view, UE’s behaviour is controlled by network. UE will perform RRC re-establishment when direct path is available due to SRB1 is configured as non-split indirect bearer. Based on network implementation, gNB knows the state of UE clearly, and choose to configure SRB1 as a non-split indirect bearer. This means, from network perspective, it is acceptable to make UE to perform RRC re-establishment when direct path is available.

- For scenario 1, the bearer type (i.e. direct bearer, indirect bearer, or multi-path split bearer) of SRB1 and SRB2 may be configured by the gNB independently. It is not necessary to mandate the same bearer type configuration of SRB1 and SRB2.

FFS issue for relay UE status report

In last meeting, RAN2 agrees that upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment. But it is FFS whether to apply the same behaviour 1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE.

For case1, as we know, handover of relay UE is decided by gNB. Before transmitting the handover decision, gNB can release the indirect path or indicate the indirect path switch to remote UE. So, we do not see the necessity for remote UE to apply same behaviour in this case.

For case 2, the uplink/downlink transmission and the length of data inactivityTimer is controlled by gNB. For the relay UE, it can be left to gNB implementation to not configure the dataInactivityTimer to relay UE. Therefore, no need to discuss the potential issues of this case any more.

- RAN2 not further discuss the following two cases:1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE.

Triggering the RRC_IDLE/RRC_INACTIVE target relay UE to initiate RRC connection establishment

In previous RAN2 meeting, during the discussion of triggering IDLE/INACTIVE relay UE enter RRC connected state, legacy R17 behaviour(RLC0 or RLC1 message triggering) will be re-used when split SRB1 is configured. For other cases, i.e. the SRB1 is not configured or suspended on indirect path, PC5-RRC signaling trigger will be used.  Additionally, one FFS issue is left, i.e. if a Rel-17 relay UE is supported for use with multi-path and how the PC5-RRC signaling trigger can be supported.

From our view, whether a R17 Relay UE is supported during indirect path addition depends on following aspects:

1. whether a R17 relay UE can be discovered by remote UE during indirect path addition,

2. whether a R17 IDLE/INACTIVE relay UE can be triggered to enter RRC connected state,

3. whether a R17 relay UE can forward indirect path traffic of MP remote UE.

For the first issue, as far as we know, for both single path and multi-path relay, the relay UE is a L2 U2N relay. The discovery message and procedure during indirect path addition re-use legacy discovery procedure, no enhancement is made. As a result, R17 relay UE can be discovered during indirect path addition procedure.

For the second issue, as we agreed in last meeting, legacy R17 behaviour is re-used for triggering relay UE enter RRC Connected state, i.e. SRB1 is configured on indirect path via split bearer or indirect bearer, and transmitting the RRCReconfigurationComplete message via SRB1 and SL-RLC1.

For the third issue, after relay UE enter into RRC connected state, the relay UE will forward the remote UE’s RRCReconfigurationComplete received from SL-RLC1 to network. After receiving RRCReconfigurationComplete  message, gNB identifies the association between the remote UE and relay UE. After that, gNB configure bearer mapping to relay UE as legacy. This procedure is similar with legacy direct to indirect path switch.

In conclusion, with legacy R17 trigger behaviour, a R17 relay UE can be supported for MP relaying.

- With legacy R17 trigger behaviour, a R17 relay UE can support MP relaying.

If an IDLE/INACTIVE relay UE is discovered during indirect path addition, both remote UE and gNB does not know whether the relay UE is Rel-17 or Rel-18 remote UE. In this case, gNB can not configure remote UE to use PC5 RRC trigger mechanism arbitrarily, i.e. only direct SRB1 is configured.

Therefore to support PC5 RRC triggers, we may need further study how to obtain the release information of relay UE or how to handle the case that R17 relay UE can not identify the PC5 RRC triggers signaling.  Corresponding study is time consuming and we do not see too much benefit to do such optimization. Therefore, considering many higher priority issues related to MP are pending on RAN2, it is suggested to revert previous RAN2 agreement to not support PC5 RRC triggers.

- To support PC5 RRC triggers, we may need further study on how to obtain the release information or how to handle the case that R17 relay UE can not identify the PC5 RRC triggers signaling.  Corresponding study is time consuming and we do not see too much benefit to do such optimization.

- It is suggested to revert previous RAN2 agreement and not support PC5 RRC triggers for RRC_IDLE/INACTIVE relay UE.

PDCP duplication

In Uu interface, when configuring Uu DRB, gNB can set the state of PDCP duplication in Uu PDCP configuration and dynamically activate/de-activate PDCP duplication by means of Uu MAC CE.

For multi-path relay, according to previous RAN2 agreement, one PDCP entity at the remote UE is configured with one direct Uu RLC channel and one indirect PC5 RLC channel. And this PDCP entity delivers PDCP PDUs to a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side. Since the PDCP entity use legacy Uu PDCP configuration, then legacy PDCP duplication activation/deactivation in PDCP configuration can be re-used for multi-path remote UE. Considering that the direct path is available for multi-path remote UE, the PDCP duplication activation/deactivation MAC CE for remote UE’s RB can be delivered via direct path as well.

- When configuring MP split bearer with duplication for remote UE, gNB can set the state of PDCP duplication (either activated or deactivated) at the time of (re-)configuration.

- The PDCP duplication (de)activation for remote UE’s multi-path split RB can be supported via the MAC CE transmitted on the direct path.

Packet split

In DC scenario, when more than one RLC entities are associated to one PDCP entity, UE can split the packet to any RLC entities to improve the data rate. However, UE may not always have high data rate requirement. In this case, a split threshold is introduced in DC to enable/disable the packet split function. UE only split the packet into different RLC entities if the data volume is above the threshold. Similar split threshold can be re-used for the multi-path remote UE.

- Uu split threshold can be re-used to trigger remote UE’s UL packet split.

It is worth noting that UE can submit the packet to any RLC entities by implementations in Uu interface. The reason why leave the decision of split packet submissions to UE implementation is that UE connect with network via two direct path in DC, UE can estimate the channel condition and data rate by itself directly.

When it comes to SL MP, for direct path, UE connects with gNB directly and can also perform channel estimation of direct path by itself. However, for indirect path, UE connects with gNB via a relay UE. The remote UE does not know the channel condition and data rate between relay UE and gNB.

- Remote UE does not know the channel condition between relay UE and gNB.

In this case, remote UE can only perform split packet submission based on the channel condition of direct path and at most PC5 link between remote UE and relay. We think this is not accurate due to the absent of channel condition between relay UE and gNB. To help remote UE performing packet split, it is suggested to keep remote UE informed of some transmission assistance information from relay UE.

- Relay UE provides assistance information of indirect path to remote UE for remote UE’s data split operation.

RLF handling

In last RAN2 meeting, RAN2 agreed that upon detection of 3GPP-defined RLF failure in one path, remote UE configured with MP can report path failure via the alternative available path if SRB1 is configured on the alternative path or split SRB1 is configured. It is FFS that which message should be used to carry the failure information.

From our view, for direct path failure, MCGFailureInformation can be re-used. Since firstly all IEs in  this RRC message are optional. The new added SL path failure information does not influence original function.  Secondly, T316 is started upon transmission of the MCGFailureInformation message. If reusing MCG failure information, we do not need to design new timer to control the reporting behaviour and perform recovery in case no response is received.

- Re-use MCGFailureInformation to report the Uu-RLF.

For indirect path failure, MCGFailureInformation can also be re-used. Considering that remote UE may only have one cell group irrespective of two paths are served by same DU or different DUs and indirect path can be also regarded as belong to MCG, it is straightforward to re-use MCGFailureInformation to report the PC5-RLF and corresponding T316 can be re-used.

- Re-use MCGFailureInformation to report the PC5-RLF.

UE aggregation requirement/capability

For scenario2, it is FFS which UE ID is used as relay UE’s ID for indirect path addition. Before we discuss the relay UE ID, we should first discuss the supported relay UE’s RRC state when remote UE report the relay UE ID.

It is straightforward that RRC connected relay UE can be supported. However, it is still not clear whether RRC IDLE and INACTIVE relay UE can be supported when remote UE report the relay UE ID to gNB. Some companies prefer to use C-RNTI as relay UE ID, which implicitly exclude the RRC IDLE and INACTIVE relay UE since IDLE and INACTIVE relay UE does not have C-RNTI.

From our view, the report of relay UE’s ID only shows the preference and capability of MP for UE aggregation. Whether remote UE can be configured with MP is decided by gNB after receiving the report. If we mandate the report of C-RNTI of relay UE by remote UE, relay UE must enter into RRC Connected before remote UE report relay UE ID to gNB. In this case, if gNB does not configure UE aggregation to remote UE immediately, relay UE has to be in RRC Connected state persistently. Taking this into account, it is suggested to also support RRC IDLE and INACTIVE Relay UE, so that when gNB may configure the multi-path transmssion for remote UE only when necessary after receiving the relay ID report.

- Remote UE may report relay UE ID of RRC IDLE and INACTIVE Relay UE to gNB for scenario 2.

For the detailed relay UE ID, following options can be considered:

- option1. C-RNTI

- option2. S-TMSI

- option3. SL-SourceIdentity-r17

- option4. A index assigned by the remote UE

- option5. new ID is allocated by the gNB

For option1, as we discussed above, option1 requires relay UE entered into RRC-connected, which we think is inefficient. Option5 re-use sidelink ID which should be avoided for UE aggregation. Option4 and option5 requires remote UE and gNB to further obtain the relationship between new assigned ID/index and relay UE. For example, for option4, after relay UE enter into RRC connected, relay UE needs to further report the index assigned by remote UE to gNB so that gNB can identify which relay UE is connected with remote UE. Option2 does not restrict the relay UE’s RRC state. And gNB can identify the relay UE via S-TMSI during relay UE’s RRC connection procedure, no extra signaling is needed.

- In addition to C-RNTI, S-TMSI can be used as relay UE ID for RRC_IDLE/INACTIVE relay UE.

During previous RAN2 meeting, for scenario1, we have following agreement:

For a MP split bearer in scenario 1, one PDCP entity at the remote UE is configured with one direct Uu RLC channel and one indirect PC5 RLC channel. - For upstream, a PDCP entity delivers to a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side. - For downstream, a PDCP entity receives from a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side.

For scenario2, whether and how to capture the protocol entities mapping in scenario2 is FFS:

FFS if we need to take decisions on the mapping of protocol entities in scenario 2.

In our opinion, the issue is how to capture the protocol entities mapping for indirect path which is a non-3GPP link. RAN2 has agreed that no adaptation layer is placed over non-3GPP link. So we suggest to describe the mapping relation implicitly and leave the details to UE implementation.

- Actually one possible implementation is that remote UE establish internal transmission tunnels with relay UE for each remote UE’s indirect or multi-path split bearer . After PDCP PDU is generated, remote UE deliver the PDCP PDU to the corresponding tunnel. Upon receiving the PDCP PDU from a given tunnel, Relay UE may deliver the received PDCP PDU to the corresponding Uu RLC channel since 1 to 1 mapping is assumed for remote UE’s RB and relay UE’s Uu RLC channel.

- For scenario2, for upstream, the PDCP entity in remote UE side delivers PDCP PDUs to a Uu RLC entity in relay UE side via non-3GPP link based on UE implementation.

Additionally，it is also FFS on how to configure the 1:1 bearer mapping over Uu link for the indirect path.

Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. Only 1:1 bearer mapping is supported over Uu link for the indirect path. FFS how to configure the mapping.

In Uu interface, the mapping between the RB and RLC bearer supports both 1:1 and 1:N mapping, i.e. one RB can be associated with one or more RLC bearer. The mapping relation is configured by servedRadioBearer included in the RLC bearer configuration as shown in following:

RLC-BearerConfig ::= SEQUENCE { logicalChannelIdentity LogicalChannelIdentity, servedRadioBearer CHOICE { srb-Identity SRB-Identity, drb-Identity DRB-Identity } OPTIONAL, -- Cond LCH-SetupOnly reestablishRLC ENUMERATED {true} OPTIONAL, -- Need N rlc-Config RLC-Config OPTIONAL, -- Cond LCH-Setup mac-LogicalChannelConfig LogicalChannelConfig OPTIONAL, -- Cond LCH-Setup ..., [[ rlc-Config-v1610 RLC-Config-v1610 OPTIONAL -- Need R ]], [[ rlc-Config-v1700 RLC-Config-v1700 OPTIONAL, -- Need R logicalChannelIdentityExt-r17 LogicalChannelIdentityExt-r17 OPTIONAL, -- Cond LCH-SetupModMRB multicastRLC-BearerConfig-r17 MulticastRLC-BearerConfig-r17 OPTIONAL, -- Cond LCH-SetupOnlyMRB servedRadioBearerSRB4-r17 SRB-Identity-v1700 OPTIONAL -- Need N ]] }

Therefore we think 1:1 mapping for scenario2 can be achieved by re-using the servedRadioBearer.

Additionally, it has been agreed that one relay UE only serve one remote UE in scenario2. For relay UE to differentiate the received RLC bearer belongs to remote UE or it’s own, following two options can be considered:

1. Uu RLC bearer configuration(RLC-BearerConfig) is used to configure Uu channel for remote UE and relay UE’s own RLC bearer. Additionally Uu RLC bearer configuration include a remote UE indication.

2. Uu RLC channel configuration(Uu-RelayRLC-ChannelConfig) is used to configure Uu channel for remote UE. Uu RLC bearer configuration(RLC-BearerConfig) is used to set relay UE’s own RLC bearer. Additionally,  to configure 1:1 bearer mapping, Uu RLC channel configuration(Uu-RelayRLC-ChannelConfig) shall include a servedRadioBearer.

From our perspective, option2 re-use signaling designed for sidelink U2N relay, which we think may confuse the UE aggregation and SL U2N relay. So we suggest to adopt option1.

For example, for indirect path Uu RLC channel, after receiving a RLC configuration including remote UE indication and served DRB ID 1 from gNB, relay UE knows this received RLC configuration is used for forwarding the packet of remote UE’s DRB1.

- It is suggested to re-use the servedRadioBearer included in Uu RLC bearer configuration to configure the 1:1 bearer mapping for scenario2.

- For relay UE to differentiate the received RLC bearer belongs to remote UE or it’s own, Uu RLC bearer configuration(RLC-BearerConfig) shall include a remote UE indication.

Multi-path configuration

- The channel condition of UEs involved in multi-path relaying may change. The traffic load and QoS requirement of remote UE may also change. Therefore, the multi-path delivery may be reconfigured. In this section, we will discuss the signalling procedure for the indirect path addition and direct path addition.

Indirect path addition

- For the remote UE, it may connect to the network via direct path initially and then add the indirect path. Figure 1 illustrates the indirect path addition procedure for L2 U2N remote UE on top of direct path. Here the relay UE is assumed in RRC_Connected state. The procedure are as follows:

- Figure 1 The addition of indirect path on top of direct path

1.	The remote UE reports one or multiple candidate relay UE(s), after it measures/discovers the candidate relay UE(s):

2.	The gNB decides to add the indirect path via relay UE to remote UE.

3. Then the gNB sends an RRCReconfiguration message to the relay UE, which can include at least remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

4.	The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message can include at least relay UE ID, PC5 Relay RLC channel configuration for relay traffic and the associated indirect/split bearer configuration. Moreover, the remote UE may be notified of indirect path addition.  For the indirect bearer, it may reuse the SL-SRAP-Config for bearer mapping between remote UE’s RB and PC5 RLC channel.

5.	The remote UE establishes PC5 connection with relay UE.

6.	The remote UE completes the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB.

7.	The data traffic from the indirect bearer or split bearer may be delivered via indirect path between the remote UE and the gNB.

- It is suggested to capture the indirect path addition signalling procedure.

Direct path addition

- For the remote UE, it may connect to the network via indirect path initially and then add the direct path. Figure 2 illustrates the direct path addition procedure for remote UE on top of indirect path. The signalling procedure is as follows:

- Figure 2 The addition of direct path on top of indirect path

1.	The Uu measurement configuration and measurement report signalling procedures are performed to evaluate both relay link measurement and Uu link measurement. The measurement results from remote UE are reported when configured measurement reporting criteria are met.

2.	The gNB decides to add the direct path to remote UE.

3.	The gNB sends RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message can include the associated direct/split bearer configuration. In addition, the remote UE may be notified of the addition of direct path.

4.	The remote UE synchronizes with the gNB and performs Random Access.

5.	The remote UE sends the RRCReconfigurationComplete to the gNB via either direct path or indirect, using the configuration provided in the RRCReconfiguration message.

6.	The gNB sends RRCReconfiguration message to the relay UE to reconfigure the connection between the relay UE and the gNB. The RRCReconfiguration message to the relay UE can be sent any time after step 3 based on gNB implementation (e.g., to modify or release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration between PC5 RLC and Uu RLC).

7.	The data packet from direct bearer or split bearer is transmitted via the direct path between the remote UE and the gNB.

- It is suggested to capture the direct path addition signalling procedure.

Conclusion

In this contribution, following proposals and observations are given:

- For scenario 1, the bearer type (i.e. direct bearer, indirect bearer, or multi-path split bearer) of SRB1 and SRB2 may be configured by the gNB independently. It is not necessary to mandate the same bearer type configuration of SRB1 and SRB2.

- RAN2 not further discuss the following two cases:1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE.

- With legacy R17 trigger behaviour, a R17 relay UE can support MP relaying.

- To support PC5 RRC triggers, we may need further study on how to obtain the release information or how to handle the case that R17 relay UE can not identify the PC5 RRC triggers signaling.  Corresponding study is time consuming and we do not see too much benefit to do such optimization.

- It is suggested to revert previous RAN2 agreement and not support PC5 RRC triggers for RRC_IDLE/INACTIVE relay UE.

- When configuring MP split bearer with duplication for remote UE, gNB can set the state of PDCP duplication (either activated or deactivated) at the time of (re-)configuration.

- The PDCP duplication (de)activation for remote UE’s multi-path split RB can be supported via the MAC CE transmitted on the direct path.

- Uu split threshold can be re-used to trigger remote UE’s UL packet split.

- Remote UE does not know the channel condition between relay UE and gNB.

- Relay UE provides assistance information of indirect path to remote UE for remote UE’s data split operation.

- Re-use MCGFailureInformation to report the Uu-RLF.

- Re-use MCGFailureInformation to report the PC5-RLF.

- Remote UE may report relay UE ID of RRC IDLE and INACTIVE Relay UE to gNB for scenario 2.

- In addition to C-RNTI, S-TMSI can be used as relay UE ID for RRC_IDLE/INACTIVE relay UE.

- For scenario2, for upstream, the PDCP entity in remote UE side delivers PDCP PDUs to a Uu RLC entity in relay UE side via non-3GPP link based on UE implementation.

- It is suggested to re-use the servedRadioBearer included in Uu RLC bearer configuration to configure the 1:1 bearer mapping for scenario2.

- For relay UE to differentiate the received RLC bearer belongs to remote UE or it’s own, Uu RLC bearer configuration(RLC-BearerConfig) shall include a remote UE indication.

- It is suggested to capture the indirect path addition signalling procedure.

- It is suggested to capture the direct path addition signalling procedure.