3GPP TSG-RAN WG2 Meeting #122											        R2-2305049

Seoul, 22-26 May, 2023

Agenda item:			7.7.4.2

Title:						Discussion on NTN handover enhancements

Source:					Continental Automotive

Document for:		Discussion, Decision

# Introduction

The work item aims at specifying enhancements for NG-RAN based NTN (Non-Terrestrial Networks) in Rel-18 [1] and has identified NTN-TN and NTN-NTN measurement/mobility and service continuity as important areas for improvements. In particular, the following objective is stated:

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4]

  - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

  - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

  - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4]

  - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between cells. [RAN3]

During RAN2#121bis-e [2] the following agreements related to RACH-less HO were made online:

Agreements:

Agreements online:

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

- Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH.

In this paper, we share further thoughts on RACH-less handover procedure and aspects related to allocation of UL grants.

# 2. Discussion

As stated in [2], many UEs may be served within a single NTN cell. Particularly in case of LEO deployments with earth moving cells, mobility and service continuity in connected mode is expected to be significantly affected by rapid satellite movements. For example, a UE may need to perform handover after 6.61 seconds [2], if served by a LEO satellite with a cell diameter of 50 km. Hence, the total number of handovers that need to be performed at a given time can very likely result in significant RACH congestion as well as signalling load in the network.

Recently, RACH-less handover was proposed to address those issues. Under the assumptions that source and target cell exchange information and that the UE knows the target cell’s Timing Advance (TA) in advance, the UE can access the target cell via pre-allocated and/or dynamic UL grants.

However, the performance of RACH-less handover may not be sufficient in all scenarios, particularly when UEs are moving at higher speeds. For example, it may happen that the target cell does not detect the initial UL transmission due to poor channel conditions or uplink synchronisation issue, where a high level of UE mobility can aggravate this issue.

Observation 1: Performance of RACH-less handover may not be sufficient in all scenarios, particularly when UEs are moving at higher speeds. The target cell may not detect the initial UL transmission due to poor channel conditions or uplink synchronisation issue, where a high level of UE mobility can aggravate this issue.

As the target cell is not aware of the UE-specific mobility status and situation when deciding to handover the UE with or without RACH procedure, this may result in suboptimal decisions, increase risk of radio link failure as well as negatively impact service continuity.

Observation 2: The target cell is not aware of the UE-specific mobility status, which may result in suboptimal decisions, increase risk of radio link failure as well as negatively impact service continuity.

In contrast, the network can enhance mobility support by considering UE-specific indications regarding its mobility. For example, stationary UEs and UEs moving at lower speeds could be preferably considered by the target cell for RACH-less procedure for reducing the risk of radio link failures, where the source cell can forward the UE-specific mobility state to the target cell.

Further, in case the UE is supposed to access the target cell via a dynamic grant in a RACH-less manner, the target cell may prefer to configure dynamic grants differently, e.g., considering UE-specific mobility.

Proposal 1: If UE mobility state can be determined by source cell, the source cell can forward this information to target cell. This information can be beneficial for mobility support.

Proposal 2: Depending on UE-specific mobility status, the target cell can decide to handover a UE with or without RACH procedure and determine which UL grant to allocate.

# Conclusions

The following observations and proposals have been made in this document:

Observation 1: Performance of RACH-less handover may not be sufficient in all scenarios, particularly when UEs are moving at higher speeds. The target cell may not detect the initial UL transmission due to poor channel conditions or uplink synchronisation issue, where a high level of UE mobility can aggravate this issue.

Observation 2: The target cell is not aware of the UE-specific mobility status, which may result in suboptimal decisions, increase risk of radio link failure as well as negatively impact service continuity.

Proposal 1: If UE mobility state can be determined by source cell, the source cell can forward this information to target cell. This information can be beneficial for mobility support.

Proposal 2: Depending on UE-specific mobility status, the target cell can decide to handover a UE with or without RACH procedure and determine which UL grant to allocate.

# References

- RP-230809 (revision of RP-223534) Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales, RAN#99, March 20-23, 2023.

- 3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), Release 16.