3GPP TSG-RAN WG2 Meeting #122	R2-2305051

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	NCR access link beam update capability

WID/SID:	NR_netcon_repeater - Release 18

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#121bis, RAN2 was able to converge on most of the remaining issues related to AI 7.1.2 (Signalling for side control information). One remaining issue was the following proposal, discussed in R2-2304484.

Proposal 1: On the issue of whether optional UE capability should be introduced to indicate if NCR-MT supports beam index update in Access Link Beam Indication MAC CE, RAN2 shall await RAN1’s decision on whether there is a need for such a functionality.

We further discuss this proposal in light of discussions from RAN1#112bis.

# 2	Discussion

## 2.1	Access link beam index update capability

In RAN1#112bis, RAN1 endorsed the initial feature list for NCR, which is copied into the Annex of this document.

Index 43-4a corresponds to beam index updates for semi-persistent beam indication. However, based on this initial list of features, RAN1 has still not determined whether a separate optional capability is necessary for beam index updates or if this capability will be merged with the capability for semi-persistent beam indication for access link (index 43-4), as captured in the following note for index 43-4a:

FFS: whether to merge this FG with 43-4

Observation 1: RAN1 is still determining whether a separate UE capability will be used to indicate support for access link beam index updates or if the capability will be merged with the semi-persistent beam indication capability.

Given this, RAN2 should wait for RAN1’s LS before making further decisions on this topic; so we may re-iterate the same proposal from R2-2304484:

Proposal 1: On the issue of whether optional UE capability should be introduced to indicate if NCR-MT supports beam index update in Access Link Beam Indication MAC CE, RAN2 shall await RAN1’s decision on whether there is a need for such a functionality.

# 3	Conclusion

This document has made the following observations:

Observation 1: RAN1 is still determining whether a separate UE capability will be used to indicate support for access link beam index updates or if the capability will be merged with the semi-persistent beam indication capability.

And proposed the following:

Proposal 1: On the issue of whether optional UE capability should be introduced to indicate if NCR-MT supports beam index update in Access Link Beam Indication MAC CE, RAN2 shall await RAN1’s decision on whether there is a need for such a functionality.

# 4	Annex: RAN1-endorsed feature list for NCR

The following feature list for NCR, copied from R1-2304282, was endorsed during RAN1#112bis.

NOTE: 	Text highlighted in yellow is still FFS.

43.	NR_netcon_repeater

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 5 candidate values: FFS] | Optional [with/without] capability signaling |
| 43. NR_netcon_repeater | 43-2 | Periodic beam indication for access link | 1.Support periodic beam indication for access link 2. Priority flag for periodic indication | 43-1 | N/A | Yes | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4a | Beam index updates for semi-persistent beam indication | 1.Support of [temporary] override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1, 43-4 | N/A | Yes | The semi-persistent beam indication cannot provide updates for the beam index(es). | Per NCR-MT | No | No | Yes | FFS: whether to merge this FG with 43-4 | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |