3GPP TSG-RAN WG2 Meeting #122	R2-2305052

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	NCR remaining RRM issues

WID/SID:	NR_netcon_repeater - Release 18

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#121bis, RAN2 made good progress on many open issues for NCR. Now, going into the final RAN2 meeting for this WI, there remain only a few issues to resolve, based on the following proposals from R2-2304420.

Proposal 1: In Rel-18, do not define “wake-up timer” IE in RRCRelease message, if needed, it can be done via OAM (no specification impact).

Proposal 4: If needed, beam monitoring for backhaul link when NCR-MT in RRC_INACTIVE state can be done by implementation.

Proposal 8: The NCR-MT in RRC_INACTIVE does not discard the stored NCR-Fwd configuration autonomously, it is up to the network to reconfigure the NCR-MT upon RRC resume procedure.

Although these proposals were already discussed at RAN2#121bis, no agreements were reached. In this contribution we discuss the proposals further.

# 2	Discussion

## 2.1	Wake-up timer

Regarding the wake-up timer, we note that in previous meetings when the timer was first being discussed most proponents argued that it would enable the network to release the NCR-MT to RRC_IDLE state—presumably for power saving—while allowing the NCR-Fwd to continue operating using the last-received configuration. This was intended as a way for an NCR-MT in RRC_IDLE to approximate the behaviour of NCR-MT in RRC_INACTIVE while compensating for the fact that the RAN cannot page the NCR-MT in RRC_IDLE state (which was seen as necessary since RRC_INACTIVE is optional for NCR and not all networks may support RRC_INACTIVE state).

However, during RAN2#121bis, it was agreed:

- NCR-Fwd is OFF when NCR-MT is in RRC_IDLE state.

Based on this agreement, NCR-Fwd behaviour when NCR-MT is in RRC_IDLE cannot mimic the behaviour when NCR-MT is in RRC_INACTIVE.

Observation 1: Since NCR-Fwd is OFF when NCR-MT is in RRC_IDLE state, a wake-up timer is not needed to mimic functionality of the NCR-MT in RRC_INACTIVE.

In addition to the above justification, companies also indicated the following motivations for the timer:

-	Temporarily releasing NCR-MT to RRC_IDLE during network maintenance.

-	Bringing the NCR-MT back from RRC_IDLE if it is accidently released due to network failure.

We observe that network maintenance is an action that is planned by an operator, and since the operator would already manage network maintenance, they could manually deactivate the NCR by OAM at the same time if required (in fact it seems to make more sense to manually control the NCR instead of using a wake-up timer in this case, since network maintenance timelines are not always predictable). Even if the NCR is not deactivated, during network maintenance the NCR’s serving cell would most likely be barred/deactivated, so the NCR-MT would likely reselect to a different cell/gNB and acquire new side control configuration anyway, regardless of a timer.

For the case where the NCR-MT is released due to a network failure or abnormal network event, it makes more sense for the NCR-MT to reconnect more-or-less immediately (similar to RLF followed by RRC re-establishment). For this type of situation, it is not clear why the network would release the NCR with a relatively long wait time instead.

Based on the above, we think the only relevant reason to release an NCR-MT to RRC_IDLE is to intentionally cease extending coverage with the NCR. Although this might be done for power saving reasons, e.g. when there are no UEs or a small number of UEs to be covered by the NCR, this could still be handled by NCR OAM through a manually scheduled OFF period and does not justify specifying a wake-up timer in our view.

Even if an OAM/implementation-based solution lacks some of the flexibility of a specified wake-up timer, we note that when NCR-MT is in RRC_CONNECTED the gNB can already release the side control configuration at the NCR-MT to turn OFF the NCR-Fwd (e.g. when there are no UEs to be covered), which saves some power even if it does not save as much power as releasing the NCR-MT to IDLE state.

Observation 2: For the other scenarios discussed, the benefit of a wake-up timer is not very clear and/or is more so an optimization than a necessity. Operator-controlled/implementation/OAM techniques for waking the NCR-MT from RRC_IDLE are already sufficient in these scenarios.

Finally, we make one last comment on OAM. Some companies in RAN2 have argued that OAM is optional for NCR because DRB is optional for NCR, and therefore OAM cannot be considered as a solution for bringing the NCR out of RRC_IDLE state. However, RAN3 already considers OAM to be mandatory at the NCR, not only for configuration management, but also for performance management, fault management, software management (e.g. upgrade SW), etc.  RAN3 only agreed that the OAM connection is not required to use a PDU session (although it may use PDU), since other proprietary solutions (e.g. fibre, Wi-Fi) could also provide OAM connections. Hence, any NCR not supporting DRB would still need to have OAM connectivity from some other technology.

Observation 3: RAN3 already considers OAM mandatory for NCR; if DRB is not supported, OAM connectivity must come from some other technology.

Proposal 1: RAN2 does not specify a wake-up timer in the RRCRelease message. If needed, wake-up can be handled by implementation/OAM without specification impact.

## 2.2	Backhaul beam monitoring in RRC_INACTIVE

The discussion at RAN2#121bis related to beam monitoring while NCR-MT is in RRC_INACTIVE was based on proposals in R2-2302893.

Proposal 1: The NCR-FWD switches OFF if the NCR-MT in RRC_INACTIVE mode detects beam failure.

Proposal 2: The NCR-MT in RRC_INACTIVE resumes connection to receive updated side control configuration if it reselects a new beam of the same camped cell

The issue at hand is whether RAN2 should specify any beam monitoring capability when the NCR-MT is in RRC_INACTIVE, since the C-link and backhaul beam configurations could not be updated dynamically if the link quality degrades. The other option is to leave such beam monitoring functionality to implementation (either at the gNB or NCR), if it is supported at all.

Currently beam monitoring while in RRC_INACTIVE is not supported by the specifications. Therefore, if RAN2 agrees to the above proposals, it would also require the involvement of other RAN working groups, e.g. RAN1 to specify beam monitoring functionality in RRC_INACTIVE and RAN4 to specify RF/RRM/demod requirements. In our view, the effort that would be required by RAN1 and RAN4 would not be trivial and should also be considered. This is especially true for RAN1 since they have no TUs left for the Rel-18 NCR WI.

Observation 4: Specifying beam monitoring for RRC_INACTIVE is not trivial and would also need to be worked on in RAN1 and RAN4. However, RAN1 has no TUs left for the Rel-18 NCR WI.

Companies that wished to have the NCR-MT perform beam monitoring in RRC_INACTIVE argued that if the behaviour is not specified it would not be easy for the NCR-MT to recover and continue providing coverage to UEs if the backhaul beam fails. However, beam recovery while in RRC_INACTIVE is not necessarily precluded from implementation. For example, if the gNB detects a sudden degradation in UE measurements this could imply a degradation in either the NCR-Fwd access link or in the NCR-Fwd backhaul link. Regardless of where the degradation occurred, the gNB would likely resume the RRC connection with the NCR-MT in order to provide it with a new side control configuration. Bringing the NCR-MT back into RRC_CONNECTED state would also enable the NCR-MT to select a new beam at the same time.

One other issue mentioned during discussions was if the NCR is not serving any connected UEs; then the gNB would not be able to infer any degradation in the backhaul. However, this seems like an edge case. If the NCR-Fwd is already ON, the gNB could detect an issue as soon as UEs try connecting through the NCR (as explained in the previous paragraph); on the other hand, if the NCR-Fwd is OFF (because there are no UEs), then the gNB would already be expected to wake the NCR and provide it with a new configuration once UEs are detected. Furthermore, if there are no UEs, there would also not be much harm in allowing the beam quality of the NCR-MT to degrade to the point where the NCR-MT just reselects a new cell: then existing mechanisms that have already been agreed by RAN2 would enable the NCR-Fwd to resume its connection on the new cell to acquire new side control configuration.

Observation 5: Specifying beam monitoring/recovery functionality for NCR-MT in RRC_INACTIVE is an optimization. The network is not precluded from inferring backhaul issues from UE measurements.

Proposal 2: RAN2 does not specify any beam monitoring/recovery functionality for NCR-MT in RRC_INACTIVE state. If needed, beam monitoring can be done by implementation without specification impact.

## 2.3	Discarding of stored NCR-Fwd configuration

In one of the offline discussions at RAN2#121bis (R2-2304420) (as well as one of the Post121 discussions, R2-2303288) it was discussed whether the NCR-MT could be allowed to discard the (old) RRC-configured side control in when it initiates RRCResume in a cell different from the cell that it was released from, e.g. following cell reselection:

Regarding whether/when to discard the received beam indication configuration (i.e. NCR-FwdConfig-r18), to discuss the following options: - Option 1: The NCR-MT in RRC_INACTIVE discards the configuration when it initiates RRC resume procedure in a cell different from the released cell (this implies delta configuration is supported only in the released cell). - Option 2: The NCR-MT in RRC_INACTIVE does not discard the configuration autonomously (this implies delta configuration is supported in any cell).

On one hand, it is quite possible that following cell reselection the old beam configuration would no longer be applicable, so there would not be a reason to maintain the old configuration. However, one corner case which was brought up during offline discussions is if an NCR-MT in RRC_INACTIVE triggers cell reselection back to its old cell during the RRCResume procedure. In this situation, the NCR-MT might discard its configuration from the old cell at the start of the RRCResume procedure, but its context would not be transferred to the new cell (since RRCResume did not complete). Then the old cell might expect that the NCR-MT still has its configuration even though the NCR-MT has already discarded it, resulting in a configuration discrepancy. This issue can be avoided by Option 2.

Observation 6: Allowing the NCR-MT in RRC_INACTIVE to discard the NCR-Fwd configuration autonomously could result in configuration discrepancies between the network and NCR.

Based on this, we support Proposal 8 from R2-2304420:

Proposal 3: The NCR-MT in RRC_INACTIVE does not discard the stored NCR-Fwd configuration autonomously; it is up to the network to reconfigure the configuration upon RRC resume procedure.

# 3	Conclusion

This document has made the following observations:

Observation 1: Since NCR-Fwd is OFF when NCR-MT is in RRC_IDLE state, a wake-up timer is not needed to mimic functionality of the NCR-MT in RRC_INACTIVE.

Observation 2: For the other scenarios discussed, the benefit of a wake-up timer is not very clear and/or is more so an optimization than a necessity. Operator-controlled/implementation/OAM techniques for waking the NCR-MT from RRC_IDLE are already sufficient in these scenarios.

Observation 3: RAN3 already considers OAM mandatory for NCR; if DRB is not supported, OAM connectivity must come from some other technology.

Observation 4: Specifying beam monitoring for RRC_INACTIVE is not trivial and would also need to be worked on in RAN1 and RAN4. However, RAN1 has no TUs left for the Rel-18 NCR WI.

Observation 5: Specifying beam monitoring/recovery functionality for NCR-MT in RRC_INACTIVE is an optimization. The network is not precluded from inferring backhaul issues from UE measurements.

Observation 6: Allowing the NCR-MT in RRC_INACTIVE to discard the NCR-Fwd configuration autonomously could result in configuration discrepancies between the network and NCR.

And proposed the following:

Proposal 1: RAN2 does not specify a wake-up timer in the RRCRelease message. If needed, wake-up can be handled by implementation/OAM without specification impact.

Proposal 2: RAN2 does not specify any beam monitoring/recovery functionality for NCR-MT in RRC_INACTIVE state. If needed, beam monitoring can be done by implementation without specification impact.

Proposal 3: The NCR-MT in RRC_INACTIVE does not discard the stored NCR-Fwd configuration autonomously; it is up to the network to reconfigure the configuration upon RRC resume procedure.