3GPP TSG-RAN WG2 Meeting #122 	R2-2305056

# Incheon, South Korea, May 22-26, 2023

Agenda item:	7.8.2

Work Item:	NR_UAV-Core

Source: 	Qualcomm Incorporated

Title: 	Measurement and reporting enhancements

Document for:     Discussion/Decision

# Introduction

3GPP studied support of Aerial Vehicles during Rel-15 LTE and documented the extensive performance evaluation results in TR 36.777 [2]. Normative work was also finished in Rel-15 for LTE, where several enhancements were added for measurement and reporting taking UAV characteristics into account.

One of the objectives in the Rel-18 WI on UAV is intended to adapt the LTE solutions to NR while developing NR-specific enhancements as needed. In this contribution, we discuss some remaining issues on the measurement and reporting aspects.

# Height-dependent configuration

During RAN2#121, based on email discussion report [Post120][312][UAV] (see R2-2300479) and report of offline [AT121][305][UAV] (see R2-2302210), following was agreed:

Agreements:

- Support configuring height-dependent more-than-one configurations targeting measurement and measurement reporting enhancement. UE applies corresponding configuration based on the UE height. The proposed solutions should aim at avoiding RAN4 impacts.  FFS how this would be configured (i.e. different MO configurations or different parameters  FFS Exact parameters and details.

During RAN2#121bis-e, this was further discussed, and based on report of [POST121][313][UAV] (see R2-2302681) and report of offline [AT121bis-e][306][UAV] (see R2-2304491), following was agreed:

Agreements

- Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

In the following, we address the highlighted FFSes.

## MO config parameter: SSB-ToMeasure

One straightforward way to specify height-specific SSB-ToMeasure in the specification is shown below, e.g. using a list of heightRange items, each heightRange corresponding to a ssb-ToMeasure field. The height range can also include a hysteresis value to avoid frequent ping-pong between the height ranges.

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

<<skip>>

]],

[[

associatedMeasGapSSB2-v1720         MeasGapId-r17                                               OPTIONAL, -- Cond AssociatedGapSSB

associatedMeasGapCSIRS2-v1720       MeasGapId-r17                                               OPTIONAL  -- Cond AssociatedGapCSIRS

]],

[[

ssb-ToMeasureHeightBasedList-r18     SetupRelease { SSB-ToMeasureHeightBasedList-r18 }    OPTIONAL  -- Need R

]]

}

SSB-ToMeasureHeightBasedList-r18 ::= SEQUENCE (SIZE (1..maxNrofHeightRanges-r18)) OF SSB-ToMeasureHeightBased-r18

SSB-ToMeasureHeightBased-r18 ::=     SEQUENCE {

heightRange-r18     SEQUENCE {

heightMin-r18       INTEGER (0..maxHeight-r18) OPTIONAL,

heightMax-r18       INTEGER (0..maxHeight-r18) OPTIONAL,

heightHyst-r18      INTEGER (0..maxHyst-r18) OPTIONAL

},

ssb-ToMeasure-r18   SSB-ToMeasure

}

ssb-ToMeasureHeightBasedList List of height-dependent ssb-ToMeasure. When this list is included, ssb-ToMeasure-r18 is applicable for the UE height range indicated by heightRange.

Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR (TP shown above can be taken as baseline).

## MR config parameters: a4-Threshold and numberOfTriggeringCells

RAN2 discussed briefly whether configuring the height dependent measurement reporting parameters is possible using combination of another event with event H1/H2.

In our view it is possible to use combination of events to configure these height-dependent MR parameters. For this discussion, let’s taking LTE RRC as baseline for H1/H2 definitions, and transform that into RRC spec. An example ASN.1 is shown below.

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

<<skip>>

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useAllowedCellList                          BOOLEAN

},

<<skip>>

[[

eventH1-r18                       SEQUENCE {

h1-ThresholdOffset-r18            INTEGER (0..300),

h1-Hysteresis-r18                 INTEGER (1..16)

},

eventH2-r18                       SEQUENCE {

h2-ThresholdOffset-r18            INTEGER (0..300),

h2-Hysteresis-r18                 INTEGER (1..16)

}

]]

},

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

<<skip>>

[[

numberOfTriggeringCells-r18        INTEGER (2..maxCellReport)  OPTIONAL   -- Cond a3a4a5

]]

}

Based on the above ASN.1 snippet, configuration for different numberOfTriggeringCells for different heights seems straightforward. E.g. if intended configuration is a value for <50m and another value for >50m, the network could configure ReportConfigNR as follows:

1. Event H1 with h1 threshold = 50m with numberOfTriggeringCells value applicable for >50m.

2. Event H2 with h2 threshold = 50m with numberOfTriggeringCells value applicable for <50m.

To configure height dependent A4 threshold, e.g. one value for <50m and another value above 50m, the network could configure ReportConfigNR as follows (depending on how RAN2 concludes on combination of events)

1. Event H1 with h1 threshold = 50m + Event A4 with a4-threshold value applicable for >50m.

2. Event H2 with h2 threshold = 50m + Event A4 with a4-threshold value applicable for <50m.

Height dependent MR configuration parameters can be configured using combination of events.

Use combination of events for the configuration of height-dependent MR configuration parameters.

# Reporting of height, location, and velocity

RAN2#121 agreed the following:

- When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results). FFS whether UAV UE’s height is mandatorily reported and which parameter/IE is used for height reporting.

In the following, we go through the open items from this agreement.

## Standalone reporting of height, location, velocity

Note that while (indefinite) periodic reporting for height reporting configuration is not supported in LTE, once an event is triggered, multiple (periodic) reports based on that trigger (as configured by reportAmount >1 and corresponding reportInterval) are possible.

However, height measurement alone cannot be sent in LTE without including it with other radio measurements. Similarly, UAV’s location and velocity measurements cannot be sent alone in LTE without including it with other radio measurements.

According to LTE procedure for H1/H2 triggered measurement reporting, height, location and/or velocity measurement reports cannot be sent alone without being included along with other radio measurements.

In some cases, the network may benefit from the ability to configure height, location and/or velocity report alone (e.g. if a large UL grant could not be provided to fit the full RRM report).

For some use cases, reporting height, location and/or velocity without accompanying RRM report is desirable.

Therefore, reporting of height, location or velocity alone should be supported for NR UAVs. This is also in-line with the RAN2#121 agreement:

When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results)

UE height, location and/or velocity reporting without being accompanied by other RRM report is supported.

## FFSes on UE height reporting

### Optional/Mandatory UE height reporting

In LTE (Rel-15), the UE height reporting was based on a RRC field heightUE which the UE includes along with other measurement reports when eventH1 or eventH2 is configured and triggered.

From TS 36.331, section 5.5.5.1

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows: 1> set the measId to the measurement identity that triggered the measurement reporting; … 1> if the triggerType is set to event; and if eventId is set to eventH1 or eventH2: 2> set the heightUE to include the altitude of the UE; …

That is, it was mandatory to report because there was no separate network configuration for the content of the measurement report. However, unlike LTE, RAN2#121 agreed to make content of the report when H1/H2 triggers for NR to be configurable by the network.

In LTE UAV, there is no configuration on whether to report UE height upon H1/H2 trigger; however, for NR, RAN2 has agreed to make the content of the measurement report configurable.

Then we do not see a strong need to stick with the LTE principal only for the UE height.

Whether UE height is included when event H1 or H2 triggers is also configurable by the network.

### Parameter/IE used for UE height reporting

As shown in previous section, in LTE (Rel-15), the UE reports height based on a RRC field heightUE which the UE includes along with other measurement reports when eventH1 or eventH2 is configured and triggered.

Later, in Rel-17, ability for the UE to report uncompensated barometric pressure was also added for LTE as TEI17, which already existed for NR.

Uncompensated barometric pressure reporting mechanism exists in NR. LTE-like height reporting (using heightUE) does not exist for NR.

So, it is reasonable to have a unified approach and use the framework of barometric pressure measurement reporting for UE height, instead of adding yet another method to NR.

H1/H2 triggered UE height reporting using uncompensated barometric pressure measurement is supported. (Height reporting using an RRC field as in LTE is not introduced.)

# New measurement reporting trigger for interference control

As noted in the WID [1], one of the objectives is the following:

4. Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling [RAN1, RAN2]: - FR1 with directional antenna at UE side

So, potentially UAV UEs can have beamforming capabilities and the UE may be able to change its antenna configuration from, e.g. omni to directional, etc. This is being discussed in RAN1.

RAN2#119e already agreed to adopt the height triggers H1/H2 to NR. However, as illustrated in Figure below, Aerial UE A may fly from low to high and cross the height threshold, which will trigger Event H1 and Aerial UE B may fly from high to low and cross the height threshold, which will trigger Event H2 to report the location/height. On the other hand, for Aerial UE C which flies horizontally with no significant change in height, no Event H1/H2 is triggered.

However, if the aerial UE C changes the antenna configurations, e.g., switch from omni to directional antenna, it may result in significant interference variation. If gNB knows UE current location/speed/antenna configuration when the antenna configuration changes, gNB can apply further beam management to the UE.

For aerial UEs capable of different antenna configurations, the antenna configuration change may result in significant inter-cell interference variation.

Figure 1 For UE C, H1/H2 is not triggered even though interference changes

Therefore, for aerial UEs capable of different antenna configurations, measurement reporting can be enhanced to include the trigger/reporting of UE’s location and antenna/beam configuration when antenna/beam configuration changes. Details can be further discussed once RAN2 agrees on the principle.

Introduce change in antenna/beam configuration as an optional trigger for reporting of UAV location, speed and/or antenna/beam configuration. Details FFS.

# No-Transmit zone (NTZ)

RAN2#121bis-e received and discussed the following LS from ETSI TC MSG/TFES:

R2-2302464	LS to 3GPP on ECC request for standardisation support related to ECC Decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands”

The referenced ECC decision (22)07 on harmonised framework on aerial UE usage in MFCN harmonised bands defines no-transmit zones as follows and further mentions the need of such no-transmit zones.

- that a no-transmit zone in this Decision is defined as a geographical area where aerial UE are not allowed to transmit for spectrum compatibility purposes in a given harmonised MFCN band or part of it; - that national studies are needed, as appropriate, to define no-transmit zones for spectrum compatibility purposes, for aerial UE operating in the relevant frequency bands; - that a mechanism is necessary to ensure that aerial UE respect no-transmit zones; <<skip>> - that no-transmit zones as described in this Decision should be defined and implemented at national level and where necessary coordinated with neighbouring countries;

RAN2 needs to study the impact of this decision and whether to introduce any mechanisms for signalling of such NTZ, whether measurement reports are triggered due to moving in/out of such zones, etc. Since there is no specific objective about this in the current WID, we think RAN2 should decide whether to study this in current release or consider this in future release.

ECC decision (22)07 describes the need of no-transmit zones for aerial UEs, however current WI doesn’t have a directly related objective.

Discuss whether impact of ECC decision (22)07 regarding no-transmit zones should be studied in Rel-18 or can be left for Rel-19.

# Summary

In this contribution, we made the following observations:

Observation 1.	Height dependent MR configuration parameters can be configured using combination of events.

Observation 2.	According to LTE procedure for H1/H2 triggered measurement reporting, height, location and/or velocity measurement reports cannot be sent alone without being included along with other radio measurements.

Observation 3.	For some use cases, reporting height, location and/or velocity without accompanying RRM report is desirable.

Observation 4.	In LTE UAV, there is no configuration on whether to report UE height upon H1/H2 trigger; however, for NR, RAN2 has agreed to make the content of the measurement report configurable.

Observation 5.	Uncompensated barometric pressure reporting mechanism exists in NR. LTE-like height reporting (using heightUE) does not exist for NR.

Observation 6.	For aerial UEs capable of different antenna configurations, the antenna configuration change may result in significant inter-cell interference variation.

Observation 7.	ECC decision (22)07 describes the need of no-transmit zones for aerial UEs, however current WI doesn’t have a directly related objective.

Based on the discussion and observations, we propose:

Proposal 1.	Add height-based list of SSB-ToMeasure with corresponding height ranges and hysteresis in MeasObjectNR (TP shown above can be taken as baseline).

Proposal 2.	Use combination of events for the configuration of height-dependent MR configuration parameters.

Proposal 3.	UE height, location and/or velocity reporting without being accompanied by other RRM report is supported.

Proposal 4.	Whether UE height is included when event H1 or H2 triggers is also configurable by the network.

Proposal 5.	H1/H2 triggered UE height reporting using uncompensated barometric pressure measurement is supported. (Height reporting using an RRC field as in LTE is not introduced.)

Proposal 6.	Introduce change in antenna/beam configuration as an optional trigger for reporting of UAV location, speed and/or antenna/beam configuration. Details FFS.

Proposal 7.	Discuss whether impact of ECC decision (22)07 regarding no-transmit zones should be studied in Rel-18 or can be left for Rel-19.

# References

[1] RP-230782, Revised WID: NR support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023

[2] TR 36.777, Study on Enhanced LTE Support for Aerial Vehicles (Release 15), Technical Report