3GPP TSG-RAN WG2 Meeting #122	R2-2305071

Incheon, South Korea, 22 May – 26 May 2023

Agenda item:	7.5.2

Source:	Apple

Title:	Views on XR-Awareness

WID/SID:	NR_XR_enh-Core

Document for:	Discussion / Decision

# Introduction

In continuation of the 3GPP work on XR in RAN1 and SA4 in Rel-17 followed by an XR study in Rel-18, RAN has approved a RAN2-led work item on XR enhancements for NR in Rel-18 [1]. For XR-Awareness, the Rel-18 XR WI now has an objective to specify RAN2 enhancements for provisioning of XR traffic assistance information by the UE. Further, RAN2 will consider the impact of identifying PDU Sets, Data bursts and PSI at the UE side as needed.

Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

Accordingly, RAN2#122 is expected to discuss how UL jitter information is reported from UE to network as well as the use of PDU Set Information in RAN. This contribution aims to discuss some of our views with respect to the normative work on XR-awareness. We also discuss what parameters could be beneficial as assistance information and how a high-level procedure might look like. Finally, we provide our view on End of Data Burst indications.

# Discussion

## Potential Set of Assistance Information Parameters

RAN2 discussed UL assistance information for XR in the RAN2#121bis-e meeting and made following agreement.

3. UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting.

This section provides a sketch of a basic parameter set, including some parameters that are based on TSCAI. The parameter set is generic and can vary for different enhancement options as well as protocol levels. We think the UE could provide at least following information as part of assistance information to the gNB:

- Periodicity

- Traffic Parameters

- Direction (UL or DL)

Although the RAN may obtain this information from the CN, it may take a long time for some of these information (especially the traffic information for UL) to reach the RAN in a timely manner. Therefore, we think it is much more efficient for the UE to provision some of these information. For the reporting granularity, information could be reported as a choice of either DRB/LCH, QFI, or in special cases, even for a particular CG/SPS or PDU Set Importance level. A potential set of traffic parameters and periodicity is as follows.

Traffic Parameters

- Data Size in bytes (e.g., within the period indicated by periodicity)

  - Data size can indicate min, max, average burst size. In case only one value is provided then Maximum Size is preferred. The size may be based on Data Burst or PDU Set.

- Arrival Time, for PDU Set or Data Burst arrival — a choice of one of the three options below:

  - Start and End: time window between first packet and last packet arrival; reported as

    - two values, one for start and one for end; or

    - start_time + max_duration

  - Arrival Time only, following the definition as in TSCAI / BAT in 23.501

  - Arrival Time + Burst Spread (based on jitter (especially for UL in tethering UC), e.g., the variation of the arrival time of the first packet)

    - Two values, one for Arrival Time and one for Burst Spread

Periodicity

- Represents a periodical packet arrival rate for bursty or quasi-periodical traffic

- May translate into a periodicity of a CG/SPS or could be based on application layer provisioning

From our perspective, XR Traffic Assistance Information should at least include data size, arrival time, and periodicity. The reporting of information for uplink traffic flows is more pronounced than the reporting for downlink traffic flows, but since activities on UL/DL flows are interrelated the UE has all the information, it might not be a big overhead to provide an option to report both directions. Besides the network can configure which information is desired including the direction and the flow.

Proposal 1: RAN2 should consider the following UE-provisioned XR traffic assistance information relating to groups of packets: periodicity, UL packet arrival time (start/stop), importance/priority, sequence, boundary, and size information.

As UL jitter can occur when media content generation and protocol processing are handled by different components that are tethered over e.g., Sidelink or non-3GPP technologies, RAN2 has confirmed that UL jitter information can be reported by the UE. In our views, UL jitter information can be represented by UL packet arrival time in Proposal 1.

Proposal 2: UL jitter information to be reported by the UE can be represented by UL packet arrival time.

## Procedure for Configuration and Reporting of XR Traffic UAI

Based on the analysis above, it makes sense for RAN2 to address the cases where TSCAI is insufficient to provide up-to-date XR traffic characteristics to the RAN, or when inherent characteristics require a fast adjustment of the current allocation. Since the XR traffic characteristics are largely influenced by user activities, we think it is easier for the UE to obtain the most up-to-date information on XR traffic conditions, and therefore UE could provide it to the gNB as a complementary source of assistance information. On a high level, the network may configure the UE to enable/disable scheduling assistance information, e.g., at QoS flow establishment or through an update at a later time.

In light of this, RAN2 can further explore the procedure of enabling UE Assistance Information for XR scheduling. Specifically, we think a procedure based on the following principle can be considered. Figure 1 shows an example for illustration purposes.

Figure 1: Example XR Traffic Assistance Information reporting procedure

The XR Assistance Information on traffic characteristics can be configured and provisioned based on the following procedure description:

- The network can first configure a UE to provide scheduling assistance information. For example, the gNB may configure/request a UE to send UAI relating to XR traffics over RRC. Alternatively, the network may also map the reporting of assistance information to a different message layer (e.g., SDAP). The configuration may further include a prohibit timer and respective thresholds.

- Based on the configuration, the UE AS can interact with the application layer (and/or the tethering module) and indicate the need of information relating to XR traffic characteristics.

- The application layer provides information to the UE AS based on the request. The information may include traffic periodicity and expected packet size etc.

- The UE AS sends the information (provided by the application layer) to gNB, based on UAI with extended parameter sets.

- Based on the received UAI, the gNB may conduct scheduling accordingly (e.g., configure or re-configure CG for UL transmission, if deemed appropriate by the gNB scheduler)

With regards to the actual data to be derived at the UE side, XR Traffic Assistance Information could be determined and reported according to on one of the following two options:

1) Input from the application and/or tethering module following an internal handshake; or

2) Detection or arrival times in the UE’s AS itself.

While the first option may require an (internal) API between UE Access Stratum (AS) and Application, the second option directly uses information locally available at the UE modem. Note that the UE in the second approach needs to filter and collect information for relevant QoS flows or logical channels. While reporting over RRC is one option for the 1st approach, SDAP might be suited for the 2nd approach, as it has a notion of the QFI. How the assistance data is derived (e.g., option 1 or 2) could be left to UE implementation, however, the desired parameter set itself could be controlled by the network as part of a configuration.

We think RAN2 can further explore the possibility of allowing the UE to provide XR assistance information for the purposes of scheduling. For reporting over UAI a straightforward option is to introduce a new XRTrafficPatternInfo IE (like a traffic pattern info used by SL) for the parameters identified in the previous section.

Proposal 3: Provision of UE Assistance Information can be made configurable by the network, including the parameter set and the message layer. RAN2 to discuss the high-level principle/procedure of UE-Assistance Information relating to XR traffic characteristics.

## End of Data Burst Indication

During RAN2#121bis-e, there have been some discussions about whether an explicit End of Data Burst (EoDB) indication is needed. As a result, RAN2 agreed to consider it further:

FFS on whether EoDB signalling is needed.

Several companies have mentioned that BSR with BS=0 can be used to indicate the EoDB and thus, explicit EoDB signalling is not needed. On the other hand, some companies prefer to have an explicit signaling and think it can be quite beneficial for e.g., UE power saving.

In our understanding, the approach relying on a BSR indicating a buffer size of zero may have some drawbacks. First, the UE may not always have a BSR transmission opportunity. If buffer size = 0, the UE will not trigger regular BSR. In fact, the UE may cancel triggered BSR when the buffer becomes zero when all the available UL grant(s) can accommodate all the data. The UE may still trigger padding BSR, but it will only happen when there are padding bits available. Nevertheless, since we will have new BS table(s) in Rel-18 to reduce quantization error, padding BSR may not be triggered as often as in previous releases due to smaller padding bits. Moreover, if the MAC PDU includes only the padding BSR and UL skipping is enabled, the MAC entity will not generate a MAC PDU for the HARQ entity. Thus, the BSR with buffer size = 0 may not reach the gNB on time in all cases.

The worst case is when the UE does not even have an UL-SCH for this BSR. Even if we can trigger a BSR in such cases, the UE would have to also trigger a SR simply for the sake of this BSR indicating BS=0. This not only causes additional delay, but it also introduces signalling overhead. Obviously, this is an overkill since the actual EoDB information at minimum is just a single bit.

Observation 1: To rely on BSR=0 for EoDB signalling is not a reasonable approach.

On the other hand, the UE and the gNB may be aware of the cadence and amount of data associated with a data burst, for example, by examining the periodicity and the nominal burst size from TSCAI / XR traffic assistance information. Such information may be considered an implicit (or pre-configured) EoDB indication, which refers the maximum Data Burst duration. In cases where the burst size does not fluctuate (much) and the pattern repeats over and over again, perhaps the gNB may infer EoDB from the nominal traffic characteristic that is known.

However, based on TS 23.501, clause 5.37.8.3, the UPF will provide a GTP-U indication for End of Data Burst after every DL burst to the RAN. In fact, one of the traffic characteristics of XR is that the amount of data is not always constant. The characteristics of XR traffic are also not totally different in DL and UL. In this case, for consistency, 3GPP should support a similar End of Data Burst mechanism in the uplink as well. For uplink, the UE could send an explicit EoDB indication to the RAN at least when the data burst ends early.

Proposal 4: Explicit signalling for EoDB is needed at least when the Data Burst ends early. The signalling can be useful in both UL and DL.

Next, we look at the options for signalling an EoDB indication. The simplest option is to signal a 1-bit indication. The signalling can be triggered by the UE (in UL) and by the RAN (in DL), by identifying/observing the last PDU of the Data Burst. As there is an association with a data packet, the signalling can occur as part of that data transmission.

Typically, the UE can know from upper (media) layers when the data burst ends. Such information may be associated with one QoS flow or a group of QoS flows. Since the SDAP layer has a notion of a QoS flow, one straight forward approach is to signal an end of traffic indicator on SDAP level. Alternatively, if end of traffic was to be indicated over multiple QoS flows, the SDAP header can be used as well, potentially along with a special QFI value or the R-bit.

Both variants can be very simple and do not require an additional octet in the SDAP header. For example, the R-bit in the first octet in the SDAP header and/or the QFI could be used for such an indication, which could be associated with the last UL SDAP Data PDU or even a separate SDAP Control PDU.

Proposal 5: The EoDB indication can be carried in the SDAP header.

Finally, a related question is whether EoDB is to be sent per QoS flow or as an aggregate value for multiple QoS flows. In our view SA2 specs do not preclude one way or another although it is not entirely clear what is the most applicable understanding. Probably that question should be addressed by SA2 first.

## SA4 LS on N6 PDU Set Identification

In a recent LS to RAN2 [3], SA4 raised a question on whether multiple additional bits are useful for end of data burst signalling from RAN2 perspective.

ACTION: - SA4 would like to kindly ask RAN2 to provide feedback on the feasibility and value of having additional signaling bits related to End of Burst and inter-burst time within Rel-18.

Based on our understanding of the SA4 perspective, additional bits for end of data burst (EDB) signalling are targeting at least three scenarios: 1) an ambiguity related with the setting of M-bits set in the RTP header; 2) to signal extra data burst information; 3) as a mitigation in case the PDU with the EoDB bit does not reach the receiver.

In the following, we briefly go over each of the scenarios.

In the first scenario, the RTP header may only have the M-bit set to 1 for the last RTP packet of a frame, where a frame consists of, or is transmitted in, e.g., multiple components or slices. The M-bit could be seen as an equivalent to EoDB; it is intended to allow significant events such as frame boundaries to be marked in the packet stream. With multiple PDU Sets (e.g., slices) making up a frame, the M-bit may be set to 1 only for the last PDU of the frame. According to RFC 3350, a profile may define additional marker bits. Additional EDB bits might allow to identify the border of a PDU Set or slice in such a case. Alternatively, a Data Burst may span over multiple frames. In this case, the RTP M-bit alone is no longer suitable to indicate EoDB – a secondary marker field (EDB) could be used in combination with the M-bit to mark the End of the Data Burst.

We think these aspects might be more in the realm of SA4 protocols. Though the UPF may use additional EDB bits to detect the end of a data burst (e.g., not only in DL but also in UL), as directed by media layers. On the UE side, application layers can detect the End of the Data Burst and/or the end of a PDU Set etc. and indicate the same to AS layers. We assume this can be done by UE implementation. Therefore, this scenario could remain confined to media layers and/or the interaction between UE and UPF or UE and Application Server.

Observation 2: Additional EDB bits may be useful on media protocol level, or between UE and UPF.

In the second scenario, additional EDB bits are used to indicate the time to the next data burst or some kind of correlation to a power state [4]. In our understanding, UE power states depend on a number of factors - many of which are internal to the chipset - and implementation can evaluate the DRX pattern. Hence, multiple EDB bits are not required for the UE to identify the most suitable power state, we think this can be left to UE implementation. We nevertheless think that multiple EDB bits can be useful in other scenarios.

Observation 3: UE implementation can evaluate the most appropriate power state without additional EDB bits.

The third scenario [5] addresses the possibility that the (single) PDU carrying the EoDB indication is not delivered to the receiver, for example, due to unstable network conditions, congestion, or insufficient protection against packet loss over the radio interface. The transition into DRX may happen at an incorrect time in such case, causing additional delay or unwanted side-effects. Multiple EDB bits are one option to mitigate such events, as the receiver can infer the end of a data burst by observing a sequence of multiple PDUs. For example, if the PDU with the EoDB is lost the receiver can infer from the Delimiter Info field in a different/next PDU (see below) that a new Data Burst has started. Thus, having additional EDB bits would increase the reliability of the DRX decision, but at the cost of additional overhead for the signalling.

In fact, a situation where the last PDU in a data burst can get lost seems more likely to occur over the radio interface than within the core network itself; also, the N6 interface might often be more reliable than the Uu interface itself (depending on where the application server sits, of course).

Another aspect is that additional EDB information carried over Uu may not need to match absolutely with EDB media layer content. For example, multiple bits could be used to indicate some kind of “delimiter” information to the RAN. The RAN may use multiple EDB bits, for example, to identify whether a PDU is the first, middle or last packet in the PDU Set or Data Burst, this can aid the gNB decision to control the DRX by adding a bit more protection. In other words, if the PDU with the final/single EoDB bit was lost the receiver can still infer the End of a Data Burst by observing more than one PDU. One example of such indication to the RAN might be the following.

- Delimiter Info (over Uu) with 2bits:

  - 00: PDU contains all bytes of a PDU Set or Data Burst

  - 01: PDU contains the first packet of a PDU Set or Data Burst

  - 10: PDU contains the last packet of a PDU Set or Data Burst

  - 11: PDU contains neither the first nor last packet of a PDU Set or Data Burst

The Delimiter Info could be sent by the UE to the RAN in UL; the UPF may do the same in DL over GTP-U.

Observation 4: Additional EDB bits can be valuable to assist the RAN in unstable network conditions.

Proposal 6: Reply to SA4 that additional signaling bits related to End of Data Burst are feasible and useful if they are used for other purposes instead of inter-burst time indication.

Proposal 7: RAN2 to adopt the Draft Reply LS to SA4 in the Annex.

# Conclusions

This contribution provides a view on XR Traffic Assistance Information. We have following observations and proposals:

Proposal 1: RAN2 should consider the following UE-provisioned XR traffic assistance information relating to groups of packets: periodicity, UL packet arrival time (start/stop), importance/priority, sequence, boundary, and size information.

Proposal 2: UL jitter information to be reported by the UE can be represented by UL packet arrival time.

Proposal 3: Provision of UE Assistance Information can be made configurable by the network, including the parameter set and the message layer. RAN2 to discuss the high-level principle/procedure of UE-Assistance Information relating to XR traffic characteristics.

Observation 1: To rely on BSR=0 for EoDB signalling is not a reasonable approach.

Proposal 4: Explicit signalling for EoDB is needed at least when the Data Burst ends early. The signalling can be useful in both UL and DL.

Proposal 5: The EoDB indication can be carried in the SDAP header.

Observation 2: Additional EDB bits may be useful on media protocol level, or between UE and UPF.

Observation 3: UE implementation can evaluate the most appropriate power state without additional EDB bits.

Observation 4: Additional EDB bits can be valuable to assist the RAN in unstable network conditions.

Proposal 6: Reply to SA4 that additional signaling bits related to End of Data Burst are feasible and useful if they are used for other purposes instead of inter-burst time indication.

Proposal 7: RAN2 to adopt the Draft Reply LS to SA4 in the Annex.

# References

- RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm, RAN#99

- R2-2300723, PDU Set Parameters and UL Jitter, Apple, RAN2#121

- R2-2304493, LS out on the N6 PDU Set Identification, SA4 LS in, To:SA2, RAN2

- S4-230666, [5G_RTP] Signaling the Time to Next Burst, Nokia, SA4#123e

- S4-230718, [5G_RTP] End-of-burst marking in PDU Set RTP HE, InterDigital, Qualcomm, SA4#123e

# Annex

## Draft Reply LS on N6 PDU Set Identification to SA4

1. Overall Description:

RAN2 thanks SA4 for the LS on N6 PDU Set Identification. For the following action:

- SA4 would like to kindly ask RAN2 to provide feedback on the feasibility and value of having additional signaling bits related to End of Burst and inter-burst time within Rel-18.

RAN2 thinks the additional signaling bits are feasible and useful, but radio interface protocols may use them for other purposes instead of inter-burst time. The power state can be evaluated based on UE implementation without additional bits. From RAN2 perspective, the additional bits can be used to handle the cases where the PDU carrying the End of Data Burst indication is not delivered to the receiver.

2. Actions:

To SA4 group.

ACTION: 	RAN2 respectfully asks SA4 to take the above into account and provide feedback if necessary.