3GPP TSG-RAN WG2 Meeting #122	R2-2305080

Incheon, South Korea, 22 May – 26 May 2023

Agenda item:	7.23.2

Source:	Apple

Title:	5GS Network Timing Synchronization in RRC_INACTIVE

WID/SID:	NR_TRS_URLLC

Document for:	Discussion / Decision

# Introduction

SA2 has studied architectural enhancements for supporting 5G Timing Resiliency and URLLC in the 5G system. To address the RAN impacts out of this work, RAN has approved a new R18 WI in the last RAN plenary at RAN#99.

One of the objectives is about 5GS network timing synchronization status and reporting.

1. 5GS network timing synchronization status and reporting [RAN3, RAN2]: a. AMF providing clock quality reporting control information per-UE to the gNB. [RAN3] b. gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3] Note 1: Details of the 5G clock quality information will be decided by RAN3. c. UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2] d. gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3]

This paper provides an initial view on 5GS Network Timing Synchronization status and reporting from a UE perspective. In particular, we consider a case where the UE is in RRC_INACTIVE state.

# Discussion

## Introduction

Use cases related to the NR_TRS_URLLC work item target network customers like smart grids and audio/video production. Rel-18 timing resiliency is meant to improve the robustness of services with a strong need for time synchronization. In the event of a clock/time synchronization degradation or failure, the 5G system can assist the end user to switch to an alternative time source. Accordingly, the 5G system will support reporting network timing synchronization status (such as divergence from UTC and 5GS network timing source degradation) to UEs and AFs. Likewise, the RAN will be able to observe 5GS network timing synchronization status and inform UEs and AFs.

In order to notify UEs of a clock quality information update, the NG-RAN includes in SIB9 an event ID when the network timing synchronization status exceeds the threshold or when the time system meets the thresholds again (following an earlier event). Either event serves as a notification to the UEs reading the SIB9 that there is new clock quality information available. The UE in RRC_INACTIVE or RRC_IDLE compares the reference report / event ID in SIB9 with the locally stored ID to determine if the last available clock quality information was already retrieved. If the UE is instructed to reconnect to the network in the case when the UE determines that event ID has changed, the UE in RRC_INACTIVE or RRC_IDLE state reconnects to the network. After the UE has reconnected to the network, the NG-RAN uses unicast RRC signalling to provision the clock quality information to the UEs.

## Time synchronization in RRC_INACTIVE state

According to SA2, to retrieve/receive the accurate (and complete) time status and updated clock quality information, a UE in RRC_INACTIVE and RRC_IDLE state needs to enter RRC CONNECTED mode for the gNB to send time information to the UE using unicast signaling. In order to go to RRC_CONNECTED mode UEs in RRC_IDLE undergo a registration. UEs in RRC_INACTIVE are meant to initiate the RRC Connection Resume procedure from RRC_INACTIVE.

TS 23.501, clause 5.27.1.12 Editor's note: Support for RRC_INACTIVE and RRC_IDLE states depends on RAN2 feedback. When activating time synchronization for a UE, TSCTSF requests the AMF (via PCF using AM policy) to instruct the UE to transition to RRC CONNECTED to the network in the case when the UE later detects that the RAN timing synchronization status has changed while the UE is in RRC_INACTIVE or RRC_IDLE state.

In some cases though, an update of time synchronization information (including clock quality information) could be achieved without entering RRC CONNECTED mode. For example, if the UE supports SDT the network can directly send an DLInformationTransfer message to a UE in RRC_INACTIVE.

Thus, in our understanding, the clock quality information update does not always require the UE to enter RRC_CONNECTED mode. That is, the necessary information exchange may be done entirely in RRC_INACTIVE state using SDT procedures itself. The decision whether to perform the clock quality information update entirely in RRC_INACTIVE or whether to send to UE to RRC_CONNECTED is at the discretion of the network. Moreover, it is also possible for the network to compensate over-the-air propagation delay in RRC_INACTIVE state. Therefore, we would like to confirm that this is the common understanding.

Proposal 1: If the UE supports SDT the clock quality information retrieval and reception of updated time synchronization information may be completed in RRC_INACTIVE state.

Proposal 2: The network can send a DLInformationTransfer message to a UE in RRC_INACTIVE state.

If the UE can use SDT and the conditions to initiate SDT transfer are fulfilled (such as, there is no data available for non-SDT radio bearers, the available amount of UL data is less than a threshold, SDT is enabled for the RB, etc.) the UE initiates a SDT session based on the SDT resources available / configured. If it is MO-SDT, where the trigger is because of the SIB indication for the need of a time sync update, then the UE can initiate RACH (for RA-SDT) or the UE can skip RACH and use CG-SDT (if possible).

For RA-SDT, the UE may indicate the type of request through a new resumeCause value. That is, unless clock quality information retrieval uses a separate Access Category or a special mapping of a new Access Identity to an Access Category, although the latter may also show as a new resumeCause value in the RRCResumeRequest or RRCResumeRequest1 message.

Proposal 3: For RA-SDT, the UE may indicate the type of request through a new resumeCause value.

For CG-SDT, the UE may include in ULInformationTransfer a request to retrieve clock quality information, for example, by providing the latest reference report / event ID as a parameter, or by setting a flag in a new field of the ULInformationTransfer message. Uplink related time synchronization information can also be provided by the NAS layer or the DS-TT via DedicatedNAS-Message.

Proposal 4: For CG-SDT, the UE may include in ULInformationTransfer a request to retrieve clock quality information.

Once the SDT session is ongoing, it is also possible to have subsequent UL or DL small data in the same SDT session. The UE may not need to enter RRC_CONNECTED mode in order for the clock quality information update to be completed.

# Conclusions

This contribution provides initial views on 5GS Network Timing Synchronization status and reporting. We have following observations and proposals:

Proposal 1: If the UE supports SDT the clock quality information retrieval and reception of updated time synchronization information may be completed in RRC_INACTIVE state.

Proposal 2: The network can send a DLInformationTransfer message to a UE in RRC_INACTIVE state.

Proposal 3: For RA-SDT, the UE may indicate the type of request through a new resumeCause value.

Proposal 4: For CG-SDT, the UE may include in ULInformationTransfer a request to retrieve clock quality information.

# References

- RP-230754, New WID on NR Timing Resiliency and URLLC enhancements, Nokia, RAN#99

- TR 23.700-25, v18.1.0, Study on timing resiliency and TSC and URLLC enhancements, Rel-18

- S2-2301461, TR 23.700 KI#1 conclusion update, CR 0002, Nokia, Qualcomm, Huawei, ZTE, Samsung