# 3GPP TSG RAN WG2 Meeting #122      	                           R2-2305081
Incheon, Korea, 22th – 26th May 2023

Agenda item:	7.3.2

Source:	Apple, InterDigital, T-Mobile USA, MediaTek Inc., Intel

Title:	Support high priority traffic in Cell DTX / DRX

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#121b-e [1], whether to allow exceptions on SR transmission in non-active duration of Cell DTX / DRX to accommodate high priority traffic was discussed. Below agreements were made with highlighted FFSs captured.

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmission

In this contribution, we discuss whether to support high priority traffic in Cell DTX / DRX, and the corresponding UE behaviors.

# 2 Discussion

## 2.1 Whether to support high priority traffic in Cell DTX / DRX

As captured in the FFS, one root cause of the divergence is that companies have different views on whether high priority traffic (e.g. voice, emergency call and SRB) needs to be supported when Cell DTX / DRX is configured. During online discussion of RAN2#121b-e [1], some companies don't prefer any exception with below reasonings:

- High priority traffic doesn't not need to be supported in cell which is configured with Cell DTX/DRX because the goal of Cell DTX / DRX is for NES. NW can handover the UEs with low latency traffic to another cell.

- gNB can configure SR transmission occasions for high priority traffic to fully fall within active duration of Cell DTX / DRX.

- Allowing exceptions on SR in non-active duration of Cell DTX/DRX will make gNB wake up frequently and lead to marginal NES gain.

- gNB may not know in advance when UE's high priority traffic arrives.

As follows, we try to address these concerns one by one.

For 1), we think it is important to support high priority traffic, especially voice, emergency call and SRB. If such high priority traffic is not supported during non-active duration of Cell DTX / DRX, the deployment of Cell DTX / DRX will be challenged as uplink resource access with Cell DRX becomes very rigid. Meanwhile, the co-existence of legacy UEs and NES capable UEs when Cell DTX / DRX is activated will be an issue.

Observation 1: If high priority traffic is not supported during non-active duration of Cell DTX / DRX, the deployment of Cell DTX / DRX will be challenged as uplink resource access with Cell DRX becomes very rigid. Meanwhile, the co-existence of legacy UEs and NES capable UEs when Cell DTX / DRX is activated will be an issue.

Thus, we propose RAN2 first confirm high priority traffic should be supported when Cell DTX/DRX is activated.

Proposal 1: NES Cell configured with Cell DTX / DRX can support high priority traffic, which includes at least voice, emergency call and SRB.

For 2), we agree that gNB can configure SR transmission occasions within active duration of Cell DTX / DRX via special RRC configurations. However, to support high priority service, gNB can only configure a short duty cycle of Cell DTX / DRX less than Packet Delay Budge (PDB) of the traffic (e.g. less than 40ms for voice). As we know, NES gain can be maximized only when the duty cycle of cell DTX / DRX is long so that gNB can enter sleep mode for a long time. Configuring SR occasions only during Cell DRX active periods can therefore limit the duration of the Cell DRX duty cycles, to accommodate emergency calls and high priority traffic, thus limiting the amount of network energy saving that can be achieved. Thus, it is conflicted with the intention to introduce Cell DTX / DRX.

Observation 2: If no exception for SR transmission in non-active duration is allowed, gNB has to configure a short duty cycle of Cell DTX / DRX less than PDB of the low latency traffic (e.g. less than 40ms for voice), which restricts NES gain because gNB can't enter sleep mode for a long time.

For 3), we think a good design of cell DTX / DRX will marginally decrease NES gain due to exceptions of some SR transmissions. Please note that RAN2 has agreed that SSB transmission, paging, SIB and RACH are not impacted by cell DTX / DRX. It means that gNB has to wake up for reception in all ROs (RACH Occasions) and wake up for transmission in all occasions for SSB/SIB/paging, even if these occasions are within non-active duration of cell DTX / DRX. Thus, as illustrated in Figure. 1, if the exceptional SR transmission occasions are configured to be close to ROs, the extra NW power consumption is marginal because gNB anyway needs to wakes up in these ROs for preamble reception. In example of Figure 1, NW can configure only SR1 is exceptional for transmission of high priority traffic. In case alignment with RO is not possible, the network can either configure such SR for emergency calls to be aligned with SR occasions of other legacy UEs, or the network can use short PUCCH formats to limit the energy saving gain reduction.

Figure.1 Example of exceptional SR transmission in non-active duration with marginal NES loss

Observation 3: Because gNB has to wake up for preamble reception in all ROs in non-active duration of Cell DRX, marginal NES loss is expected if the occasions of exceptional SR are configured close to ROs.

For 4), please note that one SR configuration is associated with one or more LCH(s) according to TS 38.331 [2]. Figure. 2 provides one illustration on the RRC configuration.

Thus, existing SR configuration already allows the Network to associated it with a logical channel with high priority. Then, through mapping between high priority traffic and logical channel priority, the Network can know whether high priority traffic arrives upon reception of the SR.

Observation 4: Existing SR configuration already allows the Network to associate it with a logical channel with high priority. Thus, gNB can know whether high priority traffic arrives upon reception of SR associated with high priority LCH(s).

What's more, the specification change to allow exceptional SR is simple:

- A list of exceptional schedulingRequestID can be included in Cell DRX configuration

  - As illustrated in Figure. 2, the mapping between logical channel and different SR configurations can be configured via IE schedulingRequestID in LogicalChannelConfig.

  - Thus, via proper configuration, gNB can only wake up to receive high priority traffic associated with LCH with high priority in the SR transmission occasions indicated by the exceptional list.

  - The UE keeps all SR configuration, but can only transmit SR in the occasions indicated by the exceptional list.

Figure.2 Existing RRC configuration on association between SR and logical channel(s)

Observation 5: The specification change to allow exceptional SR is simple: a list of exceptional schedulingRequestID can be included in Cell DRX configuration. gNB can only wake up to receive high priority traffic associated with LCH with high priority in the SR transmission occasions indicated by the exceptional list.

Thus, we propose to support configurable exceptions of SR transmission during Cell DRX non-active period.

Proposal 2: The UE can be configured per SR configuration whether to allow SR transmission associated with high priority LCH(s) during Cell DRX non-active period.

### 2.1.2 UE behaviour after SR transmission

We address below 2 FFSs.

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS#1: How to deal with any exceptions (e.g. SR if agreed and RACH)

According to TS 38.321 [3], the UE keeps monitoring PDCCH when a SR is sent and is pending in existing UE CDRX. We think the same behavior can be reused for exceptional SR transmission in Cell DRX non-active duration.

Proposal 3: Reuse UE CDRX mechanism when the UE transmits SR during non-active duration of Cell DRX, i.e. the UE keeps monitoring PDCCH when a SR is sent on PUCCH and is pending.

FFS#2: For the FFS case (i.e. allowed SR) there may be some exceptions.

For the allowed SR during non-active periods, SR transmission may be failed as legacy. We think the legacy SR retransmission mechanism in TS 38.321 [3] can be reused, i.e. the SR can be retransmitted for a configured attempt number (i.e. sr-TransMax) and RACH is triggered when retransmission number is above the threshold.

Proposal 4: For the allowed SR during non-active duration of Cell DRX, no spec impact on its transmission behavior is foreseen, i.e. the SR can be retransmitted up to sr-TransMax and RACH is triggered when retransmission number is above the threshold.

# 3 Conclusion

In this contribution, we discuss whether / how to support high priority traffic in non-active duration of Cell DTX / DRX. Our observations are:

Observation 1: If high priority traffic is not supported during non-active duration of Cell DTX / DRX, the deployment of Cell DTX / DRX will be challenged as uplink resource access with Cell DRX becomes very rigid. Meanwhile, the co-existence of legacy UEs and NES capable UEs when Cell DTX / DRX is activated will be an issue.

Observation 2: If no exception for SR transmission in non-active duration is allowed, gNB has to configure a short duty cycle of Cell DTX / DRX less than PDB of the low latency traffic (e.g. less than 40ms for voice), which restricts NES gain because gNB can't enter sleep mode for a long time.

Observation 3: Because gNB has to wake up for preamble reception in all ROs in non-active duration of Cell DRX, marginal NES loss is expected if the occasions of exceptional SR are configured close to ROs.

Observation 4: Existing SR configuration already allows the Network to associate it with a logical channel with high priority. Thus, gNB can know whether high priority traffic arrives upon reception of SR associated with high priority LCH(s).

Observation 5: The specification change to allow exceptional SR is simple: a list of exceptional schedulingRequestID can be included in Cell DRX configuration. gNB can only wake up to receive high priority traffic associated with LCH with high priority in the SR transmission occasions indicated by the exceptional list.

Based on observations, our proposals can be found below.

Proposal 1: NES Cell configured with Cell DTX / DRX can support high priority traffic, which includes at least voice, emergency call and SRB.

Proposal 2: The UE can be configured per SR configuration whether to allow SR transmission associated with high priority LCH(s) during Cell DRX non-active period.

Proposal 3: Reuse UE CDRX mechanism when the UE transmits SR during non-active duration of Cell DRX, i.e. the UE keeps monitoring PDCCH when a SR is sent on PUCCH and is pending.

Proposal 4: For the allowed SR during non-active duration of Cell DRX, no spec impact on its transmission behavior is foreseen, i.e. the SR can be retransmitted up to sr-TransMax and RACH is triggered when retransmission number is above the threshold.

# 4 References

[1] RAN2#121b-e, Chair Notes

[2] TS 38.331-v17.1.0, Radio Resource Control (RRC) protocol specification, 2022-6.

[3] TS 38.321-v17.1.0, Medium Access Control (MAC) protocol specification, 2022-6.