# 3GPP TSG RAN WG2 Meeting #122      	                           R2-2305082
Incheon, Korea, 22th – 26th May 2023

Agenda item:	7.3.2

Source:	Apple

Title:	Discussion on key open issues of Cell DTX / DRX

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

The Network Energy saving (NES) WID RP-223540 was agreed in RAN#98-e [1], the WI objective on Cell DTX / DRX is copied below:

In RAN2#121 [2] and RAN2#121b-e [3], some progress was made on Cell DTX / DRX. However, there are still some open issues.

In this contribution, we share our view on below key open issues:

- Open issue on signaling

- Open issue on UE behavior

- Alignment between Cell DTX and UE CDRX

- Measurement and reporting

Please note that whether / how to support high priority traffic is discussed in our companion contribution [4].

# 2 Discussion

## 2.1 Open issues on signalling

### 2.1.1 Details of Cell DTX/DRX configuration

In RAN2#121b-e [3], it was agreed that the Cell DTX/DRX configuration includes at least periodicity, offset and on duration, which is similar to UE CDRX.

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

We think one missed aspect is how to calculate starting time of active duration of Cell DTX / DRX pattern. Because these agreed configurations (periodicity, offset and on-duration) are quite similar to UE CDRX, we think it is straight forward to reuse formula of UE CDRX in TS 38.321 [5]. It is also helpful for the alignment between UE CDRX and Cell DTX / DRX.

Proposal 1: Reuse the below formula of UE CDRX to calculate starting time of active duration of Cell DTX / DRX pattern:

- [(SFN × 10) + subframe number] modulo (Periodicity) = StartOffset

### 2.1.2 Multiple Cell DTX/DRX configurations

Whether to support multiple Cell DTX/DRX configurations was discussed in RAN2#121 [2], and below agreement was made.

- Confirm study item agreement that we can have separate DTX and DRX configuration.   We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported.

We think the FFS on whether multiple configurations of cell DTX or DRX can be discussed when the conclusion of single configuration is clear. However, we think one important issue missed is whether Cell DTX and Cell DRX with different active duration can be activated simultaneously for the same serving cell, i.e. whether Case a) in Figure.1 is allowed.

Figure.1 Illustration of different understandings of joint Cell DTX and cell DRX operation

Our understanding is that only single pattern can be activated for one serving cell when Cell DTX and Cell DRX are jointly operated. The reasonings include:

- The NES gain of case a) over case b) is not clear because the power consumption of DL is much higher than UL according to TR38.864 [7].

- DL and UL are sometimes tightly coupled (e.g. DL transmission and its UL HARQ feedback). Allowing case a) will significantly complicate UE behaviors and introduce extra standard work (e.g. alignment mechanism between Cell DTX and Cell DRX, UE behaviors in 4 combinations of Cell DTX active/non-active and Cell DRX active/non-active).

Observation 1: When Cell DTX and Cell DRX are jointly operated, allowing different patterns brings marginal NES gain, but will significantly complicate UE behaviors and introduce extra standard work.

Thus, we propose to confirm case a) is not allowed. Please note that standalone Cell DTX and Cell DRX with different patterns (i.e. only Cell DTX or only Cell DRX is operated in one serving cell) are still allowed to provide flexibility of NW.

Proposal 2: RAN2 confirm the understanding: when Cell DTX and Cell DRX are jointly operated, only single pattern (i.e. one set of periodicity, offset and on duration) can be activated for one serving cell.

## 2.2 Open issues on UE behaviour

### 2.2.1 Exceptional DL/UL transmission in non-active duration

In RAN2#121b-e [3], whether to allow exceptions on SPS/CG/SR transmission in non-active duration of Cell DTX / DRX to accommodate high priority traffic was discussed. It was agreed that configurable SR during non-active period is FFS.

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

In our companion contribution [4], we proposed that NES Cell configured with Cell DTX / DRX can support high priority traffic, which includes at least voice, emergency call and SRB. To achieve it, configurable SR during non-active period of Cell DTX / DRX is allowed to notify NW the arrival of high priority traffic in the UE side. On the other hand, it is also possible that high priority traffic arrives at the NW side during non-active duration. Similar to exceptional SR, we think it is necessary to introduce a mechanism that NW is still allowed to transmit high priority traffic to one particular UE during non-active duration of Cell DTX.

Observation 2: High priority traffic may arrive at the NW side during non-active duration. Similar to exceptional SR, it is necessary to introduce a mechanism to allow NW to transmit high priority traffic to the UE during non-active duration of Cell DTX.

Proposal 3: Introduce a mechanism that NW is allowed to transmit high priority traffic to one particular UE during non-active duration of Cell DTX. FFS detailed mechanism.

Regarding to specific mechanism, we think it needs further discussion. In RAN2#121 [2], it was agreed whether to introduce UE specific inactivity timer is FFS.

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer .  FFS on configuration signaling and stage 3.

We think this so-called UE specific inactivity timer is similar to drx-inactivityTimer in UE CDRX, and it is one possible mechanism to achieve this purpose. According to TS 38.321 [5], the UE starts drx-inactivityTimer upon successful decoding a PDCCH indicating a new DL/UL transmission, and it keeps monitoring PDCCH when the timer is running because NW is likely to schedule data in this duration. However, we think this mechanism may not work well for Cell DTX. Different from UE CDRX, the Cell DTX is expected to have more restrictions on UE transmission / reception, and the non-active duration is expected to be long to achieve sufficient NES gain. However, UE specific inactivity timer can only work when the UE receives PDCCH around the boundary of active duration. In other word, if high priority data arrives at NW side in middle of non-active duration, NW can't reach to the UE.

Observation 3: Non-active duration of Cell DTX is expected to be long to achieve sufficient NES gain. However, inactivity timer in UE CDRX can only work when the UE receives PDCCH around the boundary of active duration. If high priority data arrives at NW side in middle of non-active duration, NW can't reach to the UE.

Thus, we prefer to pursue other approach. One possible way is to start a timer upon reception of paging message. Specifically, the NW can still page a CONNECTED UE if high priority traffic towards this UE arrives during non-active duration of Cell DTX. Upon reception of paging message with scheduling information, the UE starts a timer and monitor PDCCH when the timer is running. Details of this mechanism can be further discussed.

Proposal 4: RAN2 discuss whether NW can page a NES UE in RRC_CONNECTED state if high priority traffic towards this UE arrives during non-active duration of Cell DTX.

### 2.2.2 Retransmission in non-active duration

In RAN2#121b-e [3], it was agreed that the UE doesn't monitor PDCCH for new transmission during non-active duration of Cell DTX. But whether/how to handle retransmission is FFS.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmission

During non-active duration of Cell DTX, it is expected that the UE can only transmit / receive high priority traffic. Thus, if the new transmission is performed (e.g. PUSCH associated with the exceptional SR), it means it is a transmission/reception of high priority. Then, it makes sense to allow its retransmission. We think existing UE CDRX retransmission timer based mechanism (i.e. HARQ RTT timer and retransmission timer) can be reused.

Proposal 5: When UE performs new PDSCH/PUSCH transmission during non-active duration of Cell DTX/DRX (e.g. PUSCH associated with exceptional SR), retransmission is allowed and UE CDRX retransmission timer based mechanism (i.e. HARQ RTT timer and retransmission timer) is baseline.

### 2.2.3 Handling of CG timer

In RAN2#121b-e [3], it was agreed that the UE doesn't transmit on CG occasion during non-active duration.

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

To implement this agreement, we think the UE behaviour on configuredGrantTimer needs clarification. According to TS 38.321 [5], when configuredGrantTimer expires, the UE assumes positive acknowledge for the corresponding HARQ process.

From section 5.4.1 of TS 38.321

......

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall:

1>	if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or the PUSCH duration of a MSGA payload for this Serving Cell; or

1>	if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured (i.e. new transmission):

3>	consider the NDI bit for the corresponding HARQ process to have been toggled;

3>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

......

Observation 4: According to TS 38.321, when configuredGrantTimer expires, the UE assumes positive acknowledge for the corresponding HARQ process.

Upon entering non-active periods of Cell DRX, it is possible that the UE has pending HARQ process(es) with the corresponding configuredGrantTimer running. Then, because the UE does not transmit on CG occasions and doesn't monitor PDCCH, all the running configuredGrantTimer should be stopped. Otherwise, the above CG timer mechanism will make UE to mistakenly drop pending UL traffic in buffer. It will cause UL data loss and not sync between UE and NW.

Proposal 6: Upon entering non-active periods of Cell DRX, the UE stops all the running configuredGrantTimer.

With regarding to cg-RetransmissionTimer, we think RAN2 may deprioritize its discussion because it is only target for NR-U.

## 2.3 Alignment between Cell DTX and UE CDRX

In post-meeting email discussion 312 [6], Rapporteur drafted below related summary proposals, but it was not agreed.

Proposal 6: An aligned UE C-DRX configuration with Cell DTX means that the on-duration of C-DRX falls within Cell DTX on-duration. FFS extension of Cell DTX active time beyond Cell DTX on-duration. FFS gNB/UE behaviour during T2 from Fig.1. (15/25)

Proposal 7: The periodicity of UE C-DRX configurations in a cell should be the same or a multiple of the serving Cell’s DTX periodicity.

We think this alignment requirement for NW is too strong. If such requirement is agreed, one followed issue is that the NW may be forced to immediately reconfigure multiple UE CDRX upon Cell DTX is activated, to ensure each UE's CDRX on-duration within on-duration of Cell DTX. Such reconfiguration of UE CDRX requires the NW to send multiple RRC messages immediately, which is unnecessarily complex and incurs high signalling overhead. In our understanding, such strong requirement is unnecessary. As long as the UE behaviour in all possible overlapping duration is clear, we can leave it to NW implementation to choose appropriate configurations of UE CDRX and Cell DTX.

Observation 5: If the requirement for NW is agreed, one followed issue is that the NW may be forced to immediately reconfigure multiple UE CDRX upon Cell DTX is activated, to ensure each UE's CDRX on-duration within on-duration of Cell DTX. It is unnecessarily complex and incurs high signalling overhead.

Thus, we propose:

Proposal 7: As long as UE behavior in all possible overlapping duration is clear, it is unnecessary to introduce NW alignment requirement between Cell DTX and UE CDRX, i.e. it is up to NW implementation to choose appropriate configurations of UE CDRX and Cell DTX.

Figure.2 Illustration of issue scenarios of Cell DTX and UE CDRX alignment

Figure. 2 illustrates 3 possible overlapping durations between Cell DTX and UE CDRX. In our understanding, UE behaviour in T2 is already covered by below agreement made in RAN2#121b-e [3] (i.e. the UE doesn't monitor PDCCH).

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

However, the UE/gNB behaviour during T1 and T3 in Figure.2 is not clear. Thus, further discussion is needed.

- T1 (Part of OFF duration of UE DRX is overlapped with active duration of Cell DTX)

- T3 (OFF duration of UE DRX is overlapped with non-active duration of Cell DTX)

Our analysis for the two cases are:

- For T1, we think the reasonable UE behaviour is to follow UE CDRX behaviour (i.e. stop monitoring PDCCH) by assuming that gNB will not transmit DCI for it (i.e. gNB may transmit DCI for other UEs).

- For T3, although final agreement of UE behaviour in Cell DTX is not made yet, it seems that the common understanding is that Cell DTX has more restricted UE behaviours than UE CDRX (e.g. all SPS / CG are allowed in OFF duration of UE CDRX). Thus, we think the reasonable UE behaviour is to follow Cell DTX.

Proposal 8: RAN2 agree below UE/gNB behavior in T1 and T3 durations:

- T1 (OFF duration of UE CDRX is overlapped with active duration of Cell DTX): the UE follows UE CDRX behaviour (i.e. stop monitoring PDCCH), assuming no DCI for itself.

- T3 (OFF duration of UE CDRX is overlapped with non-active duration of Cell DTX): the UE follows behaviour specified for Cell DTX.

## 2.4 Measurement and reporting

For RRC_CONNECTED UE, it may perform the following measurements and reporting:

- SSB based L1/L3 measurements and reporting

- CSI-RS based L1/L3 measurement and reporting (including TRS)

- PRS based measurement and reporting

R1-2302131	Discussion Summary #2 for enhancements on cell DTX/DRX mechanism	Moderator (Intel Corporation)

Agreement

At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS.

- DL

  - Periodic/Semi-persistent CSI-RS (including TRS)

  - PRS

  - PDCCH scrambled with UE specific RNTI

  - PDCCH in Type-3 CSS

  - SPS-PDSCH

- UL

  - SR

  - Periodic/Semi-persistent CSI report

  - Periodic/Semi-persistent SRS

  - CG-PUSCH

Other signals/channels are not precluded

Meanwhile, RAN1 made below agreements in RAN1#112:

For SSB based measurement / reporting, according to below note in WID objective, SSB transmission is not impacted by Cell DTX / DRX.

- Note: No change for SSB transmission due to cell DTX/DRX.

However, it is not clear whether the UE can stop or relax measurement requirement during non-active duration of Cell DTX/DRX, considering gNB is sleeping. This issue should be first discussed in RAN4 and RAN1.

For CSI-RS/PRS based measurement / reporting, it is not clear whether gNB will stop transmitting CSI-RS/PRS during non-active duration of Cell DTX/DRX to save power consumption. In addition, if it is allowed, it is not clear whether the UE can stop or relax CSI-RS based measurement requirement during non-active duration of Cell DTX/DRX, considering gNB is sleeping. This issue should be first discussed in RAN1 and RAN4.

Observation 6: Although SSB transmission is not impacted by Cell DTX / DRX, whether the UE can stop or relax measurement during non-active duration of Cell DTX/DRX need to be first discussed in RAN4/RAN1.

Observation 7: For CSI-RS/TRS/PRS, whether they can be turned off and their measurement requirement during non-active duration need to be first discussed in RAN4/RAN1.

Measurement and reporting requirements are RAN4 expertise, but they may have RAN2 impact (e.g. whether/how the UE performs RRM/RLM/BFR and its signaling). Before starting work on RRM/RLM/BFR, RAN2 need to know whether any change on measurement requirements. Thus, we propose RAN2 send LS to RAN1/RAN4 on requirements of measurement and reporting.

Proposal 9: In order to start RRM/RLM/BFR work, RAN2 send LS to RAN1/RAN4 on whether any change on requirements of measurement and reporting based on SSB/CSI-RS/TRS/PRS.

# 3 Conclusion

In this contribution, we further discuss Cell DTX / DRX. Our observations are:

Observation 1: When Cell DTX and Cell DRX are jointly operated, allowing different patterns brings marginal NES gain, but will significantly complicate UE behaviors and introduce extra standard work.

Observation 2: High priority traffic may arrive at the NW side during non-active duration. Similar to exceptional SR, it is necessary to introduce a mechanism to allow NW to transmit high priority traffic to the UE during non-active duration of Cell DTX.

Observation 3: Non-active duration of Cell DTX is expected to be long to achieve sufficient NES gain. However, inactivity timer in UE CDRX can only work when the UE receives PDCCH around the boundary of active duration. If high priority data arrives at NW side in middle of non-active duration, NW can't reach to the UE.

Observation 4: According to TS 38.321, when configuredGrantTimer expires, the UE assumes positive acknowledge for the corresponding HARQ process.

Observation 5: If the requirement for NW is agreed, one followed issue is that the NW may be forced to immediately reconfigure multiple UE CDRX upon Cell DTX is activated, to ensure each UE's CDRX on-duration within on-duration of Cell DTX. It is unnecessarily complex and incurs high signalling overhead.

Observation 6: Although SSB transmission is not impacted by Cell DTX / DRX, whether the UE can stop or relax measurement during non-active duration of Cell DTX/DRX need to be first discussed in RAN4/RAN1.

Observation 7: For CSI-RS/TRS/PRS, whether they can be turned off and their measurement requirement during non-active duration need to be first discussed in RAN4/RAN1.

Based on observations, our proposals can be found below.

Open issues of signaling

Proposal 1: Reuse the below formula of UE CDRX to calculate starting time of active duration of Cell DTX / DRX pattern:

- [(SFN × 10) + subframe number] modulo (Periodicity) = StartOffset

Proposal 2: RAN2 confirm the understanding: when Cell DTX and Cell DRX are jointly operated, only single pattern (i.e. one set of periodicity, offset and on duration) can be activated for one serving cell.

Open issues of UE behavior

Proposal 3: Introduce a mechanism that NW is allowed to transmit high priority traffic to one particular UE during non-active duration of Cell DTX. FFS detailed mechanism.

Proposal 4: RAN2 discuss whether NW can page a NES UE in RRC_CONNECTED state if high priority traffic towards this UE arrives during non-active duration of Cell DTX.

Proposal 5: When UE performs new PDSCH/PUSCH transmission during non-active duration of Cell DTX/DRX (e.g. PUSCH associated with exceptional SR), retransmission is allowed and UE CDRX retransmission timer based mechanism (i.e. HARQ RTT timer and retransmission timer) is baseline.

Proposal 6: Upon entering non-active periods of Cell DRX, the UE stops all the running configuredGrantTimer.

Alignment between UE CDRX and Cell DTX/DRX

Proposal 7: As long as UE behavior in all possible overlapping duration is clear, it is unnecessary to introduce NW alignment requirement between Cell DTX and UE CDRX, i.e. it is up to NW implementation to choose appropriate configurations of UE CDRX and Cell DTX.

Proposal 8: RAN2 agree below UE/gNB behavior in T1 and T3 durations:

- T1 (OFF duration of UE CDRX is overlapped with active duration of Cell DTX): the UE follows UE CDRX behaviour (i.e. stop monitoring PDCCH), assuming no DCI for itself.

- T3 (OFF duration of UE CDRX is overlapped with non-active duration of Cell DTX): the UE follows behaviour specified for Cell DTX.

Measurement and reporting

Proposal 9: In order to start RRM/RLM/BFR work, RAN2 send LS to RAN1/RAN4 on whether any change on requirements of measurement and reporting based on SSB/CSI-RS/TRS/PRS.

# 4 References

[1] RP-223540, New WID: Network energy savings for NR, Huawei

[2] RAN2#121, Chair Notes

[3] RAN2#121b-e, Chair Notes

[4] R2-2305081, Support high priority traffic in Cell DTX / DRX, Apple, InterDigital, T-Mobile USA, MediaTek Inc., Intel.

[5] TS 38.321-v17.1.0, Medium Access Control (MAC) protocol specification, 2022-6.

[6] R2-2302796, Outcome of [POST121][312][NES] DTX/DRX - Configuration/ activation/ deactivation and alignment (Huawei), Huawei, HiSilicon.

[7] TR 38.864-v1.0.0, Study on Network energy savings for NR, 2022-12.