# 3GPP TSG RAN WG2 Meeting #122      	                           R2-2305083
Incheon, Korea, 22th – 26th May 2023

Agenda item:	7.3.3

Source:	Apple

Title:	Discussion on RAN2 work of inter-band SSB-less CA

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

The Network Energy saving (NES) WID RP-223540 was agreed in RAN#98-e [1], the WI objective on inter-band SSB-less CA is copied below:

In RAN4#106b-e, inter-band SSB-less CA was discussed and the WF has been agreed in [2]. However, RAN2 has not discussed this topic yet.

In this contribution, we share our view on RAN2 work of inter-band SSB-less CA in below aspects:

- RAN2 spec impacts on signaling

- L1 / L3 measurement and SCell activation procedure

# 2 Discussion

## 2.1 RAN2 spec impacts on signalling

Intra-band SSB-less CA was specified in NR Rel-15. RAN1/RAN4 first discussed and concluded its timing issue, and then RAN2 introduced below 2 specification changes:

- TS 38.331: In field description of IE absoluteFrequencySSB, capture RAN1/RAN4 conclusion that timing reference of SSB-less SCell is from the SpCell or an SCell of its serving cell group.

- TS 38.306: Specified UE capability of intra-band SSB-less CA.

Observation 1: RAN2 specification impacts on intra-band SSB-less CA include capturing timing assumption in TS 38.331 and UE capability in TS 38.306.

For inter-band SSB-less CA with restriction of FR1 and co-located cells, we think that RAN2 should first wait whether RAN4 conclude it is feasible. If RAN4 conclude it is feasible, we assume that the above 2 specification changes introduced for intra-band SSB-less CA can be reused for inter-band SSS-less CA. However, considering inter-band CA has some differences from intra-band CA, we don't think RAN2 can preclude new signaling if RAN4 conclude its necessity. For example, RAN4 may discuss below issues:

- Issue 1: whether TRS is necessary for FR1 SSB-less CA

  - RAN4 has discussed this issue, but no conclusion was made yet, as captured in below agreement [2].

Online session (Friday April 21, 2023) - Agreements - The following scenarios are considered for inter-band SSB-less SCell requirements definition - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell. - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

- Issue 2: whether gNB can indicate which serving cell to use as timing reference

  - For example, if PCell and SSB-less cell are inter-band CA while another SCell with SSB and SSB-less cell are intra-band CA, the UE may be confused whether PCell or SCell with SSB is its timing reference.

- Issue 3: whether TAE from gNB is needed

  - Please note that in intra-band CA, the timing alignment error (TAE) is always smaller than 0.5 CP. Thus, gNB doesn't need to indicate TAE in case of intra-band SSB-less CA. But TAE of inter-band CA may be larger than 0.5CP, even if it is co-located. Thus, further RAN4 discussion is required.

Observation 2: Considering inter-band CA has some differences from intra-band CA, RAN2 can't preclude new signaling if RAN4 conclude its necessity.

Based on above analysis, we propose that RAN2 can take signaling of intra-band SSB-less CA as baseline, but it doesn't preclude the possibility that RAN4 conclude new signaling is required.

Proposal 1: If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

## 2.2 L1/L3 measurement and SCell activation procedure

According to WID objective, L1/L3 measurement and SCell activation procedure are in scope. Among them, L3 measurement and SCell activation procedure may have some RAN2 impacts. However, our understanding is that this part was added in WID objective because it is not clear whether the existing measurement solution of intra-band SSB-less CA can be reused. Specifically, in existing intra-band SSB-less CA, the SSB measurement in PCell is reused for SSB-less SCell. Then, RAN4 need to discuss whether SSB measurement in PCell can still be reused in inter-band SSB-less-less SCell. If not, maybe CSI-RS based measurement for SSB-less SCell needs to be introduced. In this case, the SCell activation / deactivation procedure in TS 38.321 may need some spec changes.

Observation 3: L1/L3 measurement and SCell activation were added in WID objective because it is not clear whether the existing measurement solution of intra-band SSB-less CA (i.e. the SSB measurement in PCell is reused for SSB-less SCel) can be reused.

Meanwhile, please note that RAN4 has discussed this issue and agreed to further study different options for L1 and L3 measurement in [2]:

- Candidate Options for L1 measurement: - Option 1: No SSB but with CSI-RS resource for L1 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L1 measurement on the inter-band SSB-less SCell

- Candidate Options for L3 measurement: - Option 1: No SSB but with CSI-RS resource for L3 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L3 measurement on the inter-band SSB-less SCell

RAN4 also discussed latency requirement of SCell activation procedure, but no consensus was made. [2] captured two options to further study this issue and agreed to further study different options for L1 and L3 measurement in [2]. Thus, RAN2 has to wait RAN4 on the requirement.

- Candidate Options of requirements of FR1 SSB-less SCell activation: - Option 1(Huawei, CTC, ZTE): For FR1 co-located SSB-less inter-band CA, when the frequency and time synchronization information of active serving cell on FR1 inter-band is able to be applied for to-be-activated SCell, and the power difference is not larger than [TBD]dB, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms). - Option 2 (Vivo): If an SSB-less SCell is configured with CSI-RS based L3 measurement, and UE has reported one MR within a predefined period, the SSB-less SCell is known - The activation delay of a known SSB-less SCell is TfirstTRS + 5ms or TfirstA-TRS + 5ms. Otherwise, the SSB-less SCell is unknown - The activation delay of an unknow SSB-less SCell is TfirstTRS + TTRS + 5ms or TFirstATRS + Tgap + TATRS + 5ms

Observation 4: RAN4 discussed requirements of FR1 SSB-less SCell activation and how L1/L3 measurement is performed. But no consensus is achieved yet.

Based on above analysis, we think whether introducing new L1 / L3 measurement and SCell activation procedure depends on RAN4 conclusion. Thus, RAN2 should not discuss this part until RAN4 provide more input.

Proposal 2: Before RAN4 provide sufficient input, RAN2 do not start discussion on enhancement of L1 / L3 measurement and SCell activation procedures.

# 3 Conclusion

In this contribution, we share our view on RAN2 work of inter-band SSB-less CA. Our observations are:

Observation 1: RAN2 specification impacts on intra-band SSB-less CA include capturing timing assumption in TS 38.331 and UE capability in TS 38.306.

Observation 2: Considering inter-band CA has some differences from intra-band CA, RAN2 can't preclude new signaling if RAN4 conclude its necessity.

Observation 3: L1/L3 measurement and SCell activation were added in WID objective because it is not clear whether the existing measurement solution of intra-band SSB-less CA (i.e. the SSB measurement in PCell is reused for SSB-less SCel) can be reused.

Observation 4: RAN4 discussed requirements of FR1 SSB-less SCell activation and how L1/L3 measurement is performed. But no consensus is achieved yet.

Based on observations, our proposals can be found below.

Proposal 1: If RAN4 conclude SSB-less SCell for inter-band CA for FR1 and co-located cells is feasible, the signaling of intra-band CA (including RRC change on timing of SSB-less SCell and capability signaling) can be considered as its baseline. Whether other new signaling is required depends on RAN4 input.

Proposal 2: Before RAN4 provide sufficient input, RAN2 do not start discussion on enhancement of L1/ L3 measurement and SCell activation procedures.

# 4 References

[1] RP-223540, New WID: Network energy savings for NR, Huawei

[2] R4-2306369, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN4 #106-bis-e