# 3GPP TSG RAN WG2 Meeting #122      	                           R2-2305084
Incheon, Korea, 22th – 26th May 2023

Agenda item:	7.3.5

Source:	Apple

Title:	Discussion on CHO enhancement in NES

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

- Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]

The Network Energy saving (NES) WID RP-223540 was agreed in RAN#98-e [1], the WI objective on CHO enhancement for NES is copied below:

In RAN2#121b-e [2], RAN2 agreed to make enhancement in CHO based on source cell entering NES mode.

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

In this contribution, we share our view on how to enhance CHO procedure based on source cell's NES mode. And example of RRC changes is provided in Appendix.

# 2 Discussion

We first provide a brief overview on Rel-16 / Rel-17 CHO. Then, we discuss expected UE behavior upon source cell entering "NES mode". Finally, we discuss the detailed signaling options to indicate source cell entering "NES mode".

## 2.1 Brief overview on Rel-16/Rel-17 CHO

CHO was introduced in Rel-16 [3]. The basic RRC signaling and procedure can be found in Figure.1 and Figure. 2.

Figure.1 Illustration of legacy CHO RRC signaling

Figure.2 Illustration of legacy CHO procedure

In RAN2#121b-e [2], it seems there were some confusion between "CHO evaluation" and "CHO execution". We clarify their difference.

- CHO evaluation:

  - The UE starts CHO condition evaluation (i.e. conExecutionCond) upon reception of CHO command.

    - Up to 2 MeasIDs can be configured to associate with one conExecutionCond, where one MeasID is mapped to one MO and one event (A3/A5/A4/T1/D1).

    - Event T1/D1 (time/location based event) is always configured together with event A3/A4/A5 (measurement based event).

  - The UE will consider one target cell as a "triggered cell" when:

    - If only one MeasId is configured and when the entry condition is fulfilled.

    - If two MeasIDs are configured, when the entry conditions for the both events are fulfilled at the same time.

- CHO execution:

  - The UE starts CHO execution as long as one target cell is "triggered cell":

    - If there is only one triggered cell, UE applies the stored handover command immediately.

    - If more than one triggered cells, it is up to UE implementation to select the final target cell.

## 2.2 Expected UE behavior upon source cell entering "NES mode"

In RAN2#121b-e [1], below agreements were made:

Agreements

- RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

- For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

- As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

2nd and 3rd agreements mean that the timing to start CHO evaluation is same as legacy (i.e. upon reception of CHO command). However, it is not clear whether the UE can execute CHO if there is at least one "triggered cell" before source cell entering "NES mode". In our understanding, this case may happen when the UE moves to the source cell boundary. Thus, the UE shall follow legacy CHO procedure to execute CHO.

Observation 1: Before source cell entering "NES mode", the UE may move to the source cell boundary, and correspondingly one candidate target cell may satisfy CHO condition (i.e. become "triggered cell").

Proposal 1: Before source cell entering "NES mode", if there are at least one candidate target cell is triggered cell(s), the UE shall follow legacy CHO procedure to execute CHO.

Since the CHO evaluation follows legacy behavior, our understanding on 1st agreement is that RAN2 will do some enhancement on CHO execution when no triggered cell is available by the time source cell entering "NES mode". Specifically, below two cases can be considered:

- Case 1): source cell will be turned off

  - In this case, source cell will not serve any UE after it is turned off. Thus, the NES capable UE(s) served by source cell are expected to select one candidate target cell to execute CHO, even if none of them are triggered cell.

- Case 2): source cell will activate Cell DTX / DRX

  - In this case, the UE's QoS will be degraded after source cell activates Cell DTX / DRX. Thus, if none of candidate target cell are triggered cell, the NES capable UE(s) can still stay in source cell.

We think RAN2 should confirm the understanding:

Proposal 2: RAN2 confirm to enhance CHO execution when no triggered cell is available by the time source cell entering below 2 cases:

- Case 1): source cell will be turned off. Afterwards, NES capable UE(s) can't be served by source cell

- Case 2): source cell will activate Cell DTX/DRX. Afterwards, QoS of NES capable UE(s) are degraded

Then, we discuss expected UE behavior in above 2 cases. According to company contributions submitted in RAN2#121b-e [2], the proposals include:

- Alt-1: The UE evaluates candidate target cells with a CondEvent Ax which is applied only after source cell is turned off or activates Cell DTX / DRX.

- Alt-2: The UE immediately executes CHO to the candidate target cell with best RSRP.

- Alt-3: The UE reports to NW that no triggered cell is available.

For Alt-2 and Alt-3, we think they have below issues:

- Issue of Alt-2: it can only work for Case 1) (i.e. cell turning off) and such mandating execution may result in RLF.

  - The candidate cell with best RSRP may still have poor radio condition, so mandating UE to choose such cell will result in RLF.

  - If source cell just activates Cell DTX / DRX, the UE can still stay in source cell if no candidate cell with good radio condition is available.

- Issue of Alt-3: The NW can know "no triggered cell is available" without spec change

  - According to legacy CHO procedure, the UE shall execute CHO if triggered cell is available. Thus,  source cell can know the situation if the UE doesn't execute CHO.

  - In legacy CHO, source cell can still configure measurements towards candidate target cells after CHO is configured. Therefore, based on UE measurement reporting, the source cell can always know the radio condition of candidate cells.

Observation 2: For the solution of mandating the UE to immediately execute CHO to the candidate target cell with best RSRP, it may result in RLF if the candidate cell with best RSRP may still have poor radio condition.

Observation 3: No need to introduce reporting "no triggered cell available" to source cell because the NW can know the situation without spec change (e.g. by checking whether UE execution of CHO or UE's RRM reports).

For Alt-1, we think it is flexible enough to work for both Case 1) and Case 2). For example:

- For Case 1 (source cell turning off): Delayed CondEvent Ax is configured to use A4 event, so that only RSRP/RSRQ of candidate target cell is evaluated (i.e. no need to care serving cell radio quality).

- For Case 2 (source cell activates Cell DTX/DRX): Both normal and delayed CondEvent Ax are configured to use A5 event, but looser threshold is configured in delayed CondEvent Ax.

Observation 4: Alt-1 is flexible enough to cover both Case 1) and Case 2). For example:

- For Case 1 (source cell turning off): Delayed CondEvent Ax is configured to use A4 event, so that only RSRP/RSRQ of candidate target cell is evaluated (i.e. no need to care serving cell radio quality).

- For Case 2 (source cell activates Cell DTX/DRX): Both normal and delayed CondEvent Ax are configured to use A5 event, but looser threshold is configured in delayed CondEvent Ax.

Thus, we propose to introduce Alt-1.

Proposal 3: If no triggered cell is available by the time source cell entering "NES mode", the UE evaluates candidate target cells with a delayed CondEvent Ax, which may be a CondEvent A3/A5 with a looser threshold or a CondEvent A4 which only evaluates radio condition of neighbor cell.

We provide an example in Figure 3 to illustrate the UE behavior changes.

Figure.3 Procedure illustration of NES CHO enhancement (specification changes are highlighted)

## 2.3 Signaling to indicate source cell entering "NES mode"

A followed question is how the UE can detect that source cell enters "NES mode". According to company contributions submitted in RAN2#121b-e [2], the proposals include:

- Alt-1: via CondEvent T1 (assuming the source cell is planned to turn off or activate Cell DTX / DRX when its entry condition is fulfilled)

- Alt-2: via reception of a common L1/L2 signaling from gNB

- Alt-3: via indication in SIB

In Table. 1, we provide our analysis of their Pros / Cons and potential specification impacts.

|  | Pros | Cons | Protentional spec impact |
| --- | --- | --- | --- |
| Alt-1: via CondEvent T1 | Reuse existing mechanism. | 1. Require the predictability on when the source cell enters "NES mode" 2. Spec changes are needed to resolve below issues: - The NES UE may not be equipped with GNSS. So, it may not know UTC time. - In TS 38.331, only when both CondT1 and CondAx are fulfilled at the same time, UE will consider the target candidate cell as a triggered cell (i.e. AND between CondT1 and Cond Ax). - In NES CHO enhancement, it should be "OR" between CondT1 and CondAx | - Specification change in TS 38.306 to allow NES UE to apply CondT1. - Specification change in TS 38.331 to allow NES UE without equipping GNSS to apply CondT1. - For example, CondT1 can be revised to use time of source cell for NES UEs. - As another example, add procedure text that after configured with CondT1, NES UEs need to obtain SIB9 to acquire UTC time. - Specification change in TS 38.331 to use "OR" (rather than "AND") between CondT1 and CondAx to determine whether the target candidate cell as a triggered cell. - Specification change in TS 38.331 to allow up to 3 MeasID configured for one conExecutionCond. |
| Alt-2: via L1/L2 signaling | Low signaling overhead because it is cell / group common. | 1. Cross-WG efforts with RAN1 for common signaling design. 2. Security concern because L1/L2 are not security protected. | - Specification change in TS 38.331 to add indication for delayed CondEvent Ax. - Specification change in TS 38.331 to add UE behavior on evaluation of delayed CondEvent Ax. - Common L1 signaling design with RAN1. |
| Alt-3: via indication in SIB | Low signaling overhead because it is cell common. | 1. Change of the indication needs to wait SIB modification period. | - Specification change in TS 38.331 to add indication for delayed CondEvent Ax. - Specification change in TS 38.331 to add UE behavior on evaluation of delayed CondEvent Ax. |

Table. 1 Comparison between different signaling options to detect source cell entering NES mode

We suggest RAN2 to further analyze the 3 alternatives with Table. 1 as starting point.

Proposal 4: RAN2 discuss the following 3 signaling alternatives to detect the source cell enters NES mode with Table.1 as starting point.

- Alt-1: via CondEvent T1

- Alt-2: via reception of a common L1/L2 signaling from gNB

- Alt-3: via indication in SIB

Finally, we take Alt-2 as example to illustrate the potential RRC spec changes in Appendix.

# 3 Conclusion

In this contribution, we share our view on CHO procedure enhancement(s) based on source cell NES mode. Our observations are:

Observation 1: Before source cell entering "NES mode", the UE may move to the source cell boundary, and correspondingly one candidate target cell may satisfy CHO condition (i.e. become "triggered cell").

Observation 2: For the solution of mandating the UE to immediately execute CHO to the candidate target cell with best RSRP, it may result in RLF if the candidate cell with best RSRP may still have poor radio condition.

Observation 3: No need to introduce reporting "no triggered cell available" to source cell because the NW can know the situation without spec change (e.g. by checking whether UE execution of CHO or UE's RRM reports).

Observation 4: Alt-1 is flexible enough to cover both Case 1) and Case 2). For example:

- For Case 1 (source cell turning off): Delayed CondEvent Ax is configured to use A4 event, so that only RSRP/RSRQ of candidate target cell is evaluated (i.e. no need to care serving cell radio quality).

- For Case 2 (source cell activates Cell DTX/DRX): Both normal and delayed CondEvent Ax are configured to use A5 event, but looser threshold is configured in delayed CondEvent Ax.

Based on observations, our proposals can be found below. We also provide example of RRC spec changes in Appendix.

Proposal 1: Before source cell entering "NES mode", if there are at least one candidate target cell is triggered cell(s), the UE shall follow legacy CHO procedure to execute CHO.

Proposal 2: RAN2 confirm to enhance CHO execution when no triggered cell is available by the time source cell entering below 2 cases:

- Case 1): source cell will be turned off. Afterwards, NES capable UE(s) can't be served by source cell

- Case 2): source cell will activate Cell DTX/DRX. Afterwards, QoS of NES capable UE(s) are degraded

Proposal 3: If no triggered cell is available by the time source cell entering "NES mode", the UE re-evaluates candidate target cells with a delayed CondEvent Ax, which may be a CondEvent A3/A5 with a looser threshold or a CondEvent A4 which only evaluates radio condition of neighbor cell.

Proposal 4: RAN2 discuss the following 3 signaling alternatives to detect the source cell enters NES mode with Table.1 as starting point.

- Alt-1: via CondEvent T1

- Alt-2: via reception of a common L1/L2 signaling from gNB

- Alt-3: via indication in SIB

# 4 References

[1] RP-223540, New WID: Network energy savings for NR, Huawei

[2] RAN2#121b-e, Chair Notes

# Appendix (example of RRC spec change with Alt-2 in P4)

## Change in section 6.3.2 of TS 38.331

#### –	CondReconfigToAddModList

The IE CondReconfigToAddModList concerns a list of conditional reconfigurations to add or modify, with for each entry the condReconfigId and the associated condExecutionCond/condExecutionCondSCG and condRRCReconfig.

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]]

[[

DelayedcondExecutionCond-r18    INTEGER(1..2)                                             OPTIONAL,    -- Need M

]]

}

CondReconfigExecCondSCG-r17 ::=  SEQUENCE (SIZE (1..2)) OF MeasId

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

CondReconfigToAddMod field descriptions

condExecutionCond The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for CHO, CPA, intra-SN CPC without MN involvement or MN initiated inter-SN CPC. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. For CHO, if network configures condEventD1 or condEventT1 for a candidate cell network configures a second triggering event condEventA3, condEventA4 or condEventA5 for the same candidate cell. Network does not configure both condEventD1 and condEventT1 for the same candidate cell.

condExecutionCondSCG Contains execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration for SN initiated inter-SN CPC. The Meas Ids refer to the measConfig associated with the SCG. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject. For each condReconfigurationId, the network always configures either condExecutionCond or condExecutionCondSCG (not both).

condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config. For CPA and for CPC, the RRCReconfiguration message contained in condRRCReconfig cannot contain the field scg-State.

DelayedcondExecutionCond To indicate Meas Id whose associated execution condition is applied after reception of common L1 signaling. This field is presence only when configuring 2 triggering events (Meas Ids) condEventA3, condEventA4 or condEventA5 for a candidate cell.

| Conditional Presence | Explanation |
| --- | --- |
| condReconfigAdd | The field is mandatory present when a condReconfigId is being added. Otherwise the field is optional, need M. |

## Change in section 5.3.5.13.4 of TS 38.331

##### 5.3.5.13.4	Conditional reconfiguration evaluation

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	if the RRCReconfiguration within condRRCReconfig includes the masterCellGroup including the reconfigurationWithSync:

3>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell;

2>	else if the RRCReconfiguration within condRRCReconfig includes the secondaryCellGroup including the reconfigurationWithSync:

3>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the received condRRCReconfig to be applicable cell;

2>	if condExecutionCondSCG is configured:

3>	in the remainder of the procedure, consider each measId indicated in the condExecutionCondSCG as a measId in the VarMeasConfig associated with the SCG measConfig;

2>	if condExecutionCond is configured:

3>	if it is configured via SRB3 or configured within nr-SCG or within nr-SecondaryCellGroupConfig (specified in TS 36.331[10]) via SRB1:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCond as a measId in the VarMeasConfig associated with the SCG measConfig;

3>	else:

4>	in the remainder of the procedure, consider each measId indicated in the condExecutionCond as a measId in the VarMeasConfig associated with the MCG measConfig;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the condExecutionCond or condExecutionCondSCG associated to condReconfigId:

3>	if the condEventId is associated with condEventT1, and if the entry condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

3> if the condEventId is associated with condEventD1, and if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

3>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4> consider the event associated to that measId to be fulfilled;

3>	if the measId for this event associated with the condReconfigId has been modified; or

3>	if the condEventId is associated with condEventT1, and if the leaving condition applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cell; or

3> if the condEventId is associated with condEventD1, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig; or

3>	if the condEventId is associated with condEventA3, condEventA4 or condEventA5, and if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4> consider the event associated to that measId to be not fulfilled;

2> if one event within condTriggerConfig is configured with delayedcondExecutionCond for a target candidate cell within the stored condRRCReconfig:

3> if the L1 trigger signaling is received and the event configured with delayedcondExecutionCond is fulfilled, or

3> if the L1 trigger signaling is not received and the other event within condTriggerConfig is fulfilled:

4> consider the target candidate cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

4> initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

2>	if event(s) associated to all measId(s) within condTriggerConfig for a target candidate cell within the stored condRRCReconfig are fulfilled:

3>	consider the target candidate cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

3>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

NOTE 1:	Up to 2 MeasId can be configured for each condReconfigId. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold.

NOTE 2:	If multiple NR cells are triggered in conditional reconfiguration execution, it is up to UE implementation which one to select, e.g. the UE considers beams and beam quality to select one of the triggered cells for execution.