# 3GPP TSG RAN WG2 Meeting #122      	                           R2-230xxxx
Incheon, Korea, 22th – 26th May 2023

Agenda item:	7.16.2.2

Source:	Apple

Title:	Further discussion on data collection for AI/ML

WID/SID:	FS_NR_AIML_air – Release 18

Document for:	Discussion and Decision

# 1 Introduction

SID of AI/ML for NR air interface (RP-213599) was agreed in RAN#94e [1]. After several rounds of discussion, RAN2 scope mainly include AI/ML model identification, signaling of AI/ML model transfer / delivery, and procedure of LCM and data collection.

In RAN2#121 [2] data collection was discussed, and below agreements were made.

R2-2302286 	Summary of [AT121][025]: Progress table of analyzing data collection framework (Apple)	Apple

Endorse the table as a starting point (e.g. can add more columns if needed later, modify, add rows etc). Content shall be interpreted as current content.

Chair: There is significant support to aim for evaluating the data collection methods per LCM purpose

- In RAN2#121b-e [3], data collection was further discussed with below agreements.

P1: RAN2 to understand/determine/capture requirements of data collection for the LCM functionalities and document the results. FFS on the exact presentation format. Expect RAN1 to provide some related information.

P2: RAN2 to capture the analysis (see P1 above) separately for the use-cases, i.e., CSI feedback enhancement, beam management and positioning enhancement.  FFS how we do the formatting/presentation of the results.

P3: Study the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes, i.e., inference, monitoring and (offline) training. FFS how we do the formatting/presentation of the results.

P4: With more progress on architectural discussion, consider the suitability of each identified data collection framework for the termination points and mapping with the location of LCM purposes/functions (inference, monitoring, (offline) training)

- - Model sidedness (UE side, NW side, two sided) FFS

- - Use case mapping FFS

P5: RAN2 to modify the previously endorsed table by adding 3 additional columns: inference; monitoring and (offline) training. Whether to, and how to further restructure the table is FFS.

Observation: RAN2 may need to consider enhancements for AIML to existing functionality for data collection, e.g. for timing control (e.g. for MDT/RRM).

In this contribution, we further discuss data collection for AI/ML from below aspects:

- Further analysis of data collection framework

- Update data collection analysis table

# 2 Discussion

## 2.1 Further analysis of data collection framework

In RAN2#121b-e [3], it was agreed to evaluate data collection framework per LCM purpose:

P3: Study the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes, i.e., inference, monitoring and (offline) training. FFS how we do the formatting/presentation of the results.

In our understanding, RAN2 can first analyze data collection method for offline training with assumption that there is no strict latency requirement to collect data.

Proposal 1: RAN2 first analyze data collection method for offline model training with assumption that there is no strict latency requirement to collect data.

For other LCM (e.g. model inference / selection / monitor / switch / update / activation / deactivation), we think their evaluations can be done only after offline training because they highly depend on specific RAN1 requirements. Meanwhile, we also noticed that RAN1#112b-e [4] also discussed data collection signaling framework at least for inference/monitoring purpose:

Agreement

The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on

- CSI-RS configurations (No discussion on CSI-RS pattern design enhancements)

- CSI reporting configurations

- CSI report UCI mapping/priority/omission

- CSI processing procedures.

- Other aspects are not precluded.

Agreement

In CSI compression using two-sided model use case, for UE-side monitoring, further study potential specification impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE.

Working Assumption

Regarding data collection at least for model training for AI/ML based positioning, at least the following information of data with potential specification impact are identified.

- Ground truth label

  - At least for model training

  - Report from the label data generation entity

- Measurement (corresponding to model input)

  - At least for model training

  - Report from the measurement data generation entity

- Quality indicator

  - For and/or associated with ground truth label and/or measurement at least for model training

  - Report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity

- RS configuration(s)

  - At least for deriving measurement

  - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signaling to UE/PRU/TRP

  - Note1: there may not be any enhancements on top of existing RS configuration(s) or any new RS configuration(s) for positioning measurement

- Time stamp

  - At least for and/or associated with training data for model training

    - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities

  - Report from data generation entity together with training data and/or as LMF assistance signaling

  - Note2: there may not be any enhancements on top of time stamp in existing positioning measurement report or any new time stamp report for positioning measurement

- FFS other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) for data collection

- Note3: whether the above information can be applied to other aspects of AI/ML LCM (e.g., updating, monitoring, etc.) can also be discussed

- Note4: transfer of data from the entity generating data to a different entity is not precluded from RAN1 perspective

Observation 1: RAN1#112b-e has discussed data collection signaling framework at least for inference/monitoring purpose in AI/ML based CSI compression and AI/ML based positioning.

Given RAN1 has discussed data collection signaling framework for monitoring/inference purpose, we think it is better to leave this study to RAN1 because its signaling design highly depends on specific requirement in PHY.

Proposal 2: Given RAN1 has discussed data collection signaling framework for monitoring/inference, RAN2 leave this study to RAN1 because its signaling design highly depends on specific requirement in PHY.

Then, according to the endorsed data collection analysis table in [5], we think MDT framework can be prioritized for offline model training. Our justifications are:

- The data type collected by MDT is aligned with the studied 3 use cases in Rel-18 AI/ML for air interface (i.e. CSI feedback, beam management and positioning).

    - The data collected by MDT are mainly various measurements in AS layer (including L3 cell/beam measurements, location info, and sensor info).

    - Although RAN1 has not agreed details of what data to collect for the 3 use cases, they are expected to be also measurements in AS layer.

- Satisfy both security and UE privacy requirement

    - MDT measurements are reported via RRC message can be only sent from the UE after successful AS security activation.

    - User consent is required before NW configures the UE to perform logged MDT and immediate MDT.

- The data collected by MDT is visible to both RAN (e.g. for CSI and BM) and CN (e.g. for positioning)

    - The MDT data is collected in TCE/OAM, and it can also be utilized by RAN according to TS 37.320.

- Data in all RRC states can be collected in MDT

    - Immediate MDT to collect data of UEs in CONNECTED state. Logged MDT to collect data of UEs in IDLE/INACTVE state

Proposal 3: MDT framework is prioritized to study for data collect in offline training due to below reasons:

- The data type collected by MDT (i.e. various measurements in AS layer) is aligned with the studied 3 use cases in Rel-18 AI/ML for air interface (i.e. CSI feedback, beam management and positioning).

- Satisfy both security and UE privacy requirement

- The data collected by MDT is visible to both RAN (e.g. for CSI and BM) and CN (e.g. for positioning)

- Data in all RRC states can be collected in MDT

## 2.2 Update data collection analysis table

In RAN2#121b-e [3], it was agreed to add 3 additional columns on inference, monitoring and offline training for analysis requirements.

P3: Study the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes, i.e., inference, monitoring and (offline) training. FFS how we do the formatting/presentation of the results.

P5: RAN2 to modify the previously endorsed table by adding 3 additional columns: inference; monitoring and (offline) training. Whether to, and how to further restructure the table is FFS.

We provide our view in Table 1 on expected requirements.

|  | CSI | BM | Positioning |
| --- | --- | --- | --- |
| Offline Training | - High payload size - No strict latency requirement | - High payload size - No strict latency requirement | - High payload size - No strict latency requirement - Between different data generation entities (UE, PRU/TRP, LMF) |
| Monitoring | - Small payload size - High latency requirement - Between UE and gNB | - Small/medium payload size - High latency requirement - Between UE and gNB | - Medium payload size - High latency requirement - Between different data generation entities (UE, PRU/TRP, LMF) |
| Inference | - Small payload size - High latency requirement - Expected to more strict than inference - Between UE and gNB | - Small/medium payload size - High latency requirement - Expected to more strict than inference - Between UE and gNB | - Medium payload size - High latency requirement - Expected to more strict than inference - Between different data generation entities (UE, PRU/TRP, LMF) |

Table. 1 Expected data collection requirements for inference, monitoring and offline training

We propose to agree Table.1 as starting point.

Proposal 4: RAN2 agree Table. 1 as starting point of data collection “expected requirements” for training/inference/monitoring.

Finally, we think RAN2 can send LS to RAN1 on data collection requirement metrics including at least payload size, latency requirement and between which entities.

Proposal 5: RAN2 send LS to RAN1 on data collection requirement metrics including at least payload size, latency requirement and between which entities.

# 3 Conclusion

In this contribution, we share our views on data collection for Rel-18 AI/ML. Our observations are:

Observation 1: RAN1#112b-e has discussed data collection signaling framework at least for inference/monitoring purpose in AI/ML based CSI compression and AI/ML based positioning.

Based on observations, our proposals are:

Proposal 1: RAN2 first analyze data collection method for offline model training with assumption that there is no strict latency requirement to collect data.

Proposal 2: Given RAN1 has discussed data collection signaling framework for monitoring/inference, RAN2 leave this study to RAN1 because its signaling design highly depends on specific requirement in PHY.

Proposal 3: MDT framework is prioritized to study for data collect in offline training due to below reasons:

- The data type collected by MDT (i.e. various measurements in AS layer) is aligned with the studied 3 use cases in Rel-18 AI/ML for air interface (i.e. CSI feedback, beam management and positioning).

- Satisfy both security and UE privacy requirement

- The data collected by MDT is visible to both RAN (e.g. for CSI and BM) and CN (e.g. for positioning)

- Data in all RRC states can be collected in MDT

Proposal 4: RAN2 agree Table. 1 as starting point of data collection “expected requirements” for training/inference/monitoring.

Proposal 5: RAN2 send LS to RAN1 on data collection requirement metrics including at least payload size, latency requirement and between which entities.

# 4 References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN meeting #94, Electronic meeting, Dec 6-17, 2022

[2] RAN2#121, Chair Notes

[3] RAN2#121b-e, Chair Notes

[4] RAN1#112b-e, Chair Notes

[5] R2-2302286, Summary of [AT121][025]: Progress table of analyzing data collection framework (Apple), Apple