# 3GPP TSG RAN WG2 Meeting #122      	                           R2-2305089
Incheon, Korea, 22th – 26th May 2023

Agenda item:	7.15.2

Source:	Apple

Title:	Discussion on SL LCP and consistent LBT failure recovery

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#121b-e [4], RAN2 discussed LCP impact due to COT sharing. Below agreements were made:

Agreement:

If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact.

If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

RAN2 will wait for more conclusion from RAN1 on the assistance information for COT sharing.

Meanwhile, RAN2#121b-e [4] also discussed consistent LBT failure recovery, and below agreements were made:

Agreement

SL C-LBT failure is declared per RB-set

Confirm the following working assumption:

UE uses the MAC CE to report consistent LBT failure to the gNB

Uu MAC CE indicates RB set(s) where C-LBT failure happens.

Confirm the working assumption:

UE triggers SL RLF for all UC connections when UE has triggered consistent SL LBT failure in all RB sets.

In this contribution, we discuss remaining open issues on LCP impact due to COT sharing and consistent LBT failure recovery.

# 2 Discussion

## 2.1 LCP impacts due to COT sharing

According to below agreements made in RAN2#121b-e [4], our understanding is that at least a new destination selection procedure based on destination information in received COT sharing information will be specified. And RAN2 need to further discuss the condition when the new destination selection procedure is used and when the legacy priority based procedure is used.

Agreement:

If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact.

If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

So, basically it requires a different LCP procedure like: 1) There is a COT sharing (channel resource )  2) Select destination to be initiating UE  3) Multiplex LCH/MAC CEs which are of lower CPAC priority to be in MAC PDU  4) find resource (mode 2 RA or mode 1 grant).

Observation 1: A different LCP procedure is required: 1) There is a COT sharing (channel resource )  2) Select destination to be initiating UE  3) Multiplex LCH/MAC CEs which are of lower CPAC priority to be in MAC PDU  4) find resource (mode 2 RA or mode 1 grant).

Based on above analysis, we think above new LCP procedure requires below detailed specification changes:

- New Step 1) requires PHY layer of the responding UE to send the decoded COT sharing information (at least including initialling UE ID, remaining COT duration and CAPC value to acquire the resource) to its MAC layer.

- New Step 2) requires the MAC layer of the responding UE to select destination based on the COT sharing information (i.e. selecting initiating UE as destination) rather than based on logical channel priority as in legacy.

- New Step 3) requires the MAC layer of the responding UE to add a new LCP restriction for CAPC value.

- New Step 4) requires the Resource selection window to be restricted by remaining COT duration.

We think above 2)/3)/4) require MAC specification changes, and would like to discuss in detail one by one.

### 2.1.1 Enhanced destination selection and its condition

In existing TS 38.321 [3], the destination is selected based on priority of LCH and MAC-CE, as highlighted below:

###### 5.22.1.4.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	select a Destination associated to one of unicast, groupcast and broadcast, having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI:

2> SL data is available for transmission; and

2> SBj > 0, in case there is any logical channel having SBj > 0; and

2> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and

2> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and

2> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

NOTE 1: If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation.

Observation 2: In existing TS 38.321, the destination is selected based on priority of LCH and MAC-CE.

According to below agreement of RAN2#121b-e [4]:

If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

We think the enhanced destination selection means that the responding UE is allowed to select initiating UE as destination based on candidate destination information from COT sharing info (rather than priority).

Proposal 1: Enhanced destination selection procedure is that the responding UE is allowed to select initiating UE as destination based on candidate destination information from COT sharing info (rather than priority).

Then, regarding when to use the above enhanced destination selection procedure, we think it needs to at least satisfy below conditions:

- SL data or MAC-CE towards initiating UE is available for transmission;

- Remaining COT duration has available resource to complete the data transmission;

  - Especially in case that multiple MAC PDUs are required to send data for initiating UE, or multi-TTI scheduling is applied.

- SL data towards initiating UE has at least one LCH's CAPC value ≤ CAPC value indicated in COT information

Thus, we propose:

Proposal 2: The conditions to use the enhanced destination selection procedure include at least:

- SL data or MAC-CE towards initiating UE is available for transmission;

- Remaining COT duration has available resource to complete the data transmission;

- SL data towards initiating UE has at least one LCH's CAPC value ≤ CAPC value indicated in COT information.

Otherwise, the legacy destination selection procedure based on priority is applied

What's more, even if the responding UE satisfies all the above 3 conditions, it may also have SL data / MAC-CE towards other UEs with higher priority. This issue was also raised in RAN2#121 [1], but not concluded. Basically, there are two solutions:

- Solution 1: It is up to UE implementation whether to select initiating UE or the UE with highest priority SL data/MAC-CE

- Solution 2: Initiating UE is selected if highest priority of other UEs' data/MAC-CE is smaller than a threshold, similar to existing prioritization rule between UL and SL transmission

We suggest RAN2 to also discuss how to handle this case.

Proposal 3: RAN2 discuss how MAC layer of the responding UE handles the case that it has SL data / MAC-CE towards other UEs with higher priority than the SL data / MAC-CE towards initiating UE:

- Solution 1: It is up to UE implementation whether to select initiating UE or the UE with highest priority SL data/MAC-CE

- Solution 2: Initiating UE is selected if highest priority of other UEs' data/MAC-CE is smaller than a threshold, similar to existing prioritization rule between UL and SL transmission

### 2.1.2 LCP restriction

In existing TS 38.321 [3], the SL LCP restriction includes:

- Whether CG type 1 is allowed

- Allowed CG index associated to the SL grant

- HARQ feedback mode

Given the enhanced destination selection procedure, we think the new LCP restriction is straight forward. Specifically, the new SL LCP will restrict multiplexing any MAC SDU(s) of LCH(s) with CAPC value higher than the CAPC value indicated in the COT sharing information.

Proposal 4: If the initiating UE is selected as the destination, introduce a new SL LCP restriction that the responding UE shall not include any MAC SDU(s) of LCH(s) having CAPC value higher than the CAPC value indicated in the COT sharing information.

### 2.1.3 Impacts to resource selection window

According to existing TS 38.321 [3], the resource selection window is restricted by PDB of the pending SL data available in the LCH(s):

From Clause 5.22.1.1 of TS 38.321:

....

3>	if transmission based on random selection is configured by upper layers:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources pool, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

3>	else:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

....

Observation 3: According to existing TS 38.321, the resource selection window is restricted by PDB of the pending SL data available in the LCH(s).

If the initiating UE is selected as the destination, we think it is straight forward that the resource selection window needs to be further restricted by remaining COT duration.

Proposal 5: If the initiating UE is selected as the destination, MAC layer of the responding UE needs to further restrict the resource selection window within remaining COT duration besides PDB of the pending SL data available in the LCH(s).

## 2.2 Consistent LBT failure recovery

### 2.2.1 Counter/timer handling

In RAN2#119b-e [5], RAN2 agreed to reuse counter/timer mechanism of NR-U:

Agreements on timer/counter mechanism in RAN2#119b:

1:	As the general principle, reuse the consistent LBT failure detection procedure in NR-U as the baseline for SL-specific consistent LBT failure detection in SL-U.

2:	As in NR-U, introduce the following parameters and variables for the SL-specific consistent LBT failure detection in SL-U as the baseline:

- An SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER);

- An SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount);

- An SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer).

3:	Reuse the following MAC behaviors on TIMER/COUNTER handling in NR-U for SL-specific consistent LBT failure detection procedure in SL-U as the baseline:

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is incremented by one.

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, start or restart the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer)

- As in NR-U, if the SL-specific LBT failure indication counter value is equal to or larger than the SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount), consistent LBT failure is triggered/declared by the MAC entity.

- As in NR-U, if the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) expires, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

- As in NR-U, if the maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount) or SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) is reconfigured, SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

The following UE variable is used for the consistent LBT failure detection procedure:

-	LBT_COUNTER (per Serving Cell): counter for LBT failure indication which is initially set to 0.

For each activated Serving Cell configured with lbt-FailureRecoveryConfig, the MAC entity shall:

1>	if LBT failure indication has been received from lower layers:

2>	start or restart the lbt-FailureDetectionTimer;

2>	increment LBT_COUNTER by 1;

2>	if LBT_COUNTER >= lbt-FailureInstanceMaxCount:

3>	trigger consistent LBT failure for the active UL BWP in this Serving Cell;

3>	if this Serving Cell is the SpCell:

4>	if consistent LBT failure has been triggered in all UL BWPs configured with PRACH occasions on same carrier in this Serving Cell:

5>	indicate consistent LBT failure to upper layers.

4>	else:

5>	stop any ongoing Random Access procedure in this Serving Cell;

5>	switch the active UL BWP to an UL BWP, on same carrier in this Serving Cell, configured with PRACH occasion and for which consistent LBT failure has not been triggered;

5>	initiate a Random Access Procedure (as specified in clause 5.1.1).

1>	if all triggered consistent LBT failures are cancelled in this Serving Cell; or

1>	if the lbt-FailureDetectionTimer expires; or

1>	if lbt-FailureDetectionTimer or lbt-FailureInstanceMaxCount is reconfigured by upper layers:

2>	set LBT_COUNTER to 0.

However, the above agreement is just a baseline. During RAN2#119b discussion, one issue without consensus is the granularity of timer/counter (e.g. whether it is per BWP or per resource pool). In NR-U, Consistent LBT failure is detected per UL BWP but the timer/counter mechanism is running per serving cell as highlighted in below copy of TS 38.321 [3]. In other word, the LBT_COUNTER continues if gNB indicates BWP switch for one serving cell. The intention behind is that only one BWP can be activated at one time for each serving cell.

Observation 4: In NR-U, consistent LBT failure is detected per UL BWP but the timer/counter mechanism is running per serving cell because only one BWP can be activated for one serving cell at one time. In other word, the LBT_COUNTER continues if gNB indicates BWP switch.

For SL consistent LBT failure, RAN2 has agreed the granularity of LBT failure indication is per RB set. Therefore, the granularity of timer/counter can be two alternatives:

- Alt-1: per RB set

- Atl-2: per BWP/carrier, i.e. the counter continues when RB set is changed.

Between them, we prefer Alt-2 because of below two considerations:

- It is simple to maintain just one set of counter/timer per BWP/carrier rather than multiple sets

- In one SL carrier, only one resource / resource pool can be used by the TX UE at one time point, which is similar to NR-U that only one BWP can be activated at one time for each serving cell.

Thus, we propose to adopt Alt-2:

Proposal 6: For SL consistent LBT failure detection, the timer/counter mechanism is running per BWP/carrier, i.e. SL_LBT_COUNTER continues when RB set is changed (e.g. due to resource / resource pool reselection).

### 2.2.2 Which SL transmissions can result in consistent LBT failure

In NR-U, all uplink transmissions can result in consistent LBT failure, as highlighted in below copy of TS 38.321 [3]:

### 5.21.2	LBT failure detection and recovery procedure

The MAC entity may be configured by RRC with a consistent LBT failure recovery procedure. Consistent LBT failure is detected per UL BWP by counting LBT failure indications, for all UL transmissions, from the lower layers to the MAC entity.

Observation 5: In NR-U, all uplink transmissions can result in consistent LBT failure.

For SL, the above mechanism is straight forward to be reused for PSSCH/PSCCH/PSFCH. However, S-SSB needs further discussion. We suggest to preclude S-SSB transmission in consistent LBT failure detection due to below considerations:

- Even if SL consistent LBT failure can be detected for S-SSB transmission, there is no recovery solution.

Agreement (RAN1#112)

RAN1 further study the followings:

- Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

RAN1 is discussing whether/how to allow S-SSB transmission in multiple RB set(s), which is duplicated with purpose of consistent LBT failure recovery.

Observation 6: RAN1 is discussing whether/how to allow S-SSB transmission in multiple RB set(s), which is duplicated with purpose of consistent LBT failure recovery

Thus, we propose:

Proposal 7: LBT failure of PSSCH/PSCCH/PSFCH transmission leads to SL LBT failure indication, i.e. S-SSB transmission is excluded from SL LBT failure indication.

### 2.2.3 Mechanism of SL consistent LBT failure recovery

In RAN2#121b-e [4], it was agreed that granularity of SL consistent LBT failure indication is per RB set. And UE autonomous recovery via resource (re)selection is consensus.

Agreement

SL C-LBT failure is declared per RB-set

However, please note that resource / resource pool are granularity of resource (re)selection in existing TS 38.321 [3] and TS 38.213 [6]. Thus, although the granularity of LBT failure indication was agreed to be per RB set, it will lead to many spec changes on resource (re)selection procedure if RB set is used as the granularity of resource change.

Observation 7: Although the granularity of LBT failure indication was agreed to be per RB set, it will lead to many spec changes on resource (re)selection procedure if RB set is used as the granularity of resource change.

To minimize spec impacts, we prefer to keep the existing granularity of SL resource (re)selection, i.e. if one resource is overlapping with a RB-set with SL C-LBT failure detected, this resource is not usable. Otherwise, if one resource doesn't overlap with any RB-set(s) with SL C-LBT failure detected, this resource can be selected.

Proposal 8: To minimize spec impacts, keep the existing granularity of SL resource (re)selection upon detection of SL C-LBT failure, i.e. one resource candidate is precluded by MAC layer to be selected if it is overlapping with any RB set(s) where SL C-LBT failure is detected.

With regarding to details of resource (re)selection due to consistent LBT failure, we think both resource (re)selection and resource pool (re)selection can be applied to the autonomous recovery. Please note that resource pool reselection may be infeasible when bandwidth of the resource pool is 100MHz which occupies the whole SL BWP. Because SL-U is targeting for large payload transmission and WiFi supports 160/320MHz per channel, resource pool of 100MHz should not be a corner case.

Observation 8: Upon detection of consistent LBT failure, resource pool reselection may be infeasible when bandwidth of the resource pool is 100MHz which occupies the whole SL BWP

Upon detection of SL C-LBT failure, we think a reasonable recovery way is that the UE can first trigger resource (re) selection if there are available resource in current resource pool. Otherwise (i.e. if there is no available resource in current resource pool), the UE can trigger resource pool selection.

Proposal 9: Upon detection of SL C-LBT failure, the UE first triggers resource (re) selection if there is available resource in current resource pool.

Then, on detailed condition whether resource (re)selection or resource pool (re)selection is triggered, we think the simplest solution is that resource pool reselection is triggered only when there are not sufficient resource candidates (i.e. the size of S_A is lower than a threshold). Otherwise, the resource reselection is performed in current resource pool. An example of the procedure is illustrated as below:

- Based on the required number of sub-channels and RB sets excluded which are provided by MAC layer, PHY layer provides the S_A candidate set to MAC layer. Then, MAC layer checks whether the size of S_A is lower than a minimum threshold. If yes, MAC layer stops resource selection and go to step 2). Otherwise, resource selection is performed as legacy.

  - Note that the size threshold is intended to handle whether retransmission resource can be reserved.

- MAC layer conducts resource pool reselection. Then, MAC layer will trigger resource selection again with this newly selected resource pool.

Proposal 10a: Upon detection of SL C-LBT failure, the resource reselection is triggered in current resource pool if there are sufficient resource candidates not overlapping with any RB sets where SL C-LBT failure is detected (i.e. the size of S_A is lower than a threshold). Otherwise, resource pool reselection is triggered.

Proposal 10b: The candidate resource size threshold is intended to handle whether retransmission resource can be reserved.

Finally, we discuss the FFS on whether/how the triggered SL C-LBT failure is cancelled. As highlighted in copy of existing TS 38.321 [3], the triggered UL consistent LBT failure is cancelled in below 3 cases:

- Upon reporting the LBT failure MAC-CE to gNB

- Upon RACH procedure successfully completed in SpCell

- Upon reconfiguration of Uu consistent LBT failure by RRC

The MAC entity shall:

...

1>	if a MAC PDU is transmitted and LBT failure indication is not received from lower layers and this PDU includes the LBT failure MAC CE:

2>	cancel all the triggered consistent LBT failure(s) in SCell(s) for which consistent LBT failure was indicated in the transmitted LBT failure MAC CE.

1>	if consistent LBT failure is triggered and not cancelled in the SpCell; and

1>	if the Random Access procedure is considered successfully completed (see clause 5.1) in the SpCell:

2>	cancel all the triggered consistent LBT failure(s) in the SpCell.

1>	if lbt-FailureRecoveryConfig is reconfigured by upper layers for a Serving Cell:

2>	cancel all the triggered consistent LBT failure(s) in this Serving Cell.

Observation 9: In NR-U, the triggered UL consistent LBT failure is cancelled in below 3 cases:

- Upon reporting the LBT failure MAC-CE to gNB

- Upon RACH procedure successfully completed in SpCell

- Upon reconfiguration of Uu consistent LBT failure by RRC (e.g. timer/counter)

In our understanding, the intention of 1) and 2) is the assumption that gNB may recover the triggered consistent LBT failure after notified. And 3) is general handling of MAC reset upon RRC reconfiguration. Then, for the triggered SL consistent LBT failure, we think it can cancelled at least in below 4 cases:

- Upon reporting SL failure information to gNB.

    - It is similar to cancellation case 1) of NR-U.

    - It is applicable to RRC_CONNECTED UE.

- Upon reconfiguration of SL consistent LBT failure by RRC (e.g. timer/counter).

    - It is similar to cancellation case 3) of NR-U.

    - It is applicable to RRC_CONNECTED UE.

- Upon reconfiguration of resource / resource pool by RRC.

    - It is similar to cancellation case 3) of NR-U.

    - The reconfiguration means the autonomous resource change needs reset.

    - It is applicable to RRC_CONNECTED UE.

- Upon successful completion of autonomous resource or resource pool selection (i.e. the UE successfully transmits packet after switching resource / resource pool)

    - It is similar to intention of cancellation case 1) and case 2) of NR-U.

    - It is applicable to UE in all RRC states (RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED) or OOC.

We think above cancellation conditions are straight forward, and propose RAN2 to agree them. Meanwhile, whether other cancellation conditions (e.g. a new timer) are required can be further discussed.

Proposal 11: The triggered SL consistent LBT failure are cancelled at least in below 4 cases. FFS other cancellation conditions (e.g. a new timer).

- Upon reporting SL failure information to gNB

- Upon reconfiguration of SL consistent LBT failure by RRC (e.g. timer/counter)

- Upon reconfiguration of resource / resource pool by RRC

- Upon successful completion of autonomous resource or resource pool selection (i.e. the UE successfully transmits packet after switching resource / resource pool)

### 2.2.4 Signalling of SL consistent LBT failure information

Besides the confirmed WAs in RAN2#121b-e [4], we think one missed issue is the priority of SL consistent LBT failure MAC-CE in LCP procedure. According to TS 38.321 [3], priority of UL LBT failure MAC-CE is between MAC CE for Sidelink Configured Grant Confirmation and MAC CE for Timing Advance Report.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for Timing Advance Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over transmission of NR sidelink communication.

Observation 10: According to TS 38.321, priority of UL LBT failure MAC-CE is between MAC CE for Sidelink Configured Grant Confirmation and MAC CE for Timing Advance Report

We think SL consistent LBT failure MAC-CE is right after UL consistent LBT failure MAC-CE.

Proposal 12: During LCP procedure, the priority of SL consistent LBT failure MAC-CE is right after UL consistent LBT failure MAC-CE

# 3 Conclusion

In this contribution, we discuss open issues on LCP impacts due to COT sharing and consistent LBT failure. Our observations are:

Observation 1: A different LCP procedure is required: 1) There is a COT sharing (channel resource )  2) Select destination to be initiating UE  3) Multiplex LCH/MAC CEs which are of lower CPAC priority to be in MAC PDU  4) find resource (mode 2 RA or mode 1 grant).

Observation 2: In existing TS 38.321, the destination is selected based on priority of LCH and MAC-CE.

Observation 3: According to existing TS 38.321, the resource selection window is restricted by PDB of the pending SL data available in the LCH(s).

Observation 4: In NR-U, consistent LBT failure is detected per UL BWP but the timer/counter mechanism is running per serving cell because only one BWP can be activated for one serving cell at one time. In other word, the LBT_COUNTER continues if gNB indicates BWP switch.

Observation 5: In NR-U, all uplink transmissions can result in consistent LBT failure.

Observation 6: RAN1 is discussing whether/how to allow S-SSB transmission in multiple RB set(s), which is duplicated with purpose of consistent LBT failure recovery

Observation 7: Although the granularity of LBT failure indication was agreed to be per RB set, it will lead to many spec changes on resource (re)selection procedure if RB set is used as the granularity of resource change.

Observation 8: Upon detection of consistent LBT failure, resource pool reselection may be infeasible when bandwidth of the resource pool is 100MHz which occupies the whole SL BWP

Observation 9: In NR-U, the triggered UL consistent LBT failure is cancelled in below 3 cases:

- Upon reporting the LBT failure MAC-CE to gNB

- Upon RACH procedure successfully completed in SpCell

- Upon reconfiguration of Uu consistent LBT failure by RRC (e.g. timer/counter)

Observation 10: According to TS 38.321, priority of UL LBT failure MAC-CE is between MAC CE for Sidelink Configured Grant Confirmation and MAC CE for Timing Advance Report

Based on observations, our proposals are:

LCP impacts due to COT sharing

Proposal 1: Enhanced destination selection procedure is that the responding UE is allowed to select initiating UE as destination based on candidate destination information from COT sharing info (rather than priority).

Proposal 2: The conditions to use the enhanced destination selection procedure include at least:

- SL data or MAC-CE towards initiating UE is available for transmission;

- Remaining COT duration has available resource to complete the data transmission;

- SL data towards initiating UE has at least one LCH's CAPC value ≤ CAPC value indicated in COT information.

Otherwise, the legacy destination selection procedure based on priority is applied

Proposal 3: RAN2 discuss how MAC layer of the responding UE handles the case that it has SL data / MAC-CE towards other UEs with higher priority than the SL data / MAC-CE towards initiating UE:

- Solution 1: It is up to UE implementation whether to select initiating UE or the UE with highest priority SL data/MAC-CE

- Solution 2: Initiating UE is selected if highest priority of other UEs' data/MAC-CE is smaller than a threshold, similar to existing prioritization rule between UL and SL transmission

Proposal 4: If the initiating UE is selected as the destination, introduce a new SL LCP restriction that the responding UE shall not include any MAC SDU(s) of LCH(s) having CAPC value higher than the CAPC value indicated in the COT sharing information.

Proposal 5: If the initiating UE is selected as the destination, MAC layer of the responding UE needs to further restrict the resource selection window within remaining COT duration besides PDB of the pending SL data available in the LCH(s).

Consistent LBT failure recovery

Proposal 6: For SL consistent LBT failure detection, the timer/counter mechanism is running per BWP/carrier, i.e. SL_LBT_COUNTER continues when RB set is changed (e.g. due to resource / resource pool reselection).

Proposal 7: LBT failure of PSSCH/PSCCH/PSFCH transmission leads to SL LBT failure indication, i.e. S-SSB transmission is excluded from SL LBT failure indication.

Proposal 8: To minimize spec impacts, keep the existing granularity of SL resource (re)selection upon detection of SL C-LBT failure, i.e. one resource candidate is precluded by MAC layer to be selected if it is overlapping with any RB set(s) where SL C-LBT failure is detected.

Proposal 9: Upon detection of SL C-LBT failure, the UE first triggers resource (re) selection if there is available resource in current resource pool.

Proposal 10a: Upon detection of SL C-LBT failure, the resource reselection is triggered in current resource pool if there are sufficient resource candidates not overlapping with any RB sets where SL C-LBT failure is detected (i.e. the size of S_A is lower than a threshold). Otherwise, resource pool reselection is triggered.

Proposal 10b: The candidate resource size threshold is intended to handle whether retransmission resource can be reserved.

Proposal 11: The triggered SL consistent LBT failure are cancelled at least in below 4 cases. FFS other cancellation conditions (e.g. a new timer).

- Upon reporting SL failure information to gNB

- Upon reconfiguration of SL consistent LBT failure by RRC (e.g. timer/counter)

- Upon reconfiguration of resource / resource pool by RRC

- Upon successful completion of autonomous resource or resource pool selection (i.e. the UE successfully transmits packet after switching resource / resource pool)

Proposal 12: During LCP procedure, the priority of SL consistent LBT failure MAC-CE is right after UL consistent LBT failure MAC-CE

# 4 References

[1] RAN2#121, Chair Notes

[2] RAN1#111, Chair Notes

[3] TS 38.321-v17.1.0, Medium Access Control (MAC) protocol specification, 2022-6.

[4] RAN2#121b-e, Chair Notes

[5] RAN2#119b-e, Chair Notes

[6] TS 38.213-v17.1.0, Physical layer procedures for control, 2022-6.