# 3GPP TSG RAN WG2 Meeting #122      	                           R2-2305090
Incheon, Korea, 22th – 26th May 2023

Agenda item:	7.15.3

Source:	Apple

Title:	Discussion on resource (re)selection and MCSt in SL-U

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#121 [1], RAN2 discussed resource (re)selection. Below agreements were made:

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

2a:	RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

2b:	RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

3:	Will send LS to RAN1 to check if there is any concern.

The agreement 2b requires RAN2 to further discuss MAC impacts of resource (re)selection for intra-UE case.

Meanwhile, RAN2 received a LS from RAN1 on resource (re)selection of MCSt in R1-2304257 [2], which includes 3 questions for RAN2 to response.

In this contribution, we share our views on above two aspects of resource (re)selection:

- LBT impacts in resource (re)selection with LBT impacts (intra-UE case)

- Resource (re)selection for MCSt

# 2 Discussion

## 2.1 LBT impacts in resource (re)selection (intra-UE case)

We address the below highlighted agreement in this section.

Agreements on SL resource (re)selection

1: 	RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

2a:	RAN2 understands L1 handles LBT impact to/from other UEs’ reserved resources in SL candidate resource selection (inter-UE case).

2b:	RAN2 will study how MAC performs resource (re)selection with the consideration of LBT impact to its own candidate resource (intra-UE case).

3:	Will send LS to RAN1 to check if there is any concern.

According to RAN2#121 discussion [1], we understand that the intention is to ensure sufficient time gap between selected resource and the resource reserved by the same TX UE (i.e. intra-UE case). Otherwise, the candidate resource may be unavailable if LBT failure happens.

Observation 1: For LBT impact in resource (re)selection (intra-UE case), its intention is to ensure sufficient time gap between selected resource and the resource reserved by the same TX UE. Otherwise, the candidate resource may be unavailable if LBT failure happens.

We agree with the issue. However, we don't think it needs to be specified in resource (re)selection procedure. Please note that Rel-16 discussed similar issue: how to ensure the minimum time gap between the reserved resource for initial transmission and the reserved resource for retransmission. Finally, it was captured a Note that it is left to UE implementation in section 5.22.1.1 of TS 38.321 [3]:

From section 5.22.1.1 of TS 38.321

NOTE 3B:	If retransmission resource(s) cannot be selected by ensuring that the resource(s) can be indicated by the time resource assignment of a prior SCI, how to select the time and frequency resources for one or more transmission opportunities from the available resources is left for UE implementation by ensuring the minimum time gap between any two selected ‎resources in case that PSFCH is configured for this pool of ‎resources.

Observation 2: Rel-16 discussed similar issue: how to ensure the minimum time gap between the reserved resource for initial transmission and the reserved resource for retransmission. Finally, it was captured a Note that it is left to UE implementation in section 5.22.1.1 of TS 38.321

We think the same way can be reused.

Proposal 1: On the LBT impacts to its own candidate resource in resource (re)selection (i.e. intra-UE case) when MSCt and COT sharing are not applied, it is left to UE implementation in MAC layer.

## 2.2 Resource (re)selection of MCSt

RAN2 received a LS from RAN1 on resource (re)selection of MCSt in R1-2304257 [2], which includes below 3 questions for RAN2 to response:

RAN1 has discussed the following approaches to implement/achieve MCSt for SL-U communication. RAN1 would like to seek RAN2’s opinion on the following questions. Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (, remaining PDB, and ) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (, remaining PDB, and ) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required. Approach 3: “guarantee MCSt for multiple TBs” - Step 1: Higher layer triggers L1 resource (re-)selection one time for one or multiple TBs with one set of parameters (, remaining PDB, and ) + “number of slots for MCSt” which could be derived based on CAPC of the multiple TBs. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects transmission resource for the one or multiple TB(s) from the reported set of candidate multi-slot resource (SA). Question 1 (for Approach 1/ Approach 2): feasibility of selecting the resource for a single TB in MAC layer (single-slot under Approach 1, multi-slot under Approach 2) with the principle of “concatenating” across separate resource selection triggers (across TBs) Question 2 (for Approach 3): feasibility of triggering the resource selection procedures for multiple SL processes at the same time Question 3 (Approach 2/ Approach 3): feasibility of providing a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt

The description of the 3 approaches in this LS is confusing. We provide our understanding on these 3 approaches with 2 TBs as example and highlight the potential MAC layer changes in Table. 1.

Approach 1: - For the first TB, MAC layer triggers a resource selection procedure (i.e. one SL process for one TB's resource selection). - PHY layer reports single-slot candidate resources to MAC layer. - MAC layer randomly selects a resource (legacy Rel-16/Rel-17 behavior). - For the second TB, MAC layer triggers another resource selection procedure. - PHY layer reports single-slot candidate resources to MAC layer. - MAC selects a resource, which is time domain adjacent to the previous selected resources in step 3 ("Best effort for multiple TBs”). The main MAC specification impact is in Step 6. Approach 2: - For the first TB, MAC triggers a resource selection procedure "with an indication of MCSt length for 1 TB" - PHY layer reports multi-slot candidate resources for this TB to MAC layer (“Guarantee MCSt for single TB"). - MAC layer randomly selects a multi-slot candidate resource (legacy Rel-16/Rel-17 behavior). - For the second TB, MAC triggers another resource selection procedure. - PHY layer reports multi-slot candidate resources to MAC layer. - MAC layer selects a multi-slot resource, which are time domain adjacent to the previous selected multi-slot resource in Step 3 (“Best effort for multiple TBs”) The main MAC layer specification impact is in Step 6. Approach 3: - For multiple TBs, MAC triggers multiple resource selection procedures at the same time "with the indication of MCSt length for multiple TBs". - PHY layer reports multi-slot candidate resources for multiple TBs (“Guarantee MCSt for multiple TBs"). - MAC randomly selects a multi-slot resource for multiple TBs (legacy Rel-16/Rel-17 behavior). The main MAC layer specification impact is in Step 1.

Table. 1 Understanding of 3 Approaches with 2 TB as example

We propose to first confirm the understanding in Table 1 is correct.

Proposal 2: RAN2 confirm that the understanding of 3 resource (re)selection approaches for MCSt in Table. 1.

Then, we discuss their MAC spec impacts.

Approach 1 and 2

Please note that MAC layer selects resource per SL process in TS 38.321 [3]. But Approach 1 and 2 require cross-SL-process description of resource selection.

Observation 3: MAC layer selects resource per SL process in TS 38.321.

Proposal 3: For Approach 1 and 2 of MCSt, main MAC specification impact is to change from existing per-SL-process description of resource selection to cross-SL-process description of resource selection.

Approach 3

Approach 3 requires MAC layer to triggers multiple SL processes at the same time for different TBs. As mentioned above, single SL process of resource selection is described in TS 38.321 [3]. However, we think it is a kind of modeling approach. It doesn't restrict UE implementation to run simultaneous SL processes.

Proposal 4: For Approach 3 of MCSt, main MAC specification impact is to explicitly describe multiple simultaneous SL processes.

Finally, we discuss the question whether MAC layer can provide a new parameter “number of slots for MCSt” to PHY layer. In our understanding, "number of slots for MCSt” can be calculated in MAC layer similar to "number of TBs for transmission" or subchannel number. PHY layer may not calculate this parameter because it doesn't know the pending/predicted traffic information.

Observation 4: "Number of slots for MCSt” can be calculated in MAC layer similar to "number of TBs for transmission" or subchannel number. PHY layer may not calculate this parameter because it doesn't know the pending/predicted traffic information.

Thus, we propose:

Proposal 5: MAC layer can calculate "number of slots for MCSt” based on its pending/predicted traffic information.

# 3 Conclusion

In this contribution, we discuss resource (re)selection and MCSt. Our observations are:

Observation 1: For LBT impact in resource (re)selection (intra-UE case), its intention is to ensure sufficient time gap between selected resource and the reserved resource. Otherwise, the candidate resource may be unavailable if LBT failure happens.

Observation 2: Rel-16 discussed similar issue: how to ensure the minimum time gap between the reserved resource for initial transmission and the reserved resource for retransmission. Finally, it was captured a Note that it is left to UE implementation in section 5.22.1.1 of TS 38.321

Observation 3: MAC layer selects resource per SL process in TS 38.321.

Observation 4: "Number of slots for MCSt” can be calculated in MAC layer similar to "number of TBs for transmission" or subchannel number. PHY layer may not calculate this parameter because it doesn't know the pending/predicted traffic information.

Based on observations, our proposals are:

Proposal 1: On the LBT impacts to its own candidate resource in resource (re)selection (i.e. intra-UE case) when MSCt and COT sharing are not applied, it is left to UE implementation in MAC layer.

Proposal 2: RAN2 confirm that the understanding of 3 resource (re)selection approaches for MCSt in Table. 1.

Proposal 3: For Approach 1 and 2 of MCSt, main MAC specification impact is to change from existing per-SL-process description of resource selection to cross-SL-process description of resource selection.

Proposal 4: For Approach 3 of MCSt, main MAC specification impact is to explicitly describe multiple simultaneous SL processes.

Proposal 5: MAC layer can calculate "number of slots for MCSt” based on its pending/predicted traffic information.

# 4 References

[1] RAN2#121, Chair Notes

[2] R1-2304257, LS on MCSt resource (re-)selection, OPPO.

[3] TS 38.321-v17.1.0, Medium Access Control (MAC) protocol specification, 2022-6.