# 3GPP TSG RAN WG2 Meeting #122      	                           R2-2305093
Incheon, Korea, 22th – 26th May 2023

Agenda item:	7.15.6

Source:	Apple

Title:	Discussion on Sidelink CA

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN#99, SL CA was agreed to start WG work with revised scope (RP-230077 [1]). And in RAN2#121b-e [4], below agreements were made:

CA configuration for GC/BC

Proposal 3:	For groupcast/broadcast, as in LTE SL CA, the carrier(s) that can be used for transmitting data are configured by V2X layer for the L2 destination. FFS on backwards compatibility issue.

CA duplication

Proposal 8: Packet duplication for NR sidelink is performed at the PDCP layer. The duplicated PDCP PDUs of the same PDCP entity are submitted to two different RLC entities and associated to two different sidelink logical channels respectively.

Proposal 9: RAN2 agrees that LCH mapping restriction shall be defined such that the duplicated PDCP PDUs of the same PDCP entity are only allowed to be transmitted on different NR sidelink carriers.

Proposal 16: For NR sidelink PDCP duplication, reuse the hard-coded way for paired sidelink LCID to identify duplicated sidelink LCHs (i.e. for a unified design for all Bcast/Gcast). The specific SL LCID values occupied are left to Stage-3. FFS on Unicast case.

CA carrier (re)selection

Proposal 10: For TX carrier (re)selection triggers in NR sidelink CA, reuse the triggers for TX carrier (re)selection per sidelink process in LTE sidelink CA as follows at least for GC/BC

if the resource (re)selection is triggered with the sidelink process.

if there is no sidelink grant associated with the sidelink process on any carrier allowed for the STCH as indicated by upper layers (i.e., RRC layer and V2X layer).

FFS on unicast case.

Proposal 7	For LCP, only allow the LCHs having a priority whose associated CBR threshold for reselection is no lower than the CBR of the carrier when the carrier is (re-)selected. FFS on how to determine the per-carrier CBR at least for GC/BC.

FFS on unicast case.

Proposal 5	NR SL CA TX carrier (re)selection follows LTE CA solution, i.e., define 1) per-carrier-per-priority CBR threshold for carrier (re)selection, and 2) per-carrier-per-priority CBR threshold for carrier keeping. And final carrier selection is done based on the lowest CBR value across carriers. Where the priority is the LCH priority.

FFS on unicast case.

In this contribution, we further discuss SL CA from above aspects (i.e. CA configuration, CA duplication and carrier selection).

# 2 Discussion

## 2.1 CA configuration for GC/BC

In RAN2#121b-e [4], CA configuration for GC/BC is discussed, and below agreement was made with FFS on backward compatibility issue.

Proposal 3:	For groupcast/broadcast, as in LTE SL CA, the carrier(s) that can be used for transmitting data are configured by V2X layer for the L2 destination. FFS on backwards compatibility issue.

We think the FFS is the issue mentioned in WID objective:

- This feature is backwards compatible in the following regards

  - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI)

First, according to latest WID objective, we think only below two cases are valid in Rel-18:

- Case 1: RX UE is legacy Rel-16/Rel-17 UE which supports only single carrier

- Case 2: RX UE is Rel-18 UE, which always covers all possible SL carriers in Rel-18

Proposal 1: RAN2 confirm only below two cases are valid in Rel-18:

- Case 1: RX UE is legacy Rel-16/Rel-17 UE which supports only single carrier

- Case 2: RX UE is Rel-18 UE, which always covers all possible SL carriers in Rel-18

Then, we think there are basically two solutions for TX UE to determine whether the RX UE is a legacy UE which supports only single carrier:

- Alt-1: Rely on service to frequency mapping provided by upper layer (i.e. assume upper layer will provide a mapping to a single carrier if RX UE is a legacy UE supporting only single carrier)

- Alt-2: Introduce a new list for TX profiles (e.g. ENUMERATED {carrierAggregated, singleCarrier}), which corresponding to different L2 Addresses or services.

  - Note that SL DRX has adopted the similar solution.

After checking TS 24.587 [2], we think Alt-1 can't work because mapping to multiple V2X frequencies is already allowed in Rel-16/Rel-17. Thus, TX UE can't determine whether the RX UE is a legacy UE via service to frequency mapping provided by upper layer.

###### 6.1.3.2.1.1	Requirements for V2X communication over PC5

Omit...

Upon a request from upper layers to send a V2X message of a V2X service identified by a V2X service identifier using V2X communication over PC5, if:

a)	the UE is configured with V2X service identifier to V2X frequency mapping rules for V2X communication over PC5 as specified in clause 5.2.3; and

b)	there is one or more V2X frequencies associated with the V2X service identifier of the V2X service for the V2X message in the current geographical area,

then the UE passes the one or more V2X frequencies associated with the V2X service identifier of the V2X service and the communication mode which is set to broadcast mode for the V2X message to the lower layers.

Observation 1: According to TS 24.587, upper layer can provide a service mapping to multiple V2X frequencies for a Rel-16/Rel-17 legacy UE. Thus, TX UE can't determine whether the RX UE is a legacy UE via service to frequency mapping provided by upper layer.

Thus, we propose to use TX profile to resolve the backward compatibility issue.

Proposal 2: To resolve the backward compatibility issue (i.e. TX UE with broadcast/groupcast SL CA needs to determine whether the RX UE is legacy Rel-16/Rel-17 UE which only supports single carrier), introduce a new list for TX profiles corresponding to different L2 Addresses or services.

## 2.2 CA configuration for UC

In RAN2#121b-e [4], how to provide CA configuration for SL UC was discussed (e.g. whether V2X layer or AS layer can provide the carrier configuration). Finally, one LS [5] is sent to SA2 with below two questions related to the procedure after unicast PC5 link is established.

Agreement:

Proposal 1: Based on observation that section 6.1.2.12 of TS 24.587-v18.0.0 has captured V2X layer can be provisioned with service to frequency mapping for unicast. RAN2 assume it is applicable to PC5 unicast SL CA after link has been established. RAN2 notify SA2 this assumption and ask their input on identified questions.

RAN2 ask SA2 input on Question 1: According to TS 24.588, V2X layer is only provisioned with a mapping between service identifier and initial L2 address used for unicast. But service identifier is invisible to AS-layer, and the initial L2 ID will only be used in DCR and be replaced by a self-chosen L2 ID in PC5-S link establishment procedure. Then, after L2 ID changes, whether/how UE's AS layer can obtain the mapping between L2 ID and frequencies.

RAN2 ask SA2 input on Question 2: According to TS 24.587, PC5 unicast allows UEs to add/modify/remove V2X services/PC5 QoS flows to the same L2 ID pair. Then, given service info is invisible to AS layer, how can the UE ensure the modified V2X services to be transmitted only on the corresponding frequencies in the V2X layer?

Thus, we think the discussion on CA configuration for established PC5 UC link can be postponed after SA2 reply LS is received.

Proposal 3: RAN2 is suggested to postpone the discussion on CA configuration for established PC5 UC link after SA2 reply LS is received.

Meanwhile, we think one open is whether CA can be applied before PC5 unicast link is established (i.e. initial PC5-S signaling). Although it is theoretically feasible, we prefer not to apply CA (i.e. only single carrier) to initial PC5-S signaling due to below reasonings:

- If CA for initial PC5-S signaling can be used, it will incur legacy issue because legacy UE doesn't support CA. And the TX UE can't know whether RX UE supports CA until PC5 capability exchange.

- If CA for initial PC5-S signaling can be used, the UE has to keep all RX chains active for initial PC5-S signalling, which is power consuming.

- There is no requirement to enhance reliability performance of initial PC5-S signalling. Otherwise, the legacy Rel-16/Rel-17 SL system can't work.

Observation 2: If CA for initial PC5-S signaling can be used, it will incur at least below issues:

- Legacy issue because legacy UE doesn't support CA. And the TX UE can't know whether RX UE supports CA until PC5 capability exchange.

- UE has to keep all RX chains active for initial PC5-S signalling, which is power consuming.

- There is no requirement to enhance reliability performance of initial PC5-S signalling. Otherwise, the legacy Rel-16/Rel-17 SL system can't work.

Thus, we propose not to apply CA to initial PC5-S signaling. FFS how TX and RX UE align with carrier for initial PC5-S signaling.

Proposal 4: SL CA is not applied to initial PC5-S signaling. FFS how TX UE and RX UE align with one carrier for initial PC5-S signaling.

## 2.3 Carrier (re)selection

In RAN2#121b-e [4], it was agreed that the same carrier (re)selection mechanism of LTE SL CA is reused for NR SL GC/BC CA. The 2 main FFSs are:

- FFS1: whether the same mechanism is reused for NR SL unicast CA

- FFF2: FFS on how to determine the per-carrier CBR at least for GC/BC.

Proposal 10: For TX carrier (re)selection triggers in NR sidelink CA, reuse the triggers for TX carrier (re)selection per sidelink process in LTE sidelink CA as follows at least for GC/BC

if the resource (re)selection is triggered with the sidelink process.

if there is no sidelink grant associated with the sidelink process on any carrier allowed for the STCH as indicated by upper layers (i.e., RRC layer and V2X layer).

FFS on unicast case.

Proposal 7	For LCP, only allow the LCHs having a priority whose associated CBR threshold for reselection is no lower than the CBR of the carrier when the carrier is (re-)selected. FFS on how to determine the per-carrier CBR at least for GC/BC.

FFS on unicast case.

Proposal 5	NR SL CA TX carrier (re)selection follows LTE CA solution, i.e., define 1) per-carrier-per-priority CBR threshold for carrier (re)selection, and 2) per-carrier-per-priority CBR threshold for carrier keeping. And final carrier selection is done based on the lowest CBR value across carriers. Where the priority is the LCH priority.

FFS on unicast case.

FFS1: whether the same mechanism is reused for NR SL unicast CA

In Rel-18, we prefer an unified design for NR SL UC and BC/GC. One may argue that NR SL UC can introduce PC5-RRC signaling exchange so that TX UE and RX UE can negotiate a subset of carriers via AS signaling. However, we tend to think it is an optimization, which can be studied in Rel-19. In Rel-18, an unified solution same as LTE SL CA is sufficient

Proposal 5: Introduce an unified carrier (re)selection procedure for NR SL UC/BC/GC CA, i.e. the similar mechanism of LTE SL CA is reused for NR SL UC CA.

FFS2: how to determine the per-carrier CBR at least for GC/BC

In LTE SL, because the mapping between zones and resource pool can be configured, the UE selects the resource pool based on the zone where the UE is located. Thus, in LTE SL, the carrier level CBR is the corresponding resource pool level CBR based on the zone where the UE is located. However, in NR SL, there is no mapping between zones and resource pool. Because the carrier (re)selection is performed before the resource pool (re)selection, the UE may not know which resource pool will be selected for each carrier.

Observation 3: In NR SL, there is no mapping between zones and resource pool. Because the carrier (re)selection is performed before the resource pool (re)selection, the UE may not know which resource pool will be selected for each carrier.

To resolve this issue, we think the possible solutions can be:

- Alt-1: Average CBR of all resource pools in the carrier

- Alt-2: Maximum CBR of all resource pool in the carrier

- Alt-3: Minimum CBR of all resource pool in the carrier

Because the UE performs random resource selection in NR SL, we think Alt-1 makes more sense, i.e., use an average over the CBRs of all resource pools in the each carrier.

Proposal 6: For the calculation of per-carrier CBR, use an average over the CBRs of all resource pools in the each carrier for all cast-type (UC/BC/GC).

## 2.4 Packet duplication

In RAN2#121b-e [4], it was agreed that the same packet duplication framework of LTE SL CA is reused for NR SL GC/BC SL. The 2 main FFSs are:

- FFS1: whether the hard-coded LCID is reused for NR SL unicast CA

- FFF2: how to determine trigger of packet duplication

Proposal 8: Packet duplication for NR sidelink is performed at the PDCP layer. The duplicated PDCP PDUs of the same PDCP entity are submitted to two different RLC entities and associated to two different sidelink logical channels respectively.

Proposal 9: RAN2 agrees that LCH mapping restriction shall be defined such that the duplicated PDCP PDUs of the same PDCP entity are only allowed to be transmitted on different NR sidelink carriers.

Proposal 16: For NR sidelink PDCP duplication, reuse the hard-coded way for paired sidelink LCID to identify duplicated sidelink LCHs (i.e. for a unified design for all Bcast/Gcast). The specific SL LCID values occupied are left to Stage-3. FFS on Unicast case.

FFS1: whether the hard-coded LCID is reused for NR SL unicast CA

For NR SL UC CA, it is feasible to utilize PC5 RRC signaling to achieve configurable SL LCID between TX UE and RX UE, which can save LCID space. However, we don't think the LCID value space is a big issue, and an unified design for NR SL UC and BC/GC is preferred if no blocking issue is identified. Thus, we propose to reuse the hard-coded LCID solution.

Proposal 7: For NR SL PDCP duplication, introduce an unified solution for UC/BC/GC if no blocking issue is identified.

Proposal 8: For PDCP duplication of NR SL CA of UC, reuse the hard-coded LCID to identify duplicated SL LCHs .

FFS2: how to determine trigger of packet duplication

Mechanism on when to perform packet duplication in LTE SL CA is illustrated in clause 23.14.1.1 of TS 36.300 [3]:

Sidelink packet duplication is supported for V2X sidelink communication and is performed at PDCP layer of the UE. For sidelink packet duplication for transmission, a PDCP PDU is duplicated at the PDCP entity. The duplicated PDCP PDUs of the same PDCP entity are submitted to two different RLC entities and associated to two different sidelink logical channels respectively. The duplicated PDCP PDUs of the same PDCP entity are only allowed to be transmitted on different sidelink carriers. A UE can activate or deactivate sidelink packet duplication based on (pre)configuration. Sidelink packet duplication does not apply to transmission with Rel-14 transmit profile (TS 23.285 [72]). The PPPR value(s) for which sidelink packet duplication is supported can be (pre)configured via a PPPR threshold. For UE autonomous resource selection and scheduled resource allocation, the UE shall perform sidelink packet duplication for the data with the configured PPPR value(s) until packet duplication is deconfigured for these PPPR value(s). For scheduled resource allocation, the UE reports the amount of data associated with one or more PPPR values, and the destination(s) to which the data belongs via sidelink BSR(s). A mapping of PPPR values to logical channel groups can be configured by the eNB, and the PPPR value(s) are reflected by the associated logical channel group ID included in the sidelink BSR(s). A list of PPPR value(s) may be reported in Sidelink UE information by an RRC_CONNECTED UE.

As highlighted above, packet duplication in LTE SL CA is triggered based on threshold of PPPR.

Observation 4: In LTE SL CA, packet duplication is triggered based on threshold of PPPR.

Because PPPR is not introduced in NR SL, we can introduce a similar reliability threshold for sl-PacketErrorRate-r16 in PQI. Then, the whole mechanism of LTE packet duplication mechanism can be reused in NR SL CA.

Proposal 9: Introduce a reliability threshold for sl-PacketErrorRate-r16 in PQI as the criteria to determine whether packet duplication is performed.

# 3 Conclusion

In this contribution, we further SL CA. Our observations are:

Observation 1: According to TS 24.587, upper layer can provide a service mapping to multiple V2X frequencies for a Rel-16/Rel-17 legacy UE. Thus, TX UE can't determine whether the RX UE is a legacy UE via service to frequency mapping provided by upper layer.

Observation 2: If CA for initial PC5-S signaling can be used, it will incur at least below issues:

- Legacy issue because legacy UE doesn't support CA. And the TX UE can't know whether RX UE supports CA until PC5 capability exchange.

- UE has to keep all RX chains active for initial PC5-S signalling, which is power consuming.

- There is no requirement to enhance reliability performance of initial PC5-S signalling. Otherwise, the legacy Rel-16/Rel-17 SL system can't work.

Observation 3: In NR SL, there is no mapping between zones and resource pool. Because the carrier (re)selection is performed before the resource pool (re)selection, the UE may not know which resource pool will be selected for each carrier.

Observation 4: In LTE SL CA, packet duplication is triggered based on threshold of PPPR.

Based on observations, our proposals are:

Proposal 1: RAN2 confirm only below two cases are valid in Rel-18:

- Case 1: RX UE is legacy Rel-16/Rel-17 UE which supports only single carrier

- Case 2: RX UE is Rel-18 UE, which always covers all possible SL carriers in Rel-18

Proposal 2: To resolve the backward compatibility issue (i.e. TX UE with broadcast/groupcast SL CA needs to determine whether the RX UE is legacy Rel-16/Rel-17 UE which only supports single carrier), introduce a new list for TX profiles corresponding to different L2 Addresses or services.

Proposal 3: RAN2 is suggested to postpone the discussion on CA configuration for established PC5 UC link after SA2 reply LS is received.

Proposal 4: SL CA is not applied to initial PC5-S signaling. FFS how TX UE and RX UE align with one carrier for initial PC5-S signaling.

Proposal 5: Introduce an unified carrier (re)selection procedure for NR SL UC/BC/GC CA, i.e. the similar mechanism of LTE SL CA is reused for NR SL UC CA.

Proposal 6: For the calculation of per-carrier CBR, use an average over the CBRs of all resource pools in the each carrier for all cast-type (UC/BC/GC).

Proposal 7: For NR SL PDCP duplication, introduce an unified solution for UC/BC/GC if no blocking issue is identified.

Proposal 8: For PDCP duplication of NR SL CA of UC, reuse the hard-coded LCID to identify duplicated SL LCHs .

Proposal 9: Introduce a reliability threshold for sl-PacketErrorRate-r16 in PQI as the criteria to determine whether packet duplication is performed.

# 4 References

[1] 3GPP RP-230077, WID revision: NR sidelink evolution, OPPO

[2] TS 24.587-v17.5.0, Vehicle-to-Everything (V2X) services in 5G System (5GS), 2022-3.

[3] TS 36.300-v17.1.0, Overall description; Stage 2, 2022-6.

[4] RAN2#121b-e, Chair Notes.

[5] R2-2304236, LS on carrier mapping for unicast SL CA, Apple.