3GPP TSG-RAN WG2 Meeting #122	R2-2305101

Incheon, South Korea, 22-26 May 2023

Agenda item:	7.4.2.1

Source:	Apple

Title:	LTM cell switch link failure handling

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion / Decision

# Introduction

LTM is intra-CU and this aspect alone removes a lot of recovery procedures that are generally involved when the UE connects to the NW for connection re-establishment.

In RAN2-121bis-e, we made an agreement on trying to optimize the link failure handling, possibly by having the UE try other candidate LTM cells. We think that this might not always be possible, as the gNB-DUs might not be ready yet for the “reception” of this UE.

LTM supervisor timer is RRC layer timer. At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution. While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB.

So in this paper, we intend to bring the up the UE procedures that can skip re-establishment and in cases where this is not possible, revise the legacy handover (LTM cell switch) and link failure handling for LTM .

# Rel-16 CHO like fast recovery

The agreement from RAN2-121-bis-e on attempting to perform fast recovery, requires that the gNB-DUs be ready for receiving the UE as part of LTM switch. But UE can be configured with multiple candidate LTM cells, and all of them might not be assumed to be used as target cells, as the NW might be depending on L1 meas report, before the necessary inter-node co-ordination is in place that is needed before initiating LTM switch command.

Observation 1: Before LTM switch, the target gNB-DU should be informed (admission control) about the UE. So just configuring a set of candidate LTM cells, do not necessarily mean that they are ready for the UE to them as the candidate recovery cells in case the LTM switch fails. For eg., the RACH resources might not be allocated to the UE, even when they were configured in the RRC LTM configuration.

It is better for the UE to preform RACH, but provide an identification to have the UE identified (similar to C-RNTI for eg). Based on the above, we propose the below:

Proposal 1: In the case of an LTM switch failure, the UE can recover (perform LTM switch) to other candidate LTM cells, if the NW explicitly configures for such action, and this configuration is per LTM cell.

Proposal 2: In case there are no LTM cells on which the UE can perform LTM switch, the UE does not perform LTM switch, even when the potential cell it found as part of cell selection, is the LTM candidate cell.

# RRC connection re-establishment in case LTM cell switch is not possible

We now discuss the UE actions in case the cell the UE selected is an LTM cell, but the NW has not configured the UE to perform LTM switch on this cell.

The legacy procedure in LTE and NR in re-establisment of the RRC connection following a link or handover failure entails the below key steps:

- UE releases the dedicated config (reverts back to source cell common config in case of handover failure)

- UE searches for a suitable cell to camp on

- When such cell is found, initiates a re-establishment procedure with the following

  - Create a RRC re-estabishment message

    - Add the UE identity for the NW to retrieve the UE context

    - Add the security (integrity) using previous security context

    - Add necessary cause for the failure

  - Re-establish PDCP for SRB1

  - Re-establish RLC

  - Send the RRC message

- Once the restablishment related reply is received (assuming that the UE context if fond at the NW)

  - Check the security validity

  - Apply the config (likely restore configuration)

  - Re-estabish the L2/L3 datapath.

## LTM and same CU

Since the CU does not change with LTM, the PDCP state (including the associated security context) does not need to change as the UE tries to re-establish a failure LTM link.

In other words, even when the lower-layers have a failure (due to a link failure at the DU), the PDCP context and operation does not need to be reset and started: as long as the UE tries to re-connect to the same CU (via any other cell/DU as long as that DU is linked to the same CU)

Observation 2: RRC connection re-establishment can be simplified in case the UE tries to connect to the same CU.

Observation 3: Current re-establishment procedure at the UE in NR does not take any assumptions that the UE can potentially connect to the same CU. In other words, the procedure makes no assumption and is defined with the intention that the NW is provided with enough UE information to try and get the UE context, and while at the UE, the L2/L3 is reset/released and is assumed to be restored later on based on the NW response to the re-establishment message.

Observation 4: LTM inherently operates with the design that all the LTM candidate cells operate with the same CU.

On the other hand, it is intended that the UE can be configured with LTM and legacy L3 mobility configuration at the same time, as the deployment and UE location could be such that the UE might be at the border of a LTM NW with a non-LTM CU as the neighbor.

Observation 5: The UE can be configured with legacy L3 mobility at the same time as the LTM config. However, the UE is aware of the candidate LTM cells and can know if the cell it finds after a link failure, belongs to the same CU that the UE was in connection with.

## Faster recovery with same CU

In case the UE is aware that the target cell it is trying to send the re-establishment is driven by the same CU, several of the link recovery steps the UE has to take, can be removed or reduced. We list below highlighting the changes/reductions in such a case.

- UE releases the dedicated config (reverts back to source cell common config in case of handover failure) -> UE can keep the PDCP context for SRBs and DRBs

- UE searches for a suitable cell to camp on

- When such cell is found (and it is the same CU), initiates a re-establishment procedure with the following

  - Create a RRC re-estabishment message

    - Add the UE identity for the NW to retrieve the UE context

    - Add the security (integrity) using previous security context -> Continue with the existing security context

    - Add necessary cause for the failure

  - Re-establish PDCP for SRB1

  - Re-establish RLC -> do not need to re-establish in case of intra-DU

  - Send the RRC message

- Once the restablishment related reply is received (assuming that the UE context if fond at the NW)

  - Check the security validity

  - Apply the config (likely restore configuration)

  - Re-estabish the L2/L3 datapath. -> recover PDCP as needed

As can be seen above, the recovery process can be more efficient and the latency can be further reduced, as some of the unnecessary steps can be removed/modified with the view that the CU (PDCP context) has not changed.

First-up, even when the mobility handling is done at L2 and below, gNB-CU is involved in this, and while the measurement configuration and handling is still to be concluded, we assume that CU would be driving this, as well as the handover/cell switch related triggers/functions. So any failure in such actions still require that the gNB-CU is aware of, and be given a chance to re-eastablish/configure. Just leaving this L2 alone is not foolproof.

Observation 6: Even though the mobility is being driven out of L2/L1, gNB-CU is still involved in the mobility aspects (for eg meas and other mobility configuration etc) and so gNB-CU needs to be involved in link recovery. L2 based link recovery is localized and is not comprehensive, potentially needing several additional message exchanges. It is better to be controlled centrally in the CU

Based on the above, we propose the below:

Proposal 3: LTM link failure recovery is done using RRC and the recovery messages are exchanged between the UE and the gNB-CU.

Proposal 4: RAN2 to modify the RRC re-establishment with LTM procedure to make use of the fact that the CU does not change in case the recovery is using a candidate LTM cell.

## Principles of LTM link recovery

While we acknowledge that many of the LTM aspects including cell switch are under discussion, we think some basic agreements on the LTM link recovery principles can help RAN2 progress further.

We propose the below to that effect:

Proposal 5: For LTM supporting UE ,if there is a link failure in LTM, the UE does not atleast release the PDCP context for SRB and DRB and applies this if the UE finds a suitable cell that is part of the LTM candidate configuration. If the suitable cell found is not part of the LTM candidate configuration, the UE falls back to the legacy procedure.

Proposal 6: If the UE finds a suitable cell that is part of the LTM candidate configuration as part of link failure recovery, the UE directly sends the re-establishment message using the current PDCP context for SRB1.

We anticipate that their might be concern in re-using the same RRC restablishment message for the new LTM re-establishment. While do not see the need to see this as a new message, we are ok with a discussion and on creating a new RRC message if needed. But the key element here is that the NW needs to know that this is a re-establishment based on LTM.

In case the existing RRC message is used, atleast a new cause is needed to that effect.

Proposal 7: RAN2 to discuss if a new RRC message type is to be created for the re-establishment request which is sent on SRB1 or a new re-establishment cause can be added to the existing message type.

Since the UE context maintenance at the NW, as well as the datapath related timers can be different related to the legacy link recovery procedure, we think it would be better to have a configuration that informs how long the UE should keep the context before falling back to the legacy link recovery procedure.

Proposal 8: UE falls back to the legacy link recovery procedure based on the expiry of a new timer. Before this timer expires, the UE follows the LTM link recovery. RAN2 to discuss if this timer is static (specified in the spec) or configurable.

# Conclusions

Observation 1: Before LTM switch, the target gNB-DU should be informed (admission control) about the UE. So just configuring a set of candidate LTM cells, do not necessarily mean that they are ready for the UE to them as the candidate recovery cells in case the LTM switch fails. For eg., the RACH resources might not be allocated to the UE, even when they were configured in the RRC LTM configuration.

Observation 2: RRC connection re-establishment can be simplified in case the UE tries to connect to the same CU.

Observation 3: Current re-establishment procedure at the UE in NR does not take any assumptions that the UE can potentially connect to the same CU. In other words, the procedure makes no assumption and is defined with the intention that the NW is provided with enough UE information to try and get the UE context, and while at the UE, the L2/L3 is reset/released and is assumed to be restored later on based on the NW response to the re-establishment message.

Observation 4: LTM inherently operates with the design that all the LTM candidate cells operate with the same CU.

Observation 5: The UE can be configured with legacy L3 mobility at the same time as the LTM config. However, the UE is aware of the candidate LTM cells and can know if the cell it finds after a link failure, belongs to the same CU that the UE was in connection with.

Observation 6: Even though the mobility is being driven out of L2/L1, gNB-CU is still involved in the mobility aspects (for eg meas and other mobility configuration etc) and so gNB-CU needs to be involved in link recovery. L2 based link recovery is localized and is not comprehensive, potentially needing several additional message exchanges. It is better to be controlled centrally in the CU

Proposal 1: In the case of an LTM switch failure, the UE can recover (perform LTM switch) to other candidate LTM cells, if the NW explicitly configures for such action, and this configuration is per LTM cell.

Proposal 2: In case there are no LTM cells on which the UE can perform LTM switch, the UE does not perform LTM switch, even when the potential cell it found as part of cell selection, is the LTM candidate cell.

Proposal 3: LTM link failure recovery (in case there are no NW configured LTM cell on which the UE is allowed to perform LTM switch) is done using RRC and the recovery messages are exchanged between the UE and the gNB-CU.

Proposal 4: RAN2 to modify the RRC re-establishment with LTM procedure to make use of the fact that the CU does not change in case the recovery is using a candidate LTM cell.

Proposal 5: For LTM supporting UE ,if there is a link failure in LTM, the UE does not atleast release the PDCP context for SRB and DRB and applies this if the UE finds a suitable cell that is part of the LTM candidate configuration. If the suitable cell found is not part of the LTM candidate configuration, the UE falls back to the legacy procedure.

Proposal 6: If the UE finds a suitable cell that is part of the LTM candidate configuration as part of link failure recovery, the UE directly sends the re-establishment message using the current PDCP context for SRB1.

Proposal 7: RAN2 to discuss if a new RRC message type is to be created for the re-establishment request which is sent on SRB1 or a new re-establishment cause can be added to the existing message type.

Proposal 8: UE falls back to the legacy link recovery procedure based on the expiry of a new timer. Before this timer expires, the UE follows the LTM link recovery. RAN2 to discuss if this timer is static (specified in the spec) or configurable.

# References