3GPP TSG-RAN WG2 Meeting #122	R2-2305108

Incheon, South Korea, 22-26 May 2023

Agenda item:	6.1.3.2

Source:	Apple

Title:	Way forward on agg BW reporting per BC

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion / Decision

# Introduction

In RAN2-121-bis-e, RAN2 did another round of discussion on the topic of aggregated BW reporting per BC, with no clear consensus and RAN2 chair advised another round of offline with the below chair notes. In the ensuing offline discussion between RAN2-121-bis-e and RAN2-122, the companies who were opposed to the introduction of this, were main concerned on the backward compatibility and the effectiveness of this solution. But the discussion primarily revolved around FR1. In this paper, we try to propose a way forward to see if RAN2 can honour RAN4 request, where we must say, the request was done multiple times from RAN4 to RAN2.

Postpone (topic 2/3), - Allow/recommend companies to discuss offline until RAN2#122: - UE capability signalling overhead for existing UE implementations. - Additional UE capability signalling overhead caused by FBG5 and BCS4/5. - Potential signalling overhead reduction gain in light of above. - Any other relevant aspect. FFS whether there is any legacy network implementation for FBG5 or BCS4/5 that must be taken into account in further discussion.

# Areas of concern

The main areas of concern by the opposing companies were:

- The non-backward compatibility

- The effective ness of this solution

In the companion paper [7] (where we co-sourced) these were discussed/assessed from the proponent perspective for FR1. In this paper, we would like to bring to companies attention on the impact from FR2 perspective, esp for BW-class R*, where the above issues need to be looked at.

Observation 1: Opposing companies are concerned about NBC and the effective of signalling agg BW per BC in FR1.

## From FR2 perspective

Let’s look at both the issues from FR2 perspective.

### NBC issue

We start by pointing out that there are not yet any implementations that we are aware of that have implement the newly introduced R* BW classes for FR2, esp when the aggregated BW of the CA BCs is north of 1GHz!

So from this perspective, we do not actually see an NBC issue.

Observation 2: For FR2 R* BW classes which were introduced recently (as part of the R17 RAN4 WI), there are no commercial in the field implementations yet that we are aware of. There should not be any NBC issue.

### Effectiveness in FR2

Regarding the effectiveness, we really would like to point out to the opposing companies that RAN4 has and LS to RAN2 on requesting (and even suggesting!) a way of signalling the agg BW per BC. If there is an LS (there are multiple of these from RAN4), then companies in RAN4 have reached a consensus and agreed that it is indeed useful to have such signalling, and that there are UE implementation where the total BW might be smaller, but the associated carriers as reported from the R* BW-class.

Observation 3: RAN4 (through multiple LS requests) have had consensus on the useful to have such signalling, and that there are UE implementation where the total BW might be smaller, but the associated carriers as reported from the R* BW-class. Further explanation on this can be found in [8][9].

In addition, RAN4 is planning on introducing even more R* BW classes, where each carrier is expected to support either 100 or 200 MHz, and the fact that the BW can assume more than 1 possible value, it increases the necessity to have the UE report multiple permutations as explained in [8][9]!

Pulling in this signalling early can help with avoid the entire NBC discussions, as we anticipate (per vision from RAN4) that there would be an explosion of FSPerCC permutations if not now, in the newer BCs with newer BW-classes. It is imperative that RAN2 acknowledge the effort RAN4 has put in (which triggered the LSes) and try to avoid issues in the future.

Observation 4: RAN4 is planning on introducing even more R* BW classes, where each carrier is expected to support either 100 or 200 MHz, and the fact that the BW can assume more than 1 possible value, it increases the necessity to have the UE report multiple permutations as explained in [8][9]! RAN2 should acknowledge the effort RAN4 has put in (which triggered the LSes) and try to avoid issues with the entire NBC discussions that are bound to prop up in the future in FR2, with the way the R* BW classes are defined.

Due to the lack of consensus in RAN2 over repeated discussions we’ve had, we would like to atleast propose the way forward of handling this for FR2 FBG5 BW-classes and can potentially limit this to R* BW classes. So we propose the below:

Proposal 1: If no agreement can be reached for agg BW per BC reporting for FR1, RAN2 to agree on supporting this signalling for FR2 BCs where the BC has BW classes with R2-R12 and any future BW class additions to FBG5.

# Conclusions

Observation 1: Opposing companies are concerned about NBC and the effective of signalling agg BW per BC in FR1.

Observation 2: For FR2 R* BW classes which were introduced recently (as part of the R17 RAN4 WI), there are no commercial in the field implementations yet that we are aware of. There should not be any NBC issue.

Observation 3: RAN4 (through multiple LS requests) have had consensus on the useful to have such signalling, and that there are UE implementation where the total BW might be smaller, but the associated carriers as reported from the R* BW-class. Further explanation on this can be found in [8][9].

Observation 4: RAN4 is planning on introducing even more R* BW classes, where each carrier is expected to support either 100 or 200 MHz, and the fact that the BW can assume more than 1 possible value, it increases the necessity to have the UE report multiple permutations as explained in [8][9]! RAN2 should acknowledge the effort RAN4 has put in (which triggered the LSes) and try to avoid issues with the entire NBC discussions that are bound to prop up in the future in FR2, with the way the R* BW classes are defined.

Proposal 1: If no agreement can be reached for agg BW per BC reporting for FR1, RAN2 to agree on supporting this signalling for FR2 BCs where the BC has BW classes with R2-R12 and any future BW class additions to FBG5.

# References

[1] R2-2303398  On servicing RAN4 request on aggregate BW signaling for FBG5 CA BW classes

[2] R2-xxxx R2_121bis-e Chair Notes EOM

[3] R2-2209347 - LS on new contiguous BW classes for legacy networks

[4] R2-2212966 - Summary of RAN2 contributions on FBG5 BW Classes

[5] R2-2213312 - Reply LS on new contiguous BW classes for legacy networks

[6] R2-2302440 - LS on signaling for FR2 FBG5 CA BW classes

[7] R2-230xxxx Maximum aggregated bandwidth for FR1 inter-band CA

[8] R4-2212355  Signaling enhancement for FR2 new CA BW classes

[9] R4-2207680 Further views on new FR2 CA BW classes